IRIT Version 9.0 User's Manual

--------------------------------

A Solid Modeling Program

--------------------------

Copyright (C) 1989-2004 Gershon Elber

EMail: gershon@cs.technion.ac.il

Join IRIT mailing list: gershon@cs.technion.ac.il

Mailing list: irit-mail@cs.technion.ac.il

Bug reports: irit-bugs@cs.technion.ac.il

WWW Page: http://www.cs.technion.ac.il/~irit

Introduction

------------

IRIT is a solid modeler developed for educational purposes. Although

small, it is now powerful enough to create quite complex scenes.

IRIT started as a polygonal solid modeler and was originally developed

on an IBM PC under MSDOS. Version 2.0 was also ported to X11 and version 3.0

to SGI 4D systems. Version 3.0 also includes quite a few free form curves

and surfaces tools. See the UPDATE.NEW file for more detailed update

information. In Version 4.0, the display devices were enhanced, freeform

curves and surfaces are more extensively supported, functions can be defined,

and numerous improvement and optimizations are added.

COPYRIGHTS

----------

BECAUSE IRIT AND ITS SUPPORTING TOOLS AS DOCUMENTED IN THIS DOCUMENT

ARE LICENSED FREE OF CHARGE, I PROVIDE ABSOLUTELY NO WARRANTY, TO THE EXTENT

PERMITTED BY APPLICABLE STATE LAW. EXCEPT WHEN OTHERWISE STATED IN WRITING, I

GERSHON ELBER PROVIDE THE IRIT PROGRAM AND ITS SUPPORTING TOOLS "AS IS"

WITHOUT WARRANTY OF ANY KIND, EITHER EXPRESSED OR IMPLIED, INCLUDING, BUT NOT

LIMITED TO, THE IMPLIED WARRANTIES OF MERCHANTABILITY AND FITNESS FOR A

PARTICULAR PURPOSE.

THE ENTIRE RISK AS TO THE QUALITY AND PERFORMANCE OF THESE PROGRAMS IS WITH

YOU. SHOULD THE IRIT PROGRAMS PROVE DEFECTIVE, YOU ASSUME THE COST OF

ALL NECESSARY SERVICING, REPAIR OR CORRECTION.

IN NO EVENT UNLESS REQUIRED BY APPLICABLE LAW WILL GERSHON ELBER,

BE LIABLE TO YOU FOR DAMAGES, INCLUDING ANY LOST PROFITS, LOST MONIES,

OR OTHER SPECIAL, INCIDENTAL OR CONSEQUENTIAL DAMAGES ARISING OUT OF THE

USE OR INABILITY TO USE (INCLUDING BUT NOT LIMITED TO LOSS OF DATA OR A

FAILURE OF THE PROGRAMS TO OPERATE WITH PROGRAMS NOT DISTRIBUTED BY GERSHON

ELBER) THE PROGRAMS, EVEN IF YOU HAVE BEEN ADVISED OF THE POSSIBILITY OF

SUCH DAMAGES, OR FOR ANY CLAIM BY ANY OTHER PARTY.

IRIT is a freeware solid modeler. It is not public domain since I

hold copyrights on it. However, unless you are to sell or attempt to make

money from any part of this code and/or any model you made with this solid

modeler, you are free to make anything you want with it.

IRIT can be compiled and executed on numerous Unix systems as

well as OS2, Windows 95/98/NT/2000/XP and AmigaDOS. However, beware the

MSDOS support is fading away.  Also, under Windows, IRIT must be installed

at a directory/path with no spaces.

You are not obligated to me or to anyone else in any way by using IRIT.

You are encouraged to share any model you made with it, but the models

you made with it are yours, and you have no obligation to share them.

You can use this program and/or any model created with it for non

commercial and non profit purposes only. An acknowledgement on the way the

models were created would be nice but is not required.

Setup

-----

The IRIT program reads a file called irit.cfg each time it is executed.

This file configures the system. It is a regular text file with comments, so

you can edit it and properly modify it for your environment.

This file is being searched for in the directory specified by the

IRIT_PATH environment variable.

For example 'setenv IRIT_PATH /u/gershon/irit/bin/'.

Note IRIT_PATH must terminate with '/'. If the variable is not set only

the current directory is being searched for irit.cfg.

In addition, if it exists, a file by the name of iritinit.irt will be

automatically executed before any other '.irt' file. This file may contain

any IRIT command. It is the proper place to put your predefined

functions and procedures, if you have any.

This file will be searched much the same way irit.cfg is. The

name of this initialization file may be changed by setting the StartFile

entry in the configuration file.

This file is far more important starting at version 4.0, because of the new

function and procedure definition that has been added, and which is used

to emulate BEEP, VIEW, and INTERACT, for example.

The solid modeler can be executed in text mode (see the .cfg and the -t

flag below) on virtually any system with a C compiler.

Under all systems the following environment variables must be set

and updated:

path           Add to path the directory where IRIT's binaries are.

IRIT_PATH      Directory with config., help and IRIT's binary files.

IRIT_DISPLAY   The graphics driver program/options. Must be in path.

IRIT_BIN_IPC   If set, uses binary Inter Process Communication.

For example,

set path = (path /u/gershon/irit/bin)

setenv IRIT_PATH  /u/gershon/irit/bin/

setenv IRIT_DISPLAY "xgldrvs -s-"

setenv IRIT_BIN_IPC 1

to set /u/gershon/irit/bin as the binary directory and to use the sgi's

gl driver. If IRIT_DISPLAY is not set, the server (i.e., the IRIT

program) will prompt and wait for you to run a client (i.e., a display

driver). if IRIT_PATH is not set, none of the configuration files, nor

the help file will be found.

If IRIT_BIN_IPC is not set, text based IPC is used, which is far

slower.  There is no real reason not to use IRIT_BIN_IPC, unless it does

not work for you, for some reason.

In addition, the following optional environment variables may be set.

IRIT_MALLOC       If set, apply dynamic memory consistency testing.

Programs will execute much slower in this mode.

IRIT_MALLOC_PTR   Set to a pointer address and the program will scream

once this pointer is allocated, if IRIT_MALLOC is set.

IRIT_NO_SIGNALS   If set, no signals are caught by IRIT.

IRIT_SERVER_HOST  Internet Name of IRIT server (used by graphics driver).

IRIT_SERVER_PORT  Used internally to the TCP socket number. Should not

be set by users.

IRIT_TIME_OUT     Integer (seconds) for timing out when trying

to execute a display device from IRIT.  Default is 10 seconds.

IRIT_INCLUDE      A semicolon separated list of directories, in which to look

for the irt files to include.  See INCLUDE command.

LD_LIBRARY_PATH   If shared libraries are created, this variable must be

updated to point to the shared libraries' directory.

For example,

setenv IRIT_MALLOC 1

setenv IRIT_MALLOC_PTR 1234567890

setenv IRIT_NO_SIGNALS 1

setenv IRIT_SERVER_HOST irit.cs.technion.ac.il

setenv IRIT_INCLUDE "/d2/gershon/irit/irit/scripts;/tmp"

IRIT_MALLOC is useful for programmers, or when reporting a memory

fatal error occurrence.  This variable, when set as a non zero value,

will activate the following (hexadecimal bit settings with any combination

of the following):

0x01       A test for overwriting before the dynamic memory

is allocated or immediately after it. Cheap in time.

0x02       Savings of all allocated objects in a table for the

detection of freeing unallocated objects and consistency

of the entire dynamic memory. Time expensive.

0x04       Zeros every freed object, once it is freed.

IRIT_NO_SIGNALS is also useful for debugging when contorl-C is used

within a debugger. The IRIT_SERVER_HOST/PORT controls the server/client

(IRIT/Display device) communication.

IRIT_SERVER_HOST and IRIT_SERVER_PORT are used in the unix and

Window NT ports of IRIT.

See the section on graphics drivers for more details.

A session can be logged into a file as set via LogFile in the configuration

file. See also the LOGFILE command.

The following command line options are available:

IRIT [-t] [-g] [-q] [-i] [-z] {[-m ...]} [file.irt]

-t          Puts IRIT into text mode. No graphics will be displayed and

the display commands will be ignored. This is useful when

one needs to execute an IRIT file to create data on a tty

device...

-g          IRIT under GUI mode.  Should not be used by end users.

-q          Quiet mode with no regular output to stdout.

-i          IRIT under Interactive mode.  Should not be used by end users.

-z          Prints usage message and current configuration/version

information.

-m          Optional option...  If IRIT is compiled for debugging, allows

setting three addition parameters of trap pointer, search

pointer, and abort counter.

file.irt    A file to invoke directly instead of waiting to input from

stdin.

Under OS2 the IRIT_DISPLAY environment variable must be set (if set) to

os2drvs.exe without any option (-s- will be passed automatically).

os2drvs.exe must be in a directory that is in the PATH environment

variable. IRIT_BIN_IPC can be used to signal binary IPC which is faster.

Here is a complete example:

set IRIT_PATH=c:\irit\bin\

set IRIT_DISPLAY=os2drvs -s-

set IRIT_BIN_IPC=1

assuming the directory specified by IRIT_PATH holds the executables of

IRIT and is in PATH.

If IRIT_BIN_IPC is not set, text based IPC is used which is far

slower.  There is no real reason not to use IRIT_BIN_IPC unless it does

not work for you, for some reason.

The OS2 executables are typically built using the EMX port of gnu C

compiler.  The distribution of the executables does not include the EMX run

time library and any attempt to run IRIT will fail. You will get an error

message such as "File EMX does not exist".  You can get the run time from

ftp to ftp-os2.nmsu.edu (aliased also as hobbes.NMSU.Edu)

cd to os2/unix/emx09c   (or a newer version number/level)

get emxrt.zip and place its dlls in a place they would be found.

The NT port uses sockets and is, in this respect, similar to the Unix port.

The envirnoment variables IRIT_DISPLAY, IRIT_SERVER_HOST, and

IRIT_BIN_IPC should all be set in a similar way to the Unix specific

setup.

As a direct result, the server (IRIT) and the display device can run

on different hosts.  For example, the server might be running on an NT

system while the display device will be running on an SGI4D, exploiting

the graphic's hardware capabilities.  Here is a complete example:

set IRIT_PATH=c:\irit\bin\

set IRIT_DISPLAY=wntgdrvs -s-

set IRIT_BIN_IPC=1

Also, under Windows, IRIT must be installed in a directory/path with

no spaces.

Under UNIX using X11 (x11drvs driver), add the following options to

your .Xdefaults. Most are self explanatory. The Trans attributes control

the transformation window, while the View attributes control the view window.

SubWin attributes control the subwindows within the transformation window.

#if COLOR

irit*Trans*BackGround:            NavyBlue

irit*Trans*BorderColor:           Red

irit*Trans*BorderWidth:           3

irit*Trans*TextColor:             Yellow

irit*Trans*SubWin*BackGround:     DarkGreen

irit*Trans*SubWin*BorderColor:    Magenta

irit*Trans*Geometry:              =150x500+500+0

irit*Trans*CursorColor:           Green

irit*View*BackGround:             NavyBlue

irit*View*BorderColor:            Red

irit*View*BorderWidth:            3

irit*View*Geometry:               =500x500+0+0

irit*View*CursorColor:            Red

irit*MaxColors:                   15

#else

irit*Trans*Geometry:              =150x500+500+0

irit*Trans*BackGround:            Black

irit*View*Geometry:               =500x500+0+0

irit*View*BackGround:             Black

irit*MaxColors:                   1

#endif

First Usage

-----------

Commands to IRIT are entered using a textual interface, usually

from the same window from which the program was executed.

Some important commands to begin with are:

1. include("file.irt"); - will execute the commands in file.irt. Note

include can be recursive up to 10 levels. To execute the demo

(demo.irt) simply type 'include("demo.irt");'. Another way to run

the demo is by typing demo(); which is a predefined procedure defined

in iritinit.irt.

2. help(""); - will print all available commands and how to get help on

them. A file called irit.hlp will be searched as irit.cfg is

being searched (see above), to provide the help.

3. exit(); - close everything and exit IRIT.

Most operators are overloaded. This means that you can multiply

two scalars (numbers), or two vectors, or even two matrices, with the same

multiplication operator (*). To get the on-line help on the

operator '*', type 'help("*");'

The best way to learn this program (as any other program...) is by

trying it. Print the manual and study each of the commands available.

Study the demo programs (*.irt) provided, as well.

The "best" mode in which to use IRIT is via the emacs editor. With this

distribution an emacs mode for IRIT files (irt postfix) is provided

(irit.el). Make your .emacs load this file automatically. Loading file.irt

will switch emacs into an IRIT mode that supports the following keystrokes:

Meta-E             Executes the current line

Meta-R             Executes the current Region (Between Cursor and Mark)

Meta-S             Executes a single line from input buffer

Meta-H             Prints IRIT help on the current WORD the point is on using 'help("WORD");'

The first time one of the above keystrokes is hit, emacs will fork an

IRIT process so that IRIT'S stdin is controlled via the above commands.

This emacs mode was tested under various Unix environments, under OS2

2.x/3.x, and under Windows 95/98/NT/2000/XP.

Line Editing

------------

The IRIT interpreter provides full line editing capabilities.  The

following are the available control options:

^a                     Beginning of line

^e                     End of line

^f                     Forward one character

^b                     Backward one character

^d                     Delete current character

^h (Backspace)         Delete backward one character

^i (Tab)               Toggles overwrite/insert mode

^k                     Kill to end of line

^p                     Get previous history line

^n                     Get next history line

^j (LineFeed)          Done with this line

Only lines entered from stdin will enter the history queue.  The above

control capabilities are fully configurable via the irit.cfg

configuration file.

Data Types

----------

These are the Data Types recognized by the solid modeler. They are also

used to define the calling sequences of the different functions below:

ConstantType       Scalar real type that cannot be modified.

NumericType        Scalar real type.

VectorType         3D real type vector.

PointType          3D real type point.

CtlPtType          Control point of a freeform curve or surface.

MatrixType         4 by 4 matrix (homogeneous transformation matrix).

PolygonType        Object consists of polygons.

PolylineType       Object consists of polylines.

CurveType          Object consists of curves.

SurfaceType        Object consists of surfaces.

TrimSrfType        Object consists of trimmed surfaces.

TriSrfType         Object consists of triangular surfaces

TrivarType         Object consists of trivariate functions.

MultivarType       Object consists of multivariate functions.

FreeformType       One of CurveType, SurfaceType, TrimSrfType,

TrivarType, MultivarType, TriSrfType.

GeometricType      One of Polygon/lineType, FreeformType.

InstanceType       Object with a GeometryType and a Transformation.

GeometricTreeType  A list of GeometricTypes or GeometricTreeTypes.

StringType         Sequence of chars within double quotes - "A string".

Current implementation is limited to 80 chars.

AnyType            Any of the above.

ListType           List of (any of the above type) objects. List

size is dynamically increased, as needed.

Although points and vectors are not the same, IRIT does not

distinguish between them, most of the time. In this future this might change.

Commands

--------

These are all the commands and operators supported by the IRIT Solid

Modeler:

+          BZR2BSP    CRVLNDST   IRITSTATE  POWER      SMESH

-          CARCLEN    CRVPTDST   ISGEOM     PPINTER    SMOEBIUS

*          CAREA      CRVPTTAN   ISOCLINE   PPROPFTCH  SMOMENTS

/          CBEZIER    CSURFACE   KNOTCLEAN  PRINTER    SMOOTHNRML

^          CBIARC     CTANGENT   KNOTREMOVE PRINTF     SMORPH

=          CBISECTOR  CTLPT      LINTERP    PRINTFILE  SNOC

==         CBSPLINE   CTRIMSRF   LIST       PRISA      SNORMAL

!=         CCINTER    CYLIN      LN         PROCEDURE  SNRMLSRF

<          CCRVTR     CZEROS     LOAD       PROJMAT    SPARABOLC

>          CDERIVE    DIST2FF    LOFFSET    PT3BARY    SPHERE

<=         CDIVIDE    DSTPTLN    LOG        PTHMSPR    SPLITLST

>=         CEDITPT    DSTPTPLN   LOGFILE    PTLNPLN    SQRT

ABS        CENVOFF    DSTLNLN    MATDECOMP  PTPTLN     SRAISE

ACOS       CEVAL      DUALITY    MATPOSDIR  PTREGISTER SRAYCLIP

ADAPISO    CEXTREMES  ERROR      MAXEDGELEN PTS2PLLN   SREFINE

ADWIDTH    CHDIR      EVOLUTE    MBEZIER    PTSLNLN    SREGION

ALGSUM     CINFLECT   EXEC       MBISECTOR  QUADRIC    SREPARAM

ANALYFIT   CINTERP    EXIT       MBSPLINE   RANDOM     SREVERSE

ANIMEVAL   CINTEG     EXP        MDERIVE    RAYTRAP    SRF2TANS

AOFFSET    CIRCLE     EXTRUDE    MDIVIDE    RFLCTLN    SRF3TANS

ARC        CIRCPOLY   FFCOMPAT   MERGEPOLY  RFLCTMAT   SRFFFORM

ARC360     CLNTCLOSE  FFCTLPTS   MESHSIZE   RESET      SRFKERNEL

AREA       CLNTCRSR   FFEXTREME  MEVAL      RMATTR     SRINTER

ASIN       CLNTEXEC   FFKNTVEC   MFROMMESH  ROTVEC     SSINTER

ATAN       CLNTREAD   FFMATCH    MFROMMV    ROTX       STANGENT

ATAN2      CLNTWRITE  FFMERGE    MMERGE     ROTY       STRIMSRF

ATTRIB     CMESH      FFMESH     MOFFSET    ROTZ       STRIVAR

ATTRPROP   CMOEBIUS   FFMSIZE    MOMENT     ROTZ2V     SURFPREV

AWIDTH     CMORPH     FFORDER    MPOWER     ROTZ2V2    SURFREV

BBOX       CMULTIRES  FFPOLES    MPROMOTE   RRINTER    SURFREVAXS

BLHERMITE  CNORMAL    FFPTDIST   MRAISE     RULEDSRF   SURFREV2

BLSHERMITE CNRMLCRV   FFPTTYPE   MRCHCUBE   RULEDTV    SURFREVAX2

BLOSSOM    CNVXHULL   FFSPLIT    MREFINE    SACCESS    SVISIBLE

BOOLONE    COERCE     FITPMODEL  MREGION    SASPCTGRPH SVOLUME

BOOLSUM    COLOR      FIXPLGEOM  MREPARAM   SASYMPEVAL SWEEPSRF

BOUNDARY   COMMENT    FIXPLNRML  MREVERSE   SAVE       SWPSCLSRF

BOX        COMPOSE    FLOOR      MSLEEP     SBEZIER    SWUNGASUM

BSCTCONCN2 CON2       FMOD       MVEXPLICIT SBISECTOR  SYMBCPROD

BSCTCONCON CONE       FNFREE     MZERO      SBSPLINE   SYMBDIFF

BSCTCONCYL CONICSEC   FOR        NIL        SCALE      SYMBDPROD

BSCTCONLN  CONTOUR    FREE       NREF       SCRVTR     SYMBIPROD

BSCTCONPL  CONVEX     FUNCTION   NRMLCONE   SCRVTREVAL SYMBPROD

BSCTCONPT  COORD      GBOX       NTH        SDERIVE    SYMBSUM

BSCTCONSPR COS        GETATTR    OFFSET     SDIVIDE    SYSTEM

BSCTCYLCYL COVERISO   GETLINE    ORTHOTOMC  SEDITPT    TAN

BSCTCYLPL  COVERPT    GETNAME    PATTRIB    SETNAME    TBEZIER

BSCTCYLPT  CPATTR     GGINTER    PAUSE      SEVAL      TBSPLINE

BSCTCYLSPR CPOLY      GPOLYGON   PCIRCLE    SFLECNODAL TCRVTR

BSCTPLNLN  CPOWER     GPOLYLINE  PCRVTR     SFOCAL     TDERIVE

BSCTPLNPT  CRAISE     HELP       PDECIMATE  SFROMCRVS  TDIVIDE

BSCTSPRLN  CRC2CRVTAN HERMITE    PDOMAIN    SGAUSS     TEDITPT

BSCTSPRPL  CREDUCE    HOMOMAT    PLANE      SILHOUETTE TEVAL

BSCTSPRPT  CREFINE    IF         PLANECLIP  SIN        TEXTGEOM

BSCTSPRSPR CREGION    INCLUDE    PLN3PTS    SINTERP    TEXTWARP

BSCTTRSPT  CREPARAM   INSERTPOLY PMORPH     SIZEOF     TFROMSRFS

BSCTTRSSPR CROSSEC    INSTANCE   PNORMAL    SKEL2DINT  TIME

BSP2BZR    CRV2TANS   INTERACT   POINT      SMEAN      TINTERP

BUMPMAP    CRVKERNEL  IQUERY     POLY       SMERGE     THISOBJ

TMORPH     TRANS      TRIMSRF    TSEVAL     UVPOLY     VIEWSET

TNSCRCR    TREFINE    TRMSRFS    TSGREGORY  VARLIST    VOLUME

TOFFSET    TREGION    TSBEZIER   TSNORMAL   VECTOR     WHILE

TORUS      TREPARAM   TSBSPLINE  TVLOAD     VIEW       ZCOLLIDE

TRAISE     TRIANGL    TSDERIVE   TVZRJACOB  VIEWOBJ

Functions and Variables

-----------------------

Functions that return a NumericType:

ABS        COS        EXP        POWER      THISOBJ

ACOS       CLNTEXEC   FLOOR      RANDOM     VOLUME

AREA       CPOLY      FMOD       SIN

ASIN       DSTPTLN    LN         SIZEOF

ATAN       DSTPTPLN   LOG        SQRT

ATAN2      DSTLNLN    MESHSIZE   TAN

Functions that return a GeometricType:

ADAPISO    CEXTREMES  FFEXTREME  MREGION    SFLECNODAL

ALGSUM     CINFLECT   FFKNTVEC   MREPARAM   SFOCAL

ANALYFIT   CINTERP    FFMATCH    MREVERSE   SFROMCRVS

AOFFSET    CIRCLE     FFMERGE    MVEXPLICIT SGAUSS

ARC        CIRCPOLY   FFMESH     MZERO      SILHOUETTE

ARC360     CLNTCRSR   FFMSIZE    NIL        SINTERP

BBOX       CLNTREAD   FFORDER    OFFSET     SKEL2DINT

BLHERMITE  CMESH      FFPOLES    ORTHOTOMC  SMEAN

BLSHERMITE CMOEBIUS   FFPTDIST   PATTRIB    SMERGE

BLOSSOM    CMORPH     FFPTTYPE   PCIRCLE    SMESH

BOOLONE    CMULTIRES  FFSPLIT    PCRVTR     SMOEBIUS

BOOLSUM    CNORMAL    FITPMODEL  PDECIMATE  SMOMENTS

BOUNDARY   CNRMLCRV   FIXPLGEOM  PDOMAIN    SMOOTHNRML

BOX        CNVXHULL   FIXPLNRML  PLANE      SMORPH

BSCTCONCN2 COERCE     GBOX       PLANECLIP  SNORMAL

BSCTCONCON COMPOSE    GETATTR    PLN3PTS    SNRMLSRF

BSCTCONCYL CON2       GETLINE    PMORPH     SPARABOLC

BSCTCONLN  CONE       GETNAME    PNORMAL    SPHERE

BSCTCONPL  CONICSEC   GGINTER    POINT      SPLITLST

BSCTCONPT  CONTOUR    GPOLYGON   POLY       SRAISE

BSCTCONSPR CONVEX     GPOLYLINE  PPINTER    SRAYCLIP

BSCTCYLCYL COORD      HERMITE    PPROPFTCH  SREFINE

BSCTCYLPL  COVERISO   INSTANCE   PRINTER    SREGION

BSCTCYLPT  COVERPT    IRITSTATE  PRISA      SREPARAM

BSCTCYLSPR CPOWER     ISGEOM     PROCEDURE  SREVERSE

BSCTPLNLN  CRAISE     ISOCLINE   PT3BARY    SRF2TANS

BSCTPLNPT  CRC2CRVTAN KNOTCLEAN  PTHMSPR    SRF3TANS

BSCTSPRLN  CREDUCE    KNOTREMOVE PTLNPLN    SRFFFORM

BSCTSPRPL  CREFINE    LINTERP    PTPTLN     SRFKERNEL

BSCTSPRPT  CREGION    LOFFSET    PTREGISTER SRINTER

BSCTSPRSPR CREPARAM   MATDECOMP  PTSLNLN    SSINTER

BSCTTRSPT  CROSSEC    MAXEDGELEN QUADRIC    STANGENT

BSCTTRSSP  CRV2TANS   MBEZIER    RAYTRAP    STRIMSRF

BSP2BZR    CRVKERNEL  MBISECTOR  RFLCTLN    STRIVAR

BUMPMAP    CRVLNDST   MBSPLINE   RRINTER    SURFPREV

BZR2BSP    CRVPTDST   MDERIVE    RULEDSRF   SURFREV

CARCLEN    CRVPTTAN   MDIVIDE    RULEDTV    SURFREVAXS

CAREA      CSURFACE   MERGEPOLY  SACCESS    SURFREV2

CBEZIER    CTANGENT   MEVAL      SASPCTGRPH SURFREVAX2

CBIARC     CTRIMSRF   MFROMMESH  SASYMPEVAL SVISIBLE

CBISECTOR  CTLPT      MFROMMV    SBEZIER    SVOLUME

CBSPLINE   CYLIN      MMERGE     SBISECTOR  SWEEPSRF

CCINTER    CZEROS     MOFFSET    SBSPLINE   SWPSCLSRF

CCRVTR     DIST2FF    MOMENT     SCRVTR     SWUNGASUM

CDERIVE    DUALITY    MPOWER     SCRVTREVAL SYMBCPROD

CDIVIDE    EVOLUTE    MPROMOTE   SDERIVE    SYMBDIFF

CEDITPT    EXTRUDE    MRAISE     SDIVIDE    SYMBDPROD

CENVOFF    FFCOMPAT   MRCHCUBE   SEDITPT    SYMBIPROD

CEVAL      FFCTLPTS   MREFINE    SEVAL      SYMBPROD

SYMBSUM    TEDITPT    TMORPH     TREGION    TSBSPLINE  TVZRJACOB

TBEZIER    TEVAL      TNSCRCR    TREPARAM   TSDERIVE   UVPOLY

TBSPLINE   TEXTGEOM   TOFFSET    TRIANGL    TSEVAL     ZCOLLIDE

TCRVTR     TEXTWARP   TORUS      TRIMSRF    TSGREGORY

TDERIVE    TFROMSRFS  TRAISE     TRMSRFS    TSNORMAL

TDIVIDE    TINTERP    TREFINE    TSBEZIER   TVLOAD

Functions that create linear transformation matrices:

HOMOMAT    RFLCTMAT   ROTY       ROTZ2V2

MATPOSDIR  ROTVEC     ROTZ       SCALE

PROJMAT    ROTX       ROTZ2V     TRANS

Miscellaneous functions:

ADWIDTH    CPATTR     INCLUDE    NTH        SYSTEM

ANIMEVAL   ERROR      INSERTPOLY PAUSE      TIME

ATTRIB     EXEC       INTERACT   PRINTF     VARLIST

ATTRPROP   EXIT       IQUERY     PRINTFILE  VECTOR

AWIDTH     FNFREE     LIST       PROCEDURE  VIEW

CHDIR      FOR        LOAD       RESET      VIEWOBJ

CLNTCLOSE  FREE       LOGFILE    RMATTR     VIEWSET

CLNTWRITE  FUNCTION   MSLEEP     SAVE       WHILE

COLOR      HELP       NREF       SETNAME

COMMENT    IF         NRMLCONE   SNOC

Variables that are predefined in the system:

AXES              MACHINE           POLY_APPROX_TOL   PRSP_MAT

DRAWCTLPT         POLY_APPROX_OPT   POLY_APPROX_TRI   RESOLUTION

FLAT4PLY          POLY_APPROX_UV    POLY_MERGE_COPLANAR VIEW_MAT

Constants that are predefined in the system:

AMIGA        E3           LIST_TYPE    P7           SGI

APOLLO       E4           MAGENTA      P8           STRING_TYPE

BEZIER_TYPE  E5           MATRIX_TYPE  P9           SURFACE_TYPE

BLACK        E6           MSDOS        PARAM_CENTRIP SUN

BLUE         E7           MULTIVAR_TYPE PARAM_CHORD  TRIMSRF_TYPE

BSPLINE_TYPE E8           NUMERIC_TYPE PARAM_NIELFOL TRISRF_TYPE

CLIENTS_ALL  E9           OFF          PARAM_UNIFORM TRIVAR_TYPE

COL          FALSE        ON           PI           TRUE

CTLPT_TYPE   GREEN        P1           PLANE_TYPE   UNDEF_TYPE

CURVE_TYPE   HP           P2           POINT_TYPE   UNIX

CYAN         IBMOS2       P3           POLY_TYPE    VECTOR_TYPE

DEPTH        IBMNT        P4           POWER_TYPE   WHITE

E1           KV_FLOAT     P5           RED          YELLOW

E2           KV_OPEN      P6           ROW

Language Description

--------------------

The front end of the IRIT Solid Modeler is an infix parser that

mimics some C language behavior. The infix operators that are supported

are plus (+), minus (-), multiply (*), divide (/), and power (^), for

numeric operators, with the same precedence as in C.

However, unlike the C language, these operators are overloaded,

or different action is taken, based upon the different operands.

This means that one can write '1 + 2', in which the plus sign denotes a

numeric addition, or one can write 'PolyObj1 + PolyObj2', in which case the

plus sign denotes the Boolean operation of a union between two geometric

objects.

The exact way each operator is overloaded is defined below.

In this environment, reals, integers, and even Booleans, are all represented

as real types. Data are automatically promoted as necessary.

For example, the constants TRUE and FALSE are defined as 1.0 and 0.0,

respectively.

Each expression is terminated by a semicolon. An expression can be as

simple as 'a;' which prints the value of variable a, or as complex as:

for ( t = 1.1, 0.1, 1.9,

cb1 = csurface( sb, COL, t ):

color( cb1, green ):

snoc( cb1, cb_all )

);

While an expression is terminated with a semicolon, a colon is used to

terminate mini-expressions within an expression.

Once a complete expression is read in (i.e., a semicolon is detected)

and parsed correctly (i.e. no syntax errors are found), it is executed.

Before each operator or a function is executed, parameter type matching

tests are made to make sure the operator can be applied to these

operand(s), or that the function gets the correct set of arguments.

The parser is totally case insensitive, so Obj, obj, and OBJ will refer

to the same object, while MergePoly, MERGEPOLY, and mergePoly will refer

to the same function.

Objects (Variables, if you prefer) need not be declared. Simply use them

when you need them. Object names may be any alpha-numeric (and underscore)

string of at most 30 characters. When assigned to an old object, the old

object will be automatically deleted and if necessary, its type will be

modified on the fly.

Example:

V = sin( 45 * pi / 180.0 );

V = V * vector( 1, 2, 3 );

V = V * rotx( 90 );

V = V * V;

will assign to V a NumericType equal to the sine of 45 degrees, the VectorType

( 1, 2, 3 ) scaled by the sine of 45, rotate that vector around the X axis

by 90 degrees, and finally a NumericType which is the dot (inner) product of

V with itself.

The parser will read from stdin, unless a file is specified on the command

line or an INCLUDE command is executed. In both cases, when the end of file

is encountered, the parser will again wait for input from stdin. In order

to execute a file and quit at the end of the file, put an EXIT command as

the last command in the file.

Overloading Plus

----------------

The + operator is overloaded above the following domains:

NumericType + NumericType -> NumericType

PointType   + PolygonType -> PolygonType  (Point polyline profiling)

VectorType  + VectorType  -> VectorType   (Vector addition)

MatrixType  + MatrixType  -> MatrixType   (Matrix addition)

PolygonType + PolygonType -> PolygonType  (Boolean UNION operation)

CurveType   + CurveType   -> CurveType    (Curve curve profiling)

CurveType   + CtlPtType   -> CurveType    (Curve control point profiling)

CtlPtType   + CtlPtType   -> CurveType    (Control points profiling)

ListType    + ListType    -> ListType     (Append lists operator)

StringType  + StringType  -> StringType   (String concat)

StringType  + RealType    -> StringType   (String concat, real as int string)

Note: Boolean UNION of two disjoint objects (no common volume) will result

in the two objects being combined. It is the USER's responsibility to make

sure that the non intersecting objects are also disjoint - this system only

tests for no intersection.  Boolean UNION of two polyline objects will merge

the list of polylines.

Overloading Minus

-----------------

The - operator is overloaded above the following domains:

As a binary operator:

NumericType - NumericType -> NumericType

VectorType  - VectorType  -> VectorType   (Vectoric difference)

MatrixType  - MatrixType  -> MatrixType   (Matrix difference)

PolygonType - PolygonType -> PolygonType  (Boolean SUBTRACT operation)

As a unary operator:

- NumericType -> NumericType

- PointType   -> PointType     (Scale vector by -1)

- VectorType  -> VectorType    (Scale vector by -1)

- CtlPtType   -> CtlPtType     (Scale vector by -1)

- PlaneType   -> PlaneType     (Scale vector by -1)

- StringType  -> StringType    (Reverse the order of string's characters)

- MatrixType  -> MatrixType    (Scale matrix by -1)

- PolygonType -> PolygonType   (Boolean NEGATION operation)

- CurveType   -> CurveType     (Curve parameterization is reversed)

- SurfaceType -> SurfaceType   (Surface parameterization is reversed)

- TrimSrfType -> TrimSrfType   (Trim surface parameterization is reversed)

Note: Boolean SUBTRACT of two disjoint objects (no common volume) will result

in an empty object. For both a curve and a surface parameterization, reverse

operation (binary minus) causes the object normal to be flipped as a side

effect.

Overloading Astrisk

-------------------

The * operator is overloaded above the following domains:

NumericType  * NumericType   -> NumericType

VectorType   * NumericType   -> VectorType    (Vector scaling)

VectorType   * CurveType     -> CurveType     (Inner product projection)

VectorType   * SurfaceType   -> SurfaceType   (Inner product projection)

VectorType   * VectorType    -> NumericType   (Inner product)

PlaneType    * MatrixType    -> PlaneType     (Plane transformation)

MatrixType   * NumericType   -> MatrixType    (Matrix Scaling)

MatrixType   * PointType     -> PointType     (Point transformation)

MatrixType   * CtlPtType     -> CtlPtType     (Ctl Point transformation)

MatrixType   * VectorType    -> VectorType    (Vector transformation)

MatrixType   * MatrixType    -> MatrixType    (Matrix multiplication)

MatrixType   * GeometricType -> GeometricType (Object transformation)

MatrixType   * ListType      -> ListType      (Object hierarchy transform.)

PolygonType  * PolygonType   -> PolygonType   (Boolean INTERSECTION operation)

InstanceType * MatrixType    -> InstanceType  (Transform of Instance's matrix)

Note: Boolean INTERSECTION of two disjoint objects (no common volume) will

result in an empty object. Object hierarchy transform transforms any

transformable object (GeometricType) found in the list recursively. Boolean

INTERSECTION of two planar (XY plane) polyline objects will compute the

intersection points of the two lists of polylines.  Be aware that a plane

multiplied by a matrix does not always do what you might expected.

Overloading Slash

-----------------

The / operator is overloaded above the following domains:

NumericType / NumericType -> NumericType

PointType   / PointType   -> PolyType      (Polyline between two pts)

PointType   / PolygonType -> PolygonType   (Point polyline profiling)

PolygonType / PolygonType -> PolygonType   (Boolean CUT operation)

Note: Boolean CUT of two disjoint objects (no common volume) will result

with an empty object.

Overloading Caret

-----------------

The ^ operator is overloaded above the following domains:

NumericType ^ NumericType -> NumericType

VectorType  ^ VectorType  -> VectorType  (Cross product)

MatrixType  ^ NumericType -> MatrixType  (Matrix to the (int) power)

PolygonType ^ PolygonType -> PolygonType (Boolean MERGE operation)

StringType  ^ StringType  -> StringType  (String concat)

StringType  ^ RealType    -> StringType  (String concat, real as real string)

Note: Boolean MERGE simply merges the two sets of polygons without

any intersection tests. Matrix powers must be positive integers or -1

or -2, in which case the matrix inverse (if it exists) or transpose

is computed.

Overloading Equal (Assignments)

-------------------------------

Assignments are allowed as side effects, in any place in an expression.

If "Expr" is an expression, then "var = Expr" is the exact same expression

with the side effect of setting Var to that value. There is no guarantee

of the order of evaluation, so using Vars that are set within the same

expression is a bad practice. Use parentheses to force the order of

evaluation, i.e., "( var = Expr )".

Comparison operators - ==, !=, <, >, <=, >=

-------------------------------------------

The conditional comparison operators can be applied to the following

domains (o for a comparison operator):

NumericType  o NumericType  -> NumericType

StringType   o StringType   -> NumericType

PointType    o PointType    -> NumericType

VectorType   o VectorType   -> NumericType

PlaneType    o PlaneType    -> NumericType

CtlPtType    o CtlPtType    -> NumericType

MatrixType   o MatrixType   -> NumericType

CurveType    o CurveType    -> NumericType

SurfaceType  o SurfaceType  -> NumericType

TrivarType   o TrivarType   -> NumericType

TriSrfType   o TriSrfType   -> NumericType

MultivarType o MultivarType -> NumericType

The returned NumericType is non-zero if the condition holds, or zero if

not.

The comparison operators other than == and != can be used on

NumericTypes and StringType only.

Logical Operators - &&, ||, |

-----------------------------

Complex logical expressions can be defined using the logical and (&&),

logical or (||) and logical not (!). These operators can be applied

to NumericTypes that are considered Boolean results. That is, true for a

non-zero value, and false otherwise.

The returned NumericType is true if both operands are true for the and

operator, at least one is true for the or operator, and the operand is

false for the not operator. In all other cases, a false is returned.

To make sure logical expressions are readable, the and and or

operators are defined to have the same priority. Use parentheses to

disambiguate a logical expression and to make it more readable.

Geometric Boolean Operations

----------------------------

The IRIT Solid Modeling System supports Boolean operations between

polyhedra objects. Freeform objects will be automaticaly converted to

a polygonal representation when used in Boolean operations.

The +, *, and - are overloaded to denote Boolean union, intersection

and subtraction when operating on geometric entities. - can also be

used as an unary operator to reverse the object orientation inside out.

IRIT supports Boolean operations on polyhedra models.  A polyhedra

based model is simply a collection of polygons.  While a polyhedra is

simply a set of polygons, this set must conform to certain conditions:

Every polygon has known adjacent polygons, for all its edges.

The model must be a 2-manifold.  That is every edge is shared by

exactly two polygons.

intersection curves must be closed and the objects participating

in the Boleans might be open in unintersecting regions.

Every polygon has a normal that points into the model.  That

normal is inside/outside consistent with its adjacent polygons.

In other words, for every polygon, one can locally determine the

inside or the outside of the model.  Moreover, every polygon has

neighbors for all its edges, forming a closed object that

consistently delineates inside from outside, globally.

If your input geometry does not adhere to the above constrains, the Boolean

operation is likely to fail.  You can enable a special intersection-curves

mode that only compute the intersection curves between the two input

objects and does not form the output object.  This special model is

insensitive to many of the above constraints so you could use this model

to examine the intersection curves and make sure there are indeed forming

closed loops.  You can enable this intersection-curves mode via

'iritstate("intercrv", true);'.  See also IRITSTATE command.

The Boolean operations are set operations conducted between two

such models, M_1 and M_2, that delineate inside from

outside.  Boolean Union, Boolean Intersection and Boolean Subtraction

are the three common operations that resemble the exact semantic that

is expected, when treating M_1 and M_2 as

three-dimensional point sets.

The Boolean operations can be formulated into a binary tree structure

also known as a Constructive Solid Geometry (CSG) tree.

Example:

resolution = 20;

B = box(vector(-1, -1, -0.25), 2, 1.2, 0.5);

C = con2(vector(0, 0, -1.5), vector(0, 0, 3), 0.7, 0.3);

D = convex(B - C);

E = convex(C - B);

F = convex(B + C);

G = convex(B * C);

tr = rotx( -90 ) * roty( 40 ) * rotx( -30 );

All = list( D * tr * trans( vector(  0.6,  0.5, 0.0 ) ),

E * tr * trans( vector(  3.0,  0.0, 0.0 ) ),

F * tr * trans( vector( -2.0,  0.0, 0.0 ) ),

G * tr * trans( vector(  0.7, -1.0, 0.0 ) ) )

* scale( vector( 0.25, 0.25, 0.25 ) )

* trans( vector( -0.1, -0.3, 0.0 ) );

view_mat = rotx( 0 );

view( list( view_mat, All ), on );

save( "booleans", list( view_mat, All ) );

This is a complete example of how to compute the union, intersection and

both differences of a box and a truncated cone.

Special cases can be very difficult to handle when considering Boolean

operations.  Consider an axes parallel bounding cube.  Consider a

second cube rotated alpha degrees from the first cube.  At large

angles, the Boolean operations are fairly simple to

compute. Nevertheless, as alpha approaches zero, the almost coplanar

planes of the two intersecting cubes make it very difficult to

robustly and consistently compute their intersection.

There are several flags to control the Boolean operations. See

IRITSTATE command for the "InterCrv", "Coplanar", and "PolySort"

states.

Priority of operators

---------------------

The following table lists the priority of the different operators.

Lowest             Operator           Name of operator

priority           ,                  comma

:                  colon

&&, ||             logical and, logical or

=,==,!=,<=,>=,<,>  assignment, equal, not equal, less

equal, greater equal, less, greater

+, -               plus, minus

*, /               multiply, divide

Highest            ^                  power

priority           -, !               unary minus, logical not

Grammar

-------

The grammar of the IRIT parser follows guidelines similar to those of

the C language for simple expressions. However, complex statements differ.

See the IF, FOR, FUNCTION, and PROCEDURE below for the usage of these

clauses.

Function Description

--------------------

NumericType returning functions

-------------------------------

ABS

---

NumericType ABS( NumericType Operand )

returns the absolute value of the given Operand.

ACOS

----

NumericType ACOS( NumericType Operand )

returns the arc cosine value (in radians) of the given Operand.

AREA

----

NumericType AREA( PolygonType Object )

returns the area of the given Object (in object units).

The area of the polygonal object, not the area of the primitive it might

approximate,is returned.

This means that the area of a polygonal approximation of a sphere will be

returned, not the exact area of the sphere.

ASIN

----

NumericType ASIN( NumericType Operand )

returns the arc sine value (in radians) of the given Operand.

ATAN

----

NumericType ATAN( NumericType Operand )

returns the arc tangent value (in radians) of the given Operand.

ATAN2

-----

NumericType ATAN2( NumericType Operand1, NumericType Operand2 )

returns the arc tangent value (in radians) of the given ratio:

Operand1 / Operand2, over the whole circle.

COS

---

NumericType COS( NumericType Operand )

returns the cosine value of the given Operand (in radians).

CLNTEXEC

--------

NumericType CLNTEXEC( StringType ClientName )

Initiate communication channels to a client named ClientName.

ClientName is executed by this function as a sub process.

Two communication channels are opened between the IRIT server and the

new client, for read and write. See also CLNTCRSR, CLNTREAD, CLNTWRITE, and

CLNTCLOSE. If ClientName is an empty string, the user is provided

with the new communication port to be used and the server blocks for the

user to manually execute the client after setting the proper

IRIT_SERVER_HOST/PORT environment variables.

Example:

h1 = CLNTEXEC( "" );

h2 = CLNTEXEC( "nuldrvs -s-" );

executes two clients, one named nuldrvs while the other one is

prompted for by the user. As a result of the second invokation of

CLNTEXEC, the user will be prompted with a message similar to:

Irit: Startup your program - I am waiting...

setenv IRIT_SERVER_PORT 2182

and he/she will need to set the proper environment variable and execute

their client manually.

CPOLY

-----

NumericType CPOLY( PolygonType Object )

returns the number of polygons in the given polygonal Object.

DSTPTLN

-------

NumericType DSTPTLN( PointType Pt, PointType LineOrig, VectorType LineRay )

returns the distance between a given point Pt and line

LineOrig, LineRay. See also PTPTLN.

DSTPTPLN

--------

NumericType DSTPTPLN( PointType Pt, PlaneType Plane )

returns the distance between a given point Pt and plane

Plane.

DSTLNLN

-------

NumericType DSTLNLN( PointType Line1Orig, VectorType Line1Ray,

PointType Line2Orig, VectorType Line2Ray )

returns the distance between two lines defined by point LineiOrig

and ray LineiRay. See also PTSLNLN.

EXP

---

NumericType EXP( NumericType Operand )

returns the natural exponential value of the given Operand.

FLOOR

-----

NumericType FLOOR( NumericType Operand )

returns the largest integer not greater than the Operand.

FMOD

----

NumericType FMOD( NumericType Operand, NumericType Mod )

returns the floating point remainder of the division of the Operand by

Mod.

LN

--

NumericType LN( NumericType Operand )

returns the natural logarithm value of the given Operand.

LOG

---

NumericType LOG( NumericType Operand )

returns the base 10 logarithm value of the given Operand.

MESHSIZE

--------

NumericType MESHSIZE( FreeformType Freeform, ConstantType Direction )

returns the size of the Freeform's mesh in a Direction, which

will be COL, ROW or DEPTH.  For the case of a multivariate Freeform,

the Direction is an integer value starting from 0.  See also FFMSIZE.

Examples:

Len = MESHSIZE( Crv, COL );

RSize = MESHSIZE( Sphere, ROW );

CSize = MESHSIZE( Sphere, COL );

TVSize = MESHSIZE( TV, COL ) * MESHSIZE( TV, ROW ) * MESHSIZE( TV, DEPTH );

MVSize1 = MESHSIZE( MV, 1 );

POWER

-----

NumericType POWER( NumericType Operand, NumericType Exp )

returns the Operand to the power of Exp.

RANDOM

------

NumericType RANDOM( NumericType Min, NumericType Max )

returns a randomized value between Min and Max.

See also "RandomInit", in the IRITSTATE function.

SIN

---

NumericType SIN( NumericType Operand )

returns the sine value of the given Operand (in radians).

SIZEOF

------

NumericType SIZEOF( ListType List | PolygonType Poly |

CurveType Crv | StringType Str )

returns the length of a list if List, the number of polygons

if Poly, the length of the control polygon if Crv, or the

number of characters in string if Str.

If, however, only one polygon is in Poly, it returns the number

of vertices in that polygon.

Example:

len = SIZEOF( list( 1, 2, 3 ) );

numPolys = SIZEOF( axes );

numCtlpt = SIZEOF( circle( vector( 0, 0, 0 ), 1 ) );

will assign the value of 3 to the variable len, set numPolys

to the number of polylines in the axes object, and set numCtlPt to 9, the

number of control points in a circle.

SQRT

----

NumericType SQRT( NumericType Operand )

returns the square root value of the given Operand.

TAN

---

NumericType TAN( NumericType Operand )

returns the tangent value of the given Operand (in radians).

THISOBJ

-------

NumericType THISOBJ( StringType Object )

returns the object type of the given name of an Object. This can

be one of the constants,

UNDEF_TYPE        PLANE_TYPE        CTLPT_TYPE        MODEL_TYPE

POLY_TYPE         MATRIX_TYPE       LIST_TYPE         MULTIVAR_TYPE

NUMERIC_TYPE      CURVE_TYPE        TRIVAR_TYPE

POINT_TYPE        SURFACE_TYPE      TRISRF_TYPE

VECTOR_TYPE       STRING_TYPE       TRIMSRF_TYPE

This is also a way to ask if an object by a given name exists (if the

returned type is UNDEF_TYPE or not).

VOLUME

------

NumericType VOLUME( PolygonType Object )

returns the volume of the given Object (in object units). It returns

the volume of the polygonal object, not the volume of the object it might

approximate.

This routine decomposes all non-convex polygons to convex ones, as a side

effect (see CONVEX).

GeometricType returning functions

---------------------------------

ADAPISO

-------

CurveType ADAPISO( SurfaceType Srf, NumericType Dir, NumericType Eps,

NumericType FullIso, NumericType SinglePath )

constructs a coverage to Srf using isocurve in the Dir direction,

so that for any point p on surface Srf, there exists a point on one of

the isocurves that is close to p within Eps. If FullIso, the

extracted isocurves span the entire surface domain; otherwise they may

span only a subset of the domain. If SinglePath, an approximation to

a single path (Hamiltonian path) that visits all isocurves is constructed

(not supported). See also COVERPT, COVERISO.

srf = sbezier( list( list( ctlpt( E3, -0.5, -1.0,  0.0 ),

ctlpt( E3,  0.4,  0.0,  0.1 ),

ctlpt( E3, -0.5,  1.0,  0.0 ) ),

list( ctlpt( E3,  0.0, -0.7,  0.1 ),

ctlpt( E3,  0.0,  0.0,  0.0 ),

ctlpt( E3,  0.0,  0.7, -0.2 ) ),

list( ctlpt( E3,  0.5, -1.0,  0.1 ),

ctlpt( E3, -0.4,  0.0,  0.0 ),

ctlpt( E3,  0.5,  1.0, -0.2 ) ) ) );

aiso = ADAPISO( srf, COL, 0.1, FALSE, FALSE );

constructs an adaptive isocurve approximation with tolerance of 0.1 to

surface srf in direction COL. Isocurves are allowed to span a

subset of the surface domain. No single path is needed.

The SinglePath option is currently not supported.

ALGSUM

------

SurfaceType ALGSUM( CurveType Crv1, CurveType Crv2 )

Given two curves, compute a surface that is their algebraic sum:

S(u, v) = C1(u) + C2(v)

Example:

c1 = circle( vector( 0.0, 0.0, 0.0 ), 0.7 );

c2 = ctlpt( E3, -0.2, -0.5, -1.5 ) + ctlpt( E3, 0.2, 0.5, 1.5 );

s1 = algsum( c1, c2 );

c2 = cbspline( 3,

list( ctlpt( E3, 0.0, 0.0, 0.0 ),

ctlpt( E3, 0.0, 0.0, 0.7 ),

ctlpt( E3, 0.0, 1.5, 1.0 ),

ctlpt( E3, 0.0, 0.0, 1.3 ),

ctlpt( E3, 0.0, 0.0, 2.0 ) ),

list( KV_OPEN ) );

s2 = algsum( c1, c2 );

creates two algebraic sum surfaces, one in the shape of a cylinder as

a sum of a line and a circle, and one circular sweep like.

ANALYFIT

--------

ListType ANALYFIT( ListType UVPts, ListType EucPts,

NumericType FirstAtOrigin, NumericType Degree )

computes a surface fit to the given paraametrized points data.

The fitted surface will be of bi-degree Degree, fitting points

EucPts at parameters UVPts.  Needless to say EucPts

and UVPts should be lists of points of similar length.  If

FirstAtOrigin is TRUE, all points are translated so the first point

in EucPts is at the origin.  Only the first coordinates of UVPts

are used.

Example:

Fitting = nil();

Eps = 1e-2;

PtPln = nil():

for (i = 1, 1, 100,

snoc( point( random( -1, 1 ), random( -1, 1 ), random( -Eps, Eps ) ),

PtPln ) );

BilinCoefs = ANALYFIT( PtPln, PtPln, 0, 1 ) );

fits a bilinear to the given planar data with noise.  See also the COERCE

function from POWER_TYPE to BEZIER_TYPE, and FITPMODEL.

AOFFSET

-------

CurveType AOFFSET( CurveType Crv, NumericType OffsetDistance,

NumericType Epsilon, NumericType TrimLoops,

NumericType BezInterp )

or

CurveType AOFFSET( CurveType Crv, CurveType OffsetDistance,

NumericType Epsilon, NumericType TrimLoops,

NumericType BezInterp )

computes an offset of OffsetDistance with a globally bounded error

(controlled by Epsilon). The smaller Epsilon is, the better

the approximation to the offset. The bounded error is achieved by adaptive

refinement of the Crv.

If OffsetDistance is a (scalar) curve, the curve's first coordinate

is used to prescribe a variable offset amount along the curve.  Both

Crv and OffsetDistance must share the same parametric domain.

If TrimLoops is TRUE or on, the regions of the object that

self-intersect as a result of the offset operation are trimmed away.

If BezInterp is TRUE, each curve's segment is interpolated instead

of approximated.

Example:

OffCrv1 = AOFFSET( Crv, 0.5, 0.01, FALSE, FALSE );

OffCrv2 = AOFFSET( Crv, 0.5, 0.01, TRUE, FALSE );

computes an adaptive offset to Crv with OffsetDistance of

0.5 and Epsilon of 0.01 and trims the self intersection loops in

the second instance.

See also OFFSET, LOFFSET, and MOFFSET.

ARC

---

CurveType ARC( VectorType StartPos, VectorType Center, VectorType EndPos )

constructs an arc between the two end points StartPos and EndPos,

centered at Center. THe arc will always be less than 180 degrees, so the

shortest circular path from StartPos to EndPos is selected. The

case where StartPos, Center, and EndPos are collinear is

illegal, since it attempts to define a 180 degrees arc. The arc is constructed

as a single rational quadratic Bezier curve.

Example:

Arc1 = ARC( vector( 1.0, 0.0, 0.0 ),

vector( 1.0, 1.0, 0.0 ),

vector( 0.0, 1.0, 0.0 ) );

constructs a 90 degrees arc, tangent to both the X and Y axes at coordinate 1.

See also ARC360

ARC360

------

CurveType ARC360( VectorType Center, NumericType Radius,

NumericType StartAngle, NumericType EndAngle )

constructs an arc between the two angles (degrees) StartAngle and

EndAngle, centered at Center. The arc will always be less than 360

degrees.  The arc is constructed as a rational quadratic B-spline curve.

Example:

Arc2 = ARC360( vector( 0.0, 0.0, 0.0 ), 1.0, 75, 355 );

constructs a 280 degrees arc.

See also ARC.

BBOX

----

ListType BBOX( GeometricTreeType Geom )

Given a (tree of) geometry, Geom computes its bounding box and

return it as a list of six numbers: XMin/Max, YMin/Max, ZMin/Max, in this

order.

Example:

B1 = BBOX( axes );

BLHERMITE

---------

SurfaceType BLHERMITE( CurveType Bndry1, CurveType Bndry2,

CurveType Tan1, CurveType Tan2,

CurveType Sctn, CurveType Nrml )

computes a Hermite blend surface that supports an arbitrary cross section.

This constructs a surface between Bndry1 and Bndry2 so that the

first derivative continuity constraints, as prescribed by Tan1 at

Bndry1 and Tan2 at Bndry2, are preserved.  In addition,

the interior between Bndry1 and Bndry2 will follow the shape

of planar cross section curve Sctn and will be oriented along the

vector field prescribed by Nrml.

Cross section Sctn is a planar curve that must start at (-1, 0) and

end at (1, 0), and have zero speed at the ends (first control point equals

the second and is the same at the end).

Example:

c1 = ctlpt( e3, 0, 0, 0 ) + ctlpt( e3, 0, 1, 0 );

c2 = ctlpt( e3, 1, 0, 0 ) + ctlpt( e3, 1, 1, 0 );

d1 = ctlpt( e3,  1, 0,  1 ) + ctlpt( e3,  1, 0,  0.1 );

d2 = ctlpt( e3,  1, 0, -0.1 ) + ctlpt( e3,  1, 0, -1 );

s1 = hermite( c1, c2, d1, d2 );

color( s1, red );

cSec = cbspline( 3,

list( ctlpt( e2, -1,    0 ),

ctlpt( e2, -1,    0 ),

ctlpt( e2, -0.14, 0.26 ),

ctlpt( e2, -0.65, 0.51 ),

ctlpt( e2,  0,    0.76 ),

ctlpt( e2,  0.65, 0.51 ),

ctlpt( e2,  0.14, 0.26 ),

ctlpt( e2,  1,    0 ),

ctlpt( e2,  1,    0 ) ),

list( kv_open ) );

n = ctlpt( e3, 0, 0, 1 ) + ctlpt( e3, 0, 0, 1 );

s2 = blhermite( c1, c2, d1, d2, cSec2, n );

color( s2, yellow );

constructs a regular Hermite surfaces s1 and a blending Hermite that

follows the cross section cSec.  See also HERMITE and BLSHERMITE.

BLSHERMITE

----------

SurfaceType BLSHERMITE( SurfaceType Srf, CurveType PCrv,

CurveType Sctn, NumericType TanScale,

NumericType Width, NumericType Height )

computes a Hermite blend surface on Srf along parametric curve of

Srf, PCrv, the cross section Sctn, a tangent field scale

control TanScale, and the width and height control of Width

and Height.

The constructed surface, which is C1 continuous to Srf, is positioned

along PCrv, a curve in the parametric domain of Srf.  The

cross section Sctn is a planar curve that must start at (-1, 0) and

end at (1, 0), and have zero speed at the ends (first control point equals

the second and is the same at the end).  TanScale controls how rapid

the change in the tangent is, as we move away from the surface.

Example:

cSec = cbspline( 3,

list( ctlpt( e2, -1,    0 ),

ctlpt( e2, -1,    0 ),

ctlpt( e2, -0.5,  0.2 ),

ctlpt( e2, -0.7,  0.3 ),

ctlpt( e2,  0,    0.5 ),

ctlpt( e2,  0.7,  0.3 ),

ctlpt( e2,  0.5,  0.2 ),

ctlpt( e2,  1,    0 ),

ctlpt( e2,  1,    0 ) ),

list( kv_open ) );

s = -surfPRev( cregion( pcircle( vector( 0, 0, 0 ), 1 ),

0, 2 ) * rx( 90 ) );

s1 = BLSHERMITE( s, ctlpt( E2, 0, 1 ) + ctlpt( E2, 4, 1 ),

cSec, 1, 0.2, 0.5 );

s2 = BLSHERMITE( s, ctlpt( E2, 0, 1.5 ) + ctlpt( E2, 4, 1.5 ),

cSec, 0.1, 0.2, 0.5 );

s3 = BLSHERMITE( s, ctlpt( E2, 0, 0.3 ) + ctlpt( E2, 4, 0.3 ),

cSec, 1.5, 0.2, 0.5 );

places three Hermite blend surfaces s1, s2, s3 using the cross

section cSec on a unit sphere s.  See also HERMITE and

BLHERMITE.

BLOSSOM

-------

CtlPtType BLOSSOM( CurveType Crv, ListType BlossomVals )

computes the blossom of the given Crv and the given blossom values

BlossomVals.

Example:

c1 = cbezier( list( ctlpt( E2, 1.7, 0.0 ),

ctlpt( E2, 0.7, 0.7 ),

ctlpt( E2, 1.7, 0.3 ),

ctlpt( E2, 1.5, 0.8 ),

ctlpt( E2, 1.6, 1.0 ) ) );

BLOSSOM( c1, list( 0, 0, 0, 0 ) ) == coord( c1, 0 ) &&

BLOSSOM( c1, list( 0, 0, 0, 1 ) ) == coord( c1, 1 ) &&

BLOSSOM( c1, list( 0, 0, 1, 1 ) ) == coord( c1, 2 ) &&

BLOSSOM( c1, list( 0, 1, 1, 1 ) ) == coord( c1, 3 ) &&

BLOSSOM( c1, list( 1, 1, 1, 1 ) ) == coord( c1, 4 );

extracts the control points of an quadric Bezier curve via blossoming and

compares this to the results obtained via a traditional extraction

approach (via the COORD function).

BOOLONE

-------

SurfaceType BOOLONE( CurveType Crv )

Given a closed curve, the curve is subdivided into four segments equally

spaced in the parametric space that are fed into BOOLSUM. This is useful if

a surface should "fill" the area enclosed by a closed curve.

Example:

Srf = BOOLONE( circle( vector( 0.0, 0.0, 0.0 ), 1.0 ) );

creates a disk surface containing the area enclosed by the unit circle.

BOOLSUM

-------

SurfaceType BOOLSUM( CurveType Crv1, CurveType Crv2,

CurveType Crv3, CurveType Crv4 )

constructs a surface using the provided four curves as its four boundary

curves. Curves do not have to have the same order or type, and will be

promoted to their least common denominator. The end points of the four

curves should match as follows:

Crv1 start point,              to Crv3 start point.

Crv1 end point,                to Crv4 start point.

Crv2 start point,              to Crv3 end point.

Crv2 end point,                to Crv4 end point.

where Crv1 and Crv2 are the two boundaries in one parametric

direction, and Crv3 and Crv4 are the two boundaries in the other

parametric direction.

Example:

Cbzr1 = cbezier( list( ctlpt( E3, 0.1, 0.1, 0.1 ),

ctlpt( E3, 0.0, 0.5, 1.0 ),

ctlpt( E3, 0.4, 1.0, 0.4 ) ) );

Cbzr2 = cbezier( list( ctlpt( E3, 1.0, 0.2, 0.2 ),

ctlpt( E3, 1.0, 0.5, -1.0 ),

ctlpt( E3, 1.0, 1.0, 0.3 ) ) );

Cbsp3 = cbspline( 4,

list( ctlpt( E3, 0.1,  0.1, 0.1 ),

ctlpt( E3, 0.25, 0.0, -1.0 ),

ctlpt( E3, 0.5,  0.0, 2.0 ),

ctlpt( E3, 0.75, 0.0, -1.0 ),

ctlpt( E3, 1.0,  0.2, 0.2 ) ),

list( KV_OPEN ) );

Cbsp4 = cbspline( 4,

list( ctlpt( E3, 0.4,  1.0, 0.4 ),

ctlpt( E3, 0.25, 1.0, 1.0 ),

ctlpt( E3, 0.5,  1.0, -2.0 ),

ctlpt( E3, 0.75, 1.0, 1.0 ),

ctlpt( E3, 1.0,  1.0, 0.3 ) ),

list( KV_OPEN ) );

Srf = BOOLSUM( Cbzr1, Cbzr2, Cbsp3, Cbsp4 );

BOUNDARY

--------

AnyType BOUNDARY( AnyType Obj )

Given a geometric object Obj, let it be a surface, a trimed surface,

or a polygonal model, returns the boundary of the shape.  If Obj is a

polygonal object originated with a surface and all vertices has "uvvals"

attributes, by placing "SrfBoundary" attribute on Obj with the

"UMin VMin UMax VMax" surface domain, the surface boundary is will be

properly detected, even if the surface is closed.

Example:

CBndry1 = BOUNDARY( Srf );

poly_approx_uv = 1;

Pl = gpolygon( Srf, 1 );

attrib( Pl, "SrfBoundary", "0 0 1 2" );  # Domain of Srf [0, 1] x [0, 2]

CBndry2 = BOUNDARY( Pl );

returns in CBndry1, the four boundary curves of tensor product surface

Srf and in CBndry2, the edges on the tesselation of Srf,

Pl, on the boundary of Srf, even if Srf is closed.

BOX

---

PolygonType BOX( VectorType Point,

NumericType Dx, NumericType Dy, NumericType Dz )

creates a BOX polygonal object, whose boundary is coplanar with the

XY, XZ, and YZ planes. The BOX is defined by Point as

base position, and Dx, Dy, Dz as BOX dimensions. Negative dimensions

are allowed.

Example:

B = BOX( vector( 0, 0, 0 ), 1, 1, 1);

creates a unit cube from 0 to 1 in all axes.

BSCTCONCN2

----------

SurfaceType BSCTCONCN2( PointType ConeApx1, VectorType ConeDir1,

NumericType ConeAngle1,

PointType ConeApx2, VectorType ConeDir2,

NumericType ConeAngle2 )

computes the bisector surface of two cones in general position.

The cones' apexes can be found in ConeApx1 and ConeApx2

with axes directions ConeDir1 and ConeDir2 and spanning

angles of ConeAngle1 and ConeAngle2.

Example:

BisectSrf = BSCTCONCN2( Apx1, Dir1, Ang1, Apx2, Dir2, Ang2 );

See also BSCPCONCON, BSCTCONCYL, BSCTCYLCYL, BSCTCONLN, BSCTCONPL,

BSCTCONPT, BSCTCONSPR, BSCTCYLPL, BSCTCYLPT, BSCTCYLSPR, BSCTPLNLN,

BSCTPLNPT, BSCTSPRLN, BSCTSPRPL, BSCTSPRPT, BSCTSPRSPR, BSCTTRSPT,

BSCTTRSSPR, CBISECTOR, SBISECTOR,

BSCTCONCON

----------

SurfaceType | ListType BSCTCONCON( VectorType ConeDir1, NumericType ConeAngle1,

VectorType ConeDir2, NumericType ConeAngle2,

NumericType Size )

computes the bisector surface of two cones that share the same apex.

The cones' directions are ConeDir1 and ConeDir2 and the spanning

angles of ConeAngle1 and ConeAngle2. ConeDir1 and

ConeDir2 must be in the northern hemisphere; i.e. their Z coefficient

must be positive.

Size controls the portion of the (infinite) bisector actually

represented.

Example:

BisectSrf = BSCTCONCON( vector( 0, 0, 1 ), 50,

vector( 0, 0, 1 ), 20, 1.0 );

computes the bisector of two concentric cones, which is also a cone.

See also BSCTCONLN, BSCTCONPL, BSCTCONPT, BSCTCONSPR, BSCTCYLPL,

BSCPCONCON, BSCTCONCN2, BSCTCONCYL, BSCTCYLPT, BSCTCYLSPR, BSCTPLNLN,

BSCTPLNPT, BSCTSPRLN, BSCTSPRPL, BSCTSPRPT, BSCTSPRSPR, BSCTTRSPT,

BSCTTRSSPR, CBISECTOR, SBISECTOR

BSCTCONCYL

----------

SurfaceType BSCTCONCYL( PointType ConeApx1, VectorType ConeDir1,

NumericType ConeAngle1,

PointType CylPt2, VectorType CylDir2,

NumericType CylRad2 )

computes the bisector surface of a cone and a cylinder in general position.

The cones apex is in ConeApx1 with axes direction of ConeDir1

and spanning angles of ConeAngle1.  The second cylinder starts at

CylPt2, in direction CylDir2 and radius CylRad2.

Example:

BisectSrf = BSCTCONCYL( Apx1, Dir1, Ang1, Pt2, Dir2, Rad2 );

See also BSCPCONCON, BSCTCONCN2, BSCTCYLCYL, BSCTCONLN, BSCTCONPL,

BSCTCONPT, BSCTCONSPR, BSCTCYLPL, BSCTCYLPT, BSCTCYLSPR, BSCTPLNLN,

BSCTPLNPT, BSCTSPRLN, BSCTSPRPL, BSCTSPRPT, BSCTSPRSPR, BSCTTRSPT,

BSCTTRSSPR, CBISECTOR, SBISECTOR

BSCTCONLN

---------

SurfaceType | ListType BSCTCONLN( VectorType ConeDir, NumericType ConeAngle,

VectorType LineDir, NumericType Size )

computes the bisector surface of a cone and a line through its apex.

The cone's direction is ConeDir and its spanning angle is

ConeAngle.  ConeDir and LineDir must be in the northern

hemisphere; i.e. their Z coefficient must be positive.

Size controls the portion of the (infinite) bisector actually

represented.

Example:

BisectSrf = ( vector( 0, 0, 1 ), 45, vector( 0, 0.1, 1 ), 1 );

computes the bisector surface of a cone along the Z axis with spanning

angle of 45 degrees, and a line through its apex in direction ( 0, 0.1, 1 ).

See also BSCPCONCON, BSCTCONCN2, BSCTCONCYL, BSCTCONCON, BSCTCONPL,

BSCTCONPT, BSCTCONSPR, BSCTCYLPL, BSCTCYLPT, BSCTCYLSPR, BSCTPLNLN,

BSCTPLNPT, BSCTSPRLN, BSCTSPRPL, BSCTSPRPT, BSCTSPRSPR, BSCTTRSPT,

BSCTTRSSPR, CBISECTOR, SBISECTOR,

BSCTCONPL

---------

SurfaceType | ListType BSCTCONPL( PointType ConeApex, VectorType ConeDir,

NumericType ConeAngle, NumericType Size )

computes the bisector surface of a general cone and the XY plane (Z = 0

plane).  The cone's apex is at ConeApex, the cone's direction is

ConeDir and its spanning angle is ConeAngle.  Dir must be

in the northern hemisphere; i.e. their Z coefficient must be positive.

Size controls the portion of the (infinite) bisector actually

represented.

Example:

BisectSrf = BSCTCONPL( point( 0, 0, -0.3 ), vector( 1, 1, 1 ), 20, 1 );

computes the bisector surface of a cone with its apex at (0, 0, -0.3)

along the axis (1, 1, 1) with spanning angle of 20 degrees, and the plane

Z = 0.

See also BSCPCONCON, BSCTCONCN2, BSCTCONCYL, BSCTCONCON, BSCTCONLN,

BSCTCONPT, BSCTCONSPR, BSCTCYLPL, BSCTCYLPT, BSCTCYLSPR, BSCTPLNLN,

BSCTPLNPT, BSCTSPRLN, BSCTSPRPL, BSCTSPRPT, BSCTSPRSPR, BSCTTRSPT,

BSCTTRSSPR, CBISECTOR, SBISECTOR.

BSCTCONPT

---------

SurfaceType | ListType BSCTCONPT( PointType ConeApex, VectorType ConeDir,

NumericType ConeAngle, PointType Pt,

NumericType Size )

computes the bisector surface of a cone in a general position and a point,

Pt.

The cone's apex is at ConeApex, the cone's direction is ConeDir

and its spanning angle is ConeAngle.

Size controls the portion of the (infinite) bisector actually

represented.

Example:

Bisect = BSCTCONPT( point( 0, 0, 0 ), vector( 0, 0, 1 ), 22,

point( 0, 0.2, 0.7 ), 1 );

See also BSCPCONCON, BSCTCONCN2, BSCTCONCYL, BSCTCONCON, BSCTCONLN,

BSCTCONPL, BSCTCONSPR, BSCTCYLPL, BSCTCYLPT, BSCTCYLSPR, BSCTPLNLN,

BSCTPLNPT, BSCTSPRLN, BSCTSPRPL, BSCTSPRPT, BSCTSPRSPR, BSCTTRSPT,

BSCTTRSSPR, CBISECTOR, SBISECTOR.

BSCTCONSPR

----------

SurfaceType | ListType BSCTCONSPR( PointType ConeApex, VectorType ConeDir,

NumericType ConeAngle, PointType SptCntr,

NumericType SprRadius, NumericType Size )

computes the bisector surface of a cone and a sphere.

The cone's apex is at ConeApex, the cone's direction is ConeDir

and its spanning angle is ConeAngle.  The sphere is centered at

SptCntr and has a radius of SprRadius.

Size controls the portion of the (infinite) bisector actually

represented.

Example:

BisectSrf = BSCTCONSPR( point( 0, 0, 0 ), vector( 0, 0, 1 ),

45, point( 0, 0, 1 ), 0.5, 2.0 );

computes the bisector between a cone along the Z axis with a 45 degree

spanning angle and a sphere at (0, 0, 1) of radius 0.5.

See also BSCPCONCON, BSCTCONCN2, BSCTCONCYL, BSCTCONCON, BSCTCONLN,

BSCTCONPL, BSCTCONPT, BSCTCYLPL, BSCTCYLPT, BSCTCYLSPR, BSCTPLNLN,

BSCTPLNPT, BSCTSPRLN, BSCTSPRPL, BSCTSPRPT, BSCTSPRSPR, BSCTTRSPT,

BSCTTRSSPR, CBISECTOR, SBISECTOR.

BSCTCYLCYL

----------

SurfaceType BSCTCYLCYL( PointType CylPt1, VectorType CylDir1,

NumericType CylRad1,

PointType CylPt2, VectorType CylDir2,

NumericType CylRad2 )

computes the bisector surface of two cylinders in a general position.

The cylinders start at CylPt1 and CylPt2 and follow the

directions CylDir1 and  CylDir2. They have radii of CylRad1

and CylRad2.

Example:

BisectSrf = BSCTCYLCYL( Pt1, Dir1, Rad1, Pt2, Dir2, Rad2 );

See also BSCPCONCON, BSCTCONCN2, BSCTCONCYL, BSCTCONLN, BSCTCONPL,

BSCTCONPT, BSCTCONSPR, BSCTCYLPL, BSCTCYLPT, BSCTCYLSPR, BSCTPLNLN,

BSCTPLNPT, BSCTSPRLN, BSCTSPRPL, BSCTSPRPT, BSCTSPRSPR, BSCTTRSPT,

BSCTTRSSPR, CBISECTOR, SBISECTOR.

BSCTCYLPL

---------

SurfaceType | ListType BSCTCYLPL( PointType CylPos, VectorType CylDir,

NumericType CylRadius, Size )

computes the bisector surface of a a cylinder and the XY plane (plane

Z = 0).

The cylinder is located at CylPos, in the direction of CylDir

which also sets the length of the cylinder.  The radius of the cylinder

is CylRadius.

Size controls the portion of the (infinite) bisector actually

represented.

Example:

Pt = point( 0.1, 0, 0.2 );

BisectSrf = BSCTCYLPL( point( 0, 0, 0.5 ), vector( 0, 0, 1 ), 0.2, 1 );

computed the bisector surface between a cylinder of radius 0.2 along the

Z axis and the XY plane.

See also BSCPCONCON, BSCTCONCN2, BSCTCONCYL, BSCTCONCON, BSCTCONLN,

BSCTCONPL, BSCTCONPT, BSCTCONSPR, BSCTCYLPT, BSCTCYLSPR, BSCTPLNLN,

BSCTPLNPT, BSCTSPRLN, BSCTSPRPL, BSCTSPRPT, BSCTSPRSPR, BSCTTRSPT,

BSCTTRSSPR, CBISECTOR, SBISECTOR.

BSCTCYLPT

---------

SurfaceType | ListType BSCTCYLPT( PointType CylPos, VectorType CylDir,

NumericType CylRadius, PointType Pt,

NumericType Size )

computes the bisector surface of a a cylinder and a point, Pt.

The cylinder is located at CylPos, in the direction of CylDir

which also sets the length of the cylinder.  The radius of the cylinder

is CylRadius.

Size controls the portion of the (infinite) bisector actually

represented.

Example:

Pt = point( 0.1, 0, 0.2 );

BisectSrf = BSCTCYLPT( point( 0, 0, 0.5 ), vector( 0, 0, 1 ), 0.2, Pt, 1 );

computes the bisector surface between a cylinder of radius 0.2 along the

Z axis and a point at (0.1, 0, 0.2).

See also BSCPCONCON, BSCTCONCN2, BSCTCONCYL, BSCTCONCON, BSCTCONLN,

BSCTCONPL, BSCTCONPT, BSCTCONSPR, BSCTCYLPL, BSCTCYLSPR, BSCTPLNLN,

BSCTPLNPT, BSCTSPRLN, BSCTSPRPL, BSCTSPRPT, BSCTSPRSPR, BSCTTRSPT,

BSCTTRSSPR, CBISECTOR, SBISECTOR

BSCTCYLSPR

----------

SurfaceType | ListType BSCTCYLSPR( PointType CylPos, VectorType CylDir,

NumericType CylRadius, PointType SprCntr,

NumericType SprRadius, NumericType Size )

computes the bisector surface of a cylinder and a sphere.

The cylinder is located at CylPos, in the direction of CylDir

which also sets the length of the cylinder.  The radius of the cylinder

is CylRadius.  The sphere is centered at SptCntr and has a

radius of SprRadius.

Size controls the portion of the (infinite) bisector actually

represented.

Example:

BisectSrf = BSCTCYLSPR( point( 0, 0, 1.5 ), vector( 0, 0, 3 ), 0.2,

point( 0, 0, 0 ), 0.7, 3 );

computed the bisector surface between a cylinder of radius 0.2 along the

Z axis and a sphere at the origin with radius 0.7.

See also BSCPCONCON, BSCTCONCN2, BSCTCONCYL, BSCTCONCON, BSCTCONLN,

BSCTCONPL, BSCTCONPT, BSCTCONSPR, BSCTCYLPL, BSCTCYLPT, BSCTPLNLN,

BSCTPLNPT, BSCTSPRLN, BSCTSPRPL, BSCTSPRPT, BSCTSPRSPR, BSCTTRSPT,

BSCTTRSSPR, CBISECTOR, SBISECTOR.

BSCTPLNLN

---------

SurfaceType | ListType BSCTPLNLN( VectorType LineDir, NumericType Size )

computes the bisector surface of the XY plane (plane Z = 0) and a

line in direction LineDir.  The plane and the line are assumed to

intersect at the origin.

Size controls the portion of the (infinite) bisector actually

represented.

Example:

BisectSrf = BSCTPLNLN( vector( 0, 0, 1 ), 1 );

computes the bisector of the XY plane and the Z axis (a cone).

See also BSCPCONCON, BSCTCONCN2, BSCTCONCYL, BSCTCONCON, BSCTCONLN,

BSCTCONPL, BSCTCONPT, BSCTCONSPR, BSCTCYLPL, BSCTCYLPT, BSCTCYLSPR,

BSCTPLNPT, BSCTSPRLN, BSCTSPRPL, BSCTSPRPT, BSCTSPRSPR, BSCTTRSPT,

BSCTTRSSPR, CBISECTOR, SBISECTOR.

BSCTPLNPT

---------

SurfaceType | ListType BSCTPLNPT( PointType Pt, NumericType Size )

computes the bisector surface of the XY plane (plane Z = 0) and a point

Pt.  This surface is a paraboloid of revolution.

Size controls the portion of the (infinite) bisector actually

represented.

Example:

BisectSrf = BSCTPLNPT( point( 0, 0, 1 ), 1 );

computes the bisector surface of the XY plane and the point (0, 0, 1).

See also BSCPCONCON, BSCTCONCN2, BSCTCONCYL, BSCTCONCON, BSCTCONLN,

BSCTCONPL, BSCTCONPT, BSCTCONSPR, BSCTCYLPL, BSCTCYLPT, BSCTCYLSPR,

BSCTPLNLN, BSCTSPRLN, BSCTSPRPL, BSCTSPRPT, BSCTSPRSPR, BSCTTRSPT,

BSCTTRSSPR, CBISECTOR, SBISECTOR.

BSCTSPRLN

---------

SurfaceType | ListType BSCTSPRLN( PointType SprCntr, NumericType SprRadius,

NumericType Size )

computes the bisector surface of a sphere and the Z axis line.

The sphere is centered at SptCntr and has a radius of SprRadius.

Size controls the portion of the (infinite) bisector actually

represented.

Example:

BisectSrf = BSCTSPRLN( vector( 2, 0, 0 ), 0.7, 1 );

See also BSCPCONCON, BSCTCONCN2, BSCTCONCYL, BSCTCONCON, BSCTCONLN,

BSCTCONPL, BSCTCONPT, BSCTCONSPR, BSCTCYLPL, BSCTCYLPT, BSCTCYLSPR,

BSCTPLNPT, BSCTSPRLN, BSCTSPRPL, BSCTSPRPT, BSCTSPRSPR, BSCTTRSPT,

BSCTTRSSPR, CBISECTOR, SBISECTOR.

BSCTSPRPL

---------

SurfaceType | ListType BSCTSPRPL( PointType SprCntr, NumericType SprRadius,

NumericType Size )

computes the bisector surface of the XP plane (the plane Z = 0) and a

sphere.  This bisector surface is a paraboloid of revolution.

The sphere is centered at SptCntr and has a radius of SprRadius.

Size controls the portion of the (infinite) bisector actually

represented.

Example:

BisectSrf = BSCTSPRPL( point( 0, 0, 1.5 ), 0.7, 0.5 );

See also BSCPCONCON, BSCTCONCN2, BSCTCONCYL, BSCTCONCON, BSCTCONLN,

BSCTCONPL, BSCTCONPT, BSCTCONSPR, BSCTCYLPL, BSCTCYLPT, BSCTCYLSPR,

BSCTPLNLN, BSCTPLNPT, BSCTSPRLN, BSCTSPRPT, BSCTSPRSPR, BSCTTRSPT,

BSCTTRSSPR, CBISECTOR, SBISECTOR.

BSCTSPRPT

---------

SurfaceType | ListType BSCTSPRPT( PointType SprCntr, NumericType SprRadius

PointType Pt )

computes the bisector surface of a sphere and a point, Pt.

The sphere is centered at SptCntr and has a radius of SprRadius.

Example:

Pt = point( 0, 0, 1 );

BisectSrf = BSCTSPRPT( point( 0, 0, 0 ), 0.7, Pt );

computes the bisector of a sphere of radius 0.7 centered at the origin, and

the point (0, 0, 1).

See also BSCPCONCON, BSCTCONCN2, BSCTCONCYL, BSCTCONCON, BSCTCONLN,

BSCTCONPL, BSCTCONPT, BSCTCONSPR, BSCTCYLPL, BSCTCYLPT, BSCTCYLSPR,

BSCTPLNLN, BSCTPLNPT, BSCTSPRLN, BSCTSPRPL, BSCTSPRSPR, BSCTTRSPT,

BSCTTRSSPR, CBISECTOR, SBISECTOR.

BSCTSPRSPR

----------

SurfaceType | ListType BSCTSPRSPR( PointType Spr1Cntr, NumericType Spr1Radius

PointType Spr2Cntr, NumericType Spr2Radius )

computes the bisector surface of two spheres.

The spheres are centered at Spt1Cntr and Spt2Cntr and have a

radii of Spr1Radius and Spr2Radius, respectively.

Example:

BisectSrf = ( point( 0, 0, 0 ), 0.7, point( 1, 0, 0 ), 0.2 );

compute the bisectors of a sphere at the origin with radius 0.7, and

a sphere at (1, 0, 0) with a radius of 0.2.

See also BSCPCONCON, BSCTCONCN2, BSCTCONCYL, BSCTCONCON, BSCTCONLN,

BSCTCONPL, BSCTCONPT, BSCTCONSPR, BSCTCYLPL, BSCTCYLPT, BSCTCYLSPR,

BSCTPLNLN, BSCTPLNPT, BSCTSPRLN, BSCTSPRPL, BSCTSPRPT, BSCTTRSPT,

BSCTTRSSPR, CBISECTOR, SBISECTOR.

BSCTTRSPT

---------

SurfaceType | ListType BSCTTRSPT( PointType TrsPos, VectorType TrsDir,

NumericType TrsMjrRad, NumericType TrsMnrRad,

PointType Pt )

computes the bisector surface of a torus and a point, Pt.  The

torus is located at TrsPos, with its axis of symmetry TrsDir,

a major radius of TrsMajorRad and a minor radius pf TrsMinorRad.

Example:

BisectSrf = BSCTTRSPT( point( 0.0, 0.0, 0.0 ), vector( 0.0, 0.0, 1.0 ),

0.7, 0.7, Pt );

See also BSCPCONCON, BSCTCONCN2, BSCTCONCYL, BSCTCONCON, BSCTCONLN,

BSCTCONPL, BSCTCONPT, BSCTCONSPR, BSCTCYLPL, BSCTCYLPT, BSCTCYLSPR,

BSCTPLNLN, BSCTPLNPT, BSCTSPRLN, BSCTSPRPL, BSCTSPRPT, BSCTSPRSPR,

BSCTTRSSPR, CBISECTOR, SBISECTOR.

BSCTTRSSPR

----------

SurfaceType | ListType BSCTTRSSPR( PointType TrsPos, VectorType TrsDir,

NumericType TrsMjrRad, NumericType TrsMnrRad,

PointType SprCntr, NumericType SprRadius )

computes the bisector surface of a torus and a sphere.  The torus is

located at TrsPos, with its axis of symmetry TrsDir,

a major radius of TrsMajorRad and a minor radius pf TrsMinorRad.

The sphere is centered at SptCntr and has a radius of SprRadius.

Example:

BisectSrf = BSCTTRSSPR( point( 0.0, 0.0, 0.0 ), vector( 0.0, 0.0, 1.0 ),

0.7, 0.7, point( 0.7, 0.0, 0.0 ), 0.7);

See also BSCPCONCON, BSCTCONCN2, BSCTCONCYL, BSCTCONCON, BSCTCONLN,

BSCTCONPL, BSCTCONPT, BSCTCONSPR, BSCTCYLPL, BSCTCYLPT, BSCTCYLSPR,

BSCTPLNLN, BSCTPLNPT, BSCTSPRLN, BSCTSPRPL, BSCTSPRPT, BSCTSPRSPR,

BSCTTRSPT, CBISECTOR, SBISECTOR.

BUMPMAP

-------

PolyType BUMPMAP( SurfaceType BaseSrf, PolyType Bump,

NumericType UDup, NumericTye VDup, NumericTye LclUVs )

tiles a composition of Bump over surface BaseSrf

UDup by VDup times, creating a detailed bump geometry. Bump

can be any polygonal geometry whatsoever with XY coordinates that are

contained in the unit square [0, 1] x [0, 1], while Z serves as the elevation

above the surface.  The composed geometry could inherit the UV texture

ccordinates from the UV coordinates found in Bump if LclUVs is TRUE

or inherit BaseSrf UV coordinates if LclUVs is FALSE.

Example:

BaseTorus = torusSrf( 1, 0.2 );

BumpTorus = bumpmap( BaseTorus, BumpPolyObj, 6, 8, on );

constructs a bumpy BumpTorus with a bump tiled 6 x 8 times over the

surface.

BZR2BSP

-------

CurveType BZR2BSP( CurveType Crv )

or

SurfaceType BZR2BSP( SurfaceType Srf )

creates a B-spline curve or a B-spline surface from the given Bezier curve

or Bezier surface. The B-spline curve or surface is assigned an open end knot

vector(s) with no interior knots, in the parametric domain of zero to one.

Example:

BspSrf = BZR2BSP( BzrSrf );

BSP2BZR

-------

CurveType | ListType BSP2BZR( CurveType Crv )

or

SurfaceType | ListType BSP2BZR( SurfaceType Srf )

creates Bezier curve(s) or surface(s) from a given B-spline curve or a

B-spline surface. The B-spline input is subdivided at all internal knots to

create Bezier curves or surfaces. Therefore, if the input B-spline does have

internal knots, a list of Bezier curves or surfaces is returned. Otherwise,

a single Bezier curve or surface is returned.

Example:

BzrCirc = BSP2BZR( circle( vector( 0.0, 0.0, 0.0 ), 1.0 ) );

would subdivide the unit circle into four 90 degrees Bezier arcs returned

in a list.

CARCLEN

-------

CurveType CARCLEN( CurveType Crv, NumericType NumCtlPts, NumericType Order )

approximates an arc length curve out of the given curve Crv.  The

new approximated curve will have NumCtlPts control points and will be

of order Order.  The returned curve is not guaranteed to share the

exact same trace as the original curve Crv.

Example:

c2 = carclen( c, sizeof( c ) * 6, 3 );

approximates c as a quadratic arc length curve c2 with 6 times the

number of control points.

CAREA

-----

CurveType CAREA( CurveType Crv )

computes the integral area curve, ACrv, of the given curve Crv,

up to a sign.  If Crv is a closed curve with domain t0 to t1, then the

difference of ACrv(t1) - ACrv(t0) is the requested area.

Example:

Crv = pcircle( vector( 0, 0, 0 ), 1 );

ACrv = CAREA( Crv );

Pi = abs( coord( ceval( ACrv, 4 ), 1 ) - coord( ceval( ACrv, 0 ), 1 ) );

is yet another way of approximating the value of Pi.

See also SMOMENTS and SVOLUME.

CBEZIER

-------

CurveType CBEZIER( ListType CtlPtList )

creates a Bezier curve out of the provided control point list. CtlPtList

is a list of control points, all of which must be of type (E1-E9 P1-P9),

or regular PointType defining the curve's control polygon. The curve's point

type will be of a space which is the union of the spaces of all points.

Example:

s45 = sin(pi / 4);

Arc90 = CBEZIER( list( ctlpt( P2, 1.0, 0.0, 1.0 ),

ctlpt( P2, s45, s45, s45 ),

ctlpt( P1, 1.0, 1.0 ) ) );

constructs an arc of 90 degrees as a rational quadratic Bezier curve.

See also CBSPLINE, CPOWER and SBEZIER.

CBIARC

------

ListType CBIARC( CurveType Crv, NumericType Tol, NumericType MaxAngle )

computes bi-arc fitting to a given curve Crv, with a tolarence

Tol in L-infinity sense, and a maximum angular span of each arc of

at most MaxAngle degrees.  Returned is a list of arcs as rational

Bezier curves with an arc "center" point attribute to ease the reconstruction

of the analytic representation of the geometry.

Example:

C1 = cbspline( 4,

list( ctlpt( E3, -0.287, -0.286, 0 ),

ctlpt( E2, 0.0272, -0.425 ),

ctlpt( E2, 0.265, -0.0839 ),

ctlpt( E2, 0.607, -0.165 ),

ctlpt( E2, 0.832, -0.205 ),

ctlpt( E2, 0.737, 0.042 ),

ctlpt( E2, 0.357, 0.103 ),

ctlpt( E2, 0.508, 0.298 ),

ctlpt( E2, 0.814, 0.649 ),

ctlpt( E2, 0.692, 0.775 ),

ctlpt( E2, 0.411, 0.391 ),

ctlpt( E2, 0.301, 0.315 ),

ctlpt( E2, 0.625, 0.945 ),

ctlpt( E2, 0.49, 1.03 ),

ctlpt( E2, 0.369, 0.829 ),

ctlpt( E2, 0.185, 0.384 ),

ctlpt( E2, 0.194, 0.518 ),

ctlpt( E2, 0.243, 1.09 ),

ctlpt( E2, 0.0653, 1.13 ),

ctlpt( E2, 0.0644, 0.381 ),

ctlpt( E2, 0.00925, 0.496 ),

ctlpt( E2, -0.0113, 0.943 ),

ctlpt( E2, -0.202, 0.954 ),

ctlpt( E2, -0.147, 0.644 ),

ctlpt( E2, -0.162, 0.208 ),

ctlpt( E2, -0.337, -0.156 ) ),

list( kv_periodic ) );

C1 = coerce( C1, kv_open );

Arcs = CBIARCS( Crv, 0.01, 90 );

computes bi-arcs fitting to a given curve in the shape of a human hand,

with arcs with at most 90 degrees and tolerance of 0.01.

CBISECTOR

---------

{ CurveType | SurfaceType } CBISECTOR( CurveType Crv,

NumericType UseNrmlTan,

NumericType Tolerance,

NumericType NumerImprove,

NumericType SameNormal )

or

{ CurveType | SurfaceType } CBISECTOR( ListType TwoCrvs,

NumericType UseNrmlTan,

NumericType Tolerance,

NumericType NumerImprove,

NumericType SameNormal )

or

SurfaceType CBISECTOR( ListType CrvPt,

NumericType UseNrmlTan,

NumericType Tolerance,

NumericType NumerImprove,

NumericType SameNormal )

computes the self bisector curve(s) for Crv or the bisector(s)

of TwoCrvs or the bisector of a curve and a point, CrvPt. If

UseNrmlTan is equal to zero or one, the bisector curves are computed

using angular equality of tangents or normals of the given curve or curves,

respectively. Tolerance controls the accuracy of the computation,

with 10 as a good starting value. If Tolerance is negative,

NumerImprove can be either TRUE or FALSE, allowing or disabling a

final numerical improvement stage.  SameNormal can also assume a

TRUE or FALSE value, selecting only opposite facing normals, if TRUE.

Nevertheless, if UseNrmlTan equals -1, the bisector surface of

the given two curves TwoCrvs or a curve and a point CrvPt is

computed analytically.  For two 3-space curves this is indeed the bisector

surface while for two 2-space curves, the zero set of the returned surface

provides the bisector curve.  If UseNrmlTan equals -1, then

NumerImprove, and SameNormal are all ignored. Tolerance

is expected to be 1/2 for a proper bisector.  If Tolerance varies

between zero and one, an alpha-sector is created where for Tolerance

equal zero the created surface will contain the first curve, whereas for

Tolerance equal one, the created surface will contain the second curve.

If UseNrmlTan equals -2 and the two curves are E2, a different

bisector surface is computed whose zero set provides the bisector curve.

If UseNrmlTan equals -2, then Tolerance, NumerImprove,

and SameNormal are all ignored.

If UseNrmlTan equals -3 and the two curves are 3-space curves,

an alpha bisector surface is computed, whose ratio is set via Tolerance

relative to NumerImprove, such that the sum of Tolerance +

NumerImprove is not zero. If UseNrmlTan equals -3, then

SameNormal is ignored. See also SBISECTOR.

For a bisector between a curve and a point, the value of SameNormal

controls the computation of the bisector surface in R3 (SameNormal = FALSE)

or the computation of the planar bisector curve of planar curve and a point,

all in the XY plane (SameNormal = TRUE).

Example:

c1 = cbezier( list( ctlpt( E2, -0.5, -0.2 ),

ctlpt( E2,  0.0, -0.2 ),

ctlpt( E2,  0.6,  0.6 ) ) );

c2 = cbezier( list( ctlpt( E2,  0.3, -0.7 ),

ctlpt( E2, -0.2, -0.7 ),

ctlpt( E2,  0.7,  0.6 ) ) );

BisectCrvs = CBISECTOR( list( c1, c2 ), 0, 12, true, false );

All = list( c1, c2, BisectCrvs );

interact( list( All, view_mat2d ) );

c1 = creparam( pcircle( vector( 0.0, 0.0, 0.0 ), 1 ), 0, 1 );

c2 = cbezier( list( ctlpt( E3, -1.0, 0.0,  1.0 ),

ctlpt( E3,  1.0, 0.0, -1.0 ) ) );

BisectSrf = CBISECTOR( list( c1, c2 ), -1, -1, true, false );

interact( list( c1, c2, BisectSrf ) );

computes two bisectors, one for planar curves as a set of bisector curves,

and the other as a bisector surface of a Z parallel line and a circle in the

XY plane.

CBSPLINE

--------

CurveType CBSPLINE( NumericType Order, ListType CtlPtList,

ListType KnotVector )

creates a B-spline curve out of the provided control point list, the

knot vector, and the specified order. CtlPtList is a list of control

points, all of which must be of type (E1-E9 P1-P9, or regular PointType

defining the curve's control polygon. The curve's point type will be of a

space which is the union of the spaces of all points.

The length of the KnotVector must be equal to the number of control

points in CtlPtList plus the Order. If, however, the length of

the knot vector is equal to #CtlPtList + Order + Order - 1, the curve

is assumed to be periodic.

The knot vector list may be specified as either list( KV_OPEN ),

list( KV_FLOAT ) or list( KV_PERIODIC ) in which a uniform

open, uniform floating or uniform periodic knot vector with the appropriate

length is automatically constructed.

Example:

s45 = sin(pi / 4);

HalfCirc = CBSPLINE( 3,

list( ctlpt( P3,  1.0,  1.0,  0.0, 0.0 ),

ctlpt( P3,  s45,  s45,  s45, 0.0 ),

ctlpt( P3,  1.0,  0.0,  1.0, 0.0 ),

ctlpt( P3,  s45, -s45,  s45, 0.0 ),

ctlpt( P3,  1.0, -1.0,  0.0, 0.0 ) ),

list( 0, 0, 0, 1, 1, 2, 2, 2 ) );

constructs an arc of 180 degrees in the XZ plane as a rational quadratic

B-spline curve.

Example:

c = CBSPLINE( 4,

list( ctlpt( E2,  0.5,  0.5 ),

ctlpt( E2, -0.5,  0.5 ),

ctlpt( E2, -0.5, -0.5 ),

ctlpt( E2,  0.5, -0.5 ) ),

list( KV_PERIODIC ) );

color( c, red );

viewobj( c );

c1 = cregion( c, 3, 4 );

color( c1, green );

c2 = cregion( c, 4, 5 );

color( c2, yellow );

c3 = cregion( c, 5, 6 );

color( c3, cyan );

c4 = cregion( c, 6, 7 );

color( c3, magenta );

viewobj( list( c1, c2, c3, c4 ) );

creates a periodic curve and extracts its four polynomial domains as four

open end B-spline curves.

See also CBEZIER, CPOWER and SBSPLINE.

CCINTER

-------

ListType CCINTER( CurveType Crv1, CurveType Crv2, NumericType Epsilon,

NumericType SelfInter )

or

SurfaceType CCINTER( CurveType Crv1, CurveType Crv2, NumericType Epsilon,

NumericType SelfInter )

compute the intersection point(s) of Crv1 and Crv2 in the

XY plane. Since this computation involves numeric operations, Epsilon

controls the accuracy of the parametric values of the result.

It returns a list of PointTypes, each containing the parameter of Crv1

in the X coordinate, and the parameter of Crv2 in the Y coordinate.

If, however, Epsilon is negative, a scalar field surface representing

the square of the distance function is returned instead.

If SelfInter is TRUE, Crv1 and Crv2 can be the same

curve, and self intersection points are searched for instead.

Example:

crv1 = cbspline( 3,

list( ctlpt( E2, 0, 0 ),

ctlpt( E2, 0, 0.5 ),

ctlpt( E2, 0.5, 0.7 ),

ctlpt( E2, 1, 1 ) ),

list( KV_OPEN ) );

crv2 = cbspline( 3,

list( ctlpt( E2, 1, 0 ),

ctlpt( E2, 0.7, 0.25 ),

ctlpt( E2, 0.3, 0.5 ),

ctlpt( E2, 0, 1 ) ),

list( KV_OPEN ) );

inter_pts = CCINTER( crv1, crv2, 0.0001, FALSE );

computes the parameter values of the intersection point of crv1 and

crv2 to a tolerance of 0.0001.

CCRVTR

------

NumericType CCRVTR( CurveType Crv, NumericType Epsilon )

or

CurveType CCRVTR( CurveType Crv, NumericType Epsilon )

compute the extreme curvature points on Crv in the XY plane.

This set includes not only points of maximum (convexity) and mimumum

(concavity) curvature, but also points of zero curvature such as

inflection points.

Since this operation is partially numeric, Epsilon is used to set

the needed accuracy. It returns the parameter value(s) of the location(s)

with extreme curvature along the Crv.

If, however, Epsilon is negative, the curvature scalar field

curve is returned as a two-dimensional rational vector field curve, for

which the first dimension is equal to the parameter, and the second is the

curvature value at that parameter.

This function computes the curvature scalar field for planar curves as,

x' y'' - x'' y'

k(t) = ----------------

2     2  3/2

( x'  + y'  )

and computes kN for three-dimensional curves as the following vector field,

C' x C''     C'    (C' x C'') x C'

k(t) N(t) = K(t) B(t) x T(t) = -------- x ----- = ---------------

3    | C' |            4

| C'|                 | C' |

The extremum values are extracted from the computed curvature field.

This curvature field is a high order curve, even if the input geometry is

of low order. This is especially true for rational curves, for which the

quotient rule for differentiation is used and almost doubles the degree

in every differentiation.

See also CZEROS, CEXTREMES, and SCRVTR.

Example:

crv = cbezier( list( ctlpt( E2, -1.0,  0.5 ),

ctlpt( E2, -0.5, -2.0 ),

ctlpt( E2,  0.0,  1.0 ),

ctlpt( E2,  1.0,  0.0 ) ) ) * rotz( 30 );

crvtr = CCRVTR( crv, 0.001 );

pt_crvtr = nil();

pt = nil();

for ( i = 1, 1, sizeof( crvtr ),

( pt = ceval( crv, nth( crvtr, i ) ) ):

snoc( pt, pt_crvtr )

);

interact( list( crv, pt_crvtr ) );

finds the extreme curvature points in Crv and displays them all

with the curve.

CDERIVE

-------

CurveType CDERIVE( CurveType Curve )

returns a vector field curve representing the differentiated curve,

also known as the Hodograph curve.

Example:

Circ = circle( vector( 0.0, 0.0, 0.0 ), 1.0 );

Hodograph = CDERIVE( Circ );

See also CINTEG, SDERIVE, TDERIVE, and MDERIVE

CDIVIDE

-------

ListType CDIVIDE( CurveType Curve, NumericType Param )

subdivides a curve into two sub-curves at the specified parameter value.

Curve can be either a B-spline curve in which Param must be

within the Curve's parametric domain, or a Bezier curve in which Param

can be arbitrary, extrapolating if not in the range of zero to one.

It returns a list of the two sub-curves. The individual curves may be

extracted from the list using the NTH command.

Example:

CrvLst = CDIVIDE( Crv, 1.3 );

Crv1 = nth( CrvLst, 1 );

Crv2 = nth( CrvLst, 2 );

subdivides the curve Crv at the parameter value of 0.5.

See also SDIVIDE, TDIVIDE, and MDIVIDE

CEDITPT

-------

CurveType CEDITPT( CurveType Curve, CtlPtType CtlPt, NumericType Index )

provides a simple mechanism to manually modify a single control point number

Index (base count is 0) in the Curve, by substituting CtlPt

instead. CtlPt must have the same point type as the control points of

the Curve. The original curve Curve is not modified.

Example:

CPt = ctlpt( E3, 1, 2, 3 );

NewCrv = CEDITPT( Curve, CPt, 1 );

constructs a NewCrv with the second control point of Curve being

CPt.

CENVOFF

-------

SurfaceType CENVOFF( CurveType Curve,

NumericType Height, NumericType Tolerance )

or

ListType CENVOFF( CurveType Curve, NumericType Height, NumericType Tolerance )

computes the offset envelope of a given planar curve Curve.  The

offset envelope is the envelope of cones with apex on point on Curve

in the Z direction. Height is the height of the cone which also

equals the offset distance or the width of the cones.  Tolerance

controls the accuracy of the offset approximation.

If the Curve is closed, two surfaces are created in the offset envelope,

one for the inside and another for the outside.  If Curve is open, a single

envelope offset surface is computed, wrapping around both sides.

Example:

c1 = cbezier( list( ctlpt( E2, -0.8, 0.0 ),

ctlpt( E2, -0.2, 1.0 ),

ctlpt( E2,  0.2, 0.0 ),

ctlpt( E2,  0.8, 0.6 ) ) );

s1 = CENVOFF( c1, 0.5, 0.01 );

computes an envelope offset surface for a cubic Bezier curve c1 of

Height of 0.5 and Tolerance of 0.01.

CEVAL

-----

CtlPtType CEVAL( CurveType Curve, NumericType Param )

evaluates the provided Curve at the given Param value.

Param should be in the curve's parametric domain if the Curve is

a B-spline curve, or between zero and one if the Curve is a Bezier curve.

The returned control point has the same point type as the control points

of the Curve.

Example:

CPt = CEVAL( Crv, 0.25 );

evaluates Crv at the parameter value of 0.25.

See also SEVAL, MEVAL, TEVAL.

CEXTREMES

---------

ListType CEXTREMES( CurveType Crv, NumericType Epsilon, NumericType Axis )

computes the extreme set of the given Crv in the given axis (1 for X,

2 for Y, 3 for Z). Since this computation is numeric, an Epsilon is

also required to specify the desired tolerance. It returns a list of

all the parameter values (NumericType) in which the curve takes an extreme

value.

Example:

extremes = CEXTREMES( Crv, 0.0001, 1 );

computes the extreme set of curve crv, in the X axis, with

error tolerance of 0.0001. See also CZERO.

CINFLECT

--------

ListType CINFLECT( CurveType Crv, NumericType Epsilon )

or

CurveType CINFLECT( CurveType Crv, NumericType Epsilon )

computes the inflection points of Crv in the XY plane.

Since this computation is numeric, an Epsilon is also required

to specify the desired tolerance. It returns a list of all the

parameter values (NumericType) in which the curve has an inflection point.

If, however, Epsilon is negative, a scalar field curve representing

the sign of the curvature of the curve is returned instead.

The sign of curvature scalar field is equal to

s(t) = x' y'' - x'' y'

Example:

inflect = CINFLECT( crv, 0.001 );

pt_inflect = nil();

pt = nil();

for ( i = 1, 1, sizeof( inflect ),

pt = ceval( crv, nth( inflect, i ) ):

snoc( pt, pt_inflect )

);

interact( list( axes, crv, pt_inflect ), 0);

computes the set of inflection points of curve crv with error

tolerance of 0.001. This set is then scanned in a loop and

evaluated to the curve's locations which are then displayed with the crv.

See also CZEROS, CEXTREMES, and CCRVTR.

CINTEG

------

CurveType CINTEG( CurveType Crv );

returns a vector field curve representing the integral curve.

See also CDERIVE.

CINTERP

-------

CurveType CINTERP( ListType PtList, NumericType Order, NumericType Size,

ConstantType Param, NumericType Periodic)

or

CurveType CINTERP( CurveType Crv, NumericType Order, NumericType Size,

ConstantType Param, NumericType Periodic)

or

CurveType CINTERP( ListType PtList, NumericType Order, NumericType Size,

ListType Params, NumericType Periodic)

or

CurveType CINTERP( CurveType Crv, NumericType Order, NumericType Size,

ListType Params, NumericType Periodic)

compute a B-spline curve that interpolates or approximates

the list of points in PtList or a given curve Crv. The

B-spline curve will have order Order and Size control points,

and will be periodic if periodic is non zero.  The knots will be

spaced according to Param which can be one of PARAM_UNIFORM,

PARAM_CHORD, PARAM_CENTRIP or PARAM_NEILFOL or lists of parameter

values and knots (see below).  The PARAM_UNIFORM prescribes a uniform

knot sequence, PARAM_CHORD specifies knot spacing according to the chord

length and PARAM_CENTRIP according to the square root of the chord length.

Finally, PARAM_NEILFOL takes into consideration the angles between three

consecutive points.

A periodic curve will be coerced to have a PARAM_UNIFORM knot sequence.

If Params is a list object, it should contain two lists of

numerical values.  The first list contains the parameter values at which

to approximate or interpolate the data points. Hence, the length of this

list must equal the length of the PtList data.  The second list

specifies the knot vector of the construct B-spline curve.

Use of Periodic end conditions can create cases with degenerated

linear systems (determinant equal zero).  Increase or decrease of the

Order of the B-spline by one will resolve the problem.

All points in PtList must be of type (E1-E9, P1-P9) control point,

or regular PointType. If Size is equal to the number of points in

PtList, the resulting curve will interpolate the data set.

Otherwise, if Size is less than the number of points in PtList,

the point data set will be least square approximated. At no time can

Size be lower than Order. Size of zero forces

interpolation by selecting Size to be the size of the data set.

All interior knots will be distinct, preserving maximal continuity.

The resulting B-spline curve will have open end conditions.

Example:

pl = nil();

for ( x = 0, 1, 100,

snoc(point(cos(x / 5), sin(x / 5), x / 50 - 1), pl)

);

c = CINTERP( pl, 3, 21, PARAM_UNIFORM, false );

samples a helical curve at 100 points and least square fit of a quadratic

B-spline curve with 21 points to the data set. The curve will have a uniform

knot spacing and is not periodic.  See also SINTERP, TINTERP and LINTERP.

CIRCLE

------

CurveType CIRCLE( VectorType Center, NumericType Radius )

constructs a circle at the specified Center with the specified

Radius. The returned circle is a B-spline curve of four piecewise Bezier

90 degree arcs. The construced circle is always parallel to the XY plane.

Use the linear transformation routines to place the circle in the appropriate

orientation and location.

CIRCPOLY

--------

PolygonType CIRCPOLY( VectorType Normal, VectorType Trans, NumericType Radius )

defines a circular polygon in a plane perpendicular to Normal that

contains the Trans point. The constructed polygon is centered at

Trans. RESOLUTION vertices will be defined with Radius from

distance from Trans.

Alternative ways to construct a polygon are manual construction of the

vertices using POLY, or the construction of a flat ruled surface using

RULEDSRF.

CLNTCRSR

--------

ListType CLNTCRSR( NumericType TimeOut )

reads the mouse coordinates as well as mouse events from displace devices,

or times out after TimeOut miliseconds. A list object

of two sub-objects, a points and a vector, named "_PickCrsr_" is returned.

These point and vector define the three-dimensional line of the mouse in

object space.

Mouse events are typically processed by the display device.  However, by

the command "CLNTPICKCRSR" (in iritinit.irt) which sends a "PICKCRSR"

request to the display devices, mouse events will be sent to the server.

The server can be requested to keep mouse events for "CLNTCRSR" to be read

via the IritState command and the "CursorKeep" attribute.

Both the point and the vector will have a numeric attribute of "EventType"

that will have the following meaning:

1   Mouse motion event

2   Mouse down event

5   Mouse up event

In case of a time out the returned list object will be empty and will

have the name "_PickFail_".

Example:

ClntPickCrsr( clients_all );

IritState( "CursorKeep", 1 );

Quit = 0;

for ( i = 0, 1, 10,

CLNTCRSR( 10000 ) );

ClntPickDone( clients_all );

IritState( "CursorKeep", 0 );

asks all clients to send mouse events to the server, asks the server to keep

mouse events, and then reads 10 mouse events.

CLNTREAD

--------

AnyType CLNTREAD( NumericType Handler, NumericType Block )

reads one object from a client communication channel.

Handler contains the index of the communication channel opened via

CLNTEXEC. If no data is available in the communication channel, this

function will block for at most Block milliseconds until data is found

or timeout occurs. In the latter, a single StringType object is returned

with the content of "no data (timeout)".  If Handler equals -1, the

regular display device (forked via, for example, VIEWOBJ command) is used.

See also VIEWSET, CLNTWRITE, CLNTCLOSE, and CLNTEXEC.

Example:

h2 = clntexec( "xmtdrvs -s-" );

.

.

Model = CLNTREAD( h2 );

.

.

clntclose( h2,TRUE );

reads one object from client through communication channel h2 and saves it

in variable model.

CMESH

-----

CurveType CMESH( SurfaceType Srf, ConstantType Direction, NumericType Index )

returns a single ROW or COLumn as specified by the Direction and

Index (base count is 0) of the control mesh of surface Srf.

The returned curve will have the same knot vector as Srf in the

appropriate direction. See also CSURFACE.

This curve is not necessarily in the surface Srf.

Example:

Crv = CMESH( Srf, COL, 0 );

extracts the first column of surface Srf as a curve. See also

CSURFACE.  See also SMESH, MFROMMESH.

CMOEBIUS

--------

CurveType CMOEBIUS( CurveType Crv, NumericType Ratio )

rebalances the weights of a rational curve using the Moebius transformation.

The shape of the curve remains identical while the speed is modified.

Ratio controls the ratio between the last and the first weights. If

Ratio = 0, the first and last weights are made equal.

See also SMOEBIUS.

CMORPH

------

CurveType CMORPH( CurveType Crv1, CurveType Crv2,

NumericType Method, NumericType Blend )

or

ListType CMORPH( CurveType Crv1, CurveType Crv2,

NumericType Method, NumericType Blend )

create a new curve which is a metamorph of the two given curves.

The two given curves must be compatible (see FFCOMPAT) before this blend

is invoked. This is very useful if a sequence that "morphs" one curve to

another is to be created. Several metamorphosis methods are supported

according to the value of Method,

0   Simple convex blend.

1   Corner/Edge cutting scheme, scaled to same curve length.

2   Corner/Edge cutting scheme, scaled to same bounding box.

3   Same as 1 but with filtering out of tangencies.

4   Same as 2 but with filtering out of tangencies.

5   Multiresolution decomposition based metamorphosis. See CMULTRES.

In Method 1, Blend is a number between zero (Crv1)

and one (Crv2) defining the similarity to Crv1 and Crv2,

respectively. A single curve is returned.

In Methods 2 to 5, Blend is a step size for the

metamorphosis operation and a whole list describing the entire

metamorphosis operation is returned.

Examples:

for ( i = 0, 1, 300,

c = CMORPH( crv1a, crv1b, 0, i / 300.0 ):

color( c, yellow ):

viewobj( c )

);

crvs = CMORPH( crv1a, crv1b, 2, 0.003 );

snoc( crv1b, crvs );

for ( i = 1, 1, sizeof( crvs ),

c = nth( crvs, i ):

color( c, yellow ):

viewobj( c )

);

Turtle2 = ffmatch( Wolf, Turtle, 20, 100, 2, false, 2 );

ffcompat( Wolf, Turtle2 );

for ( i = 0, 1, 25,

c = CMORPH( Wolf, Turtle2, 0, i / 25 ):

color( c, yellow ):

viewobj( c )

);

creates three metamorphosis animation sequences, one that is based on

a convex blend, and two that are based on corner/edge cutting schemes.

See also PMORPH, SMORPH, TMORPH, and FFMATCH.

CMULTIRES

---------

ListType CMULTIRES( CurveType Crv, NumericType Discont,

NumericType LeastSquares )

computes a multiresolution decomposition of curve Crv using least

squares approximation, if LeastSquares is TRUE, or using B-Wavelets

if LeastSquares is FALSE.  The latter is optimal but slower.

The resulting list of curves describes an hierarchy of curves in linear

subspaces of the space in which Crv lay.

If LeastSquares is TRUE, the curves could be summed algebraically to

form Crv.  Each of the curves in the hierarchy is a least squares

approximation of Crv in the subspace in which it is defined.

If LeastSquares is FALSE, a list of orthogonal

projections of the Crv onto the prescibed subspaces (by the knot

sequences) is provided.

Finally, Discont is a Boolean flag that controls the way tangent

discontinuities are computed throughout the decomposition.

Example:

MRCrv = CMULTIRES( Animal, false, true );

sum = nth( MRCrv, 1 );

MRCrvs = list( sum * tx( 3.0 ) );

for ( ( i = 2 ), 1, sizeof( MRCrv ),

sum = symbsum( sum, nth( MRCrv, i ) ):

snoc( sum * tx( ( 3 - i ) * 1.5 ), MRCrvs )

);

All = MRCrvs * sc ( 0.25 );

view( All, on );

computes a multiresolution decomposition to curve CrossSec as

MRCrv and displays all the decomposition levels by summing them all

up. The use of none as on object name allows one to display an

object in the display device without replacing the previous object in the

display device, carrying the same name.

This creates two metamorphosis animation sequences, one based on

a convex blend and one based on a corner/edge cutting scheme.

CNORMAL

-------

VectorType CNORMAL( CurveType Crv, NumericType TParam )

computes the normal vector to curve Crv at the

parameter values TParam. The returned vector has a

unit length.

Example:

Normal = CNORMAL( Crv, 0.5 );

computes the normal to Crv at the parameter value 0.5.

See also CNRMLCRV.

CNRMLCRV

--------

CurveType CNRMLCRV( CurveType Crv )

symbolically computes a vector field curve representing the non-normalized

normals of the given curve. That is, a normal vector filed, evaluated at

t, provides a vector in the direction of the normal of the original

curve at t. The normal curve once computed is in fact equal to kN where

k is the curvature of Crv and N is its normal.

Example:

NrmlCrv = CNRMLCRV( Crv );

CNVXHULL

--------

PolygonType CNVXHULL( PolygonType Poly | PolylineType Poly,

NumericType FineNess );

or

CurveType CNVXHULL( CurveType Crv, NumericType FineNess );

compute the convex hull of the given set of Poly or Curve

with tolerance for curves only, which is governed by the polygon subdivision

accuracy as set via FineNess.  FineNess is ignored for polylines.

For curves, the result might be partial if the curve is not closed or

periodic.  See also CRV2TANS and CRVPTTAN.

Example:

Pts1 = nil();

Pts2 = nil();

for ( i = 0, 1, 7,

R = 0.2 + fmod( i, 2 ) / 2:

Pt = ctlpt( E2, R * cos( i * 2 * pi / 8 ), R * sin( i * 2 * pi / 8 ) ):

snoc( Pt, Pts1 ):

snoc( coerce( Pt, point_type ),  Pts2 ) );

Crv = coerce( cbspline( 4, Pts1, list( KV_PERIODIC ) ), KV_OPEN );

CHPts = CNVXHULL( poly( Pts2, 0 ), 0 );

CHCrv = CNVXHULL( Crv, 10 );

computes the convex hull of a given control polygon and freeform curve.

COERCE

------

AnyType COERCE( AnyType Object, ConstantType NewType )

provides a coercion mechanism between different objects or object types.

PointType, VectorType, PlaneType, and CtlPtType can be all coerced to each

other by using the NewType of POINT_TYPE, VECTOR_TYPE, PLANE_TYPE,

or one of E1-E9, P1-P9 (CtlPtType). Similarly, CurveType, SurfaceType,

TrimSrfType, TriSrfType, TrivarType, and MultivarType can all be coerced to

hold different CtlPtType control points, or even different open end

conditions from KV_PERIODIC to KV_FLOAT to KV_OPEN.  Freefroms can be

coerced to a Power, Bezier or a B-spline type via the NewType of

POWER_TYPE, BEZIER_TYPE or the BSPLINE_TYPE.

If a scalar (E1 or P1) curve is coerced to E2 or P2 curve or

a scalar (E1 or P1), the surface is coerced to an E3 or P3 surface, and the

Y (YZ) coordinate(s) is (are) updated to hold the parametric domain of the

curve (surface).  That is X = U (Y = V).

Curves, Surfaces, and Trivariates can be coerced to/from Multivariates

using the CURVE_TYPE, SURFACE_TYPE, TRIVAR_TYPE and MULTIVAR_TYPE.

Example:

CrvE2 = COERCE( Crv, E2 );

MultiVar == COERCE( COERCE( MultiVar, surface_type ), multivar_type );

BzrSrfs = COERCE( BspSrf, bezier_type );

coerces Crv to a new curve that will have an E2 CtlPtType control

points. Coerction of a projective curve (P1-P9) to a Euclidean curve

(E1-E9) does not preseve the shape of the curve.  The second example

coerces a bivariate MultiVar into a Srf and back and compares

the result to the original multivariate MultiVar...  The third

example coerces a B-spline surface BspSrf to a Bezier form, returning

one or more Bezier surfaces representing the same geometry as BspSrf.

COMPOSE

-------

CurveType COMPOSE( CurveType Crv1, CurveType Crv2 )

or

CurveType COMPOSE( SurfaceType Srf, CurveType Crv )

symbolically compute the composition curve Crv1(Crv2(t)) or

Srf(Crv(t)). In the above first form of Crv1(Crv2(t), Crv1

can be any curve while Crv2 is assumed to be a one-dimensional curve

that is either E1 or P1 (higher dimensions are ignored).

In the above second form of Srf(Crv(t)), the Srf can be any surface,

while the Crv is assumed to be a two-dimensional curve, that is either E2

or P2 (higher dimensions are ignored). Both Crv2 in the curve's composition,

and Crv is the surface's composition must be contained in the curve or

surface parametric domain.

Example:

srf = sbezier( list( list( ctlpt( E3, 0.0, 0.0, 0.0 ),

ctlpt( E3, 0.0, 0.5, 1.0 ),

ctlpt( E3, 0.0, 1.0, 0.0 ) ),

list( ctlpt( E3, 0.5, 0.0, 1.0 ),

ctlpt( E3, 0.5, 0.5, 0.0 ),

ctlpt( E3, 0.5, 1.0, 1.0 ) ),

list( ctlpt( E3, 1.0, 0.0, 1.0 ),

ctlpt( E3, 1.0, 0.5, 0.0 ),

ctlpt( E3, 1.0, 1.0, 1.0 ) ) ) );

crv = circle( vector( 0.5, 0.5, 0.0 ), 0.4 );

comp_crv = COMPOSE( srf, crv );

composes a circle Crv to be on the surface Srf.

CON2

----

PolygonType CON2( VectorType Center, VectorType Direction,

NumericType Radius1, NumericType Radius2,

NumericType Caps )

creates a truncated CONE geometric object, defined by Center as the

center of the main base of the CONE, with the Direction as both the CONE's

axis and the length of CONE, and the two radii Radius1/2 of the two bases

of the CONE. If Caps equals zero, no caps are created.  If Caps equal

one (two), only the bottom (top) cap is created.  If Caps equal three,

both the top and the bottom caps are created.

Unlike the regular cone (CONE) constructor which inherits

discontinuities in its generated normals at the apex, CON2 can be used to

form a (truncated) cone with continuous normals.

See RESOLUTION for the accuracy of the CON2 approximation as a polygonal

model. See also CONE. See IRITSTATE's "PrimRatSrfs" and "PrimRatSrfs"

state variables.

Example:

Cone2 = CON2( vector( 0, 0, -1 ), vector( 0, 0, 4 ), 2, 1, 3 );

constructs a truncated cone with bases parallel to the XY plane at

Z = -1 and Z = 3, and with radii of 2 and 1 respectively. Both

caps are created.

CONE

----

PolygonType CONE( VectorType Center, VectorType Direction,

NumericType Radius, NumericType Caps )

creates a CONE geometric object, defined by Center as the center of

the base of the CONE, Direction as the CONE's axis and height, and

Radius as the radius of the base of the CONE.  If Caps equals

zero, no cap is created.  If Caps equal one, the bottom

(top) cap is created.

See RESOLUTION for accuracy of the CONE approximation as a polygonal model.

Example:

Cone1 = CONE( vector( 0, 0, 0 ), vector( 1, 1, 1 ), 1, 1 );

constructs a cone based in an XY parallel plane, centered at the origin

with radius 1 and with tilted apex at ( 1, 1, 1 ).  Only the bottom cap is

created.

See IRITSTATE's "PrimRatSrfs" and "PrimRatSrfs" state variables.

See also CON2.

CONICSEC

--------

CurveType CONICSEC( ListType ABCDEF,

NumericType ZLevel,

PointType StartPoint,

PointType EndPoint )

or

{PolygonType | SurfaceType } CONICSEC( ListType TwoCurves,

NumericType ZLevel,

NumericType Dist,

NumericType EvalCurve )

In the first form, this will construct a quadratic form that represents the

planar conic section prescribed by the list of six coefficients ABCDEF as:

A x^2 + B xy + C y^2 + D x + E y + F = 0,

The conic will be parallel to the XY plane at Z level of Zlevel.

The section of the curve will be from StartPoint to EndPoint,

or alternatively, unlimited by specifying 'off' for StartPoint and

EndPoint.  The conic might be rational (for circles and ellipses, for

example) or intergral (for parabolas).

Alternatively, in the second form, if the first list object

parameter contains two planar curves in the XY plane, a piecewise linear

curve at Z level of Zlevel is computed that presents the

elliptic/hyperbolic distance Dist from the given two curves.

The elliptic distance refers to the sum of distance (that equal

Dist) to the two curves, while hyperbolic distance refers

to the difference of distances.  Finally, if EvalCurve = 0, the

surface whose zero set is the desired curve, is returned instead. If

EvalCurve = 1, distance curves are returned in the parametric

space as correspondence between the two curves' parameters. If

EvalCurve = 2, the conic curves themselves are returned.

Example:

Circ2 = CONICSEC( list( 1, 0, 1, 0, -0.5, -1 ), 0.0,

point(  1.0, 0.0, 0.0 ) * ty( 0.25 ),

point( -0.707, -0.707, 0.0 ) * ty( 0.25 ) );

Elp1 = CONICSEC( list( 1, 2, 4, 0.5, 2, -0.2 ), 0.0, off, off );

Prb1 = CONICSEC( list( 0.1, 0, 0, 0, 1, -1 ), 2, off, off ) * sc( 0.1 );

constructs a (portion of a) circle, an ellipse and a parabola as conic

sections, and,

c1 = cbspline( 3,

list( ctlpt( E2, -1, -1 ),

ctlpt( E2,  1, -1 ),

ctlpt( E2,  1,  1 ),

ctlpt( E2, -1,  1 ) ),

list( KV_OPEN ) );

c2 = -c1 * sx( -1 ) * tx( 5 );

view( list( c1, c2 ), 1 );

resolution = 15;

DistCrvE = list( CONICSEC( list( c1, c2 ), 1.0, 10, 2 ),

CONICSEC( list( c1, c2 ), 1.0,  9, 2 ),

CONICSEC( list( c1, c2 ), 1.0,  8, 2 ),

CONICSEC( list( c1, c2 ), 1.0,  7, 2 ),

CONICSEC( list( c1, c2 ), 1.0,  6, 2 ) );

color( DistCrvE, green );

computes the conic distance to the two curves c1 and c2 at distances

of 6, 7, 8, 9, and 10.

CONTOUR

-------

PolygonType CONTOUR( SurfaceType ContouredSrf, PlaneType ContourPlane )

or

PolygonType CONTOUR( SurfaceType ContouredSrf, PlaneType ContourPlane,

SurfaceType MappedSrf )

or

PolygonType CONTOUR( SurfaceType ContouredSrf, PlaneType ContourPlane,

SurfaceType MappedSrf, ListType ValidatePt )

contours surface ContouredSrf by intersecting plane ContourPlane

with a polygonal approximating ContouredSrf with resolution set

via variable RESOLUTION (10 is a good starting resolution value, 20 is

extreme).

If ContouredSrf is a scalar field surface of type E1 or P1 and

MappedSrf is provided, ContouredSrf is contoured above its

parametric domain (U is X, V is Y) and the resulting parametric curve

is composed with MappedSrf to yield the returned value.

Further, if ValidatePt is prescribed, it should contain two elements,

a vector, V, and an angle, A, in degrees.  Only the contour points for which

the normal of surface MappedSrf there has less than A degrees from V

are returned.

Example:

resolution = 20;

nglass = snrmlsrf( glass ) * vector( 1, 1, 1 );

sils = contour( nglass, plane( 1, 0, 0, 0 ), glass );

color( sils, cyan );

attrib( sils, "dwidth", 4 );

view( list( axes, glass, sils ), on );

computes the normal field of the surface glass, projects it onto

the viewing direction of (1, 1, 1) and contours the resulting scalar field

with the plane X = 0, to extract the silhouette curves from viewing direction

(1, 1, 1).

CONVEX

------

PolygonType CONVEX( PolygonType Object )

or

ListType CONVEX( ListType Object )

convert non-convex polygons in Object, into convex ones. New vertices

are introduced into the polygonal data during this process. The Boolean

operations require the input to have convex polygons only (although it may

return non convex polygons...) and it automatically converts non-convex input

polygons into convex ones, using this same routine.

However, some external tools (such as irit2ray and poly3d-h) require convex

polygons. This function must be used on the objects to guarantee that

only convex polygons are saved into data files for these external tools.

Example:

CnvxObj = CONVEX( Obj );

save( "data", CnvxObj );

converts non-convex polygons into convex ones, so that the data file can be

used by external tools requiring convex polygons.

COORD

-----

AnyType COORD( AnyType Object, NumericType Index )

extracts an element from a given Object, at index Index. From

a PointType, VectorType, PlaneType, CtlPtType and MatrixType. A NumericType

is returned with Index 0 for the X axis, 1 for the Y axis etc.

Index 0 denotes the weight of CtlPtType. For a PolygonType that

contains more than one polygon, the Indexth polygon is returned. For

a PolygonType that contains a single Polygon, the Indexth vertex is

returned. For a CurveType or a SurfaceType, the Indexth CtlPtType is

returned. For a ListType, COORD behaves like NTH and returns the Indexth

object in the list. For a StringType, the Indexth character is returned

as its ASCII numeric code.

Example:

a = vector( 1, 2, 3 );

vector( COORD( a, 0 ), COORD( a, 1 ), COORD( a, 2 ) );

a = ctlpt( P2, 6, 7, 8, 9 );

ctlpt( P3, coord( a, 0 ), coord( a, 1 ), coord( a, 2 ), coord( a, 3 ) );

a = plane( 10, 11, 12, 13 );

plane( COORD( a, 0 ), COORD( a, 1 ), COORD( a, 2 ), COORD( a, 3 ) );

constructs a vector/ctlpt/plane and reconstructs it by extracting the

constructed scalar components of the objects using COORD.

See also COERCE.

COVERISO

--------

CurveType COVERISO( TrivarType TV,

NumericType NewOfStrokes,

NumericType StrokeType,

PointType MinMaxPwrLen,

NumericType StepSize,

NumericType IsoVal,

VectorType ViewDir )

computes a coverage for an iso surface of a trivariate function TV,

using curves. NewOfStrokes strokes are distributed on the iso surface

with a length that is set via MinMaxPwrLen. MinMaxPwrLen is

a triplet of the form (Min, Max, Power) that determines the length of the

strokes as,

Avg = (Max + Min) / 2,             Dev = (Max - Min) / 2

Length = Avg + Dev * Random(0, 1) ^ Pwr.

StepSize controls the steps size of the piecewise linear approximation

formed and should typically be smaller than Min.

StrokeType can be one of,

1          Draw strokes along minimal principal curvature.

2          Draw strokes along maximal principal curvature.

3          Draw strokes along both principal curvatures.

4          Draw strokes along constant X planes.

5          Draw strokes along constant Y planes.

6          Draw strokes along constant Z planes.

IsoVal controls the constant value of the iso surface level.

See also ADAPISO, COVERPT, MRCHCUBE, TVLOAD.

Finally, ViewDir is the direction of view, used for silhouette computation.

Example:

IsoVal = 0.12;

Cover = CoverIso( ThreeCyls, 500, 1, vector( 3, 10, 1.0 ), 0.2, IsoVal );

draws 500 strokes on the iso surface of trivariate ThreeCyls at iso

value IsoVal and step size of 0.2.  Strokes are drawn in length of

3 to 10 along lines of curvatures of minimal curvature.

COVERPT

-------

PolygonType COVERPT( PolygonType Model,

NumericType NumOfPts,

VectorType ViewDir )

computes a uniform point distribution on the given polygonal Model.

Approximately NumOfPts points are uniformly distributed on the

model's surface, provided ViewDir is the zero vector.

If ViewDir is a non zero vector, the distribution is made to be

uniform from this given viewing direction.

In all cases, NumOfPts is an upper bound of the the real number of

distributed points, which will be in the same order.

See also ADAPISO, COVERISO, FFPTDIST.

Example:

Pts1 = CoverPt( solid1, 1000, vector( 0, 0, 0 ) );

Pts2 = CoverPt( solid1, 3000, vector( 0, 0, -1 ) );

computes two uniform distributions of 1000 and 3000 points on Solid1.

Pts1 is uniform in three space, while Pts2 is viewed

uniform from the -Z direction.

CPOWER

------

CurveType CPOWER( ListType CtlPtList )

creates a polynomial/rational curve out of the provided control point list.

The created curve is employing the monomial power basis.  CtlPtList is a

list of control points, all of which must be of type (E1-E9 P1-P9), or

regular PointType defining the curve's control polygon.  The curve's point

type will be of a space which is the union of the spaces of all points.

Example:

c = CPOWER( list( ctlpt( E3, 0, 1, 0 ),

ctlpt( E3, 1, 0, 0 ),

ctlpt( E3, 0, 0, 1 ) ) );

c == coerce( coerce( c, bezier_type ), power_type );

constructs a quadratic power basis curve, coerces it to a Bezier form,

coerces the Bezier form back to the power basis, and then compares the result

for equality.

See also CBEZIER, CBSPLINE and SPOWER.

CRAISE

------

CurveType CRAISE( CurveType Curve, NumericType NewOrder )

Raise Curve to the NewOrder Order specified.

Example:

Crv = cbezier( list( ctlpt( E2, -0.7,  0.3 ),

ctlpt( E2,  0.0,  1.0 ),

ctlpt( E2,  0.7,  0.0 ) ) );

Crv2 = CRAISE( Crv, 5 );

raises the 90 degrees corner Bezier curve Crv to be a quadratic.

See also CREDUCE, TRAISE, SRAISE, and MRAISE.

CRC2CRVTAN

----------

ListType CRC2CRVTAN( CurveType Crv1, CurveType Crv2,

NumericType Radius, NumericType Tol )

computes all circles that are bi-tangent to the given two curves Crv1

and Crv2.  The circles will posses a radius of Radius.  The

accuracy of the computation is governed by the tolerance Tol.  Returned

is a list center point locations with "Params" attributes of parameter values

of the tangent locations at the two curves.

Example:

Cntrs = CRC2CRVTAN( c1, c2, R, 1e-3 );

computes all the circles of radius 0.1 that are bi-tangent to the two

curves c1 and c2.   Tolerance of computation is 1e-3.

See also SKEL2DINT, CRV2TANS, TNSCRCR.

CREDUCE

-------

CurveType CREDUCE( CurveType Curve, NumericType NewOrder )

reduces the Curve to the NewOrder Order specified.  This function

approximates the lower order curve in the infinity norm sense, minimizing

the maximal deviation between the original curve Curve and the low

order curve NewOrder.  NewOrder will identify with Curve

only if Curve was degree raised before.

Example:

Crv = cbezier( list( ctlpt( E2, -0.7,  0.3 ),

ctlpt( E2,  0.0,  1.0 ),

ctlpt( E2,  0.7,  0.0 ) ) );

Crv2 = CREDUCE( craise( Crv, 5 ), 3 );

Crv == Crv2;

Should restore the original quadratic order.  I.e. Crv2 should

identify with Crv and "Crv == Crv2;" should return TRUE.

See also CRAISE.

CREFINE

-------

CurveType CREFINE( CurveType Curve, NumericType Replace, ListType KnotList )

provides the ability to Replace a knot vector of Curve, or refine

it. KnotList is a list of knots at which to refine Curve. All knots

should be contained in the parametric domain of the Curve. If the knot

vector is replaced, the length of KnotList should be identical to the

length of the original knot vector of the Curve. If Curve is a

Bezier curve, it is automatically promoted to be a B-spline curve.

Example:

Crv2 = CREFINE( Crv, FALSE, list( 0.25, 0.5, 0.75 ) );

refines Crv and adds three new knots at 0.25, 0.5, and 0.75.

See also SREFINE, TREFINE, and MREFINE.

CREGION

-------

CurveType CREGION( CurveType Curve, NumericType MinParam,

NumericType MaxParam )

extracts a region from the Curve between MinParam and MaxParam.

Both MinParam and MaxParam should be contained in the

parametric domain of the Curve, except for Bezier curves when

MinParam and MaxParam can be arbitrary (extrapolating if not

between zero and one).

Example:

SubCrv = CREGION( Crv, 0.3, 0.6 );

extracts the region from the Crv from the parameter value 0.3 to the

parameter value 0.6.

See also SREGION, TREGION, and MREGION.

CREPARAM

--------

CurveType CREPARAM( CurveType Curve, NumericType MinParam,

NumericType MaxParam )

reparametrizes the Curve over a new domain from MinParam to MaxParam.

This operation does not affect the geometry of the curve and only affine

transforms its knot vector. A Bezier curve will automatically be promoted

into a B-spline curve by this function.

Example:

arc1 = arc( vector( 0.0, 0.0, 0.0 ),

vector( 0.5, 2.0, 0.0 ),

vector( 1.0, 0.0, 0.0 ) );

crv1 = arc( vector( 1.0, 0.0, 0.75 ),

vector( 0.75, 0.0, 0.7 ),

vector( 0.5,  0.0, 0.85 ) ) +

arc( vector( 0.5,  0.0, 0.75 ),

vector( 0.75, 0.0, 0.8 ),

vector( 1.0,  0.0, 0.65 ) );

arc1 = CREPARAM( arc1, 0, 10 );

crv1 = CREPARAM( crv1, 0, 10 );

sets the domain of the given two curves to be from zero to ten. The

Bezier curve arc1 is promoted to a B-spline curve.

See also SREPARAM, TREPARAM, and MREPARAM.

CROSSEC

-------

PolygonType CROSSEC( PolygonType Object )

This feature is NOT implemented.

CRV2TANS

--------

ListType CRV2TANS( CurveType Crv, NumericType FineNess )

computes all the bi-tangents of Crv, in the XY plane. That

is, all lines that are tangent to Crv at two different locations.

A list of points with X and Y coefficients representing the

parametric locations on Crv of the bi-tangent is returned.  FineNess

controls the numerical accuracy of the computation. A value of 10 will

provide a good start and the larger this number is, the better the

accuracy will be. See also CRVPTTAN and CNVXHULL.

Example:

Tans = nil();

Crv2Tns = Crv2Tans( Crv, 10 );

for ( i = 1, 1, sizeof( Crv2Tns ),

pt = nth( Crv2Tns, i ):

snoc( ceval( Crv, coord( pt, 0 ) ) +

ceval( Crv, coord( pt, 1 ) ), Tans ) );

finds the bi-tangents of Crv and converts them to a set of line

segments.

See also CRC2CRVTAN, TNSCRCR

CRVKERNEL

---------

AnyType CRVKERNEL( CurveType Crv, NumericType Gamma, NumericType Euclid,

AnyType Fineness, NumericType Mode )

computes the (gamma) kernel of a given planar curve Crv.  If

Gamma is zero, regular kernel is computed.  Else, the gamma curves of

Gamma degrees is being computed.  If Euclid is TRUE then the

result is returned in the Euclidean space, or if zero, in the parametric

space.

Mode can be 0 for a (gamma) kernel solution, 1 to extract silhouette

sampling only out of the trivariate function, and 2 for the gamma kernel

surface/trivariate functions themselves to be returned.

Fineness controls the refinement that is applied to the numeric

solution.  For Mode 0 it sets the number of knots to insert into

the (x, y, t) trivariate function.  In Mode 2, it lists two numeric

values, the subdivision and numeric tolerances, of the silhouette extraction

process (See MZERO for their meaning).  In Mode 2, it controls the

extent of the surfaces/trivariates.

Example:

Krnl = CrvKernel( Crv, 0, 0, list( 2, 3, 1 ), 0 );

computes the regular kernel of curve Crv with a refinement of

(2, 3, 1) in the three (x, y, t) axes of the computed trivariate function.

CRVLNDST

--------

NumericType CRVLNDST( CurveType Crv, PointType PtOnLine, VectorType LnDir,

NumericType IsMinDist, NumericType Epsilon )

or

ListType CRVLNDST( CurveType Crv, PointType PtOnLine, VectorType LnDir,

NumericType IsMinDist, NumericType Epsilon )

compute the closest (if IsMinDist is TRUE, farthest if FALSE) point

on the Curve to the line specified by PtOnLine and LnDir as a

point on the line and a line direction.

Since this operation is partially numeric, Epsilon is used to set

the needed accuracy. It returns the parameter value of the location on

Crv closest to the line.

If, however, Epsilon is negative, -Epsilon is used instead,

and all local extrema in the distance function are returned as a list

(both minima and maxima).

If the line and the curve intersect, the point of intersection is

returned as the minimum.

Example:

Param = CRVLNDST( Crv, linePt, lineVec, TRUE, 0.001 );

finds the closest point on Crv to the line defined by linePt

and lineVec.

CRVPTDST

--------

NumericType CRVPTDST( CurveType Crv, PointType Point, NumericType IsMinDist,

NumericType Epsilon )

or

ListType CRVPTDST( CurveType Crv, PointType Point, NumericType IsMinDist,

NumericType Epsilon )

compute the closest (if IsMinDist is TRUE, farthest if FALSE) point

on Crv to Point.

Since this operation is partially numeric, Epsilon is used to set

the needed accuracy. It returns the parameter value of the location on

Crv closest to Point.

If, however, Epsilon is negative, -Epsilon is used instead,

and all local extrema in the distance function are returned as a list

(both minima and maxima).

Example:

Param = CRVPTDST( Crv, Pt, FALSE, 0.0001 );

finds the farthest point on Crv from point Pt.

CRVPTTAN

--------

ListType CRVPTTAN( CurveType Crv, PointType Pt, NumericType FineNess )

computes all the tangents to Crv that go through point Pt,

all in the XY plane. A list of points with X and Y

coefficients representing the parametric locations on Crv of the

bi-tangent is returned.  FineNess controls the numerical accuracy of

the computation. A value of 0.01 will provide a good start, and the smaller

this number is, the better the accuracy will be. See also CRV2TANS and

CNVXHULL.

Example:

Tans = nil();

Pt = point( 2, 0, 0 );

CrvPtTns = CrvPtTan( Crv, Pt, 0.01 );

for ( i = 1, 1, sizeof( CrvPtTns ),

snoc( ceval( Crv, nth( CrvPtTns, i ) ) + coerce( Pt, e3 ), Tans ) );

finds the tangents of Crv through Pt and converts them to a

set of line segments.

CSURFACE

--------

CurveType CSURFACE( SurfaceType Srf, ConstantType Direction,

NumericType Param )

or

CurveType CSURFACE( TriSrfType Srf, ConstantType Direction,

NumericType Param )

extract an isoparametric curve out of Srf in the specified

Direction (ROW or COL (or DEPTH for triangular surface) at the

specified parameter value Param.

Param must be contained in the parametric domain of Srf in

Direction direction.

The returned curve is in the surface Srf.

Example:

Crv = CSURFACE( Srf, COL, 0.45 );

extracts an isoparametric curve in the COLumn direction at the parameter

value of 0.15 from surface Srf.

See also CMESH, COMPOSE, STRIVAR, MFROMMV.

CTANGENT

--------

VectorType CTANGENT( CurveType Curve, NumericType Param )

computes the tangent vector to the Curve at the parameter value Param.

The returned vector has a unit length.

Example:

Tang = CTANGENT( Crv, 0.5 );

computes the tangent vector to Crv at the parameter value of 0.5.

CTLPT

-----

CPt = CTLPT( ConstantType PtType, NumericType Coord1, ... )

constructs a single control point to be used in the construction of curves

and surfaces. Points can have from one to five dimensions, and may be

either Euclidean or Projective (rational). Point type is set via the

constants E1 to E9 and P1 to P9. The coordinates of the point are specified

in order; weight is first if rational.

Examples:

CPt1 = CTLPT( E3, 0.0, 0.0, 0.0 );

CPt2 = CTLPT( P2, 0.707, 0.707, 0.707 );

constructs an E3 point at the origin and a P2 rational point with

a weight of 0.707. The Projective Pi points are specified as

CTLPT(Pn, W, W X1, ... , W Xn).

CTRIMSRF

--------

ListType CTRIMSRF( TrimSrfType TSrf, NumericType Parametric )

extracts the trimming curves of a trimmed surface TSrf.

If Parametric is not zero, then the trimming curves are extracted

as parametric space curves of TSrf. Otherwise, the trimming curves

are evaluated into Euclidean space as curves on the surface TSrf.

Example:

TrimCrvs = CTRIMSRF( TrimSrf, FALSE );

extracts the trimming curves of TrimSrf as Euclidean curves on

TrimSrf.

CYLIN

-----

PolygonType CYLIN( VectorType Center, VectorType Direction,

NumericType Radius, NumericType Caps )

creates a CYLINder geometric object, defined by Center as the center of

the base of the CYLINder, Direction as the CYLINder's axis and height,

and Radius as the radius of the base of the CYLINder.

If Caps equals zero, no caps are created.  If Caps equal

one (two), only the bottom (top) cap is created.  If Caps equal three,

both the top and the bottom caps are created.

See RESOLUTION for the accuracy of the CYLINder approximation as a

polygonal model. See IRITSTATE's "PrimRatSrfs" and "PrimRatSrfs" state

variables.

Example:

Cylinder1 = CYLIN( vector( 0, 0, 0 ), vector( 1, 0, 0 ), 10, 3 );

constructs a cylinder with two caps of radius 10 along the X axis

from the origin to X = 1.

CZEROS

------

ListType CZEROS( CurveType Crv, NumericType Epsilon, NumericType Axis )

computes the zero set of the given Crv in the given axis (1 for X,

2 for Y, 3 for Z). Since this computation is numeric, an Epsilon is

also required to specify the desired tolerance. It returns a list of

all the parameter values (NumericType) the curve is zero.

Example:

xzeros = CZEROS( cb, 0.001, 1 );

pt_xzeros = nil();

pt = nil();

for ( i = 1, 1, sizeof( xzeros ),

pt = ceval( cb, nth( xzeros, i ) ):

snoc( pt, pt_xzeros )

);

interact( list( axes, cb, pt_xzeros ), 0 );

computes the X zero set of curve cb with error tolerance

of 0.001. This set is then scanned in a loop and evaluated to

the curve's locations, which are then displayed.

See also CINFLECT.

DIST2FF

-------

SurfaceType DIST2FF( CurveType Crv1, CurveType Crv2, NumericType DistType )

or

MultivarType DIST2FF( CurveType Crv1, SurfaceType Srf2, NumericType DistType )

or

MultivarType DIST2FF( SurfaceType Srf1, SurfaceType Srf2, NumericType DistType )

computes the distance function between the two given freeform shapes.

The returned variety is bi-variate, tri-variate, or a four-variate, depending

on the dimensionality of the input, in order.  Based on DistType, the

following distance functions could be used:

DistType Value  Description

1          Computes the distance square function, (V1 - V2)^2.

2          Projection of the distance vector onto the normal

field of the first varietly, < V1 - V2, N1 >.

3          Projection of the distance vector onto the normal

field of the second varietly, < V1 - V2, N2 >.

In cases 2 and 3, the normal field is not a unit field.

Example:

Crv1 = cbezier( list( ctlpt( E1, .2 ),

ctlpt( E2, 0.5, 4 ),

ctlpt( E2, 1.3, 0.05 ) ) ) * sy( 0.2 );

Crv2 = cbezier( list( ctlpt( E1, -.2 ),

ctlpt( E2, 0.25, 1.9 ),

ctlpt( E2, 1.3, 0.05 ) ) ) * ty( 0.3 ) * sx( 1.5 );

bb = bbox( dist2ff( Crv1, Crv2, 1 ) );

computes a bound on the minimal and maximal distance square between the

given two curves, by computing a bounding box on this scalar distance square

field.

DUALITY

-------

CurveType DUALITY( CurveType Curve )

or

SurfaceType DUALITY( SurfaceType Srf )

computes the dual curve/surface to the given curve/surface.  The dual shape

is a mapping of every point to a line (plane) in R2 (R3).

Example:

Ellipsoid = sphereSrf( 1.1 ) * sx( 2 ) * sy( 1.2 );

DualEllip = DUALITY( Ellipsoid );

EVOLUTE

-------

CurveType EVOLUTE( CurveType Curve )

or

SurfaceType EVOLUTE( SurfaceType Srf )

compute the evolute of a curve or a surface.

For curves, the evolute is defined as,

N(t)

E(t) = C(t) + ----

k(t)

where N(t) is the unit normal of C(t), and k(t) is its curvature.

E(t) is computed symbolically as the symbolic sum of C(t) and

N(t) / k(t).

For surfaces, this function computes the mean evolute which is equal to,

n(u, v)

E(u, v) = S(u, v) + ---------

2 H(u, v)

where n(u, v) is the unit normal of S(u, v), and H(u, v) is its mean

curvature. E(u, v) is computed symbolically.

The result of this symbolic computation is exact (upto machine precision),

unlike similar operations such as OFFSET or AOFFSET, that are only

approximated.

Example:

crv = cbspline( 3,

list( ctlpt( E3, -1.0,  0.1,  0.2 ),

ctlpt( E3, -0.1,  1.0,  0.1 ),

ctlpt( E3,  0.1,  0.1,  1.0 ),

ctlpt( E3,  1.0,  0.1,  0.1 ),

ctlpt( E3,  0.1,  1.0,  0.2 ) ),

list( KV_OPEN ) );

cev = EVOLUTE( Crv );

See also SMEAN.

EXTRUDE

-------

PolygonType EXTRUDE( PolygonType Object, VectorType Dir, NumericType Caps )

or

SurfaceType EXTRUDE( CurveType Object, VectorType Dir, NumericType Caps )

or

TrivarType EXTRUDE( SurfaceType Object, VectorType Dir, NumericType Caps )

create an extrusion of the given Object. If the Object is a

PolygonObject, its first polygon is used as the base for the extrusion in

Dir direction.  If the Object is a CurveType, an extrusion surface is

constructed.  If the Object is a SurfaceType, an extrusion trivariate

is constructed.  If Caps equals zero, no caps are created.  If

Caps equal one (two), only the bottom (top) cap is created.  If

Caps equal three, both the top and the bottom caps are created.

Note that caps are created for a closed Object only, so the Object

must be either a polygon or a closed curve for caps to be generated.

Direction Dir cannot be coplanar with the polygon plane. The curve

may be nonplanar.

Example:

Cross = cbspline( 3,

list( ctlpt( E2, -0.018, 0.001 ),

ctlpt( E2,  0.018, 0.001 ),

ctlpt( E2,  0.019, 0.002 ),

ctlpt( E2,  0.018, 0.004 ),

ctlpt( E2, -0.018, 0.004 ),

ctlpt( E2, -0.019, 0.001 ) ),

list( KV_OPEN ) );

Cross = Cross + -Cross * scale( vector( 1, -1, 1 ) );

Napkin = EXTRUDE( Cross * scale( vector( 1.6, 1.6, 1.6 ) ),

vector( 0.02, 0.03, 0.2 ),

0 );

constructs a closed cross section Cross by duplicating one half of

it in reverse and merging the two sub-curves. Cross is then used as

the cross section for the extrusion operation.

FFCOMPAT

--------

FFCOMPAT( CurveType Crv1, CurveType Crv2 )

or

FFCOMPAT( SurfaceType Srf1, SurfaceType Srf2 )

make the given two curves or surfaces compatible by making them share

the same point type, the same curve type, the same degree, and the

same continuity.  The same point type is gained by promoting a lower

dimension into a higher one, and non-rational to rational

points. Bezier curves are promoted to B-spline curves if necessary,

for curve type compatibility.  Degree compatibility is achieved by

raising the degree of the lower order curve. Continuity is achieved

by refining both curves to the space with the same (unioned) knot

vector. This function returns nothing and compatibility is made  in place}.

Example:

FFCOMPAT( Srf1, Srf2 );

See also CMORPH and SMORPH.

FFCTLPTS

--------

ListType FFCTLPTS( FreeformType Freeform );

returns all the control points of the given Freeform in a single list.

See Also FFKNTVEC, FFMSIZE, FFORDER, FFPTTYPE.

Example:

Ctls = FFCTLPTS( Srf1 );

FFEXTREME

---------

CtlPtType FFEXTREME( CurveType Crv, NumericType Minimum )

or

CtlPtType FFEXTREME( SurfaceType Srf, NumericType Minimum )

compute a bound on the extreme values a curves Crv or surface

Srf can assume. Returned control points provide a bound on the

minimum (maximum) values that can be assumed if Minimum is TRUE

(FALSE).

Example:

Bound = FFEXTREME( Srf, false );

computes a bound on the maximal values Srf can assume.

FFKNTVEC

--------

ListType FFKNTVEC( FreeformType Freeform )

returns all the knot vector(s) of the given Freeform in a list of knot

vector(s).  See Also FFCTLPTS, FFMSIZE, FFORDER, FFPTTYPE.

Example:

KVs = FFKNTVEC( Srf1 );

FFMATCH

-------

FFMATCH( CurveType Crv1, CurveType Crv2, NumericType Reduce,

NumericType Samples, NumericType ReparamOrder,

NumericType Rotate, NumericType NormType )

computes a reparametrization to Crv2 so it fits Crv1, the best

under some prescribed norm, NormType. Currently the following norms

are valid for NormType

Value      Description

1          Suitable for ruled and blended curves, for modeling.

See RULEDSRF.

2          Suitable for metamorphosis of curves. See CMORPH.

3          Distance norm in "walking the dog" notion.

4          Bisector (skeleton) matching norm for two curves.

Whenever negative norms can result (for example, in cases were self

intersection cannot be prevented in ruled surface constructions), one can

allow negativity with no extra penalty by applying negative NormType.

Use of positive-only norms would yield no output at all if no matching

with positive weights can be established, whereas allowing negative

norm values would result in a globally optimal result, but with possible

self intersectiions.

The reparametrization is computed by sampling a fixed set of size

Samples off both curves, and fitting a B-spline curve of length

Reduce as the reparametrization curve. Hence, Reduce must

be less than or equal to Samples. The reparametrization curve

will have order of ReparamOrder. If Rotate is TRUE or ON,

then attempt is made to rotate the reparametrization of the

curves. Rotation can be used on closed curves only.

See RULEDSRF and CMORPH for examples.

FFMERGE

-------

CurveType FFMERGE( ListType E1Curves, NumericType PointType )

or

SurfaceType FFMERGE( ListType E1Surfaces, NumericType PointType )

or

MultivarType FFMERGE( ListType E1Multivars, NumericType PointType )

merge the scalar curves/surfaces/multivariates in the list of curves

E1Curves or list of surfaces E1Surfaces or list of

multivariates E1Multivars to one vector curve/surface/multivariate

of point type PointType.

Example:

Srf = FFMERGE( list( SrfW, SrfX, SrfY ), P2 );

merges three scalar surfaces into a single surface with point type P2.

See also FFSPLIT, FFPTTYPE.

FFMESH

------

ListType FFMESH( FreeformType Freeform )

returns the control mesh/polygon of the given Freeform in a list.

See Also FFCTLPTS, FFKNTVEC, FFORDER, FFPTTYPE, FFMSIZE.

Example:

SrfMesh = FFMESH( Srf );

FFMSIZE

-------

ListType FFMSIZE( FreeformType Freeform )

returns the size of the control mesh/polygon of the given freeform

in a list.

See Also MESHSIZE, FFMESH, FFCTLPTS, FFKNTVEC, FFORDER, FFPTTYPE, PDOMAIN.

Example:

MSizes = FFMSIZE( Srf1 );

FFORDER

-------

ListType FFORDER( FreeformType Freeform )

returns all the orders of the given Freeform in a single list.

See Also FFCTLPTS, FFKNTVEC, FFMSIZE, FFPTTYPE, PDOMAIN.

Example:

Orders = FFORDER( Srf1 );

FFPOLES

-------

NumericType FFPOLES( FreeformType Freeform );

returns TRUE if the given Freeform has poles, FALSE otherwise.

Poles are zeros in the weights of rational functions.

Example:

HasPoles = FFPOLES( Srf1 );

FFPTDIST

--------

ListType FFPTDIST( CurveType Crv, NumericType Param, NumericType NumOfPts )

or

ListType FFPTDIST( SurfaceType Srf, NumericType Param, NumericType NumOfPts )

compute a uniform point distribution for Crv or Srf.  If

Param is FALSE, the distribution is selected to be uniform in the

Euclidean space; otherwise if TRUE, the distribution is made uniform in the

parametric space. NumOfPts sets the number of points in the

distribution.

The returned list of points prescribes parameter values in the freeforms.

For Crv, the returned list is a list of reals, in the parameter space

of Crv.  For Srf, the returned list is a list of points, whose

X and Y coefficients hold the U and V parameters of Srf.

See also COVERPT.

Example:

c1 = cbezier( list( ctlpt( E2, -1.0,  0.0 ),

ctlpt( E2, -1.0,  0.1 ),

ctlpt( E2, -0.9, -0.1 ),

ctlpt( E2,  0.9,  0.0 ) ) );

color( c1, magenta );

pts = FFPTDIST( c1, true, 300 );

e2pts = nil();

for ( i = 1, 10, sizeof( pts ),

pt = ceval( c1, coord( nth( pts, i ), 0 ) ):

snoc( pt, e2pts )

);

interact( list( e2pts, c1 ) );

pts = FFPTDIST( c1, false, 300 );

e2pts = nil();

for ( i = 1, 10, sizeof( pts ),

pt = ceval( c1, coord( nth( pts, i ), 0 ) ):

snoc( pt, e2pts )

);

interact( list( e2pts, c1 ) );

computes the distribution of 100 points in curve c1 which has

highly nonuniform speed characteristics.  Two distributions are computed,

one to be uniform in the parametric space and one to be uniform in the

Euclidean space.

FFPTTYPE

--------

NumericType FFPTTYPE( FreeformType Freeform )

returns the point type (E2, P4 etc.) of the given freeform.

See Also FFCTLPTS, FFKNTVEC, FFMSIZE, FFORDER, PDOMAIN.

FFSPLIT

-------

ListType FFSPLIT( CurveType Crv )

or

ListType FFSPLIT( SurfaceType Srf )

or

ListType FFSPLIT( MultivarType MV )

split the given curve Crv or surface Srf or multivariate MV

into its scalar components that are returned as a list of scalar

curves/surfaces/multivariates.

Example:

E1Srfs = FFSPLIT( circle( vector( 0, 0, 0 ), 1 ) );

splits the circle which is a curve in P3 into four scalar curves (W, X, Y, Z)

that are returned in a single list.

See also FFMERGE, FFPTTYPE.

FITPMODEL

---------

ListType FITPMODEL( PolygonType PlObj, NumericType FitType,

NumericType Tol, NumericType NumIters )

fits a primitive object to the given polygonal model, PlObj.

The numeric fitting process is controled via a bound on the number of

iterations NumIters and the resulting tolerance of the fit that is

required, Tol.  FitType can be one of:

0          A Planar face.

1          A Sphere.

2          A Cylinder.

3          A Circle.

4          A Cone.

5          A Torus.

Example:

resolution = 20;

x1 = triangl( sphere( vector( 1, 2, 3 ), 4 ), 1 );

SprParams = FitPModel( x1, 1, 0.01, 100 );

See also ANALYFIT.

FIXPLGEOM

---------

PolygonType FIXPLGEOM( PolygonType PlObj, NumericType Oper, NumericType Eps )

or

ListType FIXPLGEOM( ListType Obj, NumericType Oper, NumericType Eps )

cleans polygonal geometry.  based on Oper, the following will be

conducted:

0          Remove identical duplicated polygons.

1          Remove zero length edges.

The clean up of an object will be applied individually to each part

found in the object list Obj.

Example:

Obj2 = FIXPLGEOM( Obj, 0 );

Obj3 = FIXPLGEOM( Obj2, 1 );

Obj4 = FIXPLNRML( Obj3, 2 );

cleans duplicated polygons, zero length edges, and then reorient the result.

See also FIXPLNRML.

FIXPLNRML

---------

PolygonType FIXPLNRML( PolygonType PlObj, NumericType TrustInfo )

or

ListType FIXPLNRML( ListType Obj, NumericType TrustInfo )

corrects inconsistencies in polygonal geometry, between normals of polygons

and normals at the vertices based on TrustInfo.  If TrustInfo is

0          Trust the normals at the vertices.

1          Trust the normals of the polygons.

2          Reorient all the polygon's normals and vertices

normals to follow the orientation of first polygon.

The computation on an object will be applied individually to each part

found in the object list Obj.  Option 2 of TrustInfo will correct

cases where adjacent polygons are not oriented the same, based on detection

of adjacencies.

Example:

Obj2 = FIXPLNRML( Obj, 2 );

See also FIXPLGEOM.

GBOX

----

PolygonType GBOX( VectorType Point,

VectorType Dx, VectorType Dy, VectorType Dz )

creates a parallelepiped - generalized BOX polygonal object, defined by

Point as its base position, and Dx, Dy, Dz as 3 3D vectors to define

the 6 faces of this generalized BOX. The regular BOX object is a special case

of GBOX where Dx = vector(Dx, 0, 0), Dy = vector(0, Dy, 0), and

Dz = vector(0, 0, Dz).

Dx, Dy, Dz must all be independent in order to create an

object with positive volume.

Example:

GB = GBOX( vector( 0.0, -0.35, 0.63 ), vector(  0.5, 0.0, 0.5 ),

vector( -0.5, 0.0, 0.5 ),

vector(  0.0, 0.7, 0.0 ) );

GETATTR

-------

AnyType GETATTR( AnyType Obj, StringType Name )

provides a mechanism to fetch an attribute named Name from object

Obj.

Example:

attrib( axes, "test", 15 );

a = GETATTR( axes, "test" );

will set the value of a to be 15.

GETLINE

-------

AnyType GETLINE( NumericType RequestedType )

provides a method to get input from the keyboard within functions and

or subroutines. RequestedType can be a NUMERIC_TYPE,

POINT_TYPE, VECTOR_TYPE, or PLANE_TYPE in which the entered line

will be parsed into one, three, or four numeric values (operated by

either spaces or commas) and the proper object will be created and

returned. In any other case, including failure to parse the numeric

input, a STRING_TYPE object will be constructed from the entered line.

Example:

Pt = GETLINE( point_type );

to read one point (three numeric values) from stdin.

GETNAME

-------

StringType GETNAME( ListType ListObj, NumericType Index )

gets the name of a sub object of index Index in list object

ListObj.  Index of the first element is one.

Example:

A = list( XX, Second, C );

GETNAME( A, 1 );

returns the name of the second element, "Second".

See also SETNAME.

GGINTER

-------

ListType GGINTER( CurveType Srf1Axis, CurveType Srf1Rad,

CurveType Srf2Aixs, CurveType Srf2Rad,

NumericType Tolerance, NumericType ZeroSetFunc )

computes the intersection curves of the given two ring surfaces,

defined as spine surfaces with axis SrfiAxis, i = 1, 2 and circular

cross section along the normal plane of the axis curve with radii

SrfiRad.

The ring ring intersection (RRI) problem is tranformed into a zero set

finding on another function.  If ZeroSetFunc is true, the function

whose zero set provides the RRIsolution is returned.  Otherwise, if

ZeroSetFunc is false, the RRI solution itself is returned.  The zero

set is computed via numerical zero set finding methods and Tolerance

controls the fineness of the approximated solution.

Example:

s1 = cylinSrf( 4, 1 ) * tz( -2 );

c1 = cbezier( list( ctlpt( E3, 0.0, 0.0, -1.0 ),

ctlpt( E3, 0.0, 0.0,  1.0 ) ) );

r1 = cbezier( list( ctlpt( E1, 1.0 ) ) );

s2 = cylinSrf( 4, 1 ) * tz( -2 ) * rx( 90 ) * tx( 0.5 );

c2 = cbezier( list( ctlpt( E3, 0.5, -1.0, 0.0 ),

ctlpt( E3, 0.5,  1.0, 0.0 ) ) );

r2 = cbezier( list( ctlpt( E1, 1.0 ) ) );

ZeroSetSrf = coerce( GGINTER( c1, r1, c2, r2, 10, true ), e3 )

* rotx( -90 ) * roty( -90 );

resolution = 100;

ZeroSet = contour( ZeroSetSrf, plane( 0, 0, 1, 0 ) );

interact( list( ZeroSetSrf * sz( 0.1 ), ZeroSet, axes ) );

c = nth( GGINTER( c1, r1, c2, r2, 100, false ), 1 );

interact( list( s1, s2, c ) );

constructs two cylinders as s1 and s2, defines the same two

cylinders as a ring surface with axes spines of c1 and c2 and

a constant radius, one in r1 and r2, and computes the zero set

of the intersection and the intersection curve itself.

GPOLYGON

--------

PolygonType GPOLYGON( GeometryTreeType Object, NumericType Normals )

approximates all Surface(s)/Trimmed surface(s)/Trivariate(s) in Object

with polygons using the POLY_APPROX_OPT, POLY_APPROX_TRI,

POLY_MERGE_COPLANAR, RESOLUTION and FLAT4PLY variables.

If POLY_APPROX_OPT is FALSE, RESOLUTION vaguely prescribes the number of

uniform (in parametric space) samples to sample the surface in each direction.

If POLY_APPROX_OPT is TRUE, POLY_APPROX_TOL prescribes the maximal

deviation of the polygonal approximation from the original surface, in object

space coordinates.

IF POLY_APPROX_TRI is TRUE, only triangles are generated on the output

set.

POLY_MERGE_COPLANAR controls the way coplanar adjacent polygons are merged

into one (or not.)

FLAT4PLY is a Boolean flag controlling the conversion of an (almost) flat

patch into four (TRUE) or two (FALSE) polygons.  Normals are computed to

polygon vertices using surface normals, so Gouraud or Phong shading can be

exploited. It returns a single polygonal object.

If Normals is set, surface normals will be evaluated at the

vertices. Otherwise flat shading and constant normals across polygons are

assumed.

Example:

Polys = GPOLYGON( list( Srf1, Srf2, Srf3 ), off );

converts to polygons the three surfaces Srf1, Srf2, and Srf3

with no normals.  See also POLY_APPROX_OPT, POLY_APPROX_TOL,

POLY_APPROX_TRI, POLY_APPROX_UV, POLY_MERGE_COPLANAR, RESOLUTION and

FLAT4PLY.

GPOLYLINE

---------

PolylineType GPOLYLINE( GeometryTreeType Object, NumericType Optimal )

converts all Curves(s), (Trimmed) Surface(s), and Trivariate(s) Object

into polylines using the RESOLUTION variable. The larger the RESOLUTION is,

the finer the resulting approximation will be. It returns a single polyline

object.

If Optimal is false, the points are sampled at equally spaced

intervals in the parametric space. If Optimal true, a better, more

expensive computationally algorithm is used to derive optimal sampling

locations so as to minimize the maximal distance between the curve and

piecewise linear approximation (L infinity norm).

Example:

Polys = GPOLYLINE( list( Srf1, Srf2, Srf3, list( Crv1, Crv2, Crv3 ) ),

on );

converts to polylines the three surfaces Srf1, Srf2, and Srf3

and the three curves Crv1, Crv2, and Crv3.

HERMITE

-------

SurfaceType HERMITE( CurveType Bndry1, CurveType Bndry2,

CurveType Tan1, CurveType Tan2 )

or

CurveType HERMITE( PointType Bndry1, PointType Bndry2,

VectorType Tan1, VectorType Tan2 )

construct a cubic fit between Bndry1 and Bndry2 so that

first derivative continuity constraints, as prescribed by Tan1 at

Bndry1 and Tan2 at Bndry2, are preserved.

It returns either a curve or a surface, according to type of input parameters.

Example:

h00 = HERMITE( point( 0, 0, 0 ),

point( 1, 1, 0 ),

vector( 1, 0, 0 ),

vector( 1, 0, 0 ) );

constructs a curve in the shape of the first basis function of the cubic

Hermite basis functions.  See also BLHERMITE and BLSHERMITE.

INSTANCE

--------

InstanceType INSTANCE( StringType GeomName, MatrixType Mat );

creates an instance of the geometry prescribed by GeomName to be

related to a different position as specified by matrix Mat.

The use of instances is advantageous where the same geometry is to be

displayed/processed in several different locations in space.  A modification

of the original geometry Geom will affect all instances that reference

it.  The reference is by the original object's name.  The original

object can be a single object or a whole hierarchy of objects.

Example:

Tea1 = INSTANCE( "Teapot", tx( 10 ) );

Tea2 = INSTANCE( "Teapot", tx( 20 ) );

Tea3 = INSTANCE( "Teapot", tx( 30 ) );

viewobj( list( Teapot, Tea1, Tea2, Tea3 ) );

will display four teapots 10 units apart along X.

IRITSTATE

---------

AnyType IRITSTATE( StringType State, AnyType Data )

sets a state variable in the IRIT Solid Modeller and returns the

old value, if applicative. Current supported state variables are:

State Name    Data Type    Comments

CmpObjEps     NumericType  Sets the epsilon to use to compare two objects.

Coplanar      NumericType  If TRUE, Coplanar polygons are handled by

the Boolean operations.

CursorKeep    NumericType  If TRUE, keep mouse events reported by the

display devices for CLNTCRSR to read.

DebugMalloc   StringType   If "Reset", memory allocation is cleared/reset.

No "Free unallocated pointer" test after

"Reset".  If "Print", all allocated blocks are

printed.  Otherwise, used as "address, n": ptr

address to search for with abort() called after

n mallocs.

DebugFunc     NumericType  >0 user func. debug information. >2 print params

on entry, ret. val. on exit. >4 global var. list

operations.

Dependency    NumericType  0 for no object dependency propagations, 1 for

automatic dependency propagation, in evaluation.

DumpLevel     NumericType  Bitmask to control the way variables/expressions

are dumped. Only object names/types if all 0.

Scalars and vectors are dumped if 0x01.

Curves and Surfaces are dumped if 0x02.

Polygons/lines are dumped if DumpLvl 0x04.

List objects are traversed recursively if 0x10.

List objects are dumped verbatim if 0x20.

Dependency information is dumped if 0x40.

EchoSource    NumericType  If TRUE, IRIT scripts are echoed to stdout.

FastPolys     NumericType  If 0x01, surface polygons are computed fast and

are only approximated.  If 0x02, surface normals

are computed fast and are only approximated.  If

0x01 | 0x02, both are fast and approximated.

FloatFrmt     StringType   Specifies a new printf floating point format.

InterCrv      NumericType  If TRUE Boolean operations creates only

intersection curves. If FALSE, full Boolean

operation results.

InterpProd    NumericType  TRUE for B-spline sym. products via

interpolation, FALSE for B-spline sym. products

via Bezier.

LoadFont      StringType   Specifies a new IRIT font file to use in

TEXTGEOM commands.

PolySort      NumericType  Axis of Polygon Intersection sweep in Boolean

operations: 0 for X axis, 1 for Y axis, 2 for

Z axis.

PrimRatSrfs   NumericType  TRUE for rational exact primitive surfaces,

FALSE for approximated polynomial (integral)

surfaces.  See also PrimSrfs.

PrimSrfs      NumericType  TRUE for primitive construction as freeform

surfaces, FALSE for polygonal primitives.

See also PrimRatSrfs.

RandomInit    NumericType  Initialize the seed random number generator

in IRIT.  See also the RANDOM function.

TrimCrvs      NumericType  Number of samples the higher order trimmed

curves are sampled, in piecewise linear

approximation.  If zero, computed

symbolically as composition.

UVBoolean     NumericType  If TRUE, Boolean between surfaces returns UV

instead of Euclidean curves.

Example:

IRITSTATE( "DebugFunc", 3 );

IRITSTATE( "FloatFrmt", "%8.5lg" );

To print parameters of user defined functions on entry, and return value on

exit. Also selects a floating point printf format of "%8.5lg".

ISGEOM

------

ListType ISGEOM( AnyType Obj, NumericType GeomType, NumericType Eps )

verifies if the given freeform geometry in Obj is a line,

circle, plane, sphere, surface of revolution, extrusion, ruled surface,

or a sweep surface, upto some tolerance Eps. GeomType

prescribes the type to check for as one of the GEOM_LINE/CIRCLE etc.

constants.

The return value is a list of two objects.  The first is a Boolean value

with the success/failure of the result.  The second is a list with the

construction entities of Obj, if any.  For example, for a detected

sphere, the center and radius will be returned.

Example:

b = nth( ISGEOM( Crv, GEOM_LINE ), 1 ) ||

nth( ISGEOM( Crv, GEOM_CIRCLE ), 1 );

checks if Crv is either line or a circle, ignoring the construction

entities.

ISOCLINE

--------

SurfaceType ISOCLINE( SurfaceType Srf, VectorType ViewDir,

NumericType Theta, NumericType Euc )

computes the isocline edges of the given Srf from the prescribed

viewing direction ViewDir.  Isocline curves are curves on the surface

at which location the surface normal forms a fixed angle, Theta,

in degrees, with the prescribed viewing direction, ViewDir.

The selection of 90 degrees for Theta results in the extraction of

silhouette edges.  The end result is a piecewise linear approximation of the

exact isocline edges, and its accuracy is controlled via the RESOLUTION

variable.

If Euc is TRUE, the isocline edges are returned on the surface,

in Euclidean space.  Otherwise, the isocline edges are returned in the

parametric space of Srf.

Example:

Resolution = 10;

Isocs = ISOCLINE( glass, vector( 1, -2, 1 ), 80, true );

computes the isocline edges forming 80 degrees between the surface

normal and the given viewing direction (1, -2, 1) for surface

glass, and returns the isocline edges in the Euclidean space.

See also SILHOUETTE.

KNOTCLEAN

---------

CurveType KNOTCLEAN( CurveType Crv, NumericType NormSamples )

cleans unnecessary knots from the given curve Crv.  The returned

curve is identical to the given curve, but in, possibly, a sub space with

less knots.  The L2 norm difference is measured via a discrete set of

NormSamples samples.  Note this function can undo refinement

operations.

c1 = pcircle( vector( 0, 0, 0 ), 1 );

c1r1 = crefine( c1, FALSE, list( 0.1, 0.3, 0.7, 1.5, 1.7, 1.7, 1.7, 1.7,

2.3, 2.3, 2.7, 3.5, 3.5, 3.5 ) );

c1r2 = KNOTCLEAN( c1r1, 100 );

c1 == c1r2;

refines a polynomial circle approximation and then restores the original

curve via the KNOTCLEAN operation.  The last line validates this cleaning.

See also KNOTREMOVE.

KNOTREMOVE

----------

CurveType KNOTREMOVE( CurveType Crv, NumericType GlobalLimit,

NumericType IterationLimit, NumericType NormSamples )

removes knots from curve Crv so as to keep the global error less than the

GlobalLimit.  This is an iterative process and in each iteration the

maximum allowed error is InterationLimit.  The error norm is computed

using a discrete set of NormSamples samples.

The GlobalLimit can be positive, in which case the error is measured

as the distance between the original and reduced curves.  Alternatively,

GlobalLimit can be negative, where it specifically states the number

of knots to remove.  In all cases the L2 norm is employed.

c1r1 = KNOTREMOVE( c1, -30, 10, 100 );

c1r2 = KNOTREMOVE( c1, 0.01, 0.002, 100 );

Assuming curve c1 has 50 knots, this reduces curve c1 to curve

c1r1 with 20 knots, by removing up to 10 knots per iteration; curve

c1r2 is the curve with the minimum number of knots possible such that the

global error (distance between c1 and c1r2) is less than

0.01 and that in each iteration no error larger than 0.002 is

introduced.  See also KNOTCLEAN.

LINTERP

-------

ListType LINTERP( ListType PtList)

computes a least squares fit of a line to a list of points, PtList.

A list of three elements, a point on the fitted line, a unit

vector in the direction of the line and the average distance between a point

and the fitted line, is returned.

Example:

R = 10;

Rx = Random( -1, 1 );

Ry = Random( -1, 1 );

Rz = Random( -1, 1 );

Pts = nil();

Len = 1.0;

NumPts = 100;

for ( i = 1, 1, NumPts,

Pt = ctlpt( E3, ( Random( -R, R ) + Rx * i * 2 ) / NumPts,

( Random( -R, R ) + Ry * i * -5 ) / NumPts,

( Random( -R, R ) + Rz * i * Pi ) / NumPts ):

snoc( Pt, Pts ) );

Pts = Pts * trans( vector( random( -10, -10 ),

random( -10, -10 ),

random( -10, -10 ) ) );

LnFit = LINTERP( Pts );

LnPos = nth( LnFit, 1 );

LnDir = nth( LnFit, 2 );

LnErr = nth( LnFit, 3 );

randomly samples 100 points to be approximately along a line and computes

a least squares fit of a line to this data. LnPos, LnDir, and

LnErr contain a point on the fitted line, the unit direction of the

fitted line and the average distance between a point and the line,

respectively.  See also CINTERP and SINTERP.

LOFFSET

-------

CurveType LOFFSET( CurveType Crv, NumericType OffsetDistance,

NumericType NumOfSamples, NumericType NumOfDOF,

NumericType Order )

approximates an offset of OffsetDistance by sampling NumOfSamples

samples along the offset curve and least square fitting them using a B-spline

curve of order Order and NumOfDOF control points.

Example:

OffCrv1 = LOFFSET( Crv, -0.4, 100, 10, 4 );

See also OFFSET, AOFFSET, and MOFFSET.

MATDECOMP

---------

ListType MATDECOMP( MatrixType Mat );

decomposes a given homogeneous transformation into its scaling and

translation vectors, and a pure (orthogonal) rotation matrix.

Example:

MATDECOMP( rx( 45 ) * sy( 3 ) * sx( 2 ) * tx( 5 ) * ty( 7 ) );

would result in the "(2, 3, 1)" scaling vector, "(5, 7, 0)" translation

vector and a rotation around X matrix of 45 degrees, all in one returned

list object.

MAXEDGELEN

----------

PolyType MAXEDGELEN( PolyType Pl, NumericTyoe MaxLen );

splits all polygons in polygonal object Pl to polygons with

edges no greater than MaxLen in length.

Example:

PlNew = MAXEDGELEN( Pl, 0.5 );

MBEZIER

-------

MultivarType MBEZIER( ListType Orders, ListType CtlPts )

creates a Bezier polynomial/rational multivariate out of the

provided control mesh.  Orders is a list of orders whose size

define the number of dimensions that the multivariate has.

CtlPts is a linear list of control points.  All control points

must be of type (E1-E9, P1-P9), or regular PointType defining the

multivariate's control mesh. The multivariate's point type will be

of a space which is the union of the spaces of all points.

Example:

MV = MBEZIER( list( 4 ),

list( ctlpt( E3, -1,  0.5, 2 ),

ctlpt( E1,  3 ),

ctlpt( E3,  0, -1.5, 0 ),

ctlpt( E2, -1,  3.5 ) ) );

constructs a univariate cubic multivariate object. See also MPOWER and

MBSPLINE

MBISECTOR

---------

ListType MBISECTOR( MultivarType MV1, MultivarType MV2, NumericType RetType,

NumericType SubdivTol, NumericType NumerTol )

computes the bisector surface in R3 of two surfaces or a curve and a

surface, posed as multivariate functions.

The returned results depend upon the value of RetType.

If RetType = 1, the algebraic constraints are returned as a list of

multivariates.  If RetType = 2, a list of points in R3 on the bisector

sheet(s) is returned.  Finally, if RetType = 3, a list of points in

(u, v, x, y, z) space, as E5 points, is returned, where (u, v) are the

respective parameter locations of the (must be) surface MV1.  These E5

points can then directly be employed by SINTERP through which to fit a

surface.

This bisector problem is posed as a set of two multivariate algebraic

constraints with three variables.  The simultaneous solution of these

constraints is computed using the MZERO function.  See MZERO for the

meaning of the SubdivTol and NumerTol tolerance.

Example:

s1 = sbezier(

list( list( ctlpt( E3, 0,  0,  0 ),

ctlpt( E3, 2,  0,  0 ) ),

list( ctlpt( E3, 0,  2,  0 ),

ctlpt( E3, 2,  2,  0 ) ) ) ) * tx( -1 ) * ty( -1 );

color( s1, red );

s2 = sbezier(

list( list( ctlpt( E3, 0,  0,  2 ),

ctlpt( E3, 1,  0,  1 ),

ctlpt( E3, 2,  0,  2 ) ),

list( ctlpt( E3, 0,  1,  1 ),

ctlpt( E3, 1,  1,  0 ),

ctlpt( E3, 2,  1,  1 ) ),

list( ctlpt( E3, 0,  2,  2 ),

ctlpt( E3, 1,  2,  1 ),

ctlpt( E3, 2,  2,  2 ) ) ) )* tx( -1 ) * ty( -1 );

color( s2, magenta );

ms1 = coerce( s1, multivar_type );

ms2 = coerce( s2, multivar_type );

mb1 = MBISECTOR( ms1, ms2, 3, 0.3, -0.001 );

b1 = sinterp( mb1, 3, 3, 4, 4, PARAM_UNIFORM );

mb2 = MBISECTOR( ms1, ms2, 2, 0.3, -0.001 );

interact( list( s1, s2, mb2, b1 ) );

c = cbezier( list( ctlpt( E3,  0,  0,  0 ),

ctlpt( E3,  0,  0,  2 ) ) );

color( c, red );

mc = coerce( c, multivar_type );

mb1 = MBISECTOR( mc, ms1, 3, 0.2, -0.001 );

b1 = sinterp( mb1, 3, 3, 8, 8, PARAM_UNIFORM );

mb2 = MBISECTOR( mc, ms1, 2, 0.2, -0.001 );

interact( list( c, s1, mb2, b1 ) );

computes two examples of a bisector between a plane and a biquadratic

surface and between a plane and a line.  The cloud of points is computed

twice, once interpolated by a surface, and also displayed as is.

MBSPLINE

--------

MultivarType MBSPLINE( ListType Lengths, ListType Orders,

ListType CtlPts, ListType KVLst )

creates a Bspline polynomial/rational multivariate out of the provided

control mesh of lengths Lengths and orders Orders in each axis.

The sizes of Lengths and Orders define the number of dimensions

that the multivariate has.

CtlPts is a linear list of control points.  All

control points must be of type (E1-E9, P1-P9), or regular PointType

defining the multivariate's control mesh. The multivariate's point

type will be of a space which is the union of the spaces of all

points.  KVLst is a list of knot sequences of the new Bspline

multivariate.

Example:

MV = MBSPLINE( list( 3, 3 ), list( 3, 3 ),

list( ctlpt( E1, 0 ),

ctlpt( E2, 0.25, 1 ),

ctlpt( E3, 0.5, 0.25, 2 ),

ctlpt( E3, 0.5, -1, 3 ),

ctlpt( E3, 0.75, 0.25, 4 ),

ctlpt( E3, 1, -0.5, 5 ),

ctlpt( E3, 1, 0, 6 ),

ctlpt( E3, 1.25, 1, 7 ),

ctlpt( E3, 1.3, 0.25, 8 ) ),

list( list( kv_open ),

list( kv_open ) ) );

constructs a bivariate quadratic multivariate object.  See also MPOWER and

MBEZIER.

MDERIVE

-------

MultivarType MDERIVE( MultivarType MV, NumericType Dir )

returns a vector field multivariate representing the differentiated

multivariate MV, in the given direction. Evaluation of the

returned multivariate at a given parameter value will return a vector

tangent to TV in Dir at that parameter value.

DMV = MDERIVE( MV, 2 );

computes the partial derivative of the multivariate MV with respect

to its second variable.  See also CDERIVE, SDERIVE, and TDERIVE.

MDIVIDE

-------

MultivarType MDIVIDE( MultivarType MV, ConstantType Direction,

NumericType Param )

subdivides a multivariate into two at the specified parameter value Param

in the specified Direction. MV can be either a B-spline

multivariate in which Param must be contained in the parametric

domain of the multivariate, or a Bezier multivariate in which Param

can be arbitrary, extrapolating if not in the range of zero to one.

It returns a list of the two sub-multivariates. The individual multivariates

may be extracted from the list using the NTH command.

Example:

MvDiv = MDIVIDE( Mv2, 3, 0.3 );

Mv2a = nth( MvDiv, 1 ) * tx( -2.2 );

Mv2b = nth( MvDiv, 2 ) * tx( 2.0 );

subdivides Mv2 at the parameter value of 0.3 in the direction 3 and

then extracts the two subdivided multivariate.

See also CDIVIDE, SDIVIDE, and TDIVIDE.

MERGEPOLY

---------

PolygonType MERGEPOLY( ListType PolyList )

merges a set of polygonal objects in the PolyList list to a single polygonal

object. All elements in the ObjectList must be of PolygonType type. This

function performs the same operation as the overloaded ^ operator

would, but may be more convenient to use under some circumstances.

Example:

Vrtx1 = vector( -3, -2, -1 );

Vrtx2 = vector( 3, -2, -1 );

Vrtx3 = vector( 3, 2, -1 );

Vrtx4 = vector( -3, 2, -1 );

Poly1 = poly( list( Vrtx1, Vrtx2, Vrtx3, Vrtx4 ), false );

Vrtx1 = vector( -3, 2, 1 );

Vrtx2 = vector( 3, 2, 1 );

Vrtx3 = vector( 3, -2, 1 );

Vrtx4 = vector( -3, -2, 1 );

Poly2 = poly( list( Vrtx1, Vrtx2, Vrtx3, Vrtx4 ), false );

Vrtx1 = vector( -3, -2, 1 );

Vrtx2 = vector( 3, -2, 1 );

Vrtx3 = vector( 3, -2, -1 );

Vrtx4 = vector( -3, -2, -1 );

Poly3 = poly( list( Vrtx1, Vrtx2, Vrtx3, Vrtx4 ), false );

PolyObj = MERGEPOLY( list( Poly1, Poly2, Poly3 ) );

MEVAL

-----

CtlPtType MEVAL( MultivarType MV, ListType Params )

evaluates the provided multivariate MV at the given Params

values. Params is a list of NumericTypes of length equal to the

dimension of the multivariate that must be contained in the multivariate

parametric domain, if MV is a B-spline multivariate, or all between zero

and one if MV is a Bezier multivariate. The returned control point has the

same type as the control points of MV.

Example:

CPt = MEVAL( MV1, list( 0.1, 0.25, 0.22, 0.7 ) );

evaluates the four-variate MV1 at the parameter values of

(0.1, 0.25, 0.22, 0.7).

See also CEVAL, SEVAL, TEVAL.

MFROMMESH

---------

MultivarType MFROMMESH( MultivarType MV, MumericType Dir, NumericType Index )

extracts a multivariate out of a multivariate, MV, as the Index's

plane of the control mesh of MV in direction Dir.

Example:

cmesh( s, row, 2 ) ==

coerce( MFROMMESH( coerce( s, multivar_type ), 1, 2 ), curve_type );

coerces surface s to a multivariate, extracts a one-dimensional-less

multivariate (a curve) from the second direction (first direction is

direction zero), at index 2 and compares the result for equality to

the curve extracted using cmesh from s.

MFROMMV

-------

MultivarType MFROMMV( MultivarType MV, NumericType Dir, NumericType Param )

extracts a multivariate of one lower dimension from multivariate MV by

extracting an iso-variate of MV in direction Dir at parameter

value Param.

Example:

MVFirst = MFROMMV( MV, 0, FirstParam );

extracts a multivariate for one less dimension than MV as the

constant first parameter of MV at parameter value FirstParam.

See also STRIVAR, CSURFACE.

MMERGE

------

MultivarType MMERGE( MultivarType MV1, MultivarType MV2, NumericType Dir,

NumericType Discont )

merges MV1 and MV2 together into one multivariate along the

direction Dir.  The first direction starts from zero.  If Discont,

the merge is assumed to be along a discontoinuous edge.

Example:

MVFirst = MMERGE( M1, M2, 2, false );

merges M1 and M2 along the third direction.

See also SMERGE,

MOFFSET

-------

CurveType MOFFSET( CurveType Crv, NumericType OffsetDistance,

NumericType AngularError )

computes an offset of OffsetDistance with a globally bounded error

(controlled by AngularError). The smaller the AngularError is, the better

the approximation to the offset. The bounded error is achieved by adaptive

refinement of the Crv.

The offset is computed via matching of the tangent fields of the

given curve Crv and an arc spanning the same angular domain.

Further, AngularError measures the angular deviation allowed

between the two tangent fields.

Example:

OffCrv1 = MOFFSET( Crv, -0.4, 10 );

OffCrv2 = MOFFSET( Crv, -0.4, 5 );

computes an offset approximation to Crv with OffsetDistance of

-0.4 and AngularError of 10 and 5 degrees, respectively.

See also OFFSET, AOFFSET, LOFFSET, and FFMATCH.

MOMENT

------

PointType MOMENT( CurveType Crv, 0 );

or

VectorType MOMENT( CurveType Crv, 1 );

approximate the zero and first moments of curve Crv.

Example:

a = circle( vector( 0, 0, 0 ), 1 );

a = cregion( a, 0, 1 );

p = moment( a, 0 );

v = moment( a, 1 );

view(list(a, p, v), on);

a = cregion( a, 0, 1 ) * rz( 45 );

p = moment( a, 0 );

v = moment( a, 1 );

view(list(a, p, v), on);

computes and displays the zero and first moments of a quarter of a circle

in two orientations.

MPOWER

------

MultivarType MPOWER( ListType Orders, ListType CtlPts )

creates a polynomial/rational multivariate out of the provided control mesh.

Orders is a list of orders whose size define the number of dimensions

that the multivariate has.

The created multivariate employs the monomial power basis. CtlPts is a

linear list of control points.

All control points must be of type (E1-E9, P1-P9), or regular PointType

defining the multivariate's control mesh. The multivariate's point type

will be of a space which is the union of the spaces of all points.

Example:

MV = MPOWER( list( 4 ),

list( ctlpt( E3, -1,  0.5, 2 ),

ctlpt( E3,  3, -1.5, 0 ),

ctlpt( E3,  0, -1.5, 0 ),

ctlpt( E3, -1,  3.5, 0 ) ) );

constructs a univariate cubic multivariate object.  See also MBEZIER and

MBSPLINE.

MRAISE

------

MultivarType MRAISE( MultivarType TV,

ConstantType Direction,

NumericType NewOrder )

raises Srf to the specified NewOrder in the specified

Direction.

Example:

MV2 = MRAISE( MRAISE( MV2, 0, 4 ), 1, 4 );

raises multivariate MV1 to a cubic in the first and second directions.

See also TRAISE, SRAISE, and CRAISE.

MRCHCUBE

--------

PolygonType MRCHCUBE( ListType VolumeSpec,

PointType CubeDim,

NumericType SkipFactor,

NumericType IsoVal )

applies (a variation of) the marching cubes algorithm (see

W. E. Lorensen and H. E. Cline. "Marching Cubes: A High Resolution 3D

Surface Construction Algorithm." Computer Graphics (SIGGRAPH '87

Proceedings), Vol. 21, No. 4, pp 163-169, July 1987.) to the given

volumetric data set or trivariate.  VolumeSpec can be a list of

four or five objects as follows:

4          a 4-tuple of the form

(TrivarType TV, NumericType Axis, NumericType SamplingFactor,

NumericType TVNormal)

5          a 5-tuple of the form

(StringType FileName, NumericType DataType, NumericType Width,

NumericType Height, NumericType Depth )

In the first case, the trivariate TV is iso surface contoured at level

IsoVal along the prescribed Axis (Note a trivariate need not

be a scalar function, whereas Marching Cubes assumes a scalar function).

The sampling rate of the trivariate is governed by SamplingFactor

with  SamplingFactor equal 1.0 sets sampling rate that equates with

the dimensions of the trivaariate (control mesh volume size).

If TVNormals is not zero, much more accurate normals are derived using

the trivariate function though it is also slower.  Otherwise, first order

differencing on the cubes is employed for normal estimation.

In the second case, the volume file prescribed by FileName is

loaded and iso surface contoured.  The file is assumed to hold

Width * Height * Depth (Width first, Depth order last) scalar

numeric values of type DataType:

1          Regular float or int ASCII (separated by white spaces)

2          Two bytes short integer.

3          Four bytes long integer.

4          One byte (char) integer.

5          Four bytes float.

6          Eight bytes double.

Beware of the little vs big Endian problem!  We assume here that you have read

the volume in the same machine type in which this file was written.

CubeDim allows the user to prescribe the real cell size (not

necessarily cubical). SkipFactor allows the skipping of data in large

data sets. SkipFactor = 1 skips nothing. SkipFactor = 2, skips

every other scalar value, reducing in half all dimensions, etc.

Last but not least, IsoVal sets the iso surface level.

See also COVERISO, TVLOAD, and TMORPH.

Example:

IsoSrf = MRCHCUBE( list( ThreeCyls, 1, 1, TRUE ), point( 1, 1, 1 ), 1, 0.12 );

iso surface contours the X axis of trivariate ThreeCyls and uses

the trivariate to get better normals' estimations.  Cell size is unit

cube like, no dat is skipped and the iso surface level is 0.12.

MREFINE

-------

MultivarType MREFINE( MultivarType TV, ConstantType Direction,

NumericType Replace, ListType KnotList )

provides the ability to Replace a knot vector of MV or refine

it in the specified direction Direction.

KnotList is a list of knots at which to refine MV. All knots should be

contained in the parametric domain of MV in Direction. If the knot

vector is replaced, the length of KnotList should be identical to the

length of the original knot vector of MV in Direction. If MV

is a Bezier multivariate, it is automatically promoted to be a B-spline

multivariate.

Example:

MV = MREFINE( MREFINE( MREFINE( MV,

0, FALSE, list( 0.333, 0.667 ) ),

1, FALSE, list( 0.333, 0.667 ) ),

2, FALSE, list( 0.333, 0.667 ) );

refines MV in the first three directions by adding two more knots at

0.333 and 0.667. See also CREFINE, SREFINE, and TREFINE.

MREGION

-------

MultivarType MREGION( MultivarType MV, ConstantType Direction,

NumericType MinParam, NumericType MaxParam )

extracts a region of MV between MinParam and MaxParam

in the specified Direction. Both MinParam and MaxParam

should be contained in the parametric domain of MV in Direction.

Example:

MV1r1 = TREGION( MV1, 3, 0.1, 0.2 );

MV1r2 = TREGION( MV1, 3, 0.4, 0.6 );

MV1r3 = TREGION( MV1, 3, 0.99, 1.0 );

extracts three regions of MV1 along the 4th (directions are counted

from zero) direction.  See also CREGION, SREGION, and TREGION.

MREPARAM

--------

MultivarType MREPARAM( MultivarType MV, ConstantType Direction,

NumericType MinParam, NumericType MaxParam )

reparametrizes MV over a new domain from MinParam to MaxParam,

in the prescribed Direction. This operation does not affect the geometry

of the multivariate and only affine transforms its knot vectors. A Bezier

multivariate will automatically be promoted into a B-spline surface by this

function.

Example:

MV = MREPARAM( MREPARAM( MV, 0, 0.1, 1.9 ),

1, 0.1, 0.9 );

ensures that the multivariate MV is defined over [0.1, 0.9] in the first

two directions.  See also CREPARAM, SREPARAM, and TREPARAM.

MREVERSE

--------

MultivarType MREVERSE( MultivarType MV, NumericType Dir1, NumericType Dir2 )

reverses MV by flipping the given two parametric directions, Dir1

and Dir2, (starting to count directions from zero).

Example:

RevMV = MREVERSE( MV, 2, 4 );

reverses MV by flipping the third and fifth directions of MV.

See also SREVERSE.

MVEXPLICIT

----------

MultivarType MVEXPLICIT( StringType Expression )

constructs a multivariate power basis from the given polynomial

Expression. The Expression can be any infix notational expression

using +-/*^ with no parenthesis.  The parameters are the 26 letters A-Z.

The dimension of the multivariate is governed by the highest letter found,

so having A,B,C in the expression constructs a trivariate, whereas having only

A constructs a univariate.  Having a higher letter with lower letter missing

constitutes an error.

Example:

M1 = coerce( mvexplicit( "A^2 + B^2 - 1" ), bezier_type );

M2 = coerce( mvexplicit( "4 * A^2 + B^2 / 4 - 1" ), bezier_type );

constructs two scalar saddle Bezier bivariate surfaces, represented as

multivariates.

MZERO

-----

ListType MZERO( ListType MVs, NumericType SubdivTol, NumericType NumerTol )

computes the simultaneous zeros of several scalar multivariate functions, in

MVs.  SubdivTol specifies the subdivision tolerance in the

parametric domain of the multivariates, whereas NumerTol prescribes the

tolerance of the numerical improvement stage.  A numerical improvement stage

is applied if |NumerTol| < SubdivTol.  If NumerTol is

negative, and a numeric improvement stage is indeed applied, all points that

fail to improve to the requested accuracy are purged away.

A list of control points, each designating one location in the

parameter space of the multivariates, is returned.

The number of multivariates cannot exceed the dimension of the

multivariates.  That is, if the MVs are trivariates, then, at most, three

of them may be provided.  If less are provided, then the dimension of the

solution space is larger than zero and a finite cloud of points

sampled from that solution space will be returned.

Example:

ZeroMVs = MZERO( list( MV1, MV2, MV3 ), 0.01, -1e-6 );

See also CONTOUR.  See MZERO for the meaning of SubdivTol and

NumerTol.

MPROMOTE

--------

PromMV = MPROMOTE( MultivarType MV, ListType AddDir );

or

PromMV = MPROMOTE( MultivarType MV, ListType NewDimStartAxis );

promote the multivariate MV to a higher dimension. In the first form

(a list of one numeric value), the multivariate will be promoted to have one

more dimension (i.e. a bivariate would become a trivariate).  The new added

axis will be AddDir.

The second form (a list of two numeric values) allows the original

multivariate to be placed at axes from StartAxis and have a new

dimensional NewDim.

Example:

ms = coerce( srf, multivar_type );

coerce( mfrommv( MPROMOTE( ms, list( 0 ) ), 0, 0.5 ), surface_type ) == srf;

coerces a surface to a multivariate, promotes it to a trivariate-multivariate,

extracts an iso-surface bivariate-multivariate along the new introduced axis

from the trivariate-multivariate and compares it to the original surface.  It

should be equal!

NIL

---

ListType NIL()

creates an empty list so data can be accumulated in it.

See CINFLECT or CZEROS for examples. See also LIST and SNOC.

OFFSET

------

PolygonType OFFSET( PolygonType Poly, NumericType OffsetDistance,

NumericType Tolerance, NumericType BezInterp )

or

CurveType OFFSET( CurveType Crv, NumericType OffsetDistance,

NumericType Tolerance, NumericType BezInterp )

or

CurveType OFFSET( CurveType Crv, CurveType OffsetDistance,

NumericType Tolerance, NumericType BezInterp )

or

SurfaceType OFFSET( SurfaceType Srf, NumericType OffsetDistance,

NumericType Tolerance, NumericType BezInterp )

or

TrimSrfType OFFSET( TrimSrfType TrimSrf, NumericType OffsetDistance,

NumericType Tolerance, NumericType BezInterp )

offset Poly, Crv, Srf or a TrimSrf, by translating

all the (control) points in the direction of the normal of the poly/curve

or of the (trimmed) surface by an OffsetDistance amount.

For a Poly object, the result is an offset of the original polygon/line

in the XY plane and is exact. Hence, Tolerance and BezInterp

parameters are ignored.  The result is a mitter joint based offset.

Otherwise, each control point has a node parameter value associated with

it, which is used to compute the normal. The returned curve or surface only

approximates the real offset. If the resulting approximation does not satisfy

the accuracy required by Tolerance, Crv or Srf or

TrimSrf is subdivided and an offset approximation fit is computed for

the two halves. For curves, one can request a Bezier interpolation scheme in

the offset approximation by setting BezInterp. BezInterp is

not yet supported for (trimmed) surfaces.

Negative OffsetDistance denotes offset in the reversed direction of the

normal.

If OffsetDistance is a (scalar) curve, the curve's first coordinate

is used to prescribe a variable offset amount along the curve for which we

compute the variable offset.  Both Crv and OffsetDistance must share

the same parametric domain.

Example:

OffCrv = OFFSET( Crv, -0.4, 0.1, off );

offsets Crv by the amount of -0.4 in the reversed normal direction,

Tolerance of 0.1 and no Bezier interpolation.

See also AOFFSET, LOFFSET and MOFFSET.

ORTHOTOMC

---------

CurveType ORTHOTOMC( CurveType Crv, PointType Pt, NumericType K )

or,

SurfaceType ORTHOTOMC( SurfaceType Srf, PointType Pt, NumericType K )

compute the K-orthotomic of freeform curves and surfaces.  See

Fundamentals of Computer Aided Geometric Design, by J. Hoschek and

D. Lasser.

Example:

pt = point( 0, 0.35, 0 );

crv = cbezier( list( ctlpt( E2, -0.8, -0.6 ),

ctlpt( E2, -0.3, -0.2 ),

ctlpt( E2,  0.0,  0.0 ),

ctlpt( E2,  0.8, -0.6 ) ) );

Orth = ORTHOTOMC( crv, pt, 2 );

interact( list( Orth, crv, pt ) * tx( 0.5 ) ) );

computes the orthotomic of a cubic Bezier curve that has an inflection

point.  Note that inflection points are reduced to cusps in the orthotomic

result.

PATTRIB

-------

AnyType PATTRIB( PolyType Poly, NumericType Index,

StringType Name, AnyType Value )

provides a mechanism to set/get an attribute to a vertex of a polygon.

Unlike the regular ATTRIB/RMATTR functions, PATTRIB allows access to the

Index vertex in polygon Poly, access that is otherwise impossible.

Index starts at zero for the first vertex.

The attribute will have a name Name and a value Value.  If Value

is NIL(), no attributes are set and the named attribute, if any, is returned.

This PATTRIB function only allows numeric values or strings as Value.

For example,

PATTRIB( Tri, 0, "rgb", "255,0,0");

PATTRIB( Tri, 1, "rgb", "0,255,0");

PATTRIB( Tri, 2, "rgb", "0,0,255");

sets the RGB values of the three vertices of triangle Tri.

See also PNORMAL, ATTRIB, ATTRPROP, GETATTR, RMATTR, CPATTR.

PCIRCLE

-------

CurveType PCIRCLE( VectorType Center, NumericType Radius )

is the same as CIRCLE but approximates the circle as a polynomial curve.

See also CIRCLE.

PCRVTR

------

PolyType PCRVTR( PolyType Pl, NumericType NumOfRings )

estimates curvature properties of given polygonal model Pl, assuming

Pl originated from a continuous freeform surfaces. NumOfRings

sets the number of rings around a vertex that will be used to estimate the

curvature properties of the vertex.

The return polygonal object is identical to Pl, but with the

following attributes set at each vertex:

"K1Curv"          First principal curvature value

"K2Curv"          Second principal curvature value

"KCurv"           The Gaussian Curvature

"HCurv"           The Mean Curvature

"D1"              The first principal direction

"D2"              The second principal direction

See also PPROPFTCH.

PDECIMATE

---------

PolygonType PDECIMATE( PolygonType Obj, NumericType DecimType,

NumericType Threshold )

Given a polygonal model, Obj, decimate and merge polygons, effectively

reducing the size of the data subject to a maximal deviation distance as

controlled via Threshold and DecimType.  DecimType can be

either TRUE when Threshold has a continuous zero to one control over

the output size or FALSE when Threshold prescribes the exact number of

polygons desired.

Example:

gcross = cbspline( 3,

list( ctlpt( E3, 0.3, 0.0, 0.0 ),

ctlpt( E3, 0.1, 0.0, 0.1 ),

ctlpt( E3, 0.1, 0.0, 0.4 ),

ctlpt( E3, 0.5, 0.0, 0.5 ),

ctlpt( E3, 0.6, 0.0, 0.8 ) ),

list( KV_OPEN ) );

resolution = 30;

glass = surfprev( gcross );

pglass = gpolygon( glass, false );

dglass = PDECIMATE( pglass, false, 0.5 );

creates a surface of a glass, approximates it with polygons and then

decimates the latter.

PDOMAIN

-------

ListType PDOMAIN( FreeformType Freeform )

returns the parametric domain of the given Freeform.

See also MESHSIZE, FFCTLPTS, FFKNTVEC, FFMESH, FFMSIZE, FFPTTYPE, FFORDER.

Example:

circ_domain = PDOMAIN( circle( vector( 0.0, 0.0, 0.0 ), 1.0 ) );

PLANE

-----

PointType PLANE( NumericType A, NumericType B, NumericType C, NumericType D )

creates a plane type object, using the four provided NumericType

coefficients. See also VECTOR, POINT.

PLANECLIP

---------

ListType PLANECLIP( PolyType Poly, PlaneType Pln )

clips a polygonal model Poly against a plane Pln.

Three polygonal objects are returned in a list: polygons on the positive side

of the plane, polygons that intersect the plane, and polygons on the negative

side of the plane, in this order.

If one of these lists is empty, a numeric zero is substituted.

Example:

Pls = PLANECLIP( Pl, plane( 1, 1, 0, 0 ) );

clips polygonal object Pl against the plane X+Y=0.

PLN3PTS

-------

PlaneType PLN3PTS( PointType Pt1, PointType Pt2, PointType Pt3 )

computes a plane out of three points.

Example:

Pl1 = PLN3PTS( point( 0, 0, 0 ), point( 0, 1, 0 ), point( 1, 0, 0 ) );

PMORPH

------

PlaneType PMORPH( PolyType Pl1, PolyType Pl2, NumericType Blend )

creates a new polygonal object which is a metamorph of the two given

polygonal objects that share the same topology.  That is, Pl1 and Pl2

must share the same number of polygons and the i'th polygon in Pl1 must

be equal in its number of vertices to the i'th polygon of Pl2.

This is very useful if a sequence that "morphs" one polygonal model to

another is to be created.

Example:

Pl1 = con2( vector( 0.0, -0.5, -0.5 ), vector( 0.0, 0.0, 1.0 ), 0.4, 0.1, 3 );

Pl2 = con2( vector( 0.0, 0.5, 0.0 ), vector( 0.0, 0.0, 1.0 ), 0.1, 0.4, 3 );

Pl = PMORPH( Pl1, Pl2, 0.5 );

creates a cylinder out of two truncated cones, using PMORPH.  See also

CMORPH and SMORPH.

PNORMAL

-------

PointType PNORMAL( PolyType Poly, NumericType Index, VectorType Normal )

provides a mechanism to set/get the normal of vertex number Index

in a polygon Poly.  Index starts at zero for the first vertex.

Normal replaces the current normal that is also returned.  If

Normal is not a VectorType, no new normal is set but the current

normal is still returned, allowing normals to be queried.

For example,

PNORMAL( Tri, 0, vector( 1, 0, 0 ) );

PNORMAL( Tri, 1, vector( 0, 1, 0 ) );

sets the normals of the first two vertices in triangle Tri to be

the X and Y axes, respectively.

See also PATTRIB.

POINT

-----

PointType POINT( NumericType X, NumericType Y, NumericType Z )

creates a point type object, using the three provided NumericType scalars.

See also VECTOR, PLANE.

POLY

----

PolygonType POLY( ListType VrtxList, NumericType IsPolyline )

creates a single polygon/polyline (and therefore open) object, defined by

the vertices in VrtxList (see LIST). All elements in VrtxList

must be one of PointType, VectorType, CtlPtType, or PolygonType types.

If IsPolyline, a polyline is created; otherwise, a polygon.

Example:

V1  = vector( 0.0, 0.0, 0.0 );

V2  = vector( 0.3, 0.0, 0.0 );

V3  = vector( 0.3, 0.0, 0.1 );

V4  = vector( 0.2, 0.0, 0.1 );

V5  = vector( 0.2, 0.0, 0.5 );

V6  = vector( 0.3, 0.0, 0.5 );

V7  = vector( 0.3, 0.0, 0.6 );

V8  = vector( 0.0, 0.0, 0.6 );

V9  = vector( 0.0, 0.0, 0.5 );

V10 = vector( 0.1, 0.0, 0.5 );

V11 = vector( 0.1, 0.0, 0.1 );

V12 = vector( 0.0, 0.0, 0.1 );

I = POLY( list( V1, V2, V3, V4, V5, V6, V7, V8, V9, V10, V11, V12 ),

FALSE );

constructs an object with a single polygon in the shape of the letter I.

PPINTER

-------

ListType PPINTER( PolyType Pl1, PolyType Pl2 )

computes the intersection of two individual polygons in R3, Pl1 and

Pl2.  Similar results can also be obtained via Boolean operations.

Example:

Pl1 = poly( list( point( -1, -1, 0 ),

point( -1,  1, 0 ),

point(  1,  1, 0 ),

point(  1, -1, 0 ) ), false );

Pl2 = Pl1 * rx( 70 ) * tx( 0.5 );

Inter1 = PPINTER( Pl1, Pl2 );

iritstate( "intercrv", true );

Inter2 = Pl1 * Pl2;

computes the intersection edge of two polygons in two different ways. Note,

however, that while PPINTER considers only the first polygon in a

polygonal object, the Boolean operations considers them all.

PPROPFTCH

---------

PolyType PPROPFTCH( PolyType Pl, NumericType PropType, ListType PropParam )

computes piecwise linear curves over polygonal mesh Pl.  The extracted

curves could be one of,

Property         PropType        PropParam

Attribute Value  0               list( AttrName, AttrValue )

Isophotes        1               list( ViewDir, InclinationAngle )

Gaussian Crvtr   2               list( NumRingCrvtrAprx, CrvtrVal )

Mean Crvtr       3               list( NumRingCrvtrAprx, CrvtrVal )

The NumRingCrvtrAprx specifies how many rings around a vertex should be

considered when the curvature of the vertex is estimated.  Typically 1.

Example:

Pl1 = PPropFtch( Srf, 1, list( normalize( vector( 1,  1, 1 ) ), 90 ) );

Pl2 = PPropFtch( Srf, 1, list( normalize( vector( 1, -1, 1 ) ), 90 ) );

Pl3 = PPropFtch( Srf, 1, list( normalize( vector( 1,  0, 1 ) ), 90 ) );

extracts silhouettes from surface Srf (note an InclinationAngle of 90

degrees extract silhouettes), from three different viewing direction.

See also PCRVTR, SILHOUETTE, ISOCLINE, PPROPFTCH and SASPCTGRPH.

PRINTER

--------

ListType PRINTER( PolyType Pl, NumericType RayPt, NumericType RayDir )

computes the number of XY planar intersection of ray (RayPt, RayDir)

with a single polygon Pl.  Returned is the number of interesections

found.

PRISA

-----

ListType PRISA( SurfaceType Srfs, NumericType SamplesPerCurve,

NumericType Epsilon, ConstantType Dir, VectorType Space,

NumericType CrossSecs )

or

ListType PRISA( TrimSrfType TrimSrfs, NumericType SamplesPerCurve,

NumericType Epsilon, ConstantType Dir, VectorType Space,

NumericType CrossSecs )

compute a layout (prisa) of the given surface(s) Srfs or {TrimSrfs},

and return a list of (trimmed) surface objects representing the layout.

The surface is approximated to within Epsilon in direction Dir

into a set of ruled surfaces, and then developable surfaces that are laid out

flat onto the XY plane. If Epsilon is negative, the piecewise ruled

surface approximation in 3-space is returned.

SamplesPerCurve controls the piecewise linear approximation of the

boundary of the ruled/developable surfaces. Space is a vector whose

X component controls the space between the different surfaces' layout, and

whose Y component controls the space between different layout pieces.

If CrossSecs is not zero, the 3D cross sections, approximated as

planar, of each laid out region are also provided.

Example:

cross = cbspline( 3,

list( ctlpt( E3, 0.7, 0.0, 0. ),

ctlpt( E3, 0.7, 0.0, 0.06 ),

ctlpt( E3, 0.1, 0.0, 0.1 ),

ctlpt( E3, 0.1, 0.0, 0.6 ),

ctlpt( E3, 0.6, 0.0, 0.6 ),

ctlpt( E3, 0.8, 0.0, 0.8 ),

ctlpt( E3, 0.8, 0.0, 1.4 ),

ctlpt( E3, 0.6, 0.0, 1.6 ) ),

list( KV_OPEN ) );

wglass = surfrev( cross );

wgl_ruled = PRISA( wglass, 6, -0.1, COL, vector( 0, 0.25, 0.0 ), false );

wgl_prisa = PRISA( wglass, 6, 0.1, COL, vector( 0, 0.25, 0.0 ), true );

computes a layout of a wine glass in wgl_prisa and a three-dimensional

ruled surface approximation of wglass in wgl_ruled.

PT3BARY

-------

VectorType PT3BARY( PointType Pt1, PointType Pt2, PointType Pt3,

PointType InteriorPt )

computes the barycentric coordinates of InterPt with respect

to the triangle defined by Pt1, Pt2, Pt3.

A vector of three coefficents, which are the weights of the three

points of the triangle, are returned. InteriorPt is assumed to be in the

triangle.

Example:

Coeffs = PT3BARY( point( 0, 0, 0 ),

point( 1, 0, 0 ),

point( 0, 1, 0 ),

point( 0.25, 0.25, 0.0 ) );

PTHMSPR

-------

ListType PTHMSPR( NumericType Size )

computes a fairly uniform distribution of points on a hemisphere.

Size hints at the distance between adjacent placed points.

Example:

Pts = PTHMSPR( 0.1 );

PTLNPLN

-------

VectorType PTLNPLN( PointType LineOrig, VectorType LineRay, PlaneType Plane )

computes the point of intersection of given line LineOrig,

LineRay with plane Plane.

Example:

InterPt = PtLnPln( point( 1, 0, 1 ), vector( 1, 1, 1 ), Plane( 0, 0, 1, 0 ) );

PTPTLN

------

VectorType PTPTLN( PointType Point, PointType LineOrig, VectorType LineRay )

computes the point on line LineOrig, LineRay that is closest

to point Point.  See also DSTPTLN.

Example:

ClosestPt = PTPTLN( point( 0, 0, 0 ), point( 1, 1, 0 ), vector( 1, 1, 1 ) );

PTREGISTER

----------

MatrixType PTREGISTER( ListType PtSet1, ListType PtSet2,

NumericType StepSize, NumericType Tolerance )

registers one points set, PtSet1, with another, PtSet2.

The two points sets are assumed to be rigid motion of one another.

StepSize controls the step size of the numerical process and must be

a positive real less than 1.0.  The larger  StepSize is, the faster the

convergance with less stability. Finally, Tolerance prescribes the

necessary accuraacy in L-infinity sense.  This function will converge for

small rotational deviations only.

Pt1 = nil();

for (i = 0, 1, 15,

Pt = point( random( -.7, .7 ), random( -.7, .7 ), random( -.7, .7 ) ):

snoc( Pt * tx( 0 ), Pt1 ) );

Pt2 = Pt1 * rx( 13 ) * ry( 5 ) * rz( 11 )

* tx( 0.1 ) * ty( 0.03 ) * tz( -0.05 );

Tr = PTREGISTER( Pt1, Pt2, 1, 1e-6 );

PTS2PLLN

--------

ListType PTS2PLLN( ListType Points, NumericType MaxMatchDist )

matches the given cloud of points in a list of polylines.

MaxMatchDist is used as the maximal distance between two adjacent

points to connect.

Example:

Pts = nil();

for ( i = 0, 1, 100,

t = random( 0, 2 * Pi ):

snoc( point( cos( t ), sin( t ), 0 ), Pts ) );

Pll = PTS2PLLN( Pts, 0.1 );

connects 100 random points on the unit circle into a polyline

approximating an (almost) complete circle.

PTSLNLN

-------

ListType PTSLNLN( PointType Line1Orig, VectorType Line1Ray,

PointType Line2Orig, VectorType Line2Ray )

computes the closest two points on the two lines defined by point

LineiOrig and ray LineiRay. See also DSTLNLN.

A list object with the two points is returned.

Example:

ClosestPts = PtsLnLn( point( 1, 0, 0 ), vector( 0, 1, 0 ),

point( 0, 1, 0 ), vector( 1, 0, 0 ) );

QUADRIC

-------

ListType QUADRIC( ListType ABCDEFGHIJ ) )

constructs a quadric parametric surface whose coefficients are the ten

coefficients in the list ABCDEFGHIJ:

A x^2 + B y^2 + C z^2 + D xy + E xz + F yz + G x + H y + I z + J = 0.

Example:

Sph   = quadric( list( 1,  1 , 1 , 0,  0,  0,  0,  0,  0, -1 ) );

Hyp1s = quadric( list( 1,  1, -1,  0,  0,  0,  0,  0,  0, -1 ) );

Hyp2s = quadric( list( 1, -1, -1,  0,  0,  0,  0,  0,  0, -1 ) );

constructs three quadric surfaces, a portion of the sphere, a portion of

a hyperboloid of one sheet and a portion of a hyperboloid of two sheets.

Because the parametrization of the quadric is predetermined, one might need

to use SREGION and SMOEBIUS to extract subregions and/or reparametrize the

surface.

RAYTRAP

-------

ListType RAYTRAP( ListType Crvs,

NumericType SubdivTol, NumericType NumerTol )

computes locations on the given planar curves that would bounce rays

from one curve to the next, in an infinite cycle.  Such traps are denoted

ray traps.  Ray-traps are computed for the given list of

Crvs, in the given order.  The ray-trap problem is posed as a

set of n multivariate algebraic constraints with n variables, given

n curves prescribed in Crvs.  The simultaneous solution of these

constraints is computed using the MZERO function.  See MZERO for the

meaning of the SubdivTol and NumerTol tolerance.

Example:

Crv1 = pcircle( vector( -0.75, -0.75, 0 ), 0.5 );

Crv2 = Crv1 * sc( 1.5 ) * tx( 2 );

Crv3 = Crv1 * sc( 0.5 ) * tx( 0.2 ) * ty( 0.6 );

Tris = RayTrapTris( list( Crv1, Crv2, Crv3 ), 0.1, -1e-6 );

computes the ray-traps between three circles.

RFLCTLN

-------

ListType RFLCTLN( SurfaceType Srf, VectorType ViewDir,

ListType LinesSprs, NumericType Euclidean )

computes reflection lines/ovals to the given surface Srf as

seen from view direction ViewDir.  The resulting piecewise linear

curves are in Euclidean space if Euclidean is TRUE and in Srf

parameter space, otherwise.

The reflection/ovals themselves are defined via LinesSprs.

For reflection lines, LinesSprs consists of

list( LineDir, list( LinePos1, LinePos2, ... , LinePosN ) );

defining n parallel lines with direction LineDir through

point LinePos1 to LinePosN.

For reflection ovals, LinesSprs consists of

list( SprCntr, list( Rad1, Rad2, ... , RadN ) );

defining n co-spherical spheres, all located at SprCntr with

radii of Rad1 to RadN.

Example:

resolution = 20;

RefLns = RflctLn( Srf,

vector( 0, 1, 2 ),

list( vector( 0, 0, 1 ),

list( point( -3.0, 2, 0 ),

point( -1.5, 2, 0 ),

point(  0.0, 2, 0 ),

point(  1.5, 2, 0 ),

point(  3.0, 2, 0 ) ) ),

true );

RefOvals = RflctLn( Srf,

vector( 1, 1, 0 ),

list( point( 0, 2, 0 ),

list( 5, 25, 45, 65, 85 ) ),

true );

computes the reflection lines of surface Srf from viewing

direction ( 0, 1, 2 ) having five reflected lines and computes five

reflection ovals from viewing direction ( 0, 1, 2 ).

See also ReflectLns attributes in the display devices.

RRINTER

-------

ListType RRINTER( CurveType Srf1Crv1, CurveType Srf1Crv2,

CurveType Srf2Crv1, CurveType Srf2Crv2,

NumericType Tolerance, NumericType ZeroSetFunc )

computes the intersection curves of the given two ruled surfaces,

defined as

SrfiCrv1 * v + SrfiCrv2 * (1 - v)", i = 1, 2, v in [0,1].

SrfiCrv1 * v~ +~ SrfiCrv2 * (1 - v),~~ i = 1, 2,~~v in [0,1].

The ruled ruled intersection (RRI) problem is tranformed into a zero set

finding on another function.  If ZeroSetFunc is true, the function

whose zero set provides the RRIsolution is returned.  Otherwise, if

ZeroSetFunc is false, the RRI solution itself is returned.  The zero

set is computed via numerical zero set finding methods and Tolerance

controls the fineness of the approximated solution.  If Tolerance is

negative, the absolute value is employed as Tolerance but the

intersection curves are computed as if the two ruled surfaces are infinite

(i.e. v is unbounded).

Example:

c1 = cbezier( list( ctlpt( E3, -1.0,  -1.0, -1.0 ),

ctlpt( E3, -0.5,   8.0, -1.0 ),

ctlpt( E3,  0.0, -15.0, -1.0 ),

ctlpt( E3,  0.5,   8.0, -1.0 ),

ctlpt( E3,  1.0,  -1.0, -1.0 ) ) );

c2 = c1 * sc( 0.7 ) * tz( 1.7 );

r1 = ruledSrf( c1, c2 );

c1 = pcircle( vector( 0, 0, 0 ), 0.3 ) * tz( 2 );

c2 = c1 * sc( 0.5 ) * tz( -3 );

r2 = ruledSrf( c1, c2 ) * ry( 90 );

c = RRINTER( cMesh( r1, row, 0 ),

cMesh( r1, row, 1 ),

cMesh( r2, row, 0 ),

cMesh( r2, row, 1 ),

25, false );

interact( list( r1, r2, nth( c, 1 ) ) );

RULEDSRF

--------

SurfaceType RULEDSRF( CurveType Crv1, CurveType Crv2 )

or

PolygonType RULEDSRF( PolygonType Poly1, PolygonType Poly2 )

construct a ruled surface between the two curves Crv1 and Crv2

or two polylines Poly1 and Poly2.

The curves do not have to have the same order or type, and will be promoted

to their least common denominator.  The polys must have the same number of

points and both must be either polygons or polylines.

Example:

c1 = cbspline( 3,

list( ctlpt(E3, 1.7, 0.0 , 0  ),

ctlpt(E3, 0.7, 0.7 , 0  ),

ctlpt(E3, 1.7, 0.3 , 0  ),

ctlpt(E3, 1.5, 0.8 , 0  ),

ctlpt(E3, 1.6, 1.0 , 0  ) ),

list( KV_OPEN ) );

c2 = cbspline( 3,

list( ctlpt(E3, 0.7, 0.0 , 0  ),

ctlpt(E3,-0.7, 0.2 , 0  ),

ctlpt(E3, 0.7, 0.5 , 0  ),

ctlpt(E3,-0.7, 0.7 , 0  ),

ctlpt(E3, 0.7, 1.0 , 0  ) ) ,

list( KV_OPEN ) );

srf1 = RULEDSRF( c1, c2 );

interact( list( c1, c2, srf1 ), on );

c2a = ffmatch( c1, c2, 50, 100, 2, false, 1 );

srf2 = RULEDSRF( c1, c2a );

interact( list( c1, c2, srf2 ), on );

constructs a planar ruled surface between two curves, c1 and c2.

The naive construction causes self intersection, but by employing FFMATCH

the self intersection can be resolved.

See also FFMATCH.

RULEDTV

-------

TrivarType RULEDTV( SurfaceType Srf1, SurfaceType Srf2 )

constructs a ruled trivariate between the two surfaces Srf1 and

Srf2.

The surfaces do not have to have the same order or type, and will be

promoted to their least common denominator.

Example:

s1 = boolone( pcircle( vector( 0, 0, 0 ), 1 ) );

s2 = boolone( pcircle( vector( 0, 0, 1 ), 0.5 ) );

tv = RULEDTV( s1, s2 );

constructs a truncated cone-volume as a ruled trivariate between two surfaces,

s1 and s2.

See also EXTRUDE, TFROMSRFS.

SACCESS

-------

ListType SACCESS( SurfaceType AccessSrf,

AnyType OrientFieldSrf,

SurfaceType CheckSrf,

AnyType AccessLimitDir,

NumericType SubdivTol,

NumericType NumericTol )

computes the domain on the AccessSrf surface that is accessible from the

orientation that is optionally prescribed by OrientFieldSrf, without

gouging into the CheckSrf surface.  If OrientFieldSrf is not

a surface, the normal field of AccessSrf is employed.

AccessSrf and OrientFieldSrf must share a (u, v) domain,

whereas CheckSrf can present a different (s, t) domain.

If AccessLimitDir is indeed a vector, the access is limited for

direction V such that the inner product of V and AccessLimitDir is

positive.

The accuracy of the computation is governed by a two stage solution,

a subdivision stage with tolerance SubdivTol followed by a numerical

improvement stage with NumericTol accuracy.  The second, numeric,

stage is invoked only if NumericTol < SubdivTol.

The returned results are a set of points on the boundary of the accessible

region.  The points are in E4 space as (u, v, s, t) 4-tuples.

Example:

c = cregion( pcircle( vector( 0, 0, 0 ), 1 ), 1, 3 ) * ry( 90 );

pSphere = surfPRev( c ) * sc( 0.3 ) * tz( 1 );

Pln = ruledSrf( ctlpt( E3, -1, -1, 0 ) + ctlpt( E3, -1,  1, 0 ),

ctlpt( E3,  1, -1, 0 ) + ctlpt( E3,  1,  1, 0 ) );

Pts = SACCESS( Pln, 0, pSphere, 0, 0.1, 1e-5 );

sPts = nil();

sPtsErr = nil();

for ( i = 1, 1, sizeof( Pts ),

Pt = nth( Pts, i ):

Err = getAttr( Pt, "Error"):

if ( Err > 1e-5,

snoc( seval( Pln, coord( Pt, 1 ), coord( Pt, 2 ) ), sPtsErr ),

snoc( seval( Pln, coord( Pt, 1 ), coord( Pt, 2 ) ), sPts ) ) );

color( sPts, green );

color( sPtsErr, red );

interact( list( pSphere, Pln, sPts, sPtsErr ) );

computes the access domain of plane Pln along the normal, Z, direction

while preventing gouging into the check surface pSphere.

See MZERO for the meaning of SubdivTol and NumerTol.

SASPCTGRPH

----------

PolyType SASPCTGRPH( SurfaceType Srf )

approximates the aspect graph of surface Srf by computing the

principal directions with zero curvature at the parabolic points of Srf.

The aspect graph is defined over the unit sphere and identifies all

direction from which the silhouette curves of Srf change topology.

Example:

AG = SAspctGrph( Srf );

See also SILHOUETTE.

SASYMPEVAL

----------

ListType SASYMPEVAL( SurfaceType Srf, NumericType U, NumericType V,

NumericType Euclidean )

evalutes the asymptotic direction of surface Srf at parametric

location (U, V), if any.  If Euclidean is not zero, the directions

are returned in Euclidean space, otherwise, in parametric space.  Returned

is a list of upto two vectors.

Example:

AsympDir = SAsympEval( Srf, u, v, true );

See also SCRVTR.

SBEZIER

-------

SurfaceType SBEZIER( ListType CtlMesh )

creates a Bezier surface using the provided control mesh. CtlMesh is a

list of rows, each of which is a list of control points. All control points

must be of type (E1-E9, P1-P9), or regular PointType defining the surface's

control mesh. The surface's point type will be of a space which is the

union of the spaces of all points.

Example:

Srf = SBEZIER( list ( list( ctlpt( E3, 0.0, 0.0, 1.0 ),

ctlpt( E3, 0.0, 1.0, 0.0 ),

ctlpt( E3, 0.0, 2.0, 1.0 ) ),

list( ctlpt( E3, 1.0, 0.0, 0.0 ),

ctlpt( E3, 1.0, 1.0, 2.0 ),

ctlpt( E3, 1.0, 2.0, 0.0 ) ),

list( ctlpt( E3, 2.0, 0.0, 2.0 ),

ctlpt( E3, 2.0, 1.0, 0.0 ),

ctlpt( E3, 2.0, 2.0, 2.0 ) ),

list( ctlpt( E3, 3.0, 0.0, 0.0 ),

ctlpt( E3, 3.0, 1.0, 2.0 ),

ctlpt( E3, 3.0, 2.0, 0.0 ) ),

list( ctlpt( E3, 4.0, 0.0, 1.0 ),

ctlpt( E3, 4.0, 1.0, 0.0 ),

ctlpt( E3, 4.0, 2.0, 1.0 ) ) ) );

See also CBEZIER, SBSPLINE and SPOWER.

SBISECTOR

---------

SurfaceType SBISECTOR( SurfaceType Srf, PointType Pt )

computes the bisector surface of a given surface to a point.

See also CBISECTOR.

Example:

s = ruledSrf( ctlpt( E3, -1.0, -1.0, 0.0 ) + ctlpt( E3,  1.0, -1.0, 0.0 ),

ctlpt( E3, -1.0,  1.0, 0.0 ) + ctlpt( E3,  1.0,  1.0, 0.0 ) );

pt = point( 0.0, 0.0, 1.0 );

bisect = SBISECTOR( s, pt );

interact( list( s, pt, bisect ) );

computes the bisector surface of a plane and a point.

SBSPLINE

--------

SurfaceType SBSPLINE( NumericType UOrder, NumericType VOrder,

ListType CtlMesh, ListType KnotVectors )

creates a B-spline surface from the provided UOrder and VOrder

orders, the control mesh CtlMesh, and the two knot vectors KnotVectors.

CtlMesh is a list of rows, each of which is a list of control points.

All control points must be of point type (E1-E9, P1-P9), or regular

PointType defining the surface's control mesh. The surface's point type will

be of a space which is the union of the spaces of all points.

KnotVectors is a list of two knot vectors. Each knot vector is a

list of NumericType knots of length #CtlPtList plus the Order.

If, however, the length of the knot vector is equal to #CtlPtList +

Order + Order - 1}, the curve is assumed to be periodic.

The knot vector may also be a list of a single constant KV_OPEN or

KV_FLOAT or KV_PERIODIC, in which a uniform knot vector with the

appropriate length and with an open, floating or periodic end condition

will be constructed automatically.

Example:

Mesh = list ( list( ctlpt( E3, 0.0, 0.0, 1.0 ),

ctlpt( E3, 0.0, 1.0, 0.0 ),

ctlpt( E3, 0.0, 2.0, 1.0 ) ),

list( ctlpt( E3, 1.0, 0.0, 0.0 ),

ctlpt( E3, 1.0, 1.0, 2.0 ),

ctlpt( E3, 1.0, 2.0, 0.0 ) ),

list( ctlpt( E3, 2.0, 0.0, 2.0 ),

ctlpt( E3, 2.0, 1.0, 0.0 ),

ctlpt( E3, 2.0, 2.0, 2.0 ) ),

list( ctlpt( E3, 3.0, 0.0, 0.0 ),

ctlpt( E3, 3.0, 1.0, 2.0 ),

ctlpt( E3, 3.0, 2.0, 0.0 ) ),

list( ctlpt( E3, 4.0, 0.0, 1.0 ),

ctlpt( E3, 4.0, 1.0, 0.0 ),

ctlpt( E3, 4.0, 2.0, 1.0 ) ) );

Srf = SBSPLINE( 3, 3, Mesh, list( list( KV_OPEN ),

list( 3, 3, 3, 4, 5, 6, 6, 6 ) ) );

constructs a bi-quadratic B-spline surface with its first knot vector

having a uniform knot spacing with open end conditions.

See also CBSPLINE, SBEZIER and SPOWER.

SCRVTR

------

SurfaceType SCRVTR( SurfaceType Srf, ConstType PtType, ConstType Dir )

symbolically computes the extreme curvature bound on Srf.

If Dir is either ROW or COL, then the normal curvature square

of Srf in Dir is computed symbolically and returned.

Otherwise, an upper bound on the sum of the squares of the two

principle curvatures is symbolically computed and returned.

The returned value is a surface that can be evaluated to the curvature

bound, given a UV location. The returned surface value is a scalar field

of point type P1 (scalar rational).

However, if PtType is one of E1, P1, E3, or P3, the returned surface

is coerced to this given type. If the types are one of E3, or P3, then the

Y and Z axes are set to be equivalent to the U and V parametric domains.

This function computes the square of the normal curvature scalar

field for surfaces as (in the U parametric direction, same for V),

2

d S

< n , --- >

2

u               du

k (u, v) = ------------

n

dS   ds

< -- , -- >

du   du

and computes the sum of the squares of the principle curvatures as,

2    2  ( g11 l22 + g22 l11 - 2 g12 l12 )^2 - 2 |G| |L|

k  + k = -----------------------------------------------

1    2                   |G|^2 ||n||^2

See also CCRVTR, SASYMPEVAL.

Example:

cross = cbspline( 3,

list( ctlpt( E2,  0.0,  0.0 ),

ctlpt( E2,  0.8,  0.0 ),

ctlpt( E2,  0.8,  0.2 ),

ctlpt( E2,  0.07, 1.4 ),

ctlpt( E2, -0.07, 1.4 ),

ctlpt( E2, -0.8,  0.2 ),

ctlpt( E2, -0.8,  0.0 ),

ctlpt( E2,  0.0,  0.0 ) ),

list( KV_OPEN ) );

cross = coerce( cross, e3 );

s = sFromCrvs( list( cross,

cross * trans( vector( 0.5, 0, 1 ) ),

cross * trans( vector( 0, 0, 2 ) ) ), 3, KV_OPEN );

view( list( s, axes ), on );

UCrvtrZXY = scrvtr( s, E3, row );

VCrvtrZXY = scrvtr( s, E3, col );

UCrvtrXYZ = UCrvtrZXY * rotx( -90 ) * roty( -90 ) * scale( vector( 1, 1, 0.001 ) );

VCrvtrXYZ = VCrvtrZXY * rotx( -90 ) * roty( -90 ) * scale( vector( 1, 1, 10 ) );

color( UCrvtrXYZ, red );

color( VCrvtrXYZ, magenta );

view( list( UCrvtrXYZ, VCrvtrXYZ ), off );

CrvtrZXY = scrvtr( s, E3, off );

CrvtrXYZ = CrvtrZXY * rotx( -90 ) * roty( -90 ) * scale( vector( 1, 1, 0.001 ) );

color( CrvtrXYZ, green );

view( CrvtrXYZ, off );

computes the square of the normal curvature in the U and V directions, flips

its scalar value from X to Z using rotations and scales the fields to

reasonable values, and then displays them.

It also displays a total bound on the normal curvature.

Due to the large degree of the resulting fields, be aware that rational

surfaces will compute into large degree curvature bound fields. See also

IRITSTATE("InterpProd", FALSE); for faster symbolic computation.

SCRVTREVAL

----------

ListType SCRVTREVAL( SurfaceType Srf, NumericType U, NumericType V )

computes the principle curvatures and directions of surface Srf

at parametric location (U, V).  A list of four elements

(k1, V1, k2, V2), with k1/V1 being the first principle curvature/direction

and k2/V2 being the second, is returned.

Consecutive calls with the same surface Srf to SCRVTREVAL will yield

more efficient evaluations as derivative data is cached.

Example:

Crvtr = SCRVTREVAL( Srf, 0.5, 0.5 );

K = nth( Crvtr, 1 ) * nth( Crvtr, 3 );

computes the Total (Gaussian) curvatures, K = k1 * k2, of Srf at (0.5, 0.5).

See also SCRVTR.

SDERIVE

-------

SurfaceType SDERIVE( SurfaceType Srf, NumericType Dir )

returns a vector field surface representing the differentiated surface

in the given direction (ROW or COL). Evaluation of the returned surface at

a given parameter value will return a vector tangent to Srf in

Dir at that parameter value.

DuSrf = SDERIVE( Srf, ROW );

DvSrf = SDERIVE( Srf, COL );

Normal = coerce( seval( DuSrf, 0.5, 0.5 ), VECTOR_TYPE ) ^

coerce( seval( DvSrf, 0.5, 0.5 ), VECTOR_TYPE );

computes the two partial derivatives of the surface Srf and computes

its normal as their cross product, at the parametric location (0.5, 0.5).

See also CDERIVE, TDERIVE, and MDERIVE.

SDIVIDE

-------

SurfaceType SDIVIDE( SurfaceType Srf, ConstantType Direction,

NumericType Param )

or

TrimSrfType SDIVIDE( TrimSrfType Srf, ConstantType Direction,

NumericType Param )

subdivide a (possibly trimmed) surface into two at the specified parameter

value Param in the specified Direction (ROW or COL). Srf

can be either a B-spline surface in which Param must be contained in

the parametric domain of the surface, or a Bezier surface in which  Param} can be arbitrary, extrapolating if not in the range of zero to one.

It returns a list of upto two sub-surfaces. The individual surfaces may be

extracted from the list using the NTH command. If Srf is a trimmed

surface, it may be the case that one of the two subdivided surfaces is

completely trimmed out, and hence only one surface will be returned.

Example:

SrfLst = SDIVIDE( Srf, ROW, 0.5 );

Srf1 = nth( SrfLst, 1 );

Srf2 = nth( SrfLst, 2 );

subdivides Srf at the parameter value of 0.5 in the ROW direction.

See also CDIVIDE, TDIVIDE, and MDIVIDE

SEDITPT

-------

SurfaceType SEDITPT( SurfaceType Srf, CtlPtType CPt, NumericType UIndex,

NumericType VIndex )

provides a simple mechanism to manually modify a single control point number

UIndex and VIndex (base count is 0) in the control mesh of Srf

by substituting CtlPt instead. CtlPt must have the same point type as

the control points of Srf. The original surface Srf is not modified.

Example:

CPt = ctlpt( E3, 1, 2, 3 );

NewSrf = SEDITPT( Srf, CPt, 0, 0 );

constructs a NewSrf with the first control point of Srf being

CPt.

SEVAL

-----

CtlPtType SEVAL( SurfaceType Srf, NumericType UParam, NumericType VParam )

or

CtlPtType SEVAL( TrimSrfType Srf, NumericType UParam, NumericType VParam )

evaluates the provided (possibly trimmed) surface Srf at the given

UParam and VParam parameters. Both UParam and VParam

should be contained in the surface parametric domain if Srf is a

B-spline surface, or between zero and one if Srf is a Bezier surface.

The returned control point has the same type as the control points of

Srf.

Example:

CPt = SEVAL( Srf, 0.25, 0.22 );

evaluates Srf at the parameter values of (0.25, 0.22).

See also CEVAL, MEVAL, TEVAL.

SFLECNODAL

----------

SurfaceType SFLECNODAL( SurfaceType Srf, NumericType SubdivTol,

NumericType NumericTol, NumericType MergeTol,

NumericType ContactOrder )

computes the flecnodal curves over a given freeform geometry, Srf.

The flecnodal curves are curves of contact of order four with a line in an

asymptotic direction.  SubdivTol and NumericTol controls the

subdivision and numeric tolerances of the approximation.  Typically the

subdivision tolerance is fairly coarse.  MergeTol prescribes the

tolerance of merging individual points into polylines.  A negative value

will prescribe no merge.  The ContactOrder should be 3 to compute

flecnodal curves and 4 to compute flecnodal points.

Example:

flecs = SFlecnodal( srf, 0.05, -1e-6, 1e-1, 3 );

See MZERO for the meaning of SubdivTol and NumerTol.

SFOCAL

------

SurfaceType SFOCAL( SurfaceType Srf, NumericType Dir )

evaluates the focal surface field of surface Srf using the normal

curvature in the isoparametric direction as given by Dir (either

ROW or COL). Note this function is not using the principal curvatures as

is generaly the case for focal surfaces.

Example:

gcross = cbspline( 3,

list( ctlpt( E3, 0.3, 0.0, 0.0 ),

ctlpt( E3, 0.1, 0.0, 0.1 ),

ctlpt( E3, 0.1, 0.0, 0.4 ),

ctlpt( E3, 0.5, 0.0, 0.5 ),

ctlpt( E3, 0.6, 0.0, 0.8 ) ),

list( KV_OPEN ) );

glass = surfprev( gcross );

color( glass, red );

gfocal = SFOCAL(glass, col);

evaluates the focal surface using the COL isoparametric direction's

normal curvature of the glass surface.

SFROMCRVS

---------

SurfaceType SFROMCRVS( ListType CrvList,

NumericType OtherOrder,

NumericType OtherEndCond )

constructs a surface by substituting the curves in CrvList as rows

in a control mesh of a surface. The curves in CrvList are made compatible

by promoting Bezier curves to B-splines if necessary, and raising the degrees

and refining as required before substituting the control polygons of the

curves as rows in the mesh. The other direction order is set by

OtherOrder, which cannot be larger than the number of curves.

If B-spline (OtherOrder is smaller than number of curves) end

conditions are set via OtherEndCond and can be one of KV_OPEN,

KV_FLOAT orKV_PERIODIC.

The surface interpolates the first and last curves only, if a Bezier

or open end conditions are selected; otherwise, no curve is interpolated.

Example:

Crv1 = cbspline( 3,

list( ctlpt( E3, 0.0, 0.0, 0.0 ),

ctlpt( E3, 1.0, 0.0, 0.0 ),

ctlpt( E3, 1.0, 1.0, 0.0 ) ),

list( KV_OPEN ) );

Crv2 = Crv1 * trans( vector( 0.0, 0.0, 1.0 ) );

Crv3 = Crv2 * trans( vector( 0.0, 1.0, 0.0 ) );

Srf = SFROMCRVS( list( Crv1, Crv2, Crv3 ), 3, KV_OPEN );

SGAUSS

------

SurfaceType SGAUSS( SurfaceType Srf, NumericType NumerOnly )

evaluates the Gaussian curvature (K) field of surface Srf.  If

NumerOnly is TRUE, only the numerator of the Gaussian curvature

is derived.  Otherwise, if NumerOnly is FALSE, the full exact

Gaussian field is derived.  NumerOnly TRUE may be used in cases

where the zero set of K is needed (parabolic lines).

Example:

Srf1 = hermite( cbezier( list( ctlpt( E3, 0.0, 0.0, 0.0 ),

ctlpt( E3, 0.5, 0.2, 0.0 ),

ctlpt( E3, 1.0, 0.0, 0.0 ) ) ),

cbezier( list( ctlpt( E3, 0.0, 1.0, 0.0 ),

ctlpt( E3, 0.5, 0.8, 0.0 ),

ctlpt( E3, 1.0, 1.0, 0.5 ) ) ),

cbezier( list( ctlpt( E3, 0.0, 2.0, 0.0 ),

ctlpt( E3, 0.0, 2.0, 0.0 ),

ctlpt( E3, 0.0, 2.0, 0.0 ) ) ),

cbezier( list( ctlpt( E3, 0.0, 2.0, 0.0 ),

ctlpt( E3, 0.0, 2.0, 0.0 ),

ctlpt( E3, 0.0, 2.0, 0.0 ) ) ) );

SGauss = SGAUSS( Srf1, false );

evaluates the Gaussian curvaure of Srf1.

See also EVOLUTE and SMEAN.

SILHOUETTE

----------

PolyType SILHOUETTE( SurfaceType Srf, VectorType ViewDir, NumericType Euc )

or

PolyType SILHOUETTE( PolyType Pl, VectorType ViewDir, NumericType Euc )

compute the silhouette edges of the given Srf or Pl from the

prescribed viewing direction ViewDir.  The end result is a piecewise

linear approximation of the exact silhouette, and its accuracy is controlled

via the RESOLUTION variable, in the case of a freeform surface Srf.

If Euc is TRUE, the silhouette curves are returned on the surface,

in Euclidean space.  Otherwise, the silhouette curves are returned in the

parametric space of Srf.

Both Euc and the RESOLUTION variables have no affect in the case of

a polygonal model Pl.

Example:

Resolution = 10;

Sils = SILHOUETTE( glass, vector( 1, -2, 1 ), true );

computes the silhouette curves of surface glass as viewed from

viewing direction (1, -2, 1), and returns the silhouette curves

in Euclidean space.  See also ISOCLINE, PPROPFTCH and SASPCTGRPH.

SINTERP

-------

SurfaceType SINTERP( ListType PtList, NumericType UOrder, NumericType VOrder,

NumericType USize, NumericType VSize,

ConstantType Param)

computes a B-spline polynomial surface that interpolates or approximates

the rectangular grid or scattered set of points in PtList. The

B-spline surface will have orders UOrder and VOrder and mesh

of size USize by VSize control points. If the data is on a grid,

the knots will be spaced according to Param which can be one of

PARAM_UNIFORM, PARAM_CHORD, PARAM_CENTRIP or PARAM_NEILFOL. Currently,

only PARAM_UNIFORM is supported. For a scattered point set, the Param

parameter is ignored.

PtList is a list of points for grid data in which all

lists carry the same amount of points, thereby defining a

rectangular grid.  For scattered data, PtList is a linear list

of points. All points in PtList must be of type (E1-E9, P1-P9)

control point, or regular PointType.  If USize and VSize

are equal to the number of points in the grid data set of PtList,

the resulting surface will interpolate the data set. Otherwise,

if USize or VSize is less than the number of points in

the grid of PtList, the point data set will be least square

approximated.  At no time can USize or VSize be larger that the

number of points in PtList or lower than UOrder and VOrder,

respectively. If USize or VSize are zero, the grid size

is used, forcing an interpolation of the data set.

If PtList contains a linear list of points, these points are

treated as scattered.  Each scattered point is assumed to be holding the

parameteric location at which to interpolate its first two coefficients.

The other coefficients are the interpolation values.  In other words,

to interpolate scattered data of type E3, E5 control points in a linear

list must be provided in (u, v, x, y, z) format.  Scattered data is

interpolated over a unit square (0 to 1) parameteric domain in both u

and v.

All interior knots will be distinctly preserving maximal continuity.

The resulting B-spline surface will have open end conditions.

Example:

pl = nil();

pll = nil();

for ( x = -5, 1, 5,

pl = nil():

for ( y = -5, 1, 5,

snoc( point( x, y, sin( x * Pi / 2 ) * cos( y * Pi / 2 ) ),

pl )

):

snoc( pl, pll ) );

s1 = sinterp( pll, 3, 3, 8, 8, PARAM_UNIFORM );

s2 = sinterp( pll, 3, 3, 11, 11, PARAM_UNIFORM );

samples an explicit surface sin(x) * cos(y) at a grid of 11 by 11 points,

least square fit with a grid of size of 8 by 8 surface s1, and

interpolates surface s2 using this data set.   See also CINTERP and

LINTERP.

SKEL2DINT

---------

ListType SKEL2DINT( CurveType Crv1 | PointType Pt1 | CtlPtType Pt1,

CurveType Crv2 | PointType Pt2 | CtlPtType Pt1,

CurveType Crv3 | PointType Pt3 | CtlPtType Pt1,

NumericType OutExtent, NumericType Epsilon,

NumericType FineNess )

computes locations in the plane of points that are equadistant from the

three given entities.  Entities can be points or control points or curves,

all in the XY plane.  The equadistant points are computed as the mutual

intersection of the bisectors of the entities.  Infinite bisectors (such

as the bisector of two points) are extended up to OutExtent.

Epsilon controls the tolerances while FineNess controls the

subdivision fineness in the bisector intersection computations.

Exanple:

Crv1 = pcircle( vector( -0.5,  0.7, 0.0 ), 0.3 );

Crv2 = pcircle( vector( -0.4, -0.6, 0.0 ), 0.5 );

Crv3 = pcircle( vector(  0.3,  0.2, 0.0 ), 0.4 );

EquaPt = SKEL2DINT( Crv1, Crv2, Crv3, 100, 0.1, 150 ):

computes the eight points that are equadistant to three circles.

See also CRC2CRVTAN.

SMEAN

-----

SurfaceType SMEAN( SurfaceType Srf, NumericType NumerOnly )

evaluates the mean curvature field of surface Srf as follows:

if NumerOnly is true, it computes the numerator of only the Mean curvature.

Otherwise, if NumerOnly is false, the square of the exact Mean

curvature field is derived.  NumerOnly TRUE may be used in cases

where the zero set of H is needed (k1 == -k2 points).

Example:

Srf1 = hermite( cbezier( list( ctlpt( E3, 0.0, 0.0, 0.0 ),

ctlpt( E3, 0.5, 0.2, 0.0 ),

ctlpt( E3, 1.0, 0.0, 0.0 ) ) ),

cbezier( list( ctlpt( E3, 0.0, 1.0, 0.0 ),

ctlpt( E3, 0.5, 0.8, 0.0 ),

ctlpt( E3, 1.0, 1.0, 0.5 ) ) ),

cbezier( list( ctlpt( E3, 0.0, 2.0, 0.0 ),

ctlpt( E3, 0.0, 2.0, 0.0 ),

ctlpt( E3, 0.0, 2.0, 0.0 ) ) ),

cbezier( list( ctlpt( E3, 0.0, 2.0, 0.0 ),

ctlpt( E3, 0.0, 2.0, 0.0 ),

ctlpt( E3, 0.0, 2.0, 0.0 ) ) ) );

SMean = SMEAN( Srf1, false );

evaluates the square of the mean curvature of Srf1.

See also EVOLUTE and SGAUSS.

SMERGE

------

SurfaceType SMERGE( SurfaceType Srf1, SurfaceType Srf2,

NumericType Dir, NumericType SameEdge )

merges two surfaces along the requested direction (ROW or COL). If

SameEdge is non-zero (ON or TRUE), then the common edge is assumed to be

identical and copied only once. Otherwise (OFF or FALSE), a ruled surface

is constructed between the two surfaces along the (not) common edge.

Example:

MergedSrf = SMERGE( Srf1, Srf2, ROW, TRUE );

See also MMERGE.

SMESH

-----

SurfaceType SMESH( TrivarType TV, MumericType Dir, NumericType Index )

extracts a surface out of a trivariate, TV, as the Index's plane

of the control mesh of TV in direction Dir. Dir can be

one of COL, ROW, DEPTH.

Example:

tv = tbezier( list( list( list( ctlpt( E3, 0.1, 0.0, 0.8 ),

ctlpt( E3, 0.2, 0.1, 2.4 ) ),

list( ctlpt( E3, 0.3, 2.2, 0.2 ),

ctlpt( E3, 0.4, 2.3, 2.0 ) ) ),

list( list( ctlpt( E3, 2.4, 0.8, 0.1 ),

ctlpt( E3, 2.2, 0.7, 2.3 ) ),

list( ctlpt( E3, 2.3, 2.6, 0.5 ),

ctlpt( E3, 2.1, 2.5, 2.7) ) ) ) );

s0 = SMESH( tv, col, 0 );

s1 = SMESH( tv, col, 1 );

extracts the two (first and last) planes in direction col out of

trivariate tv.

See also STRIVAR, CMESH, MFROMMESH.

SMOEBIUS

--------

CurveType SMOEBIUS( CurveType Crv, NumericType Ratio, NumericType Dir )

rebalances the weights of a rational surface using the Moebius

transformation.  The shape of the surface remains identical, while the

speed is modified in the direction Dir.

Ratio controls the ratio between the last and first weights of

the first row/column. If Ratio = 0, the first and last weights are

made equal.

See also CMOEBIUS.

SMOOTHNRML

----------

ListType SMOOTHNRML( ListType Obj, NumericType MaxAngle )

or

PolygonType SMOOTHNRML( PolygonType Obj, NumericType MaxAngle )

Given a (list of) polygonal object(s), Obj, compute normals to

the vertices by averaging the normals of the polygons that share the

vertices.  Only vertices where the deviation between the polygons' normals

and the averaged normal is less than MaxAngle are updated.

This is useful for polygonal data sets that have no vertex normals.

Example:

A = box( vector( -1, -1, -1 ), 2, 2, 2 );

B = SMOOTHNRML( A, 90 );

computes average normals to a curve resulting in the smoothly shaded display

of a cube.

SMOMENTS

--------

SurfaceType SMOMENTS( SurfaceType Srf, NumericType Moment,

NumericType Axis1, NumericType Axis2,

NumericType Eval )

or

NumericType SMOMENTS( SurfaceType Srf, NumericType Moment,

NumericType Axis1, NumericType Axis2,

NumericType Eval )

compute the integral moment surface, MSrf, of the given surface

Srf, up to a sign.  The computed moment can be either a first order

moment when Moment = 1, or a second order moment when Moment = 2.

If Srf is a closed surface with domain (u0, v0) to (u1, v1), then

the difference of MSrf(u1, v1) - MSrf(u0, v0) is the requested

moment.  Otherwise, the computation is for the volume occupied between the

surface Srf and the XY plane.

If Eval is TRUE, the actual numerical value of the moment is returned.

The moment integral surface is returned if Eval is FALSE.

Axis1 and Axis2 prescribe the two axes to compute the moments

for a second order moment computation.  For a first order moment

computation only Axis1 is considered.

Example:

Spr = surfPRev( cregion( pcircle( vector( 0, 0, 0 ), 1 ), 1, 3 )

* ry( 90 ) );

SMOMENTS( Spr, 2, 1, 1, 2, 1 );

computes the second order XX moment of a polynomial approximation of a

unit sphere, using method one.  See also SVOLUME and CAREA.

SMORPH

------

SurfaceType SMORPH( SurfaceType Srf1, SurfaceType Srf2, NumericType Blend )

creates a new surface which is a convex blend of the two given surfaces.

The two given surfaces must be compatible (see FFCOMPAT) before this blend

is invoked. This is very useful if a sequence that "morphs" one surface to

another is to be created.

Example:

for ( i = 0.0, 1.0, 11.0,

Msrf = SMORPH( Srf1, Srf2, i / 11.0 ):

color( Msrf, white ):

attrib( Msrf, "rgb", "255,255,255" ):

attrib( Msrf, "reflect", "0.7" ):

save( "morp1-" + i, Msrf )

);

creates a sequence of 12 surfaces, morphed from Srf1 to Srf2

and saves them in the files "morph-0.itd" to "morph-11.itd".

See also PMORPH, CMORPH and TMORPH.

SNORMAL

-------

VectorType SNORMAL( SurfaceType Srf, NumericType UParam, NumericType VParam )

or

VectorType SNORMAL( TrimSrfType Srf, NumericType UParam, NumericType VParam )

compute the normal vector to (possibly trimmed) surface Srf at the

parameter values UParam and VParam. The returned vector has a

unit length.

Example:

Normal = SNORMAL( Srf, 0.5, 0.5 );

computes the normal to Srf at the parameter values (0.5, 0.5).

See also SNRMLSRF.

SNRMLSRF

--------

SurfaceType SNRMLSRF( SurfaceType Srf )

symbolically computes a vector field surface representing the non-normalized

normals of the given surface. That is, the normal surface, evaluated at

(u, v), provides a vector in the direction of the normal of the original

surface at (u, v). The normal surface is computed as the symbolic cross

product of the two surfaces representing the partial derivatives of the

original surface.

Example:

NrmlSrf = SNRMLSRF( Srf );

SPARABOLC

---------

ListType SPARABOLC( SurfaceType Srf, NumericType Euclidean )

computes the parabolic edges of a freeform surface, Srf, as

the zero set of the Gaussian curvature.  A scalar field with the sign of

the Gauss curvature is computed and its zero is derived.

If Euclidean is false, the list of (piecewise linear) parabolic

curves is returned in the parametric space of Srf.  Otherwise, if

Euclidean is true, the parabolic curves are mapped onto Srf.

Example:

pl = nil();

pll = nil();

for ( x = -3, 1, 3,

pl = nil():

for ( y = -3, 1, 3,

snoc( point( x, y, sin( x * Pi / 2 ) * cos( y * Pi / 2 ) ),

pl ) ):

snoc( pl, pll ) );

EggBase = sinterp( pll, 4, 4, 0, 0, PARAM_UNIFORM );

Resolution = 15;

Parab = SPARABOLC( EggBase, true );

constructs a surface in the shape of an egg carton's base and then derives its

parabolic edges in Euclidean space.

SPHERE

------

PolygonType SPHERE( VectorType Center, NumericType Radius )

creates a SPHERE geometric object, defined by Center as the center of

the SPHERE, and with Radius as the radius of the SPHERE.

See RESOLUTION for accuracy of SPHERE approximation as a polygonal model.

See IRITSTATE's "PrimRatSrfs" and "PrimRatSrfs" state variables.

SPLISTLST

---------

ListType SPLISTLST( AnyType LinkedListObj )

splits an objected of several linked list data elements such as polygons,

curves, or suraces, into a list object that contains an object for each of

the individual objects.

Example:

ObjLst = SPLISTLST( axes );

splits the axes object into a list object of several objects each holding

a single polyline.

SPOWER

------

SurfaceType SPOWER( ListType CtlMesh )

creates a polynomial/rational surface out of the provided control mesh.

The created surface employs the monomial power basis. CtlMesh is a

list of rows, each of which is a list of control points. All control points

must be of type (E1-E9, P1-P9), or regular PointType defining the surface's

control mesh. The surface's point type will be of a space which is the

union of the spaces of all points.

Example:

s = SPOWER( list( list( ctlpt( E3, 1, 0, 1 ),

ctlpt( E3, 0, 1, 1 ) ),

list( ctlpt( E3, 0, 0, 1 ),

ctlpt( E3, 0, 0, 1 ) ) ) );

s == coerce( coerce( s, bezier_type ), power_type );

constructs a bilinear power basis surface, coerces it to a Bezier form,

coerces the Bezier form back to a power basis, and then compares the result

for equality.

See also CBEZIER, SBSPLINE and SPOWER.

SRAISE

------

SurfaceType SRAISE( SurfaceType Srf, ConstantType Direction,

NumericType NewOrder )

raises Srf to the specified NewOrder in the specified

Direction.

Example:

Srf = ruledSrf( cbezier( list( ctlpt( E3, -0.5, -0.5, 0.0 ),

ctlpt( E3,  0.5, -0.5, 0.0 ) ) ),

cbezier( list( ctlpt( E3, -0.5,  0.5, 0.0 ),

ctlpt( E3,  0.5,  0.5, 0.0 ) ) ) );

Srf = SRAISE( SRAISE( Srf, ROW, 3 ), COL, 3 );

constructs a bilinear flat-ruled surface and raises both its directions to be

a bi-quadratic surface.

See also TRAISE, MRAISE, and CRAISE.

SRAYCLIP

--------

ListType SRAYCLIP( PointType Pt, VectorType Dir, SurfaceType Srf )

computes the intersection of ray (Pt, Dir) with Bezier surface

Srf, using the Bezier clipping scheme.  The returned list is of the

form "list( NumInters, UV0, EucPt0, ..., UVn, EucPtn )".

Example:

InterPts = SRayClip( point( 0, 0, 0 ), vector( 0, 0, 1 ), Srf );

computes the intersection of surface Srf with the positive Z axis.

SREFINE

-------

SurfaceType SREFINE( SurfaceType Srf, ConstantType Direction,

NumericType Replace, ListType KnotList )

provides the ability to Replace a knot vector of Srf or refine

it in the specified direction Direction (ROW or COL).

KnotList is a list of knots at which to refine Srf. All knots should

be contained in the parametric domain of Srf in Direction. If the knot

vector is replaced, the length of KnotList should be identical to the

length of the original knot vector of Srf in Direction. If Srf

is a Bezier surface, it is automatically promoted to be a B-spline surface.

Example:

Srf = SREFINE( SREFINE( Srf,

ROW, FALSE, list( 0.333, 0.667 ) ),

COL, FALSE, list( 0.333, 0.667 ) );

refines Srf in both directions by adding two more knots at 0.333 and

0.667.   See also CREFINE, TREFINE, and MREFINE.

SREGION

-------

SurfaceType SREGION( SurfaceType Srf, ConstantType Direction,

NumericType MinParam, NumericType MaxParam )

or

TrimSrfType SREGION( TrimSrfType Srf, ConstantType Direction,

NumericType MinParam, NumericType MaxParam )

extract a region of Srf between MinParam and MaxParam

in the specified Direction. Both MinParam and MaxParam

should be contained in the parametric domain of Srf in Direction,

except for Bezier surfaces when MinParam and MaxParam can be

arbitrary (extrapolating if not between zero and one).

Example:

Srf = ruledSrf( cbezier( list( ctlpt( E3, -0.5, -0.5, 0.5 ),

ctlpt( E3,  0.0,  0.5, 0.0 ),

ctlpt( E3,  0.5, -0.5, 0.0 ) ) ),

cbezier( list( ctlpt( E3, -0.5,  0.5, 0.0 ),

ctlpt( E3,  0.0,  0.0, 0.0 ),

ctlpt( E3,  0.5,  0.5, 0.5 ) ) ) );

SubSrf = SREGION( Srf, ROW, 0.3, 0.6 );

extracts the region of Srf from the parameter value 0.3 to the

parameter value 0.6 along the ROW direction. The COLumn direction is

extracted as a whole.

See also CREGION, TREGION, and MREGION.

SREPARAM

--------

SurfaceType SREPARAM( SurfaceType Srf, ConstantType Direction,

NumericType MinParam, NumericType MaxParam )

or

TrimSrfType SREPARAM( TrimSrfType Srf, ConstantType Direction,

NumericType MinParam, NumericType MaxParam )

reparametrize Srf over a new domain from MinParam to MaxParam,

in the prescribed Direction. This operation does not affect the geometry

of the (trimmed) surface and only affine transforms its knot vectors. A

Bezier (trimmed) surface will automatically be promoted into a B-spline

surface by this function.

Example:

srf = sbspline( 2, 4,

list( list( ctlpt( E3, 0.0, 0.0, 1.0 ),

ctlpt( E2, 0.0, 1.0 ),

ctlpt( E3, 0.0, 2.0, 1.0 ) ),

list( ctlpt( E2, 1.0, 0.0 ),

ctlpt( E3, 1.0, 1.0, 2.0 ),

ctlpt( E2, 1.0, 2.0 ) ),

list( ctlpt( E3, 2.0, 0.0, 2.0 ),

ctlpt( E2, 2.0, 1.0 ),

ctlpt( E3, 2.0, 2.0, 2.0 ) ),

list( ctlpt( E2, 3.0, 0.0 ),

ctlpt( E3, 3.0, 1.0, 2.0 ),

ctlpt( E2, 3.0, 2.0 ) ),

list( ctlpt( E3, 4.0, 0.0, 1.0 ),

ctlpt( E2, 4.0, 1.0 ),

ctlpt( E3, 4.0, 2.0, 1.0 ) ) ),

list( list( KV_OPEN ),

list( KV_OPEN ) ) );

srf = sreparam( sreparam( srf, ROW, 0, 1 ), COL, 0, 1 );

ensures that the (trimmed) B-spline surface is defined over the unit size

parametric domain.  See also CREPARAM, TREPARAM, and MREPARAM.

SREVERSE

--------

SurfaceType SREVERSE( SurfaceType Srf )

or

TrimSrfType SREVERSE( TrimSrfType Srf )

reverse Srf by flipping the U and V parametric directions.

Note that the unary minus (i.e -Srf) also reverses the surface by

reversing the U parametric direction.  If the surface is a trimmed surface,

the trimming curves are flipped accordingly to yield the same shape.

RevSrf = SREVERSE( Srf );

See also MREVERSE.

SRF2TANS

--------

ListType SRF2TANS( SurfaceType Srfs, NumericType Orientation,

NumericType SubdivTol, NumericType NumericTol,

NumericType MergeTol )

computes the developable sheet(s) bi-tangent to given two surfaces

Srfs.  Srfs can be either a list of two surfaces or a single

surface in which self bi-tangencies are being sought.

If Orientation is 0 all bi-tangent sheet(s) are returned.  Otherwise, if

Orientation equal +1 or -1, bi-tangent sheets(s) with same or different

tangency orienation are returned, respectively.  See MZERO for the meaning

of the SubdivTol and NumerTol tolerance.

Returns are lists, one per developable sheet, of sample points in E4, having

two pairs of parameter values of the bi-tangent points in the two surfaces,

in their parametric domain.

Example:

c1 = cbspline( 3,

list( ctlpt( E2, -1, -1 ),

ctlpt( E2,  1, -1 ),

ctlpt( E2,  1,  1 ),

ctlpt( E2, -1,  1 ) ),

list( kv_periodic ) );

c1 = coerce( c1, kv_open );

c2 = cbspline( 3,

list( ctlpt( E3,  0.8,  -0.2,  -0.3 ),

ctlpt( E3,  0.5,   0.0,  -0.2 ),

ctlpt( E2, -0.45, -0.21 ),

ctlpt( E2, -0.45,  0.32 ),

ctlpt( E3,  0.5,  -0.0,   0.2 ),

ctlpt( E3,  0.8,   0.28,  0.3 ) ),

list( kv_open ) );

s = sregion( sweepSrf( c1 * sc( 0.1 ), c2, off ), col, 0, 0.5 );

BiTans = SRF2TANS( list( s, s ), 0, 0.1, -1e-6, 1e-1 );

computes the self bi-tangencies of a given bottle-like surface.

See also SRF3TANS.

SRF3TANS

--------

ListType SRF3TANS( ListType Srfs, NumericType Orientation,

NumericType SubdivTol, NumericType NumericTol )

computes the plane(s) tri-tangent to given three surfaces Srfs.

Srfs can be either a list of three surfaces or a list of one

surface in which self bi-tangencies are being sought.

If Orientation is 0 all tri-tangent planes are returned. Otherwise, if

Orientation equal +1 or -1, tri-tangent sheets(s)with same or different

tangency orienation are returned, respectively.  See MZERO for the meaning

of the SubdivTol and NumerTol tolerance.

Returns are lists, one per developable sheet, of sample points in E6, having

three pairs of parameter values of the tri-tangent points in the three

surfaces, in their parametric domain.

Example:

c2 = cbspline( 3,

list( ctlpt( E3,  0.8,  -0.2,  -0.3 ),

ctlpt( E3,  0.5,   0.0,  -0.2 ),

ctlpt( E2, -0.45, -0.21 ),

ctlpt( E2, -0.45,  0.32 ),

ctlpt( E3,  0.5,  -0.0,   0.2 ),

ctlpt( E3,  0.8,   0.28,  0.3 ) ),

list( kv_open ) );

s1 = sFromCrvs( list( c2 * sc( 0.001 ),

c2,

c2 * tz( 1.0 ),

c2 * sc( 0.001 ) * tz( 1.0 ) ),

3, kv_open ) * sc( 0.1 );

s2 = s1 * ry( 14 ) * tx( 0.6 ) * tz( 0.02 );

s3 = s1 * rx( 12 ) * ty( 0.6 ) * tx( 0.3 ) * tz( 0.01 );

TriTans = SRF3TANS( list( s1, s2, s3 ) * sz( 1 ), 1, 0.5, -1e-6 );

Edges = nil();

for ( i = 1, 1, sizeof( TriTans ),

Pt = nth( TriTans, i ):

snoc( seval( s1, coord( Pt, 1 ), coord( Pt, 2 ) ) +

seval( s2, coord( Pt, 3 ), coord( Pt, 4 ) ), Edges ):

snoc( seval( s1, coord( Pt, 1 ), coord( Pt, 2 ) ) +

seval( s3, coord( Pt, 5 ), coord( Pt, 6 ) ), Edges ):

snoc( seval( s2, coord( Pt, 3 ), coord( Pt, 4 ) ) +

seval( s3, coord( Pt, 5 ), coord( Pt, 6 ) ), Edges ) );

computes the two outer oriented tri-tangencies to three approximate

ellipsoids.  Extract and draw the two tri-tangent triangles.

See also SRF2TANS.

SRFFFORM

--------

ListType SRFFFORM( SurfaceType Srf, NumericType Form )

derives the four coefficients of the 1st, 2nd or 3rd surface fundamental

forms.  Form can be one of 1, 2, or 3 only, designating the requested

form.  Since this 2x2 matrix is symmetric, only three coefficients are

returned as a list of three scalar surfaces as (A11, A12 == A21, A22).

Example:

FFF = SRFFFORM( Srf, 1 );

SFF = SRFFFORM( Srf, 2 );

TFF = SRFFFORM( Srf, 3 );

computes the three fundamental forms of Srf.

SRFKERNEL

---------

PolyType SRFKERNEL( SurfaceType Srf, NumericType Fineness,

NumericType SkipRate )

approximates the kernel of (closed and continuous) surface Srf by

deriving the parabolic points of Srf and intersecting half planes

tangent to Srf and placed at a sampled set of parabolic locations.  The

fineness of the parabolic curves' approximation is governed by Fineness

and the sampling rate is controled by SkipRate.

Example:

Krnl = SRFKERNEL( Srf, 40, 15 );

estimates the kernel of Srf with Fineness of 40 and SkipRate

of 15.

SRINTER

-------

PointType SRINTER( SurfaceType Srf, PointType RayOrigin,

VectorType RayDirection )

computes the first intersection, if any, of the prescribed ray originating

from RayOrigin in direction RayDirection with surface Srf.

It returns the intersection point in the parametric space of Srf with

the U and V coordinates as the X and Y coefficients of the returned value.

The intersection is computed between the ray and a polygonal approximation

of the surface Srf as set via the RESOLUTION variable.

Example:

RayOrigin = point( 2, 0.1, 0.3 );

RayDir = vector( -4, 0, 0 );

RayLine = coerce( RayOrigin, E3 ) + coerce( RayOrigin + RayDir, E3 );

color( RayLine, magenta );

attrib( RayLine, "dwidth", 2 );

resolution = 5;

InterPt = SRINTER( glass, RayOrigin, RayDir );

InterPtE3 = seval( glass, coord( InterPt, 0 ), coord( InterPt, 1 ) );

color( InterPtE3, cyan );

attrib( InterPtE3, "dwidth", 3 );

view( list( InterPtE3, RayLine, glass, axes ), 1 );

resolution = 80;

InterPt = SRINTER( glass, RayOrigin, RayDir );

InterPtE3 = seval( glass, coord( InterPt, 0 ), coord( InterPt, 1 ) );

color( InterPtE3, cyan );

attrib( InterPtE3, "dwidth", 3 );

view( list( InterPtE3, RayLine, glass, axes ), 1 );

This is a complete example of constructing a ray and intersecting it against a

surface of a glass at two different resolutions, resulting in two different

accuracies. See also RESOLUTION.

SSINTER

-------

ListType SSINTER( SurfaceType Srf1, SurfaceType Srf2,

NumericType Euclidean, NumericType Epsilon,

NumericType Alignment )

computes the intersection curve of two surfaces, Srf1 and Srf2,

up to Epsilon accuracy.  The returned data is in Euclidean space if

Euclidean is true; otherwise it is in the parametric space.

A list of two lists (for the two surfaces) of n curves each,

where n is the number of intersection curves, is returned.

If Alignment is true, the surfaces are rotated to that one bbox

of one surface whose axes are aligned, increasing the probability of detecting

disjoint cases.

Example:

s1 = sphereSrf( 0.35 ) * trans( vector( 0.0, 0.1, 0.2 ) );

s2 = coneSrf( 1, 0.5 );

Inter = nth( SSINTER( s1, s2, true, 0.1, false ), 1 );

computes the Euclidean intersection curves of a cone and a sphere,

in general positions.  The Euclidean curves on the first surface are

extracted while purging the Euclidean curves on the second surface.

STANGENT

--------

VectorType STANGENT( SurfaceType Srf, ConstantType Direction,

NumericType UParam, NumericType VParam )

or

VectorType STANGENT( TrimSrfType Srf, ConstantType Direction,

NumericType UParam, NumericType VParam )

compute the tangent vector to the (possibly trimmed) surface Srf at the

parameter values UParam and VParam in Direction. The

returned vector has a unit length.

Example:

Tang = STANGENT( Srf, ROW, 0.5, 0.6 );

computes the tangent to Srf in the ROW direction at the parameter

values (0.5, 0.6).

STRIMSRF

--------

SurfaceType STRIMSRF( TrimSrfType TSrf )

extracts the surface of a trimmed surface TSrf.

Example:

Srf = STRIMSRF( TrimSrf );

extracts the surface of TrimSrf.

STRIVAR

-------

SurfaceType STRIVAR( TrivarType TV, ConstantType Direction,

NumericType Param ) )

extracts an iso surface from a trivariate function TV in the specified

Direction (ROW or COL or DEPTH) at the specified parameter value

Param.

Param must be contained in the parametric domain of TV in

Direction direction.

The returned surface is in the trivariate TV.

Example:

TV1 = tbezier( list( list( list( ctlpt( E3, 0.1, 0.0, 0.8 ),

ctlpt( E3, 0.2, 0.1, 2.4 ) ),

list( ctlpt( E3, 0.3, 2.2, 0.2 ),

ctlpt( E3, 0.4, 2.3, 2.0 ) ) ),

list( list( ctlpt( E3, 2.4, 0.8, 0.1 ),

ctlpt( E3, 2.2, 0.7, 2.3 ) ),

list( ctlpt( E3, 2.3, 2.6, 0.5 ),

ctlpt( E3, 2.1, 2.5, 2.7) ) ) ) );

Srf = STRIVAR( TV1, col, 0.4 );

extracts an iso surface of TV1, in the col direction at

parameter value 0.4.

See also SMESH, CSURFACE, MFROMMV.

SURFPREV

--------

SurfaceType SURFPREV( CurveType Object )

This is the same as SURFREV but approximates the surface of revolution as a

polynomial surface. The object must be a polynomial curve.

See SURFREV.

SURFREV

-------

PolygonType SURFREV( PolygonType Object )

or

SurfaceType SURFREV( CurveType Object )

create a surface of revolution by rotating the first polygon/curve of the

given Object, around the Z axis. Use the linear transformation functions

to position a surface of revolution in a different orientation.

Example:

VTailAntn = SURFREV( ctlpt( E3, 0.001, 0.0, 1.0 ) +

ctlpt( E3, 0.01,  0.0, 1.0 ) +

ctlpt( E3, 0.01,  0.0, 0.8 ) +

ctlpt( E3, 0.03,  0.0, 0.7 ) +

ctlpt( E3, 0.03,  0.0, 0.3 ) +

ctlpt( E3, 0.001, 0.0, 0.0 ) );

constructs a piecewise linear B-spline curve in the XZ plane and uses it to

construct a surface of revolution by rotating it around the Z axis.

See also SURFPREV, SURFREVAXS, SURFREV2, SURFREVAX2.

SURFREVAXS

----------

PolygonType SURFREVAXS( PolygonType Object, VectorType Axis )

or

SurfaceType SURFREVAXS( CurveType Object, VectorType Axis )

create a surface of revolution by rotating the first polygon/curve of the

given Object, around the Axis axis. Use the linear transformation

functions to position a surface of revolution in a different location.

Example:

Glass = SURFREVAXS( GCross, vector( 1, 0, 1 ) );

constructs a surface of revolution by rotating GCross around the axis of (1, 0, 1).

See also SURFPREV, SURFREV, SURFREV2, SURFREVAX2.

SURFREV2

--------

PolygonType SURFREV2( PolygonType Object,

NumericType StartAngle,

NumericType EndAngle )

or

SurfaceType SURFREV2( CurveType Object,

NumericType StartAngle,

NumericType EndAngle )

create a surface of revolution by rotating the first polygon/curve of the

given Object, around the Z axis.  The rotation does not form a complete

circle and is from the StartAngle to the EndAngle only, in degrees,

starting from the X axis toward the Y axis, in the XY plane.  Use the linear

transformation functions to position a surface of revolution in a different

orientation.

Example:

Glass = SURFREV2( GCross, 45, 180 );

constructs a surface of revolution by rotating it around the Z axis from 45

to 180 degrees.

See also SURFPREV, SURFREVAXS, SURFREV, SURFREVAX2.

SURFREVAX2

----------

PolygonType SURFREVAX2( PolygonType Object,

NumericType StartAngle,

NumericType EndAngle,

VectorType Axis )

or

SurfaceType SURFREVAX2( CurveType Object,

NumericType StartAngle,

NumericType EndAngle,

VectorType Axis )

create a surface of revolution by rotating the first polygon/curve of the

given Object, around the Axis axis.  The rotation does not form

a complete circle and is from the StartAngle to the EndAngle only, in

degrees, starting from the X axis toward the Y axis, in the XY plane.

Use the linear transformation functions to position a surface of revolution

in a different location.

Example:

T4 = SURFREVAX2( PolyCross, 90, 360, vector( 1, 0, 1 ) );

constructs a polygonal surface of revolution by rotating PolygonType

PolyCross around the axis (1, 0, 1), from 45 to 180 degrees.

See also SURFPREV, SURFREVAXS, SURFREV2, SURFREV.

SVISIBLE

--------

ListType SVISIBLE( SurfaceType Srf,

NumericType Resolution,

NumericType ConeSize )

computes a decomposition of a freeform surface Srf into regions, each

visible with a cone visibility of ConeSize degrees from one direction.

In other words, all points in one region have angular deviation of their

surface normal of less than ConeSize degrees from the set viewing

direction. Resolution controls the accuracy of the computation; the higher

this value is, more exact the result. 20 is a good starting value.  Each

returned region is a trimmed surface that has a "ViewDir" attribute that

contains the viewing direction of this region.

Example:

c1 = cbezier( list( ctlpt( E3, 1.0, 0.0,  0.5 ),

ctlpt( E3, 1.1, 0.0,  0.0 ),

ctlpt( E3, 1.0, 0.0, -0.5 ) ) );

Simp = sregion( surfPRev( c1 ), col, 0.0, 1.0 ) * rz( 45 ) * rx( 90 );

Decomp = SVISIBLE( Simp, 20, 30 * pi / 180 );

SimDecomp = nil();

Mod = 5;

for ( i = 1, 1, sizeof( Decomp ),

o = nth( Decomp, i ):

v = getattr( o, "ViewDir" ):

l = ( ctlpt( E3, 0, 0, 0 ) + coerce( v, e3 ) ) * sc( 1.5 ):

j = floor( ( i - 1 ) / Mod ):

snoc( list( o, Simp, l, axes )

* view_mat * tx( ( i - 1 - j * Mod ) * 2 - 4 )

* ty( -j * 2 ),

SimDecomp ) );

view( SimDecomp, on );

decomposes a given surface Simp into regions of 30 degrees at most,

goes over the decomposed regions and orders them five in a row.

SVOLUME

-------

SurfaceType SVOLUME( SurfaceType Srf, NumericType Method, NumericType Eval )

or

NumericType SVOLUME( SurfaceType Srf, NumericType Method, NumericType Eval )

computes the integral volume surface, VSrf, of the given surface

Srf, up to a sign.

If Srf is a closed surface with domain (u0, v0) to (u1, v1), then

the difference of VSrf(u1, v1) - VSrf(u0, v0) is the requested

volume.  Otherwise, the computation is for the volume occupied between the

surface Srf and the XY plane if Method equals one, and the

volume occupied between the surface Srf and the

origin if Method equals two.

If Eval is TRUE, the actual numerical value of the volume is returned.

The volume integral surface is returned if Eval is FALSE.

Example:

Spr = surfPRev( cregion( pcircle( vector( 0, 0, 0 ), 1 ), 1, 3 )

* ry( 90 ) );

SVOLUME( Spr, 1, 1 ) * 3 / 4;

SVOLUME( Spr, 2, 1 ) * 3 / 4;

are yet another two ways of approximating the value of Pi.

See also SMOMENTS and CAREA.

SWEEPSRF

--------

SurfaceType SWEEPSRF( CurveType CrossSection | ListType CrossSectionList,

CurveType Axis,

CurveType FrameCrv | VectorType FrameVec

| ConstType OFF )

constructs a generalized cylinder surface. This function sweeps a specified

cross section CrossSection along the provided Axis.  If a list

of curves CrossSectionList is specified instead, the cross sections

are blended along the Axis of the curve so that the first/last cross

section in the list fits the first/last location on the Axis.

By default, when frame specification is OFF, the orientation

of the cross section is computed using the Axis curve tangent and

normal. However, unlike the Frenet frame, attempt is made to minimize

the normal change, as can happen along inflection points in Axis.

If a VectorType FrameVec is provided as a frame orientation setting,

it is used to fix the binormal direction to this value. In other words, the

orientation frame has a fixed binormal. If a CurveType FrameCrv is

specified as a frame orientation setting, this vector field curve is

evaluated at each placement of the cross section to yield the needed binormal.

The resulting sweep is only an approximation of the real sweep. The

resulting sweep surface will not be exact, in general.

Refinement of the axis curve at the proper location, where accuracy is

important, should improve the accuracy of the output. The parametric domains

of FrameCrv do not have to match the parametric domain of Axis,

and its parametric domain is automatically made compatible by this function.

Example:

Cross = arc( vector( 0.2, 0.0, 0.0 ),

vector( 0.2, 0.2, 0.0 ),

vector( 0.0, 0.2, 0.0 ) ) +

arc( vector( 0.0, 0.4, 0.0 ),

vector( 0.1, 0.4, 0.0 ),

vector( 0.1, 0.5, 0.0 ) ) +

arc( vector( 0.8, 0.5, 0.0 ),

vector( 0.8, 0.3, 0.0 ),

vector( 1.0, 0.3, 0.0 ) ) +

arc( vector( 1.0, 0.1, 0.0 ),

vector( 0.9, 0.1, 0.0 ),

vector( 0.9, 0.0, 0.0 ) ) +

ctlpt( E2, 0.2, 0.0 );

Axis = arc( vector( -1.0, 0.0, 0.0 ),

vector(  0.0, 0.0, 0.1 ),

vector(  1.0, 0.0, 0.0 ) );

Axis = crefine( Axis, FALSE, list( 0.25, 0.5, 0.75 ) );

Srf1 = SWEEPSRF( Cross, Axis, OFF );

Srf2 = SWEEPSRF( Cross, Axis, vector( 0.0, 1.0, 1.0 ) );

Srf3 = SWEEPSRF( Cross, Axis,

cbezier( list( ctlpt( E3,  1.0, 0.0, 0.0 ),

ctlpt( E3,  0.0, 1.0, 0.0 ),

ctlpt( E3, -1.0, 0.0, 0.0 ) ) ) );

constructs a rounded rectangle cross section and sweeps it along an arc,

while orienting it several ways. The axis curve Axis is manually

refined to better approximate the requested shape.

See also SWPSCLSRF for sweep with scale.

SWPSCLSRF

---------

SurfaceType SWPSCLSRF( CurveType CrossSection | ListType CrossSectionList,

CurveType Axis, NumericType Scale | CurveType ScaleCrv,

CurveType FrameCrv | VectorType FrameVec

| ConstType OFF,

NumericType ScaleRefine )

constructs a generalized cylinder surface. This function sweeps a specified

cross section CrossSection along the provided Axis.  If a list

of curves CrossSectionList is specified instead, the cross sections

are blended along the Axis of the curve so that the first/last cross

section in the list fits the first/last location on the Axis.

The cross section may be scaled by a constant value Scale, or scaled

along the Axis parametric direction via a scaling curve ScaleCrv.

By default, when frame specification is OFF, the orientation

of the cross section is computed using the Axis curve tangent and

normal. However, unlike the Frenet frame, attempt is made to minimize

the normal change, as can happen along inflection points in Axis.

If a VectorType FrameVec is provided as a frame orientation setting,

it is used to fix the binormal direction to this value. In other words, the

orientation frame has a fixed binormal. If a CurveType FrameCrv is

specified as a frame orientation setting, this vector field curve is

evaluated at each placement of the cross section to yield the needed binormal.

ScaleRefine is an integer value to define possible refinement of the

Axis to reflect the information in ScalingCrv. A value of zero will

force no refinement while a value of n > 0 will insert n times the

number of control points in ScaleCrv into Axis, better emulating

the scaling requested.

The resulting sweep is only an approximation of the real sweep. The

scaling and axis placement will not be exact, in general.

Manual refinement (in addition to ScaleRefine) of the axis curve at

the proper location, where accuracy is important, should improve the accuracy

of the output. The parametric domains of ScaleCrv and FrameCrv do

not have to match the parametric domain of Axis, and their domains are

made compatible by this function.

Example:

Cross = arc( vector( -0.11, -0.1,  0.0 ),

vector( -0.1,  -0.1,  0.0 ),

vector( -0.1,  -0.11, 0.0 ) ) +

arc( vector(  0.1,  -0.11, 0.0 ),

vector(  0.1,  -0.1,  0.0 ),

vector(  0.11, -0.1,  0.0 ) ) +

arc( vector(  0.11,  0.1,  0.0 ),

vector(  0.1,   0.1,  0.0 ),

vector(  0.1,   0.11, 0.0 ) ) +

arc( vector( -0.1,   0.11, 0.0 ),

vector( -0.1,   0.1,  0.0 ),

vector( -0.11,  0.1,  0.0 ) ) +

ctlpt( E2, -0.11, -0.1 );

scaleCrv = cbspline( 3,

list( ctlpt( E2, 0.05, 1.0 ),

ctlpt( E2, 0.1,  0.0 ),

ctlpt( E2, 0.2,  2.0 ),

ctlpt( E2, 0.3,  0.0 ),

ctlpt( E2, 0.4,  2.0 ),

ctlpt( E2, 0.5,  0.0 ),

ctlpt( E2, 0.6,  2.0 ),

ctlpt( E2, 0.7,  0.0 ),

ctlpt( E2, 0.8,  2.0 ),

ctlpt( E2, 0.85, 1.0 ) ),

list( KV_OPEN ) );

Axis = circle( vector( 0, 0, 0 ), 1 );

Frame = circle( vector( 0, 0, 0 ), 1 )

* rotx( 90 ) * trans( vector( 1.5, 0.0, 0.0 ) );

Srf1 = SWPSCLSRF( Cross, Axis, scaleCrv, off, 0 );

Srf2 = SWPSCLSRF( Cross, Axis, scaleCrv, off, 2 );

Srf3 = SWPSCLSRF( Cross, Axis, 1.0, Frame, 0 );

constructs a rounded rectangle cross section and sweeps it along a circle,

while scaling and orienting in several ways. The axis curve Axis is

automatically refined in Srf2 to better approximate the requested

scaling.

See also SWEEPSRF for sweep with no scale.

SWUNGASUM

---------

SurfaceType SWUNGASUM( CurveType Crv1, CurveType Crv2 )

Given two curves, compute a swung surface that equals:

S(r, t) = (x1(r) x2(t), x1(r) y2(t), y1(r))

Example:

circ = circle( vector( 0.0, 0.0, 0.0 ), 1.5 ) * ry( 90 );

arc1 = arc( vector( 0.0, 1.0, 0.0 ),

vector( 0.0, 0.0, 0.0 ),

vector( 1.0, 0.0, 0.0 ) );

as1 = SWUNGASUM( circ * ry( -90 ), arc1 );

arc1 = cregion( circle( vector( 0.0, 0.0, 0.0 ), 1.5 ), 0, 2 ) * rz( 90 );

c2 = coerce( cbspline( 3,

list( ctlpt( E2,  1.0,  0.0 ),

ctlpt( E2,  0.2,  0.2 ),

ctlpt( E2,  0.0,  1.0 ),

ctlpt( E2, -0.2,  0.2 ),

ctlpt( E2, -1.0,  0.0 ),

ctlpt( E2, -0.2, -0.2 ),

ctlpt( E2,  0.0, -1.0 ),

ctlpt( E2,  0.2, -0.2 ) ),

list( KV_PERIODIC ) ),

KV_OPEN );

as2 = SWUNGASUM( arc1, c2 );

creates two algebraic sum surfaces, one in the shape of a cylinder as

a sum of a line and a circle, and one circular sweep.

SYMBCPROD

---------

CurveType SYMBCPROD( CurveType Crv1, CurveType Crv2 )

or

SurfaceType SYMBCPROD( SurfaceType Srf1, SurfaceType Srf2 )

or

MultivarType SYMBCPROD( MultivarType MV1, MultivarType MV2 )

compute the symbolic cross product of the two given

curves/surfaces/multivariates as a curve, surface or multivariate.

Example:

NrmlSrf = SYMBCPROD( sderive( Srf, ROW ), sderive( Srf, COL ) )

computes a normal surface as the cross product of the two surface partial

derivatives (see SNRMLSRF).

SYMBDIFF

--------

CurveType SYMBDIFF( CurveType Crv1, CurveType Crv2 )

or

SurfaceType SYMBDIFF( SurfaceType Srf1, SurfaceType Srf2 )

or

MultivarType SYMBDIFF( MultivarType MV1, MultivarType MV2 )

compute the symbolic difference of the two given

curves/surfaces/multivariates as a curve, surface or multivariate.

The difference is computed coordinate-wise.

Example:

DiffCrv = SYMBDIFF( Crv1, Crv2 )

DistSqrCrv = symbdprod( DiffCrv, DiffCrv )

SYMBDPROD

---------

CurveType SYMBDPROD( CurveType Crv1, CurveType Crv2 )

or

CurveType SYMBDPROD( CurveType Crv1, VectorType Vec2 )

or

SurfaceType SYMBDPROD( SurfaceType Srf1, SurfaceType Srf2 )

or

SurfaceType SYMBDPROD( SurfaceType Srf1, VectorType Vec2 )

or

MultivarType SYMBDPROD( MultivarType MV1, MultivarType MV2 )

or

MultivarType SYMBDPROD( MultivarType MV1, VectorType Vec2 )

compute the symbolic dot (inner) product of the two given

curves/surfaces/multivariates as a scalar

curve/surface/multivariate.  As an alternative, one parameter can

also be a regular vector.

Example:

DiffCrv = symbdiff( Crv1, Crv2 )

DistSqrCrv = SYMBDPROD( DiffCrv, DiffCrv )

computes a scalar curve that at parameter t is equal to the distance

square between Crv1 at t and Crv2.

SYMBIPROD

---------

NumericType SYMBIPROD( CurveType Crv, NumericType Order1, NumericType Order2 )

or

NumericType SYMBIPROD( NumericType Dummy, NumericType Idx1, NumericType Idx2 )

compute the inner product of two B-spline basis functions.  The first form

defines the function space to be the same as the function space of Crv

of order Order1 (first basis function) by Order2. The second

basis function in the inner product is defined as,

_

|

| Bi,o1(t) Bj,o2(t) dt.

|

-

The second form prescribes the indices of the two basis functions,

i and j.  The first form returns zero in case of an error.  The second

form returns the result of the inner product.

Example:

SYMBIPROD( Crv = pcircle( vector( 0, 0, 0 ), 1 ), 4, 4 );

for ( i = 0, 1, nth( ffmsize( Crv ), 1 ) - 1,

for ( j = 0, 1, nth( ffmsize( Crv ), 1 ) - 2,

printf( "%3.3f ", list( SYMBIPROD( 0, i, j ) ) ) ):

printf( "\n", nil() ) );

prints all possible inner products of the B-spline function space of

pcircle, of cubics vs. cubics.

SYMBPROD

--------

CurveType SYMBPROD( CurveType Crv1, CurveType Crv2 )

or

SurfaceType SYMBPROD( SurfaceType Srf1, SurfaceType Srf2 )

or

MultivarType SYMBPROD( MultivarType MV1, MultivarType MV2 )

compute the symbolic product of the two given curves/surfaces/multivariates

as a curve, surface or multivariate. The product is computed coordinate-wise.

Example:

ProdSrf = SYMBPROD( Srf1, Srf2 )

SYMBSUM

-------

CurveType SYMBSUM( CurveType Crv1, CurveType Crv2 )

or

SurfaceType SYMBSUM( SurfaceType Srf1, SurfaceType Srf2 )

or

MultivarType SYMBSUM( MultivarType MV1, MultivarType MV2 )

compute the symbolic sum of the two given curves/surfaces/multivariates as

a curve, surface or multivariate. The sum is computed coordinate-wise.

Example:

SumCrv = SYMBSUM( Crv1, Crv2 )

TBEZIER

-------

TrivarType TBEZIER( ListType CtlMesh )

creates a Bezier trivariate using the provided control mesh. CtlMesh

is a list of planes, each of which is a list of rows, each of which is a list

of control points. All control points must be of type (E1-E9, P1-P9), or

regular PointType defining the trivariate's control mesh. The surface point

type will be of a space which is the union of the spaces of all points.

Example:

TV = TBEZIER( list( list( list( ctlpt( E3, 0.1, 0.1, 0.0 ),

ctlpt( E3, 0.2, 0.5, 1.1 ),

ctlpt( E3, 0.3, 0.1, 2.2 ) ),

list( ctlpt( E3, 0.4, 1.3, 0.5 ),

ctlpt( E3, 0.5, 1.7, 1.7 ),

ctlpt( E3, 0.6, 1.3, 2.9 ) ),

list( ctlpt( E3, 0.7, 2.4, 0.5 ),

ctlpt( E3, 0.8, 2.6, 1.4 ),

ctlpt( E3, 0.9, 2.8, 2.3 ) ) ),

list( list( ctlpt( E3, 1.1, 0.1, 0.5 ),

ctlpt( E3, 1.3, 0.2, 1.7 ),

ctlpt( E3, 1.5, 0.3, 2.9 ) ),

list( ctlpt( E3, 1.7, 1.2, 0.0 ),

ctlpt( E3, 1.9, 1.4, 1.2 ),

ctlpt( E3, 1.2, 1.6, 2.4 ) ),

list( ctlpt( E3, 1.4, 2.3, 0.9 ),

ctlpt( E3, 1.6, 2.5, 1.7 ),

ctlpt( E3, 1.8, 2.7, 2.5 ) ) ) ) );

creates a trivariate Bezier which is linear in the first direction, and

quadratic in the second and third.

TBSPLINE

--------

TrivarType TBSPLINE( NumericType UOrder,

NumericType VOrder,

NumericType WOrder,

ListType CtlMesh,

ListType KnotVectors )

creates a B-spline trivariate with the provided UOrder, VOrder

and WOrder orders, the control mesh CtlMesh, and the three knot

vectors in KnotVectors.

CtlMesh is a list of planes, each of which is a list of rows, each

of which is a list of control points. All control points must be of point

type (E1-E9, P1-P9), or regular PointType defining the trivariate's control

mesh. Trivariate point type will be of a space which is the union of the

spaces of all points.

KnotVectors is a list of three knot vectors. Each knot vector is a

list of NumericType knots of length #CtlPtList plus the Order.

If, however, the length of the knot vector is equal to #CtlPtList +

Order + Order - 1}, the curve is assumed to be periodic.

The knot vector may also be a list of a single constant, KV_OPEN,

KV_FLOAT or KV_PERIODIC, in which a uniform knot vector with the

appropriate length and with open, floating or periodic end conditions

will be constructed automatically.

Example:

TV = TBSPLINE( 2, 2, 2,

list( list( list( ctlpt( E3, 0.1, 0.1, 0.0 ),

ctlpt( E3, 0.2, 0.5, 1.1 ),

ctlpt( E3, 0.3, 0.1, 2.2 ) ),

list( ctlpt( E3, 0.4, 1.3, 0.5 ),

ctlpt( E3, 0.5, 1.7, 1.7 ),

ctlpt( E3, 0.6, 1.3, 2.9 ) ),

list( ctlpt( E3, 0.7, 2.4, 0.5 ),

ctlpt( E3, 0.8, 2.6, 1.4 ),

ctlpt( E3, 0.9, 2.8, 2.3 ) ) ),

list( list( ctlpt( E3, 1.1, 0.1, 0.5 ),

ctlpt( E3, 1.3, 0.2, 1.7 ),

ctlpt( E3, 1.5, 0.3, 2.9 ) ),

list( ctlpt( E3, 1.7, 1.2, 0.0 ),

ctlpt( E3, 1.9, 1.4, 1.2 ),

ctlpt( E3, 1.2, 1.6, 2.4 ) ),

list( ctlpt( E3, 1.4, 2.3, 0.9 ),

ctlpt( E3, 1.6, 2.5, 1.7 ),

ctlpt( E3, 1.8, 2.7, 2.5 ) ) ) ),

list( list( KV_OPEN ),

list( KV_OPEN ),

list( KV_OPEN ) ) );

constructs a trilinear B-spline trivariate with open end conditions.

TCRVTR

TCRVTR

------

AnyType TCRVTR( TrivarType TV, PointType Pos, NumericType ComputeWhat )

computes differential curvature properties of an isosurface of

the given trivariate TV at the given (parameteric) location

Pos. Following the value of ComputeWhat, the result equals,

-1             Initialization (a must prelude)

0              Conclusion (a must postlude)

1              Returns a vector hold of the gradient

2              Returns a list of three vectors

equal to the Hessian of this location

3              Returns a list of two scalar values

(Principle curvatures) and two vectors

(Principal directions).

Every evaluation must start with an invocation of ComputeWhat

equal to -1 and terminate with ComputeWhat 0.  In both cases, 1 is

returned in case of success.

Example:

TCRVTR( TV, point( 0, 0, 0 ), -1 ); # Prelude

Grad1 = TCRVTR( TV, point( 0, 0, 0 ),  1 );

Grad2 = TCRVTR( TV, point( 0, 0, 1 ),  1 );

Grad3 = TCRVTR( TV, point( 0, 1, 0 ),  1 );

Grad4 = TCRVTR( TV, point( 1, 0, 0 ),  1 );

TCRVTR( TV, point( 0, 0, 0 ), 0 ); #Postlude

TDERIVE

-------

TrivarType TDERIVE( TrivarType TV, NumericType Dir )

returns a vector field trivariate representing the differentiated

trivariate in the given direction (ROW, COL, or DEPTH). Evaluation of the

returned trivariate at a given parameter value will return a vector

representing the partial derivative of TV in Dir at that

parameter value.

TV = tbezier( list( list( list( ctlpt( E1, 0.1 ),

ctlpt( E1, 0.2 ) ),

list( ctlpt( E1, 0.3 ),

ctlpt( E1, 0.4 ) ) ),

list( list( ctlpt( E1, 2.4 ),

ctlpt( E1, 2.2 ) ),

list( ctlpt( E1, 2.3 ),

ctlpt( E1, 2.1 ) ) ) ) );

DuTV = TDERIVE( TV, ROW );

DvTV = TDERIVE( TV, COL );

DwTV = TDERIVE( TV, DEPTH );

computes the gradiate of a scalar trivariate field, by computing its

partials with respect to u, v, and w.

See also CDERIVE, SDERIVE, and MDERIVE.

TDIVIDE

-------

TrivarType TDIVIDE( TrivarType TV, ConstantType Direction,

NumericType Param )

subdivides a trivariate into two at the specified parameter value Param

in the specified Direction (ROW, COL, or DEPTH). TV can be either

a B-spline trivariate in which Param must be contained in the parametric

domain of the trivariate, or a Bezier trivariate in which Param must be

in the range of zero to one.

It returns a list of the two sub-trivariates. The individual trivariates

may be extracted from the list using the NTH command.

Example:

TvDiv = TDIVIDE( Tv2, depth, 0.3 );

Tv2a = nth( TvDiv, 1 ) * tx( -2.2 );

Tv2b = nth( TvDiv, 2 ) * tx( 2.0 );

subdivides Tv2 at the parameter value of 0.3 in the DEPTH direction,

See also CDIVIDE, SDIVIDE, and MDIVIDE

TEDITPT

-------

TrivarType TEDITPT( TrivarType TV, CtlPtType CPt, NumericType UIndex,

NumericType VIndex )

NumericType WIndex )

provides a simple mechanism to manually modify a single control point number

UIndex, VIndex and WIndex (base count is 0) in the control mesh

of Srf by substituting CtlPt instead. CtlPt must have the same

point type as the control points of Srf. Original surface Srf is

not modified.

Example:

CPt = ctlpt( E3, 1, 2, 3 );

NewTV = TEDITPT( TV, CPt, 0, 0, 0 );

constructs a NewTV with the first control point of TV being

CPt.

TEVAL

-----

CtlPtType TEVAL( TrivarType TV,

NumericType UParam,

NumericType VParam,

NumericType WParam )

evaluates the provided trivariate TV at the given UParam,

VParam and WParam values. UParam, VParam,

WParam must be contained in the surface parametric domain if TV

is a B-spline trivariate, or between zero and one if TV is a Bezier

trivariate. The returned control point has the same type as the control

points of TV.

Example:

CPt = TEVAL( TV1, 0.25, 0.22, 0.7 );

evaluates TV at the parameter values of (0.25, 0.22, 0.7).

See also CEVAL, SEVAL, MEVAL.

TEXTGEOM

--------

AnyType TEXTGEOM( StringType Str, VectorType Spacing, NumericType Scaling )

creates a displayable geometry that represents the text in Str, with

Spacing space between individual characters. Each character is scaled

by Scaling where scaling of one generates a close to unit size

character.

Example:

a = TEXTGEOM("Text", vector( 0.12, 0, 0 ), 0.1 );

b = TEXTGEOM("IRIT", vector( 0, -0.12, 0 ), 0.1 );

creates a horizontal Text and a vertical top to bottom IRIT,

both as geometrical objects.  See TEXTWARP and IRITSTATE's "LoadFont"

state variable.

TEXTWARP

--------

AnyType TEXTWARP( Surface Srf, StringType Text, NumericType HSpace,

NumericType VBase, NumericType VTop, NumericType Ligature )

warps the given text, Text, using surface Srf as warping

function with HSpace setting the horizontal spacing between

characters, VBase and VTop controls the vertical spacing

of the characters in Srf, and Ligature, if not zero, sets

the amount to contract the distance between two adjacent characters.

Example:

c = cbezier( list( ctlpt( e2, -1.5, -0.5 ),

ctlpt( e2, -2,    0 ),

ctlpt( e2, -1,    1 ),

ctlpt( e2,  0,   -2 ),

ctlpt( e2,  1,    0 ) ) );

s = sreparam( ruledSrf( c, offset( c, -0.4, 0.02, off ) ), col, 0, 6 );

Txt = TEXTWARP( s, "Text Warping Toolkit", 0.08, 0.25, 0.75, 0 );

See also TEXTGEOM and IRITSTATE's "LoadFont" state variable.

TFROMSRFS

---------

TrivarType TFROMSRFS( ListType SrfList, NumericType OtherOrder )

constructs a trivariate by substituting the surfaces in SrfList as

planes in a control mesh of a trivariate. Surfaces in SrfList are

made compatible by promoting Bezier surfaces to B-splines if necessary,

and raising degree and refining as required before substituting the

control meshes of the surfaces as planes in the mesh of the trivariate.

The other, third, direction order is set by OtherOrder, which cannot

be larger than the number of surfaces.

The trivariate interpolates the first and last surfaces only.

Example:

s1 = sbezier( list( list( ctlpt( E3, -0.5, -0.5, 0 ),

ctlpt( E3, -0.5,  0.5, 0 ) ),

list( ctlpt( E3,  0.5, -0.5, 0 ),

ctlpt( E3,  0.5,  0.5, 0 ) ) ) ) * sc( 0.3 );

Srfs = list( s1 * sc( 2.0 ),

s1 * sx( 1.4 ) * ry( 45 ) * tz( 1.0 ),

s1 * ry( 90 ) * trans( vector( 1.0, 0.0, 1.1 ) ),

s1 * sx( 1.4 ) * ry( 135 ) * trans( vector( 2.0, 0.0, 1.0 ) ),

s1 * sc( 2.0 ) * ry( 180 ) * trans( vector( 2.0, 0.0, 0.0 ) ) );

color( Srfs, red );

ts = tfromsrfs( Srfs, 3 );

color( ts, green );

view( list( Srfs, ts ), on );

constructs a trivariate from five planar surfaces and displays both the

trivariate and the five planar surfaces, in different colors.

See also EXTRUDE, RULEDTV.

TINTERP

-------

TrivarType TINTERP( TrivarType TV,

NumericType ULength, NumericType VLength, NumericType WLength,

NumericType UOrder, NumericType VOrder, NumericType WOrder );

or

TrivarType TINTERP( ListType PtList,

NumericType ULength, NumericType VLength, NumericType WLength,

NumericType UOrder, NumericType VOrder, NumericType WOrder );

Given a trivariate data structure or a list of points in R3, the above

computes a fitted trivariate in the prescribed function space (i.e.

U/V/WLength and U/V/WOrder) that interpolates/least squares approximates the

given trivariate, TV, at the node parameter values.

PtList is a list of points in Rn, n > 3.  The first three coordinates

of each points in PtList prescribes the (u, v, w) parametric value and

the rest, the interpolation values.  To construct a mapping from R3 to R3,

the points of PtList should be in R6.  To construct a scalar trivariate

function, R4 points are expected.  The (u, v, w) points are assumed to be

containted in a unit curve paramteric space - zero to one in all three

dimensions.

If U/V/WOrder are zero and the first parameter is a trivariate, the

respective order is taken for TV.

If U/V/WLength are zero and the first parameter is a trivariate, the

respective length is taken for TV.

Example:

tv = tbspline( 3, 3, 2,

list( list( list( ctlpt( E3, 0.1, 0.1, 0.0 ),

ctlpt( E3, 0.2, 0.5, 1.1 ),

ctlpt( E3, 0.3, 0.1, 2.2 ) ),

list( ctlpt( E3, 0.4, 1.3, 0.5 ),

ctlpt( E3, 0.5, 1.7, 1.7 ),

ctlpt( E3, 0.6, 1.3, 2.9 ) ),

list( ctlpt( E3, 0.7, 2.4, 0.5 ),

ctlpt( E3, 0.8, 2.6, 1.4 ),

ctlpt( E3, 0.9, 2.8, 2.3 ) ) ),

list( list( ctlpt( E3, 1.1, 0.1, 0.5 ),

ctlpt( E3, 1.3, 0.2, 1.7 ),

ctlpt( E3, 1.5, 0.3, 2.9 ) ),

list( ctlpt( E3, 1.7, 1.2, 0.0 ),

ctlpt( E3, 1.9, 1.4, 1.2 ),

ctlpt( E3, 1.2, 1.6, 2.4 ) ),

list( ctlpt( E3, 1.4, 2.3, 0.9 ),

ctlpt( E3, 1.6, 2.5, 1.7 ),

ctlpt( E3, 1.8, 2.7, 2.5 ) ) ),

list( list( ctlpt( E3, 2.8, 0.1, 0.4 ),

ctlpt( E3, 2.6, 0.7, 1.3 ),

ctlpt( E3, 2.4, 0.2, 2.2 ) ),

list( ctlpt( E3, 2.2, 1.1, 0.4 ),

ctlpt( E3, 2.9, 1.2, 1.5 ),

ctlpt( E3, 2.7, 1.3, 2.6 ) ),

list( ctlpt( E3, 2.5, 2.9, 0.7 ),

ctlpt( E3, 2.3, 2.8, 1.7 ),

ctlpt( E3, 2.1, 2.7, 2.7 ) ) ) ),

list( list( KV_OPEN ),

list( KV_OPEN ),

list( KV_OPEN ) ) );

tvi = TINTERP( tv, 0, 0, 0, 0, 0, 0 );

creates a quadratic by quaratic by linear trivariate tvi that

interpolates the control points of tv at the node parameter values.

TMORPH

------

TrivarType TMORPH( TrivarType TV1, SurfaceType TV2, NumericType Blend )

creates a new trivariate which is a convex blend of the two given

trivariates.  The two given trivariates must be compatible (see FFCOMPAT)

before this blend is invoked. This isv ery useful if a sequence that "morphs"

one trivariate to another is to be created and in combination with MRCHCUBE.

Example:

Size = 0.05;

for ( i = 0, step, 1.0,

Tv = TMORPH( Tv1, Tv2, i ):

view( mrchcube( list( Tv, 1, off ),

point( Size, Size, Size ), 1, IsoVal ), on ) );

creates a sequence of 1/step trivariates, morphed from Tv1 to Tv2

and displays an extracted iso surface at level IsoVal.

See also MRCHCUBE, PMORPH, CMORPH and SMORPH.

TNSCRCR

-------

ListType TNSCRCR( PointType Cntr1, NumericType Rad1,

PointType Cntr2, NumericType Rad2, NumericType OuterTans )

computes the two outer, if OuterTans TRUE, or the two inner

if OuterTans FALSE, bitangents between the prescribed two circles.

Note the bitangents might no exist of one circle is containt in the other.

Example:

T1 = TnsCrCr( point( -2, 0.3, 0 ), 0.7, point( 1, 0, 0 ), 1, 0 );

T2 = TnsCrCr( point( -2, 0.3, 0 ), 0.7, point( 1, 0, 0 ), 1, 1 );

See also CRC2CRVTAN.

TOFFSET

-------

ListType TOFFSET( CurveType Crv, CurveType OffCrv, NumericType Tolerance,

NumericType TrimAmount, NumericType NumerTol )

trims local and global self intersections in curve OffCrv that

is an offset approximation to curve Crv with tolerance Tolerance

Curve OffCrv is trimmed if found closer than TrimAmount to

Crv. OffCrv could be computed from Crv using any available

offset approximation method such as OFFSET, AOFFSET, LOFFSET and MOFFSET.

If NumerTol is positive, a numerical step is applied to the

end points of the trimmed pieces to match them with greater

precision.

Example:

c0 = cbspline( 3,

list( ctlpt( E2, -1,   3 ),

ctlpt( E2, -0.3, 0 ),

ctlpt( E2,  0.3, 0 ),

ctlpt( E2,  1,   3 ) ),

list( kv_open ) );

for ( i = -5, 1, 5,

if ( i != 0,

ofst = 0.15 * i:

co = offset( c0, ofst, 0.0001, off ):

none = TOFFSET( c0, co, 15, abs( ofst * 0.999 ), 0.001 ):

color( none, i + 6 ):

viewobj( none ) ) );

approximates several offset curves at offset amounts of 0.15 * i to

curve c0 and trim the self intersections detected in them.

TORUS

-----

PolygonType TORUS( VectorType Center, VectorType Normal,

NumericType MRadius, NumericType mRadius )

creates a TORUS geometric object, defined by Center as the center

of the TORUS, Normal as the normal to the main plane of the TORUS,

MRadius and mRadius as the major and minor radii of the TORUS.

See RESOLUTION for the accuracy of the TORUS approximation as a polygonal

model. See IRITSTATE's "PrimRatSrfs" and "PrimRatSrfs" state variables.

Example:

T = TORUS( vector( 0.0, 0.0, 0.0), vector( 0.0, 0.0, 1.0), 0.5, 0.2 );

constructs a torus with its major plane as the XY plane, major radius of

0.5, and minor radius of 0.2.

TRAISE

------

TrivarType TRAISE( TrivarType TV, ConstantType Direction,

NumericType NewOrder )

raises TV to the specified NewOrder in the specified

Direction.

Example:

tv1r = TRAISE( traise( traise( tv1, row, 4 ), col, 4 ), depth, 4 );

ensures that the trivariate is a tricubic.

See also MRAISE, SRAISE, and CRAISE.

TREFINE

-------

TrivarType TREFINE( TrivarType TV, ConstantType Direction,

NumericType Replace, ListType KnotList )

provides the ability to Replace a knot vector of TV or refine

it in the specified direction Direction (ROW, COL, or DEPTH).

KnotList is a list of knots at which to refine TV. All knots should be

contained in the parametric domain of TV in Direction. If the knot

vector is replaced, the length of KnotList should be identical to the

length of the original knot vector of TV in the Direction. If TV

is a Bezier trivariate, it is automatically promoted to be a B-spline

trivariate.

Example:

TV = TREFINE( TREFINE( TREFINE( TV,

ROW, FALSE, list( 0.333, 0.667 ) ),

COL, FALSE, list( 0.333, 0.667 ) ),

DEPTH, FALSE, list( 0.333, 0.667 ) );

refines TV in all directions by adding two more knots at 0.333 and

0.667.   See also CREFINE, SREFINE, and MREFINE.

TREGION

-------

TrivarType TREGION( TrivarType TV, ConstantType Direction,

NumericType MinParam, NumericType MaxParam )

extracts a region of TV between MinParam and MaxParam

in the specified Direction. Both MinParam and MaxParam

should be contained in the parametric domain of TV in the Direction.

Example:

Tv1 = tbezier( list( list( list( ctlpt( E3, 0.1, 0.0, 0.8 ),

ctlpt( E3, 0.2, 0.1, 2.4 ) ),

list( ctlpt( E3, 0.3, 2.2, 0.2 ),

ctlpt( E3, 0.4, 2.3, 2.0 ) ) ),

list( list( ctlpt( E3, 2.4, 0.8, 0.1 ),

ctlpt( E3, 2.2, 0.7, 2.3 ) ),

list( ctlpt( E3, 2.3, 2.6, 0.5 ),

ctlpt( E3, 2.1, 2.5, 2.7) ) ) ) );

Tv1r1 = TREGION( Tv1, row, 0.1, 0.2 );

Tv1r2 = TREGION( Tv1, row, 0.4, 0.6 );

Tv1r3 = TREGION( Tv1, row, 0.99, 1.0 );

extracts three regions of Tv1 along the ROW direction.

See also CREGION, SREGION, and MREGION.

TREPARAM

--------

TrivarType TREPARAM( TrivarType TV, ConstantType Direction,

NumericType MinParam, NumericType MaxParam )

reparametrizes TV over a new domain from MinParam to MaxParam,

in the prescribed Direction. This operation does not affect the geometry

of the trivariate and only affine transforms its knot vectors. A Bezier

trivariate will automatically be promoted into a B-spline surface by this

function.

Example:

Tv = TREPARAM( TREPARAM( TREPARAM( tv, row, 0, 1 ),

col, 0, 1 ),

depth, 0, 1 );

ensures that the trivariate is defined over the unit size parametric

cube.   See also CREPARAM, SREPARAM, and MREPARAM.

TRIANGL

-------

PolygonType TRIANGL( PolygonType Model, NumericType Regular )

converts Model into a new model with exactly the same shape that holds

only triangles.  If the Regular is not zero, the object is regularized as

well.

Example:

final2 = triangl( final, false );

TRIMSRF

-------

TrimSrfType TRIMSRF( SurfaceType Srf,

CurveType TrimCrv,

NumericType HasUpperLevel )

or

TrimSrfType TRIMSRF( SurfaceType Srf,

ListType TrimCrvs,

NumericType HasUpperLevel )

create a trimmed surface from the provided surface Srf and the

trimming curve TrimCrv or curves TrimCrvs. If HasUpperLevel

is FALSE, an additional trimming curve is automatically added that

contains the entire parametric domain of Srf. No validity test is

performed on the trimming curves which are assumed to be two-dimensional

curves contained in the parametric domain of Srf.

Example:

spts = list( list( ctlpt( E3, 0.1, 0.0, 1.0 ),

ctlpt( E3, 0.3, 1.0, 0.0 ),

ctlpt( E3, 0.0, 2.0, 1.0 ) ),

list( ctlpt( E3, 1.1, 0.0, 0.0 ),

ctlpt( E3, 1.3, 1.5, 2.0 ),

ctlpt( E3, 1.0, 2.1, 0.0 ) ),

list( ctlpt( E3, 2.1, 0.0, 2.0 ),

ctlpt( E3, 2.3, 1.0, 0.0 ),

ctlpt( E3, 2.0, 2.0, 2.0 ) ),

list( ctlpt( E3, 3.1, 0.0, 0.0 ),

ctlpt( E3, 3.3, 1.5, 2.0 ),

ctlpt( E3, 3.0, 2.1, 0.0 ) ),

list( ctlpt( E3, 4.1, 0.0, 1.0 ),

ctlpt( E3, 4.3, 1.0, 0.0 ),

ctlpt( E3, 4.0, 2.0, 1.0 ) ) );

sb = sbspline( 3, 3, spts, list( list( KV_OPEN ), list( KV_OPEN ) ) );

TCrv1 = cbspline( 2,

list( ctlpt( E2, 0.3, 0.3 ),

ctlpt( E2, 0.7, 0.3 ),

ctlpt( E2, 0.7, 0.7 ),

ctlpt( E2, 0.3, 0.7 ),

ctlpt( E2, 0.3, 0.3 ) ),

list( KV_OPEN ) );

TCrv2 = circle( vector( 0.5, 0.5, 0.0 ), 0.25 );

TCrv3 = cbspline( 3,

list( ctlpt( E2, 0.3, 0.3 ),

ctlpt( E2, 0.7, 0.3 ),

ctlpt( E2, 0.7, 0.7 ),

ctlpt( E2, 0.3, 0.7 ) ),

list( KV_PERIODIC ) );

TSrf1 = TRIMSRF( sb, TCrv1, false );

TSrf2 = TRIMSRF( sb, TCrv1, true );

TSrf3 = TRIMSRF( sb, list( TCrv1, TcRv2 * ty( 1 ), TCrv3 * ty( 2 ) ),

false );

constructs three trimmed surfaces. Tsrf1 contains the outer boundary

and excludes what is inside TCrv1, TSrf2 contains only the

domain inside TCrv1. TCrv3 has three holes corresponding to the

three trimming curves.  See also TRMSRFS.

TRMSRFS

-------

TrimSrfType TRMSRFS( SurfaceType Srf, CurveType Cntrs )

or

TrimSrfType TRMSRFS( SurfaceType Srf, PolyType Cntrs )

or

TrimSrfType TRMSRFS( SurfaceType Srf, ListType Cntrs )

create a set of trimmed surfaces from the provided surface Srf and

the set of contours Cntrs in Srf's parametric domain.

The contours in Cntrs can be polylines, curves, or a list of

such entities.  The contours in Cntrs must be either closed or start

and end on the boundary of the parametric domain of Srf.  Further,

these contours must be (self) intersection free.

The returned result is a (list of) trimmed surfaces, each defining one

sub-region that results from Cntrs's trimming.

Example:

tsrfs = TRMSRFS( srf,

list( poly( list( point( 0.0, 0.2, 0.0 ),

point( 1.0, 0.5, 0.0 ) ), true ),

cbezier( list( ctlpt( E2, 0.0, 2.5 ),

ctlpt( E2, 0.5, 2.5 ),

ctlpt( E2, 0.5, 3.0 ) ) ) ) );

interact( list( nth( tsrfs, 1 ) * tz( -0.2 ),

nth( tsrfs, 2 ) * tz(  0.0 ),

nth( tsrfs, 3 ) * tz(  0.2 ) ) );

constructs trimmed surfaces using two contours. One contour is a polyline

with two points, and the other is a quadratic Bezier curve. See also TRIMSRF.

TSBEZIER

--------

SurfaceType TSBEZIER( NumericType Order, ListType CtlMesh )

creates a triangular Bezier surface of order Order using the

provided control mesh. CtlMesh is a list of control points of

size (Order + 1) * Order / 2. All control points

must be of type (E1-E9, P1-P9), or regular PointType defining the surface's

control polygon. The surface point type will be of a space which is the union

of the spaces of all points.

Example:

b = TSBEZIER( 3,

list( ctlpt( E3, 0.0,  0.0,  0.4 ),

ctlpt( E3, 0.3,  0.0,  0.3 ),

ctlpt( E3, 0.7,  0.0,  0.8 ),

ctlpt( E3, 0.2,  0.4,  1.0 ),

ctlpt( E3, 0.4,  0.5,  1.0 ),

ctlpt( E3, 0.5,  1.0,  0.7 ) ) );

See also TSGREGORY and TSBSPLINE.

TSBSPLINE

---------

TriSrfType TSBSPLINE( NumericType Order, NumericType Length,

ListType CtlMesh, ListType KnotVector )

creates a B-spline surface from the provided Order and Length,

the control mesh CtlMesh, and the knot vector KnotVector.

CtlMesh is a list of control points of size (Length + 1) *

Length / 2.  All control points must be of point type (E1-E9,

P1-P9), or regular PointType defining the surface's control mesh.

The surface point type will be of a space which is the union of the spaces

of all points.

KnotVector is a list of NumericType knots of length Length

plus the Order.

The knot vector may also be a list of a single constant KV_OPEN or

KV_FLOAT, in which a uniform knot vector with the appropriate length

and with an open or floating end condition will be constructed automatically.

Not fully supported at this time.   See also TSBEZIER and TSGREGORY.

TSDERIVE

--------

TriSrfType TSDERIVE( TriSrfType Srf, NumericType Dir )

returns a vector field surface representing the differentiated triangular

surface in the given direction (ROW, COL, or DEPTH). Evaluation of

the returned surface at a given parameter value will return a vector

tangent to Srf in Dir at that parameter value.

DuSrf = TSDERIVE( Srf, ROW );

DvSrf = TSDERIVE( Srf, COL );

Normal = coerce( tseval( DuSrf, 0.5, 0.25, 0.25 ), VECTOR_TYPE ) ^

coerce( tseval( DvSrf, 0.5, 0.25, 0.25 ), VECTOR_TYPE );

computes two partial derivatives of the surface Srf and computes

its normal as their cross product, at the parametric location

(0.5, 0.25, 0.25).

See also TSNORMAL

TSEVAL

------

CtlPtType TSEVAL( TriSrfType Srf,

NumericType UParam, NumericType VParam, NumericType WParam )

evaluates the provided triangular surface Srf at the given

UParam, VParam, WParam parameters.

UParam, VParam, and WParam must all be non negative

and must sum to one for a Bezier triangular surface or to the maximum

domain, if a B-spline surface.

Example:

CPt = TSEVAL( Srf, u, v, 1.0 - u - v );

evaluates Srf at the parameter values prescribed by u and v.

TSGREGORY

---------

SurfaceType TSGREGORY( NumericType Order, ListType CtlMesh )

cCreates a triangular Gregory surface of order Order using the

provided control mesh. CtlMesh is a list of control points of

size (Order + 1) * Order / 2 + 3. All control points

must be of type (E1-E9, P1-P9), or regular PointType defining the surface's

control polygon.  The surface point type will be of a space which is the union

of the spaces of all points.

Example:

Srf = tsgregory( 5,

list( ctlpt( E3, 2, -1, 0 ),

ctlpt( E3, 2.3, -1, 0.25 ),

ctlpt( E3, 2.6, -1, 0.25 ),

ctlpt( E3, 2.8, -1, 0.13 ),

ctlpt( E3, 3, -1, 0 ),

ctlpt( E3, 2.25, -0.7, 0.25 ),

ctlpt( E3, 2.5, -0.7, -0.25 ),

ctlpt( E3, 2.6, -0.7, -0.15 ),

ctlpt( E3, 2.75, -0.7, 0.25 ),

ctlpt( E3, 2.4, -0.4, 0.25 ),

ctlpt( E3, 2.5, -0.4, 0 ),

ctlpt( E3, 2.6, -0.4, -0.25 ),

ctlpt( E3, 2.45, -0.2, 0.12 ),

ctlpt( E3, 2.55, -0.2, -0.12 ),

ctlpt( E3, 2.5, 0, 0 ),

ctlpt( E3, 2.5, -0.7, -0.25 ),

ctlpt( E3, 2.6, -0.7, -0.15 ),

ctlpt( E3, 2.5, -0.4, 0 ) ) );

Not fully supported at this time.   See also TSBEZIER and TSBSPLINE.

TSNORMAL

--------

VectorType TSNORMAL( TriSrfType Srf,

NumericType UParam, NumericType VParam, NumericType WParam)

computes the normal vector to a triangular surface Srf at the

parameter values UParam, VParam, WParam. The returned

vector has a unit length.

UParam, VParam, and WParam must all be non

negative and must sum to one for a Bezier triangular surface or to

the maximum domain, if a B-spline surface.

Example:

Normal = TSNORMAL( Srf, 0.5, 0.5, 0.0 );

computes the normal to Srf at the parameter values (0.5, 0.5, 0.0).

TVLOAD

------

TrivarType TVLOAD( StringType FileName,

NumericType DataType,

VectorType VolSize,

VectorType Orders )

loads a volumetric data set from file FileName in as a

trivariate of orders Orders.  DataType can be one of:

1          Regular ASCII (separated by white spaces).

2          Two bytes short integer.

3          Four bytes long integer.

4          One byte (char) integer.

5          Four bytes float.

6          Eight bytes double.

Beware of the little vs big Endian problem! We assume here you have read the

volume in the same machine type in which this file was written.

VolSize provides the dimensions of the volume, with width first and

depth last.  Uniform open end condition knot vectors are constructed to all

three axes.

Example:

Tv = TVLOAD( "3dhead", 1, vector( 32, 32, 13 ), vector( 3, 3, 3 ) );

loads the data set "3dhead" of size (32, 32, 13) as a triquadratic

function.  THe sata set is assumed to contain ASCII numeric values.

See also MRCHCUBE.

TVZRJACOB

---------

PolyType TVZRJACOB( TrivarType TV,

NumericType Euclidean,

NumericType SkipRate,

NumericType Fineness )

computes the zero set of the Jacobian of the given trivariate, TV.

This zero set is the implicit boundary of the trivariate and, for example,

equals the envelop of the sweep of a bivariate surface in space (see

example below).  The zero set is returned as a polygonal data set

approximation with Fineness tolerance.  If Euclidean, the

resulting polygons are in Euclidean space.  Otherwise, the polygons are

returned in the trivariate's parametric domain. Finally, SkipRate

provides a mechanism to skip to every SkipRate row, column and plane

while a SkipRate skips nothing.

Example:

Tv = tfromsrfs( list( Srf,

Srf * tx( 3 ) * ty( 3 ),

Srf * tx( 6 ) ), 3 );

Tv1ZeroJacobian = TVZRJACOB( Tv, 1, 1, 0 );

A trivariate TV is constructed as a sweep of surface Srf

along a quadratic Bezier curve with (0, 0), (3, 3), (6, 0) as control points,

and then the zero set of the Jacobian is derived to yield the envelope of

this motion of Srf.

UVPOLY

------

PolyType UVPOLY( PolyType Obj, ListType Scales );

Sets UV coordinates to polygonal object Obj.  The UV coordinates

are set using the XY Euclidean coordinates if Scales is a list that

holds two scaling factors (XScale, YScale), or the UV coordinates are

set via the two largest span in XYZ for each polygon, if Scales is

a list of three scaling factors (XScale, YScale, ZScale).

Needless to say, the ?Scale factors scales the Euclidean coordinates before

being sets as UV texture coordinates.

Example:

UVCube = UVPOLY( Cube, scale( 1, 1, 1 ) );

sets UV coordinates to the six faces of the cube, each face with UV

values between zero and one.

ZCOLLIDE

--------

NumericType ZCOLLIDE( GeometricTreeType Obj1,

GeometricTreeType Obj2,

NumericType Fineness,

NumericType NumOfIters );

Given two objects, Obj1 and Obj2, where Obj1 is assumed to

be above (in the Z direction) Obj2, this function computes the amount

that Obj1 could be moved down, the -Z direction, until it collides with

Obj2.  The collision detection is considered using a polygonal

approximation that has a Fineness resolution (see RESOLUTION variable).

The computation cost is linear in NumOfIters with quadratic accuracy

convergence.  Values of ten for both Fineness and NumOfIters are

reasonable selections.  While Obj1 is considered in its exact form, in

Obj2, only the bbox of the shape is considered.

Example:

view( chair, 1 );

for ( x = 0, 1, 5,

b = box( vector( x / 10, 0, 2 ), 0.1, 0.1, 0.1 ):

view( b * tz( ZCOLLIDE( chair, b, 10, 10 ) ), 0 ) );

places and draws six different cubes on top of the object called chair.

Object transformation functions

-------------------------------

All the routines in this section construct a 4 by 4 homogeneous

transformation matrix representing the required transform. These matrices

may be concatenated to achieve more complex transforms using the matrix

multiplication operator *. For example, the expression

m = trans( vector( -1, 0, 0 ) ) * rotx( 45 ) * trans( vector( 1, 0, 0 ) );

constructs a transform to rotate an object around the X = 1 line, 45 degrees.

A matrix representing the inverse transformation can be computed as:

InvM = m ^ -1

See also overloading of the - operator.

HOMOMAT

-------

MatrixType HOMOMAT( ListType MatData )

creates an arbitrary homogeneous transformation matrix by manually providing

its 16 coefficients.

Example:

step = 10;

for ( a = 1, 1, 720 / step,

view_mat = save_mat *

HOMOMAT( list( list( 1, 0, 0, 0 ),

list( 0, 1, 0, 0 ),

list( 0, 0, 1, -a * step / 500 ),

list( 0, 0, 0, 1 ) ) ):

view( list( view_mat, axes ), on )

);

looping and viewing through a sequence of perspective transforms, created

using the HOMOMAT constructor.  See also RFLCTMAT and PROJMAP.

MATPOSDIR

---------

MatrixType MATPOSDIR( PointType Pos, VectorType Dir, VectorType UpDir )

creates a viewing transformation matrix of a viewer at Pos, looking

at direction Dir and upper view of UpDir.

Example:

step = 10;

for ( a = 1, 1, 720 / step,

view_mat = MATPOSDIR( point( 0.5, 0.1, 0.5 ),

vector( 0.0, 1.0, 0.0 ),

vector( cos( a * step * Pi / 360 ), 0,

sin( a * step * Pi / 360 ) ) ):

view( list( view_mat, axes ), on )

);

looping and viewing through a sequence of transforms, created

using the MATPOSDIR constructor.

PROJMAT

-------

MatrixType PROJMAT( PlaneType ProjPlane,

VectorType EyePos,

NumericType EyeInf )

constructs a projection matrix to project the universe onto the given

projection plane ProjPlane, with the eye position at EyePos

(divided by EyeInf).  Note that if EyeInf is zero, the eye is

at infinity.

Example:

PMat = PROJMAT( plane( 0, 0, 1, -0.1 ), vector( 1, 1, 1 ), 0 );

contstructs a projection matrix PMat onto the Z = -0.1 plane

with a view direction of ( 1, 1, 1 ).  See also RFLCTMAP, HOMOMAT.

RFLCTMAT

--------

MatrixType RFLCTMAT( PlaneType RflctPlane )

constructs a reflection matrix to reflect the universe along the given

reflection plane RflctPlane.

Example:

PMat = RFLCTMAT( plane( 0, 0, 1, 0 ) );

constructs a reflection matrix PMat around the Z = 0 plane.

See also PROJMAP, HOMOMAT.

ROTVEC

------

MatrixType ROTVEC( VectorType Vec, NumericType Angle )

creates a rotation around the vector Vec matrix with Angle degrees.

ROTX

----

MatrixType ROTX( NumericType Angle )

creates a rotation around the X transformation matrix with Angle degrees.

ROTY

----

MatrixType ROTY( NumericType Angle )

creates a rotation around the Y transformation matrix with Angle degrees.

ROTZ

----

MatrixType ROTZ( NumericType Angle )

creates a rotation around the Z transformation matrix with Angle degrees.

ROTZ2V

------

MatrixType ROTZ2V( VectorType Dir )

creates a rotation matrix that takes Z axis into Dir. Length of Dir

is ignored.

ROTZ2V2

-------

MatrixType ROTZ2V2( VectorType Dir, VectorType Dir2 )

creates a rotation matrix that takes the Z axis into Dir, while the X

axis is aligned with Dir2. The lengths of Dir and Dir2 are

ignored.

SCALE

-----

MatrixType SCALE( VectorType ScaleFactors )

creates a scaling by the ScaleFactors transformation matrix.

TRANS

-----

MatrixType TRANS( VectorType TransFactors )

creates a translation by the TransFactors transformation matrix.

General purpose functions

---------------------------

ADWIDTH

-------

ADWIDTH( GeometricType Object, NumericType DWidth )

sets the width of the object.  This display width is used in pixels in

display devices for width of line drawing, if supported by the display device.

See also ATTRIB, COLOR, and AWIDTH.

This function is equivalent to using,

ATTRIB( Object, "dwidth", Width );

ANIMEVAL

--------

ANIMEVAL( NumericType Time, AnyType Object )

evaluates the animation curves in Object at time Time.  The

transformations for time Time are saved at the respective sub objects

of Object as "animation_mat" matrices.

For example,

mov_x = cbezier( list( ctlpt( E1, 0.0 ), ctlpt( E1, 1.0 ) ) );

attrib( axes, "animation", list( mov_x ) );

ANIMEVAL( 0.5, axes );

and axes will have a matrix in attribute "animation_mat" of translation in x

of 1/2.

ATTRIB

------

ATTRIB( AnyType Object, StringType Name, AnyType Value )

provides a mechanism to add an attribute of any type to an Object,

with name Name and value Value. This ATTRIB function is tuned

and optimized toward numeric values or strings as Value although any

other object type can be saved as attribute.

These attributes may be used to pass information to other programs about

this object, and are saved with the objects in data files.

Attributes placed on a list object or even a whole hierarchy of

objects will be propagated into all items in the list or hierarchy.

There are a few exception to this propagation. The "animation" attribute

is not propagated and is kept in the internal nodes, forming a hierachy of

animation commands for all the objects contained in the list/hierarchy.

The "invisible" attribute is saved at all levels of the hierarchy, used to

denote a complete sub tree that is invisible (yet can serve as a source at

which instances can point).

For example,

ATTRIB(Glass, "rgb", "255,0,0");

ATTRIB(Glass, "refract", "1.4");

.

.

.

RmAttr(Glass, "rgb");   # Removes "rgb" attribute.

sets the RGB color and refraction index of the Glass object and

later removes the RGB attribute.

Attribute names are case insensitive. Spaces are allowed in the Value

string, as well as the double quote itself, although the latter must be

escaped:

ATTRIB(Glass, "text", "Say "this is me"");

See also RMATTR for removal of attributes, CPATTR for copying them,

GETATTR to get an attribute, ATTRPROP for setting attributes on all

subtrees of parts, as well as AWIDTH, ADWIDTH, COLOR and PATTRIB.

ATTRPROP

--------

ATTRPROP( AnyType Object, StringType Name, AnyType Value )

Same as ATTRIB but propagates the attributes to all sub-parts of the

object.

Example:

Glass1 = list( Base, Handle, Wine );

Glass2 = list( Base, Handle, Wine );

attrib( Glass1, "ptexture", "marble1.gif" );

ATTRPROP( Glass2, "ptexture", "marble1.gif" );

In Glass1, only Glass1 will be set with "texture" while in

Glass2, the "texture" attribute will propagate to the sub-parts of

Glass2, namely to the Base, Handle, Wine.

AWIDTH

------

AWIDTH( GeometricType Object, NumericType Width )

sets the width of the object to one of those specified below.  This width

is used in real object side dimensions in tools such as scan converters and

rendering tools for rendering lines and curves, as well as postscript.

See also ATTRIB, COLOR, and ADWIDTH.

This function is equivament to using,

ATTRIB( Object, "width", Width );

CHDIR

-----

CHDIR( StringType NewDir )

sets the current working directory to be NewDir.

CLNTCLOSE

---------

CLNTCLOSE( NumericType Handler, NumericType Kill )

closes a communication channel to a client. Handler contains the

index of the communication channel opened via CLNTEXEC. If Kill, the

client is sent an  exit request for it to die. Otherwise, the communication

is closed and the client runs standing alone.  See also VIEWOBJ, VIEWSET,

CLNTREAD, CLNTWRITE, and CLNTEXEC.

Example:

h2 = clntexec( "nuldrvs -s-" );

.

.

.

CLNTCLOSE( h2,TRUE );

closes the connection to the nuldrvs client, opened via CLNTEXEC.

CLNTWRITE

---------

CLNTWRITE( NumericType Handler, AnyType Object )

writes one object Object to a communication channel of a client.

Handler contains the index of the communication channel opened via

CLNTEXEC.  If the Handler equals -1, the regular display device

(forked via, for example, VIEWOBJ command) is used.  If Handler equals

CLIENTS_ALL, a broadcast of Object to all clients is performed.

See also VIEWOBJ, VIEWSET, CLNTREAD, CLNTCLOSE, and CLNTEXEC.

Example:

h2 = clntexec( "nuldrvs -s-" );

.

.

CLNTWRITE( h2, Model );

.

.

clntclose( h2,TRUE );

writes the object named Model to client through communication channel h2.

COLOR

-----

COLOR( GeometricType Object, NumericType Color )

sets the color of the object to one of those specified below. Note that an

object has a default color (see irit.cfg file) according to its origin -

loaded with the LOAD command, PRIMITIVE, or a BOOLEAN operation result.

The system internally supports colors (although you may have a B&W

system) and the colors recognized are:

BLACK, BLUE, GREEN, CYAN, RED, MAGENTA, YELLOW, and WHITE.

See the ATTRIB command for more fine control of colors using the RGB

attribute. See also AWIDTH and AWIDTH.

This function is equivament to using,

ATTRIB( Object, "color", Width );

COMMENT

-------

COMMENT

Two types of comments are allowed:

1. One-line comment: starts anywhere in a line at the '#' character, up to

the end of the line.

2. Block comment: starts at the COMMENT keyword followed by a unique

character (anything but white space), up to the second occurrence of that

character. This is a fast way to comment out large blocks.

Example:

COMMENT

This is a comment

CPATTR

------

CPATTR( AnyType DestObj, AnyType SrcObj )

copies all attribute from object SrcObj into object DestObj. All

attributes, if any, in DestObj are purged.  Needless to say, both objects

must exist at the time of attribute copy.

See also ATTRIB, ATTRPROP, GETATTR, RMATTR.

ERROR

-----

ERROR( StringType Message);

breaks the execution and returns to the IRIT main loop, after printing a

Message to the screen. This may be useful in user defined functions to

break execution in cases of fatal errors.

EXEC

----

EXEC( StringType Command );

executes a string Command in the IRIT interepreter, indirectly.

Example:

Univariate2Bezier = function( Polynom, Deg ): x: f:

return = nil():

f = 1:

for ( x = 0, 0.05 / Deg, 1,

EXEC( "f = " + Polynom ):

snoc( ctlpt( E1, f ), return ) ):

return = coerce( cinterp( return, Deg + 1, Deg + 1, PARAM_UNIFORM, FALSE ),

bezier_type );

defines a function that converts univariate expressions into explicit, E1,

Bezier curves.  For example

"Univariate2Bezier( "3 * x ^ 2 - 2 * x + 5", 3 );"

would return a cubic Bezier curve representing "3 * x ^ 2 - 2 * x + 5".

EXIT

----

EXIT()

exits from the solid modeler. NO warning is given!

FOR

---

FOR( NumericType Start, NumericType Increment, NumericType End, AnyType Body )

executes the Body (see below), while the FOR loop conditions hold.

Start, Increment, End are evaluated first, and the loop is executed

while <= End if Increment > 0, or while >= End if Increment < 0.

If Start is of the form "Variable = Expression", then that variable is

updated on each iteration, and can be used within the body.

The body may consist of any number of regular commands, separated by

COLONs, including nesting FOR loops to an arbitrary level.

Example:

step = 10;

rotstepx = rotx(step);

FOR ( a = 1, 1, 360 / step,

view_mat = rotstepx * view_mat:

view( list( view_mat, axes ), ON )

);

displays axes with a view direction that is rotated 10 degrees at a

time around the X axis.

HELP

----

HELP( StringType Subject )

provides help on the specified Subject.

Example:

HELP("");

will list all IRIT help subjects.

FNFREE

------

FNFREE( StringType UserFuncName )

frees a user defined function named UserFuncName. See also FREE.

FREE

----

FREE( GeometricType Object )

Because of the usually huge size of geometric objects, this procedure

may be used to free them. Reassigning a value (even of different type)

to a variable automatically releases the old variable's allocated space

as well.  See also FNFREE.

FUNCTION

--------

FuncName = FUNCTION(Prm1, Prm2, ... , PrmN):LclVal1:LclVar2: ... :LclVarM:

FuncBody;

defines a function named FuncName with N parameters and M local variables

(N, M >= 0). Here is a (simple) example of a function with no local variables

and a single parameter that computes the square of a number:

sqr = FUNCTION(x):

return = x * x;

Functions can be defined with optional parameters and optional local

variables. A function's body may contain an arbitrary set of expressions

including for/while loops, (user) function calls, or even recursive function

calls, all separated by colons.

The returned value of the function is the value of an automatically defined

local variable named return. The return variable is a regular local variable

within the scope of the function and can be used as any other variable.

If a variable's name is found in neither the local variable list nor

the parameter list, it is searched for in the global variable list (outside

the scope of the function). Binding of names of variables is static as in the

C programming language.

Because binding of variables is performed in execution time, there is a

somewhat less restrictive type checking of parameters of functions that are

invoked within a user's defined function.

A function can invoke itself, i.e., it can be recursive. However, since a

function should be defined when it is called, a dummy function should be

defined before the recursive one is defined:

factorial = function(x):return = x; # Dummy function.

factorial = function(x):

if (x <= 1, return = 1, return = x * factorial(x - 1));

Overloading is valid inside a function as it is outside. For example, for

add = FUNCTION(x, y):

return = x + y;

the following function calls are all valid:

add(1, 2);

add(vector(1,2,3), point(1,2,3));

add(box(vector(-3, -2, -1), 6, 4, 2), box(vector(-4, -3, -2), 2, 2, 4));

Finally, here is a more interesting example that computes an approximation

of the length of a curve, using the sqr function defined above:

distptpt = FUNCTION(pt1, pt2):

return = sqrt(sqr(coord(pt1, 1) - coord(pt2, 1)) +

sqr(coord(pt1, 2) - coord(pt2, 2)) +

sqr(coord(pt1, 3) - coord(pt2, 3)));

crvlength = FUNCTION(crv, n):pd:t:t1:t2:dt:pt1:pt2:i:

return = 0.0:

pd = pdomain(crv):

t1 = nth(pd, 1):

t2 = nth(pd, 2):

dt = (t2 - t1) / n:

pt1 = coerce(ceval(crv, t1), e3):

for (i = 1, 1, n,

pt2 = coerce(ceval(crv, t1 + dt * i), e3):

return = return + distptpt(pt1, pt2):

pt1 = pt2);

Try, for example:

crvlength(circle(vector(0.0, 0.0, 0.0), 1.0), 30) / 2;

crvlength(circle(vector(0.0, 0.0, 0.0), 1.0), 100) / 2;

crvlength(circle(vector(0.0, 0.0, 0.0), 1.0), 300) / 2;

See PROCEDURE and IRITSTATE's "DebugFunc" for more.

IF

--

IF( NumericType Cond, AnyType TrueBody { , AnyType FalseBody } )

executes TrueBody (a group of regular commands, separated by COLONs -

see FOR loop) if the Cond holds, i.e., it is a numeric value other than

zero, or optionally, if it exists, executes FalseBody. If the Cond

does not hold, i.e., it evaluates to a numeric value equal to zero.

Examples:

IF ( machine == IBMOS2, resolution = 5, resolution = 10 );

IF ( a > b, max = a, max = b );

sets the resolution to 10, unless running on an IBMOS2 system, in which

case the RESOLUTION variable will be set to 5 in the first statement, and

set to max to the maximum of a and b in the second statement.

INCLUDE

-------

INCLUDE( StringType FileName )

executes the script file FileName. Nesting of an include file is allowed

up to 10 levels deep. If an error occurs, all open files in all nested files

are closed and data are waited for at the top level (standard input).  Files

are searched for inclusion in the current directory.  If not found, and the

inclusion is from a different file at some directory, that directory is

searched as well.  Finally, if all the above fails, the directories specified

via the IRIT_INCLUDE environment variable are also searched.

A script file can contain any command the solid modeler supports.

Example:

INCLUDE( "/tmp/general.irt" );

includes the file "/tmp/general.irt".  Any inclusion inside general.irt will

search for the included file in the current directory, then in /tmp, and then

in the directories specified via IRIT_INCLUDE.

INSERTPOLY

----------

INSERTPOLY( PolyType Poly, PolyType Polys )

inserts, in place, Poly as a new polygon of object Polys.

After the completion of this function Poly is unmodified but Polys

has a new polygon in it.

Example:

X = poly( list( point( 0, 0, 0 ),

point( 0, 1, 0 ),

point( 1, 1, 0 ),

point( 1, 0, 0 ) ), false );

Y = X * tz( 1 );

INSERTPOLY( Y, X );

At the end of the execution of this sequence of command, X contains

two polygons, one at Z = 0 and one at Z = 1.

INTERACT

--------

INTERACT( GeometryTreeType Object )

This is a user-defined function (see iritinit.irt) that does the

following, in order outlined:

Clear the display device.

Display the given Object.

Pause for a keystroke.

This user-defined function in version 4.0 of IRIT is an

emulation of the INTERACT function that used to exist in previous versions.

Example:

INTERACT( list( view_mat, Axes, Obj ) );

displays and interacts with the object Obj and the predefined object

Axes. VIEW_MAT will be used to set the starting transformation.

See VIEW and VIEWOBJ for more.

IQUERY

------

IQUERY( NumericType QueryType )

A low level query tool for checking the current state of the IRIT internal

tables.  According to the values of QueryType the following is printed

to stdout:

QueryType       Printed content

1               All the known functions/user defined functions/constants

and parameters/returned values (if any).

2               All the knwon keywords

LIST

----

ListType LIST( AnyType Elem1, AnyType Elem2, ... )

constructs an object as a list of several other objects. Only a reference

is made to the Elements, so modifying Elem1 after being included in the list

will affect Elem1 in that list next time list is used!

Each inclusion of an object in a list increases its internal used

reference. The object is freed iff theused reference is zero.

As a result, attempt to delete a variable (using FREE) which is referenced

in a list removes the variable, but the object itself is freed only when the

list is freed.

LOAD

----

AnyType LOAD( StringType FileName )

loads an object from the given FileName. The object may be any object

defined in the system, including lists, in which the structure is recovered

and reconstructed as well (internal objects are inserted into the global

system object list if they have names). If no file type is provided, ".itd"

is assumed.

This command can also be used to load binary files. ASCII regular data

files usually take longer to load than binary files due to

the required parsing. Binary data files can be loaded directly, like ASCII

files in IRIT, but can only be inspected through IRIT tools such

as dat2irit. A binary data file must have a ".ibd" (IRIT Binary Data) type in

its name.

Compressed files can be loaded if the given file name has a postfix

of ".Z" or .".gz". The gnu utility "gzip" will be invoked via a pipe

for that purpose.

LOGFILE

-------

LOGFILE( NumericType Set )

or

LOGFILE( StringType FileName )

If Set is non zero (see TRUE/FALSE and ON/OFF), then everything

printed in the input window will go to the log file specified in the

irit.cfg configuration file. This file will be created the first time

logfile is turned ON. If a string FileName is provided, it will

be used as a log file name from now on. It also closes the current log

file. A "LOGFILE( on );" must be issued after a log file name change.

Example:

LOGFILE( "Data1" );

LOGFILE( on );

printf( "Resolution = %lf\n", list( resolution ) );

LOGFILE( off );

to print the current resolution level into file Data1.

MSLEEP

------

MSLEEP( NumericType MilliSeconds )

causes the solid modeller to sleep for the prescribed time in milliseconds.

Example:

for ( i = 1, 1, sizeof( crvs ),

c = nth( crvs, i ):

color( c, yellow ):

msleep(20):

viewobj( c )

);

displays an animation sequence and sleeps for 20 milliseconds between

iterations.

NREF

----

AnyType NREF( ListType ListObject, NumericType Index )

returns a reference to the Index (base count 1) element of the

list ListObject.  The reference points to the original object and

hence can be used to modify (add attributes for example) to objects in

lists.  Assignment of this reference to a new object would result in a copy

of the object.  In contrast, a FREE of a reference to an object would have

an undefined result.

Example:

Lst = list( a, b, c );

attrib( NREF( Lst, 2 ), "NewAttr", on );

adds a new attribute to the second element of Lst.  See also NTH.

NRMLCONE

--------

ListType NRMLCONE( SurfaceType Srf )

computes a cone that bounds all normals of surface Srf.  A

list of two objects, the axis vector of the cone and the opening radius,

in radians, is returned.

Example:

NCone = NRMLCONE( Srf );

Cn = Cone( vector( 0, 0, 0 ), normalize( nth( NCone, 1 ) ),

nth( NCone, 2 ), 0 ) * tz( 1.0 ) * sc( 0.75 );

computes a normals' cone for surface Srf and builds a real cone following

these limits.

NTH

---

AnyType NTH( ListType ListObject, NumericType Index )

returns the Index (base count 1) element of the list ListObject.

Example:

Lst = list( a, list( b, c ), d );

Lst2 = NTH( Lst, 2 );

and now Lst2 is equal to 'list( b, c )'.  See also NREF.

PAUSE

-----

PAUSE( NumericType Flush )

waits for a keystroke. This is nice to have if a temporary stop in a middle

of an included file (see INCLUDE) is required. If Flush is TRUE, then

the input is first flushed to guarantee that the actual stop will occur.

PRINTF

------

PRINTF( StringType CtrlStr, ListType Data )

This results in a formatted printing routine, following the concepts of the C

programming language's printf routine. CtrlStr is a string object

for which the following special '%' commands are supported:

%d, %i, %u      Prints the numeric object as an integer or unsigned integer.

%o, %x, %X      Prints the numeric object as an octal or hexadecimal integer.

%e, %f, %g,     Prints the numeric object in several formats of

%E, %F          floating point numbers.

%s              Prints the string object as a string.

%pe, %pf, %pg   Prints the three coordinates of the point object.

%ve, %vf, %vg   Prints the three coordinates of the vector object.

%Pe, %Pf, %Pg,  Prints the four coordinates of the plane object.

%De, %Df, %Dg,  Prints the given object in IRIT's data file format.

All the '%' commands can include any modifier that is valid in the C

programming language PRINTF routine, including l (long), prefix

character(s), size, etc. The point, vector, plane, and object commands

can also be modified in a similar way, to set the format of the

numeric data printed.

Also supported are the newline and tab using the backslash escape

character:

PRINTF("\\tThis is the char \"\\%\"\\n", nil());

Backslashes should be escaped themselves as can be seen in the above example.

Here are few more examples:

PRINTF("this is a string \"%s\" and this is an integer %8d.\\n",

list("STRING", 1987));

PRINTF("this is a vector [%8.5lvf]\\n", list(vector(1,2,3)));

IritState("DumpLevel", 9);

PRINTF("this is a object %8.6lDf...\\n", list(axes));

PRINTF("this is a object %10.8lDg...\\n", list(axes));

This implementation of PRINTF is somewhat different than the C programming

language's version, because the backslash always escapes the next

character during the processing stage of IRIT's parser. That is, the string

'\\tThis is the char \"\\%\"\\n'

is actually parsed by the IRIT's parser into

'\tThis is the char "\%"\n'

because this is the way the IRIT parser processes strings. The latter

string is the one that PRINTF actually sees.

See also PRINTFILE for ways to redirect PRINTF to a file..

PRINTFILE

---------

PRINTFILE( StringType FileName );

Sets the file PRINTF prints to to FileName.  Any newer call to

PRINTFILE will close the current file used so far.  If FileName is

an empty string, PRINTF will print to stdout.

PROCEDURE

---------

ProcName = PROCEDURE(Prm1, Prm2, ... , PrmN):LclVal1:LclVar2: ... :LclVarM:

ProcBody;

A procedure is a function that does not return a value, and therefore the

returned variable (see FUNCTION) should not be used. A procedure is

identical to a function in every other way. See FUNCTION for more.

RESET

-----

RESET()

clears all variables and initializes the environment to the starting state.

User defined functions, however, are kept intact.

RMATTR

------

RMATTR( AnyType Object, StringType Name )

removes attribute named Name from object Object. This function

will have no affect on the Object if the Object has no attribute named

Name.

See also ATTRIB, ATTRPROP, GETATTR, CPATTR.

SAVE

----

SAVE( StringType FileName, AnyType Object )

saves the provided Object in the specified file name FileName.

No extension type is needed (ignored if specified), and ".itd" is supplied

by default. The Object can be any object type, including list, in which

the structure is saved recursively. See also LOAD. If a display device is

actively running at the time SAVE is invoked, its transformation matrix

will be saved with the same name but with extension type of ".imd" instead

of ".itd".

This command can also be used to save binary files. ASCII regular data

files usually take longer to load than binary files due to

the required parsing. Binary data files can be loaded directly like ASCII

files in IRIT, but must be inspected through IRIT tools such

as dat2irit. A binary data file must have a ".ibd" (IRIT Binary Data) type in

its name.

Files will be saved compressed if the given file name has a postfix of

".Z" or ".gz". The gnu "gzip" utility will be invoked via a pipe for that

purpose.

Example:

SAVE( "Obj1.ibd.Z", Obj1 );

Saves Obj1 in the file Obj1.ibd.Z as compressed binary file.

SETNAME

-------

SETNAME( ListType ListObj, NumericType Index, StringType NewName )

sets the name of a sub object of index Index in list object

ListObj to a new name NewName.  The index of the first element is

zero.

Example:

A = list( 1, 2, 3 );

SETNAME( A, 0, "First" );

sets the name of the first element in object A to "First".

While it is not a good idea to modify names of objects in the top level

global space, one can use this function to do exactly that.  To rename

the object "Axes" to "XYZ", do:

SETNAME( list( Axes ), 0, "XYZ" );

See also GETNAME.

SNOC

----

SNOC( AnyType Object, ListType ListObject )

This is similar to the lisp cons operator but puts the new Object in

the end of the list ListObject instead of at the beginning.

Example:

Lst = list( axes );

SNOC( Srf, Lst );

and now Lst is equal to the list 'list( axes, Srf )'.

SYSTEM

------

SYSTEM( StringType Command )

executes a system command Command. For example,

SYSTEM( "ls -l" );

TIME

----

TIME( NumericType Reset )

returns the time in seconds from the last time TIME was called with

Reset TRUE. This time is CPU time if such support is available

from the system (times function), and otherwise, is real time (time

function).

The time is automatically reset at the beginning of the execution of this

program.

Example:

Dummy = TIME( TRUE );

.

.

.

TIME( FALSE );

prints the time in seconds between the above two time function calls.

VARLIST

-------

VARLIST()

lists all the currently defined objects in the system.

VECTOR

------

VectorType VECTOR( NumericType X, NumericType Y, NumericType Z )

sreates a vector type object, using the three provided NumericType scalars.

See also PLANE, POINT.

VIEW

----

VIEW( GeometricTreeType Object, NumericType ClearWindow )

displays the (geometric) object(s) as given in Object.

If ClearWindow is non zero (see TRUE/FALSE and ON/OFF), the window is

first cleared (before drawing the objects).

Example:

VIEW( Axes, FALSE );

displays the predefined object Axes in the viewing window on top of

what is drawn already.

In version 4.0, this function is emulated (see iritinit.irt) using the

VIEWOBJ function. In order to use the current viewing matrix, VIEW_MAT

should be provided as an additional parameter. For example,

VIEW( list( view_mat, Obj ), TRUE );

However, since VIEW is a user defined function, the following will not

use VIEW_MAT as one would expect:

VIEW( view_mat, TRUE );

because VIEW_MAT will be renamed inside the VIEW user defined function to

a local (to the user defined function) variable.

In iritinit.irt one can find several other useful VIEW related functions:

VIEWCLEAR       Clears all data displayed on the display device.

VIEWREMOVE      Removes the object specified by name from display.

VIEWDISC        Disconnects from display device (which is still running)

while allowing IRIT to connect to a new device.

VIEWEXIT        Forces the display device to exit.

VIEWSAVE        Requests the display device to save transformation matrix.

BEEP            An emulation of the BEEP command of versions prior to 4.0.

VIEWSTATE       Allows change to the state of the display device.

For the above VIEW related functions, only VIEWREMOVE, VIEWSAVE, and

VIEWSTATE require parameters, which are the file name and view state,

respectively. The view state can be one of several commands. See the

display device section for more.

Examples:

VIEWCLEAR();

VIEW( axes, off );

VIEWSTATE( "LngrVecs" );

VIEWSTATE( "DrawSolid" );

VIEWSAVE( "matrix1" );

VIEWREMOVE( "axes" );

VIEWDISC();

VIEWOBJ

-------

VIEWOBJ( GeometricTreeType Object )

displays the (geometric) object(s) as given in Object.

Object may be any GeometricType or a list of other

GeometricTypes nested to an arbitrary level.

Unlike IRIT versions prior to 4.0, VIEW_MAT is not explicitly used

as the transformation matrix. In order to display with a VIEW_MAT view,

VIEW_MAT should be listed as an argument (in that exact name) to

VIEWOBJ. The same is true for the perspective matrix PRSP_MAT.

Example:

VIEWOBJ( list( view_mat, Axes ) );

displays the predefined object Axes in the viewing window using the

viewing matrix VIEW_MAT.

VIEWSET

-------

VIEWSET( NumericType DispHandle )

sets the current display device to be DispHandle. DispHandle is

returned by the CLNTEXEC command.  The use of the reserved constant of

CLIENTS_ALL would broadcast the viewing commands to all objects.

Example:

h1 = clntexec( DispDeviceName );

h2 = clntexec( DispDeviceName );

clntwrite( h1, sphere( vector( 0, 0, 0 ), 1 ) );

clntwrite( h2, axes );

pause();

VIEWSET( h1 );

viewclear();

viewobj( list( sphere( vector( 0, 0, 0 ), 1 ), axes ) );

VIEWSET( h2 );

viewclear();

viewobj( list( sphere( vector( 0, 0, 0 ), 1 ), axes ) );

pause();

VIEWSET( CLIENTS_ALL );

viewobj( axes );

pause();

viewexit();

opens two display devices, and displays a unit sphere to the first, and the

axes object, to the second.  After a pause, displays both objects on both

display devices, then pauses and exits from both.

See also VIEWOBJ, CLNTEXEC, CLNTCLOSE, CLNTREAD, CLNTWRITE.

WHILE

-----

WHILE( NumericType Cond, AnyType Body )

executes the Body (see below), while the WHILE loop condition

Cond is evaluated into a non zero value. Cond is

evaluated before each iteration.

The body may consist of any number of regular commands, separated by

COLONs, including nesting loops to an arbitrary level.

Example:

deg = 0;

rotstepx = rotx( 10 );

WHILE ( deg < 360,

deg = deg + 10:

view_mat = rotstepx * view_mat:

view( list( view_mat, axes ), ON )

);

displays axes with a view direction that is rotated 10 degrees at a

time around the X axis.

System variables

----------------

System variables are predefined objects in the system. Any time IRIT is

executed, these variable are automatically defined and set to values which

are sometimes machine dependent. These are regular objects in any other

sense, including the ability to be deleted or overwritten. One can modify,

delete, or introduce other objects using the iritinit.irt file.

AXES

----

Predefined polyline object (PolylineType) that describes the XYZ axes.

DRAWCTLPT

---------

Predefined Boolean variable (NumericType) that controls whether curves'

control polygons and surfaces' control meshes are drawn (TRUE) or not

(FALSE). Default is FALSE.

FLAT4PLY

--------

Predefined Boolean object (NumericType) that controls the way almost flat

surface patches are converted to polygons: four polygons (TRUE) or only

two polygons (FALSE). Default value is FALSE.

MACHINE

-------

Predefined numeric object (NumericType) holding the machine type as one of

the following constants: MSDOS, SGI, HP, SUN, APOLLO, UNIX, IBMOS2, IBMNT,

and AMIGA.

POLY_APPROX_OPT

---------------

A variable controlling the algorithm to tesselate surfaces into polygons.

If FALSE, that is, uniform, in parametric space, sampling is used.  If TRUE,

maximal deviation between the polygonal approximation and the surface is

used, with distance as prescribed by POLY_APPROX_TOL.

POLY_APPROX_UV

--------------

A Boolean predefined variable. If TRUE, UV values of surface polygonal

approximation are placed on the attribute lists of vertices.

POLY_APPROX_TOL

---------------

A numeric predefined tesselation control on the distance between the surface

and its polygonal approximation in POLY_APPROX_OPT settings.

POLY_APPROX_TRI

---------------

A numeric predefined tesselation control.  If TRUE, only triangles are

generated in surface tesselations.

POLY_MERGE_COPLANAR

-------------------

A numeric predefined surface tesselation control.  If TRUE, coplanar

adjacent polygons are merged into one.

PRSP_MAT

--------

Predefined matrix object (MatrixType) to hold the perspective matrix

used/set by VIEW and/or INTERACT commands. See also VIEW_MAT.

RESOLUTION

----------

Predefined numeric object (NumericType) that sets the accuracy of the

polygonal primitive geometric objects and the approximation of curves and

surfaces. It holds the number of divisions into which a circle is divided

(with minimum value of 4). If, for example, RESOLUTION is set to 6, then a

generated CONE will effectively be a six-sided pyramid.

It also controls the fineness of freeform curves and surfaces when they are

approximated as piecewise linear polylines, and the fineness of freeform

surfaces when they are approximated as polygons.

VIEW_MAT

--------

Predefined matrix object (MatrixType) to hold the viewing matrix used/set

by VIEW and/or INTERACT commands. See also PRSP_MAT.

System constants

----------------

The following constants are used by the various functions of the system to

signal certain conditions. Internally, they are represented numerically,

although, in general, their exact value is unimportant and may be changed

in future versions. In the rare circumstance that you need to know their

values, simply type the constant as an expression.

Example:

MAGENTA;

AMIGA

-----

A constant designating an AMIGA system, in the MACHINE variable.

APOLLO

------

A constant designating an APOLLO system, in the MACHINE variable.

BEZIER_TYPE

-----------

A constant defining a Bezier freeform geometry.

BLACK

-----

A constant defining a BLACK color.

BLUE

----

A constant defining a BLUE color.

BSPLINE_TYPE

------------

A constant defining a B-spline freeform geometry.

CLIENTS_ALL

-----------

A constant defining a request to address (broadcast to) all clients.

COL

---

A constant defining the COLumn or U direction of a surface or a

trivariate mesh.

CTLPT_TYPE

----------

A constant defining an object of type control point.

CURVE_TYPE

----------

A constant defining an object of type curve.

CYAN

----

A constant defining a CYAN color.

DEPTH

-----

A constant defining the DEPTH direction of a trivariate mesh.

See TBEZIER, TBSPLINE.

E1

--

A constant defining an E1 (X only coordinate) control point type.

E2

--

A constant defining an E2 (X and Y coordinates) control point type.

E3

--

A constant defining an E3 (X, Y and Z coordinates) control point type.

E4

--

A constant defining an E4 control point type.

E5

--

A constant defining an E5 control point type.

E6

--

A constant defining an E6 control point type.

E7

--

A constant defining an E7 control point type.

E8

--

A constant defining an E8 control point type.

E9

--

A constant defining an E9 control point type.

FALSE

-----

A zero constant. May be used as a Boolean operand.

GEOM_CONST

----------

Designates a constant shape.

GEOM_LINEAR

-----------

Designates a shape of a (piecewise) linear curve.

GEOM_CIRCULAR

-------------

Designates a shape of a circle/arc.

GEOM_PLANAR

-----------

Designates a planar shape.

GEOM_SPHERICAL

--------------

Designates a spherical shape.

GEOM_SRF_OF_REV

---------------

Designates a shape that is (a portion of) a surface of revolution..

GEOM_EXTRUSION

--------------

Designates a shape that is an extrusion surface.

GEOM_RULED_SRF

--------------

Designates a shape that is a ruled surface.

GEOM_DEVELOP_SRF

----------------

Designates a shape that is a ruled surface.

GEOM_SWEEP

----------

Designates a shape that is a sweep surface.

GREEN

-----

A constant defining a GREEN color.

HP

--

A constant designating an HP system, in the MACHINE variable.

IBMOS

-----

A constant designating an IBM system running under OS2, in the MACHINE

variable.

IBMNT

-----

A constant designating an IBM system running under Windows NT, in the MACHINE

variable.

KV_DISC_OPEN

------------

A constant defining an open end condition with a discontinuous uniformly

spaced knot vector.  That is, all interior knots are of multiplicity order -1

and are equally spaced.

KV_FLOAT

--------

A constant defining a floating end condition uniformly spaced knot vector.

KV_OPEN

-------

A constant defining an open end condition uniformly spaced knot vector.

KV_PERIODIC

-----------

A constant defining a periodic end condition with a uniformly spaced knot

vector.

LIST_TYPE

---------

A constant defining an object of type list.

MAGENTA

-------

A constant defining a MAGENTA color.

MATRIX_TYPE

-----------

A constant defining an object of type matrix.

MSDOS

-----

A constant designating an MSDOS system, in the MACHINE variable.

MULTIVAR_TYPE

-------------

A constant defining an object of type multivariate function.

NUMERIC_TYPE

------------

A constant defining an object of type numeric.

OFF

---

Synonym for FALSE.

ON

--

Synonym for TRUE.

P2

--

A constant defining a P1 (W and WX coordinates, in that order) rational

control point type.

P2

--

A constant defining a P2 (W, WX, and WY coordinates, in that order) rational

control point type.

P3

--

A constant defining a P3 (W, WX, WY, and WZ coordinates, in that order)

rational control point type.

P4

--

A constant defining a P4 rational control

point type.

P5

--

A constant defining a P5 rational control

point type.

P6

--

A constant defining a P6 rational control

point type.

P7

--

A constant defining a P7 rational control

point type.

P8

--

A constant defining a P8 rational control

point type.

P9

--

A constant defining a P9 rational control

point type.

PARAM_CENTRIP

-------------

A constant defining a centripetal length parametrization.

PARAM_CHORD

-----------

A constant defining a chord length parametrization.

PARAM_NIELFOL

-------------

A constant defining a Neilson-Foley parametrization.

PARAM_UNIFORM

-------------

A constant defining an uniform parametrization.

PI

--

The constant of 3.141592...

PLANE_TYPE

----------

A constant defining an object of type plane.

POINT_TYPE

----------

A constant defining an object of type point.

POLY_TYPE

---------

A constant defining an object of type poly.

POWER_TYPE

----------

A constant defining a power basis freeform geometry.

RED

---

A constant defining a RED color.

ROW

---

A constant defining the ROW or V direction of a surface or a trivariate mesh.

SGI

---

A constant designating an SGI system, in the MACHINE variable.

STRING_TYPE

-----------

A constant defining an object of type string.

SURFACE_TYPE

------------

A constant defining an object of type surface.

SUN

---

A constant designating a SUN system, in the MACHINE variable.

TRIMSRF_TYPE

------------

A constant defining an object of type trimmed surface.

TRIMSRF_TYPE

------------

A constant defining an object of type triangular surface.

TRIVAR_TYPE

-----------

A constant defining an object of type trivariate function.

TRUE

----

A non zero constant. May be used as a Boolean operand.

UNDEF_TYPE

----------

A constant defining an object of no type (yet).

UNIX

----

A constant designating a generic UNIX system, in the MACHINE variable.

VECTOR_TYPE

-----------

A constant defining an object of type vector.

WHITE

-----

A constant defining a WHITE color.

YELLOW

------

A constant defining a YELLOW color.

Animation

---------

The animation tool adds the capability of animating objects using

forward kinematics, exploiting animation curves.  Each object has

different attributes, that prescribe its motion, scale, and visibility

as a function of time. Every attribute has a name, which designates

its role. For instance, an attribute animation curve named MOV_X

describes a translation motion along the X axis.

How to create animation curves in IRIT

--------------------------------------

Let OBJ be an object in IRIT which we want to animate.

Animation curves are either scalar (E1/P1) curves or three-dimensional

(E3/P3) curves with one of the following name prefixes:

MOV_X, MOV_Y, MOV_Z            Translation along one axis

MOV_XYZ                        Arbitrary translation along all three axes

ROT_X, ROT_Y, ROT_Z            Rotating around a single axis (degrees)

SCL_X, SCL_Y, SCL_Z            Scale along a single axis

SCL                            Global scale

VISIBLE                        Visibility

The visibility curve is a scalar curve that enables the display of

the object if the visibility curve is positive at time t and disables

the display (hides) the object if the visibility curve is negative at

time t.  A positive visibility value between zero and one also hints at

the opacity of the object, if supported; one means fully opaque.

The animation curves are all attached as an attribute named "animation"

to the object OBJ.

Example:

mov_x = cbezier( list( ctlpt( E1, 0.0 ),

ctlpt( E1, 1.0 ) ) );

scl   = cbezier( list( ctlpt( E1, 1.0 ),

ctlpt( E1, 0.1 ) ) );

rot_y = cbezier( list( ctlpt( E1, 0.0 ),

ctlpt( E1, 0.0 ) );

ctlpt( E1, 360.0 ) ) );

attrib(OBJ, "animation", list( mov_x, scl, rot_y ) );

The above will animate OBJ between time zero and one (Bezier curves are always

between zero and one), by moving it a unit size in the X direction,

scaling it to %10 of its original size and rotating it at increasing

angular speed from zero to 360 degrees.

OBJ can now be saved into a file or displayed via one of the regular

viewing commands in IRIT (i.e. VIEWOBJ).

Animation is not always between zero and one. To that end, one can

apply the CREPARAM function to modify the parametric domain of the

animation curve. The convention is that if the time is below the

starting value of the parametric domain, the starting value of the

curve is used.  Similarly, if the time is beyond the end of the

parameter domain of the animation curve, the end value of the

animation curve is used.

Example:

CREPARAM( mov_x, 3.0, 5.0 );

to set the time of the motion in the x axis to be from t = 3 to

t = 5.  For t < 3, use mov_x(3), and for t > 5, use mov_x(5).

The animation curves are regular objects in the IRIT system. Hence,

only one object named mov_x or scl can exist at one time. If you

create a new object named mov_x, the old one is overwritten! To

preserve old animation curves you can detach the old ones by executing

'free(mov_x)' which will remove the object named mov_x from IRIT's object

list but not from its previously used locations within other list

objects, if any.  A different way to do this is to call the animation curves

mov_x1, mov_x2 etc. as only the prefix of the name is verified.

For example:

mov_x = cbezier( list( ctlpt( E1, 0.0 ),

ctlpt( E1, 1.0 ) ) );

attrib(obj1, "animation", list( mov_x ) );

free(mov_x);

mov_x1 = cbezier( list( ctlpt( E1, 2.0 ),

ctlpt( E1, 3.0 ) ) );

mov_x2 = cbezier( list( ctlpt( E1, 2.0 ),

ctlpt( E1, 3.0 ) ) );

attrib(obj2, "animation", list( mov_x1, mov_x2 ) );

free(mov_x);

Notice the way we have two animation curves translating obj2 in x.

This is somewhat artificial but makes more sense if other

transformations appear in between.

One can evaluate an object with animation curves at a certain time, only

to find the proper expected transformation matrix at that time on the

object as an "animation_mat" attribute.  The following example defines

a user defined TransformAnim function that creates a transformed

object out of object that was evaluated with ANIMEVAL.  Then, a simple loop

(slowly) animates the scene...

TransformAnim = function( Obj ):

return = 0;

TransformAnim = function( Obj ): m: i:

if ( thisobj( "Obj" ) == list_type,

return = nil():

for ( i = 1, 1, sizeof( Obj ),

snoc( TransformAnim( nth( Obj, i ) ), return ) ),

return = Obj * tx( 0 ) ):

m = getattr( Obj, "animation_mat" ):

if ( thisobj( "m" ) == matrix_type,

return = return * m );

for ( t = 0, 0.1, 1,

ANIMEVAL( t, Object ):

view( TransformAnim( Object ), 1 ) );

A more complete animation example

---------------------------------

a = box( vector( 0, 0, 0 ), 1, 1, 1 );

b = box( vector( 0, 0, 0 ), 1, 1, 1 );

c = box( vector( 0, 0, 0 ), 1, 1, 1 );

d = sphere( vector( 0, 0, 0), 0.7 );

pt0   =  ctlpt( e1,  0.0 );

pt1   =  ctlpt( e1,  1.0 );

pt2   =  ctlpt( e1,  2.0 );

pt6   =  ctlpt( e1,  6.0 );

pt360 =  ctlpt( e1,  360.0 );

pt10 = ctlpt( e1, -4.0 );

pt11 = ctlpt( e1,  1.0 );

pt12 = ctlpt( e1,  4.0 );

pt13 = ctlpt( e1, -1.0 );

visible = creparam( cbezier( list( pt10,  pt11 ) ), 0.0, 5.0 );

mov_x   = creparam( cbezier( list( pt0, pt6, pt2 ) ), 0.0, 1.2 );

mov_y   = mov_x;

mov_z   = mov_x;

rot_x   = creparam( cbspline( 2,

list( pt0, pt360, pt0 ),

list( KV_OPEN ) ),

1.2, 2.5 );

rot_y   = rot_x;

rot_z   = rot_x;

scl     = creparam( cbezier( list( pt1, pt2, pt1, pt2, pt1 ) ),

2.5, 4.0 );

scl_x   = scl;

scl_y   = scl;

scl_z   = scl;

mov_xyz = creparam( circle( vector( 0, 0, 0 ), 2.0 ), 4.0, 5.0 );

attrib( d, "animation", list( mov_xyz, visible ) );

free( visible );

visible = creparam( cbezier( list( pt12,  pt13 ) ), 0.0, 5.0 );

attrib( a, "animation", list( rot_x, mov_x, scl, scl_x, visible ) );

attrib( b, "animation", list( rot_y, mov_y, scl, scl_y, visible ) );

attrib( c, "animation", list( rot_z, mov_z, scl, scl_z, visible ) );

color( a, red );

color( b, green );

color( c, blue );

color( d, cyan );

demo = list( a, b, c, d );

interact( demo );

viewanim( 0, 5, 0.01 );

In this example, we create four objects, three cubes and one sphere.

Animation curves to translate the three cubes along the three axes for

time period of t = 0 to t = 1.2 are created. Rotation curves to

rotate the three cubes along the three axes are then created for time

period t = 1.2 to t = 2.5. Finally, for time period t = 2.5

to t = 4.0. the cubes are (not only) unifomly scaled. For time

period t = 4 to t = 5, the cubes become invisible and the sphere,

which becomes visible, is rotated along a circle of radius 2.

Another complete animation example

----------------------------------

This example demonstrates the ability to put "animation" attributes

on internal nodes of a hierarchy, thereb, affecting the entire set of objects

in the hierachy.   Herein, we present an robotic arm with three edges

and two joints.

BoxLength = 2;

BoxWidth = 2;

BoxHeight = 10;

LowerBox = box( vector( -BoxLength / 2, -BoxWidth / 2, 0 ),

BoxLength, BoxWidth, BoxHeight);

MiddleBox = box( vector( -BoxLength / 2, -BoxWidth / 2, 0 ),

BoxLength, BoxWidth, BoxHeight);

UpperBox = box( vector( -BoxLength / 2, -BoxWidth / 2, 0 ),

BoxLength, BoxWidth, BoxHeight);

Cn1 = cone( vector( 0, 0, 0 ), vector( 0, BoxHeight / 3, 0 ), 1 );

color( LowerBox, magenta );

color( MiddleBox, yellow );

color( UpperBox, cyan );

color( Cn1, green );

rot_x1 = creparam( cbspline( 3,

list( ctlpt( E1,  0 ),

ctlpt( E1,  -200 ),

ctlpt( E1,  200 ),

ctlpt( E1,  0 ) ),

list( KV_OPEN ) ),

0, 3 );

rot_x2 = creparam( cbspline( 4,

list( ctlpt( E1,  0 ),

ctlpt( E1,  400 ),

ctlpt( E1,  -400 ),

ctlpt( E1,  0 ) ),

list( KV_OPEN ) ),

0, 3 );

rot_y = creparam( cbspline( 2,

list( ctlpt( E1,  0 ),

ctlpt( E1,  100 ),

ctlpt( E1, -100 ),

ctlpt( E1,  0 ) ),

list( KV_OPEN ) ),

0, 3 );

rot_z = creparam( cbspline( 2,

list( ctlpt( E1,  0 ),

ctlpt( E1,  1440 ) ),

list( KV_OPEN ) ),

0, 3 );

Translate = trans( vector( 0, 0, BoxHeight ) );

attrib( Cn1, "animation", list( rot_z, Translate ) );

Upr = list( Cn1, UpperBox );

attrib( Upr, "animation", list( rot_y, Translate ) );

Mid = list( Upr, MiddleBox );

attrib( Mid, "animation", list( rot_x2, Translate ) );

rbt_hand = list( Mid, LowerBox );

attrib( rbt_hand, "animation", list( rot_x1 ) );

view( rbt_hand, 1 );

In this example, we create four objects, three cubes and one cone,

simulating a robotic hand with three edges an a gripper (the cone).

The animation is defined hierarchically, making it very easy to model

the robot.

Display devices

---------------

The following display device drivers are available,

Device Name            Invocation             Environment

xgldrvs                xgldrvs -s-            SGI 4D GL regular driver.

xogldrvs               xogldrvs -s-           SGI 4D Open GL/Motif driver.

xgladap                xgladap -s-            SGI 4D GL adaptive isocurve

experimental driver.

x11drvs                x11drvs -s-            X11 driver.

xmtdrvs                xmtdrvs -s-            X11 Motif driver.

xglmdrvs               xglmdrvs -s-           SGI 4D GL and X11/Motif driver.

wntdrvs                wntdrvs -s-            IBM PC Windows NT driver.

wntgdrvs               wntgdrvs -s-           IBM PC Windows NT Open GL driver.

wntgaiso               wntgaiso -s-           IBM PC OGL Adap. Iso. driver.

os2drvs                os2drvs -s-            IBM PC OS2 2.x/3.x driver.

amidrvs                amidrvs -s-            AmigaDOS 2.04+ driver.

nuldrvs                nuldrvs -s- [-d] [-D]   A device to print the

object stream to stdout.

All display devices are clients communicating with the (IRIT) server

using IPC (inter process communication). On Unix and Windows NT, sockets are

used. A Windows NT client can talk to a server (IRIT) on a Unix host if

hooked to the same network. On OS2 pipes are used, and both the client and

server must run on the same machine. On AmigaDOS exec messages are used,

and both the client and server must run on the same machine.

While all display devices support object(s) transformations via a

transformation control window, many of the display devices allow one

to click and drag on the viewing window to rotate (Left Button) and

to translate (Right Button).  This mode exploits the mouse's two degrees of

freedom to provide intuitive dual axis rotation and

translation.  Most display devices supports two levels of fineness.

A rough display is used when in the middle of a transformation operation

(i.e. the mouse button is down/dragged), while a fine object display is

employed when the display is idle (mouse button is up). See also option

'-E'.

The (IRIT) server will automatically start a client display device

if the IRIT_DISPLAY environment variable is set to the name and options of

the display device to run. For example:

setenv IRIT_DISPLAY xgldrvs -s-

The display device must be in a directory that is in the

environment variable path. Most display devices require the '-s-' flags

to run in a non-standalone mode, or a client-server mode. Most drivers can

also be used to display data in a standalone mode (i.e., no server).  For

example:

xgldrvs -s solid1.itd irit.imd

Effectively, all the display devices are also data display programs.

Therefore, some functionality is not always as expected. For

example, the Quit button will always force the display device to quit,

even if popped up from IRIT, but will not cause IRIT to

quit as might logically expected. In fact, the next time IRIT will

try to communicate with the display device, it will find the broken

connection and will start up a new display device.

Most display devices recognize attributes found on objects. The following

attributes are usually recognized (depending on the device capability):

COLOR: Selects the drawn color of the object to be one of the 8/16

predefined colors in the IRIT system: white, red, green, blue,

yellow, cyan, magenta, black.

RGB: Overwrites (if supported) the COLOR attribute (if given) and

sets the color of the object to the exact prescribed RGB set.

DWIDTH: Sets the width in pixels of the drawn object, when drawn as

a wireframe.

ReflectLns: Allows the display of reflection lines off a freeform

surface.  The "ReflectLns" attribute is a list object of two

subobjects, a vector and a list of points.  The vector is the

reflection lines' direction (all reflection lines are parallel)

and the list of points is a list of points on the different

reflection lines. For example,

attrib( S, "RflctLines",

list( vector( 0, 0, 1 ),

list( point( -1.6, 2, 0 ),

point( -0.8, 2, 0 ),

point(  0.0, 2, 0 ),

point(  0.8, 2, 0 ),

point(  1.6, 2, 0 ) ) ) );

defines five reflection lines to be reflected off surface S,

all in the direction of (0, 0, 1) and on the plane Y = 2.

See also RFLCTLN command.

All display devices recognize all the command line flags and all the

configuration options in a configuration file, as described below. The display

devices will attempt to honor the requests, to the best of their ability.

For example, only gl and OpenGL devices can render shaded models, and so

only they will honor DrawSolid configuration options.

Command Line Options

--------------------

???drvs [-s] [-u] [-n] [-N] [-i] [-c] [-C] [-m] [-a] [-g "x1,x2,y1,y2"]

[-G "x1,x2,y1,y2"] [-I #IsoLines] [-F PlgnOpti PlgnFineNess] [-R]

[-f PllnOpti PllnFineNess] [-E RelLowRes] [-p] [-l LineWidth]

[-r] [-A Shader] [-B] [-2] [-d] [-D] [-L NormalLen] [-4]

[-k SketchType Sil Shd Imp] [-b "R,B,G (background)"]

[-S "x,y,z,w{,a,d,s} (LgtSrcPosADS)"] [-e PickDist]

[-O PickObjType] [-Z ZMin ZMax] [-M] [-W WireSetup] [-P] [-o]

[-x ExecAnimCmd] [-X Min,Max,Dt,R{,flags}] [-w InitWidget] [-T]

[-z] DFiles

-s: Runs the driver in a standalone mode. Otherwise, the driver will

attempt to communicate with the IRIT server.

-u: Forces a unit matrix. That is, input data are not

transformed at all.

-n: Draws normals of vertices.

-N: Draws normals of polygons.

-i: Draws internal edges (created by IRIT) - default is not to

display them; this option will also force their display.

-c: Sets depth cueing on. Drawings that are closer to the viewer will

be drawn in more intense color.

-C: Caches the piecewise linear geometry so curves and surface can

be redisplayed faster. Purging it will free memory, on the other hand.

-m: Provides some more information on the parsed data file(s).

-a: Sets the support of antialiased lines.

-g x1,x2,y1,y2: Prescribes the position and location of the

transformation window by prescribing the domain of the window in

screen space pixels.

-G x1,x2,y1,y2: Prescribes the position and location of the

viewing window by prescribing the domain of the window in

screen space pixels.

-I #IsoLines: Specifies the number of isolines per surface, per

direction.

A specification of zero isolines is possible only on the command line

and it denotes the obvious.

-F PolyOpti FineNess: Controls the method used to approximate surfaces

into polygons. See the variable POLY_APPROX_OPT for the meaning of

FineNess. See also -4.

-R:  Use optimized polygonal strips instead of lists of polygons, if

possible.  This feasibility depends on the support of the underlying

hardware/graphics libraries.

-f PolyOpti SampTol: Controls the method used to approximate curves

into polylines. If PolyOpti == 0, equally spaced intervals are

used.  For PolyOpti == 1, SampTol (real number) specifies the

maximal allowed dveiation tolerance of the piecewise linear

approximation from the original curve.

Default is 0 64 (uniform sampling with 64 samples).

-E RelLowRes:  Sets the relative fineness of curves and surface

while the input device is active, such as in a drag operation.

-p:  Sets the width of drawn points.

-l LineWidth: Sets the linewidth, in pixels. Default is one pixel wide.

-r: Rendered mode. Draws object as solid.

-A Shader: Shader can be one of 0 (None), 1 (Background), 2 (Flat),

3 (Gouraud), or 4 (Phong).

-B: Back face culling of polygons.

-2: Double buffering. Prevents screen flicker at the possible cost of

fewer colors.

-d: Debug objects. Prints to stderr all objects read from the

communication port with the server IRIT.

-D: Debug input. Prints to stderr all characters read from communcation

port with the server IRIT. Lowest level of communication.

-L NormalLen: Sets the length of the drawn normals in thousandths of

a unit.

-4: Forces four polygons per almost flat region in the surface to

polygon conversion. Otherwise two polygons only.

-k SketchType Sil Shd Imp: Sets the strokes type (one of 1

(isoparametric curves), 2 (lines of curvature), 3 (silhoutees)),

and the silhouette and shader powers (between zero and one) and

strokes improtance factor, in interactive line art strokes (See -W).

-b BackGround: Sets the background color as three RGB integers in the

range of 0 to 255.

-S x,y,z,w{,a,d,s} (LgtSrcPosADS): Sets the lighting by setting the

light source position as well as the optional Ambient, Diffuse, and

Specular intensities.

-e PickDist: Sets the distance to the near and far Z clipping planes.

-O PickObjType: A binary mask that controls which object can be

picked:

bit 0 - not used, bit 1 - poly, bit 2 - numeric,

bit 3 - point, bit 4 - vector, bit 5 - Plane, bit 6 - matrix,

bit 7 - curve, bit 8 - surface, bit 9 - string, bit 10 - list object,

bit 11 - ctl pt, bit 12 - trimmed srf, bit 13 - trivariate,

bit 14 - instance, bit 15 - triangular srf, bit 16 - model,

bit 17 - multivariate.

-Z ZMin ZMax: Sets the near and far Z clipping planes.

-M: Draw control mesh/polygon of curves and surfaces, as well.

-W WireSetup: Controls the line drawing of the freeforms where

WireSetup is a mask that controls:

bit 0: Draw curves and surfaces using a set of isocurves (see -I and

-f),

bit 1: Draw boundary curves of surfaces,

bit 2: Draw silhouette curves of surfaces,

bit 3: Draw surfaces in sketch style line art (see -k).

bit 4: Draw surfaces' reflection lines (surface also must have a

"ReflectLns" attribute - see attributes above).

-P: Draws curves and surfaces using a set of polygons (see -F).

-o: Reverses the Orientation by flipping all normals (see -n, -N).

-x ExecAnimCmd: Command to execute as a subprocess every iteration of

display of an animation sequence. This command can, for example, save

the display into an image file, saving the animation sequence.

One parameter, which is an running index starting from one, is passed.

-X Min,Max,Dt,R{,flags}: Executes an animation sequence between

Min time to Max time in steps of Dt. R repetitions of the animations

are executed.  Flags could be any combination of:

's': Flag to specify the saving of the animation as individual

data files, one per frame, for high quality rendering.

't': Two way animation - bounce back and forth.

'b': Reset the animation back to its starting position.

'x': Flag to force the display device to exit upon completion of the

animation.

-w InitWidget: Sets the widgets that are displayed initially

(as an or'ed mask):

1 - Environment widget,            2 - Animation widget,

4 - Curves widget,                 8 - surfaces widget,

16 - Shading widget,               32 - Pick objects widget,

64 - Object transforms widget.

-T: Enable continuous moTion.  Objects continue to move indefinitely

following the last transformation applied.

-z: Prints version number and current defaults.

Configuration Options

---------------------

The configuration file is read before the command line options are

processed. Therefore, all options in this section can be overridden

by the appropriate command line option, if any.

TransPrefPos: Preferred location (Xmin, YMin, Xmax, Ymax) of the

transformation window.

ViewPrefPos: Preferred location (Xmin, YMin, Xmax, Ymax) of the

viewing window.

BackGround: Background color. Same as '-b'.

Internal: Draws internal edges. Same as '-i'.

LightSrcPos: Sets the location of the (first) light source as a

rational four coefficient location. W of zero sets the light source

at infinity.

ExecAnimCmd: Executes a command at each step of the animation. Same as

'-x'.

ExecAnimation: Executes an animation sequence on startup. Same as '-X'.

DrawVNormal: Draws normals of vertices. Same as '-n'.

DrawPNormal: Draws normals of polygons. Same as '-n'.

MoreVerbose: Provides some more information on the parsed data file(s).

Same as '-m'.

UnitMatrix: Forces a unit matrix. That is, input data are not

transformed at all. Same as '-u'.

DrawSolid: Requests a shaded surface rendering, as opposed to isocurve

surface representation.

BFaceCull: Requests the removal of back facing polygons, for better

visibility.

DoubleBuffer: Requests drawing using a double buffer, if any.

DebugObjects: Debugs objects. Prints to stderr all objects read

from the communication port with the server IRIT. Same as '-d'.

DebugEchoInput: Debugs input. Prints to stderr all characters read

from the communication port with the server IRIT. Lowest level of

communication.

DepthCue: Sets depth cueing on. Drawings that are closer to the

viewer will be drawn in more intense color. Same as '-c'.

CacheGeom: Normally piecewise linear approximation of freefroms is

cached. By setting this option to FALSE, no such auxiliary data is

saved, reducing the memory overhead. Same as '-C'.

FourPerFlat: Forces four polygons per almost flat region in the

surface to polygon conversion. Otherwise two polygons only. Same as

'-4'.

AntiAlias: Requests the drawing of antialiased lines.

DrawSurfaceMesh: Draws control mesh/polygon of curves and surfaces,

as well. Same as '-M'.

DrawSurfacePoly: Draws curves and surfaces (surfaces are drawn using

a set of isocurves, see -I, or polygons, see -f). Same as '-P'.

StandAlone: Runs the driver in a standalone mode. Otherwise, the

driver will attempt to communicate with the IRIT server. Same

as '-s'.

PolyStrips: Renders using polygonal strips, if possible. Same

as '-R'.

ContMotion: Renders using continuous motions. Objects continue to

move indefinitely, following the last transformation applied. Same

as '-T'.

NumOfIsolines:  Specifies number of isolines per surface, per

direction. Same as '-I'.

PllnFineNess: Specifies the samples per (iso)curve or tolerance of

approximation. See '-f'.

LineWidth: Sets the linewidth, in pixels. Default is one pixel

wide. Same as '-l'

AdapIsoDir: Selects the direction of the adaptive isoline

rendering.

PolygonOpti: Controls the method used to subdivide a surface into

polygons that approximate it. See '-F'.

PolylineOpti: Controls the method used to subdivide a curve into

polylines that approximate it. See '-f'.

ShadingModel: One of 1 (Flat), 2 (Gouraud), or 3 (Phong). Same as

'-A'.

TransMode: Selects between object space transformations and screen

space transformation.

ViewMode: Selects between perspective and orthographic views.

NormalLength: Sets the length of the drawn normals in thousandths of

a unit. Same as '-L'.

ZClipMin: Sets the minimal clipping plane in Z. Same as '-Z'.

ZClipMax: Sets the maximal clipping plane in Z. Same as '-Z'.

PlgnFineNess: Controls the fineness of the surface to polygon

subdivision.  See '-F'.

Interactive mode setup

----------------------

Commands that affect the status of the display device can also be sent

via the communication port with the IRIT server. The following commands

are recognized as string objects with object name of "COMMAND_":

ANIMATE TMin TMax Dt   Animates current scene from TMin to TMax in Dt

steps.

BEEP                 Makes some sound.

CLEAR                Clears the display area. All objects are deleted.

DCLEAR               Delays clear. Same as CLEAR but delayed until next

object is sent from the server. Useful for animation.

DISCONNECT           Closes connection with the server, but does not quit.

EXIT                 Closes connection with the server and quits.

GETOBJ NAME          Requests the object named NAME that is returned

in the output channel to the server.

HIGHLIGHT1 NAME      Color the object named NAME with highlight1 color.

HIGHLIGHT2 NAME      Color the object named NAME with highlight2 color.

MSAVE NAME           Save the current matrix in a file named NAME.

PICKCRSR             Requests to interactively sample mouse/cursor events

for mouse-up, mouse-down, and mouse-move-while-down.

PICKDONE             Stop interactive pick reports to server.  Stops all

PICKCRSR, PICKNAME and PICKOBJ modes.

PICKNAME             Requests to interactively pick an object by name that

is returned in the output channel to the server.

PICKOBJ              Requests to interactively pick an object that is

returned in the output channel to the server.

MSAVE NAME           Saves the transformation matrix file by the name

NAME.

REMOVE NAME          Requests the removal of object named NAME from

display.

STATE COMMAND        Changes the state of the display device. See below.

UNHIGHLIGHT          Unhighlight all highlighted objects.

The following commands are valid for the STATE COMMAND above,

MouseSense:       Mouse sensitivity control.

ScrnObjct:        Controls screen/object transformation mode.

PerspOrtho:       Controls perspective/orthographic trans. mode.

DepthCue:         Controls depth cueing drawing.

CacheGeom:        Cache the created piecewise linear geometry.

DrawSolid:        Controls isocurve/shaded solid drawing.

ShadingMdl:       Controls shading model for solid solid drawing.

BFaceCull:        Cull backfacing polygons.

DblBuffer:        Controls single/double buffer mode.

AntiAlias:        Controls antialiased lines.

DrawIntrnl:       Controls drawing of internal lines.

DrawVNrml:        Controls drawing of normals of vertices.

DrawPNrml:        Controls drawing of normals of polygons.

DrawPlgns:        Controls drawing of polygonal objects as polygons.

DSrfMesh:         Controls drawing of control meshes/polygons.

DSrfWire:         Controls drawing of curves/surfaces as wireframes.

DSrfBndry:        Controls drawing of boundary curves of surfaces.

DSrfSilh:         Controls drawing of silhouette curves of surfaces.

DSrfPoly:         Controls drawing of curves/surfaces as polygons.

DSrfSktch:        Controls drawing of surfaces as sketches.

4PerFlat:         Controls 2/4 polygons per flat surface regions.

NumIsos:          Controls the number of isocurves in a surface.

PolyAprx:         Controls the surface tesselation fineness.

PllnAprx:         Controls the curves to polylines fineness.

LenVecs:          Controls the length of displayed normal vectors.

WidthLines:       Controls the width of the drawn lines.

WidthPts:         Controls the width of the cross of drawn points.

Front:            Selects a front view.

Side:             Selects a side view.

Top:              Selects a top view.

Isometry:         Selects an isometric view.

4Views:           Selects a four views mode.

Clear:            Clears the viewing area.

ResAdapIso:       Controls the resolution of a number of adaptive isocurves.

ResRldSrf:        Controls the resolution of ruled srfs in adaptive isocurves.

RuledSrfApx:      Controls the ruled surface approx. in adaptive isocurves.

AdapIsoDir:       Controls the row/col direction of adaptive isocurves.

LowResRatio:      Controls the low/high resolution ratios.

Obviously not all state options are valid for all drivers.

The IRIT server defines in iritinit.irt several user-defined

functions that exercise some of the above state commands, such as

VIEWSTATE and VIEWSAVE.  VIEWSTATE accepts a second parameter which

can be -1 to toggle the value, 0 to reset the value or 1 to set it.

If the state value is real, 1 doubles its value and 0 halfs it.

In addition to state modification via communication with the IRIT

server, modes can be interactively modified on most of the display devices

using a pop-up menu that is activated using the right button in the

transformation window}.

This pop-up menu is somewhat different in different drivers, but its

entries closely follow the entries of the above state command table.

All driver support three special matrices.  The VIEW_MAT can set

the current viewing direction and PRSP_MAT can set the current perspective

view.  Finally, CONT_MAT can set the current continuous motion (see also

'-T' option).

Animation Mode

--------------

All the display drivers are now able to animate objects with animation

curve attributes on them. For more on the way animation curves can be

created, see the Animation Section of this manual.

Once a scene with animation curve attributes is being loaded into

a display device, one can enter "animation" mode using the "Animation"

button available in all display devices. The user is then prompted (either

graphically or in a textual based interface) for the starting time,

termination time and step size of the animation. The parameter space of

the animation curve serves as the time domain. The default starting

and terminating times are set as the minimal and maximal parametric

domain values of all animation curves. An object at time t below the

minimal parametric value will be placed at the starting value of the

animation curve. Similarly, an object at time t above the

maximal parametric value will be placed at the termination value of the

animation curve. The user can also set a bouncing back and forth mode,

the number of repetitions, and if desired, request the saving of all

the different scenes in the animation as separate files so a high quality

animation can be created.

A string object can be viewed as the text of selected PS font (See -N).

The string position is set via a "StrPos" vector attribute (default to the

origin), and "StrScale" real attribute to control the string height in

world unit (default to 0.1).  Text will always be in a plane parallel to

the XY plane.

Specific Comments

-----------------

The x11drvs supports the following X Defaults (searched at ~/.Xdefaults):

#ifndef COLOR

irit*MaxColors:                    1

irit*Trans*BackGround:             Black

irit*Trans*BorderColor:            White

irit*Trans*TextColor:              White

irit*Trans*SubWin*BackGround:      Black

irit*Trans*SubWin*BorderColor:     White

irit*Trans*CursorColor:            White

irit*View*BackGround:              Black

irit*View*BorderColor:             White

irit*View*CursorColor:             White

#else

irit*MaxColors:                    15

irit*Trans*BackGround:             NavyBlue

irit*Trans*BorderColor:            Red

irit*Trans*TextColor:              Yellow

irit*Trans*SubWin*BackGround:      DarkGreen

irit*Trans*SubWin*BorderColor:     Magenta

irit*Trans*CursorColor:            Green

irit*View*BackGround:              NavyBlue

irit*View*BorderColor:             Red

irit*View*CursorColor:             Red

#endif

irit*Trans*BorderWidth:            3

irit*Trans*Geometry:               =150x500+510+0

irit*View*BorderWidth:             3

irit*View*Geometry:                =500x500+0+0

The Motif-based display drivers contain three types of gadgets which can be

operated in the following manner. Scales: can be dragged or clicked outside

for single (mouse's middle button) or continuous (mouse's left

button) action. Pushbuttons: activated by clicking the mouse's left button.

The control panel: allows rotation, translation of the objects in three

axes, determining of the perspective ratio, viewing an object from top, side,

front or isometrically, determining scale factor and clipping settings, and

operating the matrix stack.

The environment window toggles between screen or object transformation,

depth cue on or off, orthographic or perspective projection, wireframe or

solid display, single or double buffering, showing or hiding normals,

including or excluding the surface's mesh and curve's control polygon,

surface drawing using isolines or polygons, and four or two polygons per

flat patch. Some display devices allow for the inclusion or exclusion of

internal edges, and enable or disable of antialiased lines.

Scales in the X11/Motif based devices set normals length, lines width,

control sensitivity, the number of islolines and samples, etc.

The locations of windows as set via [-g] and [-G] and/or via the

configuration file overwrite in x11drvs the Geometry X11 defaults.

To use the Geometry X11 default, use '-G " "' and '-g " "' or set the

string to empty size in the configuration file.

In os2drvs, only -G is used to specify the dimensions of the parent window

that holds both the viewing and the transformation window.

In os2drvs, the following key strokes are available as shortcuts:

Key        Function

^x         Quit

^s         Save

^f         Front View

^d         Side View

^t         Top View

^i         Isometric VIew

^p         Perspetive/Orthographic

^n         View Internal Edges

^v         View Vertices' Normals

^g         View Polygons' Normals

^b         Backface Culling

^c         Depth Cue

^m         View Control Mesh/Poly

Examples

--------

xglmdrvs -z

prints all the options and their current values.

xglmdrvs -B -i -l 3 solid1.itd

displays the model of solid1.itd using backface culling ('-B'), with

internal edges ('-i'), and line width of 3.

xglmdrvs -r -A flat wiggle.itd

displays the model of wiggle.itd shaded ('-r') using flat shading ('-A').

xglmdrvs -I 40 -u -b 255 255 255 wiggle.itd

displays the model of wiggle.itd using isolines' density of 40 ('-I'), using

unit matrix to begin with ('-u'), and a white background ('-b').

xglmdrvs -X 0,2,0.1,sx -r anim.itd

executes the animation in anim.itd, from time 0 to time 2 in steps of 0.1.

The animation is saved in one frame per file (flag 's' in '-X') and

the display device exists once the animation has terminated (flag 'x'

in '-X')).  The animation will be shaded ('-r').

Utilities - General Usage

-------------------------

The IRIT Solid Modeler is accompanied by quite a few utilities. They

can be subdivided into two major groups. The first includes auxiliary tools

such as illustrt and poly3d-h. The second includes filters such as irit2ray

and irit2ps.

All these tools operate on input files, and most of the time produce

data files. In all utilities that read files, the dash ('-') can be used

to read stdin.

Example:

poly3d-h solid1.itd | irit2ps - > solid1.ps

All the utilities have command line options. If an option is set by a '-x',

then '-x-' resets the option. The command line options overwrite the settings

in config files, and the reset option is useful for cases where the option

is set by default, in the configuration file.

All utilities can read a sequence of data files. However, the last

transformation matrices found (VIEW_MAT and PRSP_MAT) are actually used.

Example:

poly3d-h solid1.itd | x11drvs solid1.itd - solid1.imd

x11drvs will display the original solid1.itd file with its hidden version,

as computed by poly3d-h, all with the solid1.imd, ignoring all other matrices

in the data stream.

Compressed files with a postfix ".Z" or ".gz" will be automatically

uncompressed on read and write. The following is legal:

poly3d-h solid1.itd.Z | x11drvs solid1.itd.Z - solid1.imd

where solid1.itd.Z was saved from within IRIT using the command

save( "solid1.itd.Z", solid1 );

or similarly. The gnu utility "gzip" is used for the purpose of

(un)compressing the data via pipes.  See also SAVE and LOAD.

Poly3d-h - Hidden Line Removing Program

---------------------------------------

Introduction

------------

poly3d-h is a program to remove hidden lines from a given polygonal model.

Freeform objects are preprocessed into polygons with controlled fineness.

The program performs 4 passes over the input:

1. Preprocesses and maps all polygons in a scene, and sorts them.

2. Generates edges out of the polygonal model and sorts them (preprocessing

for the scan line algorithm) into buckets.

3. Intersects edges, and splits edges with non-homogeneous visibility (the

scan line algorithm).

4. Applies a visibility test on each edge.

This program can handle CONVEX polygons only. From IRIT one can

ensure that a model consists of convex polygons only, using the CONVEX

command:

CnvxObj = convex( Obj );

just before saving it into a file. Surfaces are always decomposed into

triangles.

poly3d-h output is in the form of polylines. It is a regular IRIT

data file that can be viewed using any of the display devices, for example.

Command line options

--------------------

poly3d-h [-b] [-m] [-i] [-e #Edges] [-H] [-4] [-W Width]

[-F PolyOpti FineNess] [-q] [-o OutName] [-t AnimTime]

[-c] [-z] DFiles > OutFile

-b: BackFacing - if an object is closed (such as most models created by

IRIT), backfacing polygons can be deleted, thereby speeding up

the process by at least a factor of two.

-m: More - provides some more information on the parsed data file(s).

-i: Internal edges (created by IRIT) - default is not to

display them, and this option will force their display, as well.

-e n: Number of edges to use from each given polygon (default all).

Handy as '-e 1 -4' for freeform data.

-H: Dumps both visible lines and hidden lines as separated objects.

Hidden lines will be dumped using a different (dimmer) color and (a

narrower) line width.

-4: Forces four polygons per almost flat region in the surface to

polygon conversion. Otherwise two polygons only.

-W Width: Selects a default width for visible lines in inches.

-F PolyOpti FineNess: Optimality of polygonal approximation of

surfaces. See the variable POLY_APPROX_OPT for the meaning of

FineNess. See also -4.

-q: Quiet mode. No printing aside from fatal errors. Disables -m.

-o OutName: Name of output file. Default is stdout.

-t AnimTime: If the data contains animation curves, evaluate and

process the scene at time AnimTime.

-z: Prints version number and current defaults.

-c: Clips data to screen (default). If disabled ('-c-'), data

outside the view screen ([-1, 1] in x and y) are also processed.

Some of the options may be turned on in poly3d-h.cfg. They can then be

turned off in the command line as '-?-'.

Configuration

-------------

The program can be configured using a configuration file named poly3d-h.cfg.

This is a plain ASCII file you can edit directly and set the parameters

according to the comments there. 'poly3d-h -z' will display the current

configuration as read from the configuration file.

The configuration file is searched in the directory specified by the

IRIT_PATH environment variable. For example,

'setenv IRIT_PATH /u/gershon/irit/bin/'.

If the IRIT_PATH variable is not set, the current directory is searched.

Usage

-----

As this program is not interactive, usage is quite simple, and the only

control available is the command line options.

If a certain surface should be polygonized into a finer/coarser

set of polygons than the rest of the scene, one can set a "resolution"

attribute which specifies the relative FineNess resolution of this

specific surface. Further, "u_resolution" and "v_resolution" might be

similarly used to set relative resolution for the u or v direction only.

The "crv_resolution" attribute controls the relative fineness of curves

as polylines.  The "num_of_isolines" attribute controls the relative number

of isoparametric curves.

See also IHidden.

Poly3d-r - A Simple Data Rendering Program

------------------------------------------

Retired. Sources can be found in the contrib directory, but this program is

no longer supported. See irender program instead.

Illustrt - Simple line illustration filter

------------------------------------------

Introduction

------------

illustrt is a filter that processes IRIT data files and dumps out modified

IRIT data files. illustrt can be used to make simple, nice illustrations of

data. The features of illustrt include depth sorting, hidden line clipping

at intersection points, and vertex enhancements. illustrt is designed to

closely interact with irit2ps, although it is not neceessary to use irit2ps

on illustrt output.

Command line options

--------------------

illustrt [-I #UIso[:#VIso[:#WIso]]] [-f PolyOpti SampTol] [-s] [-M] [-P]

[-p] [-O] [-l MaxLnLen] [-a] [-t TrimInter] [-o OutName]

[-Z InterSameZ] [-m] [-T AnimTime] [-z] DFiles

-I #UIso[:#VIso[:#WIso]]: Specifies the number of isolines per

surface/trivariate, per direction. If #VIso is not specified,

#UIso is used for #VIso as well and so no.

-f PolyOpti SampTol:  Controls the method used to approximate curves

into polylines. If PolyOpti == 0, equally spaced intervals are

used.  For PolyOpti == 1, SampTol (real number) specifies the

maximal allowed deviation tolerance of the piecewise linear

approximation from the original curve.

Default is 0 64 (uniform sampling with 64 samples).

-s: sorts the data in Z depth order that emulates hidden line removal

once the data are drawn.

-M: Dumps the control mesh/polygon as well.

-P: Dumps the curve/surface as isocurves.

-p: Dumps vertices of polygons/lines as points.

-O: Handles polygonal objects as possibly open.  This will generate

two identical edges for an edge shared by two adjacent polygons.

This can be useful for open or isolated polygons.

-l MaxLnLen: Breaks long lines into shorter ones with maximal length

of MaxLnLen. This option is necessary to achieve good depth depending

on line width in the '-d' option of irit2ps.

-a: Takes into account the angle between the two (poly)lines that

intersect when computing how much to trim. See also -t.

-t TrimInter: Each time two (poly)line segments intersect in the

projection plane, the (poly)line that is farther away from the

viewer is clipped by the TrimInter amount from both sides. See also -a.

-o OutName: Name of output file. Default is stdout.

-Z InterSameZ: The maximal Z depth difference of intersection curves

to be be considered invalid.

-m: More talkative mode. Prints processing information.

-T AnimTime: If the data contain animation curves, evaluate and

process the scene at time AnimTime.

-z: Prints version number and current defaults.

Usage

-----

illustrt is a simple line illustration tool. It processes geometry such

as polylines and surfaces and dumps geometry with attributes that will make

nice line illustrations. illustrt is geared mainly toward its use with

irit2ps to create postscript illustrations. Here is a simple example:

illustrt -s -l 0.1 solid1.itd | irit2ps -W 0.05 -d 0.2 0.6 -u - > solid.ps

makes sure all segments piped into irit2ps are shorter than 0.1 and sorts them

in order to make sure hidden surface removal is correctly applied. Irit2ps

is invoked with depth cueing activated, and a default width of 0.05.

illustrt dumps out regular IRIT data files, so output can be handled

like any other data set. illustrt does the following processing to the input

data set:

Converts surfaces to isocurves ('-I' flag) and isocurves and curves to

polylines ('-S' flag), and converts polygons to polylines.

Polygonal objects are considered closed and even though each edge is

shared by two polygons, only a single one is generated.

Finds the intersection location in the projection plane of all segments in

the input data set and trims away the far segment at both sides of the

intersection point by an amount controlled by the '-t' and '-a' flags.

Breaks polylines and long lines into short segments, as specified via the

'-l' flag, so that width depth cueing can be applied more accurately

(see irit2ps's '-d' flag) as well as the Z sorting.

Generates vertices of polygons in the input data set as points in output data

controlled via the '-p' flag.

set.

Applies a Z sort to the output data, if '-s', so drawing in order of the data

will produce a properly hidden surface removal drawing.

Here is a more complex example. Make sure tubular is properly set via

"attrib(solid1, "tubular", 0.7);" and invoke:

illustrt -s -p -l 0.1 -t 0.05 solid1.itd |

irit2ps -W 0.05 -d 0.2 0.6 -p h 0.05 -u - > solid.ps

makes sure all segments piped into irit2ps are shorter than 0.1, generates

points for the vertices, sorts the data in order to make sure hidden surface

removal is correctly applied, and trims the far edge by 0.05 at an

intersection point. Irit2ps is invoked with depth cueing activated and a

default width of 0.05, points are drawn as hollowed circles of default

size 0.05, and lines are drawn tubular.

Objects in the input stream that have an attribute by the name of

"IllustrtNoProcess" are passed to the output unmodified.

Objects in the input stream that have an attribute by the name of

"SpeedWave" will have a linear segment added that emulates fast

motion with the following attributes,

"Randomness,DirX,DirY,DirZ,Len,Dist,LenRandom,DistRandom, width".

Objects in the input stream that have an attribute by the name of

"HeatWave" will have a spiral curves added that emulate a heat wave

in the +Z axis with the following attributes,

"Randomness,Len,Dist,LenRandom,DistRandom, width".

Examples:

attrib(Axis, "IllustrtNoProcess", "");

attrib(Obj, "SpeedWave", "0.0005,1,0,0,5,3,3,2,0.05");

attrib(Obj, "HeatWave", "0.015,0.1,0.03,0.06,0.03,0.002");

Aisoshad - Simple line illustration filter

------------------------------------------

Introduction

------------

Aisoshad is a filter that processes IRIT data files of freeform shapes

and dumps out modified IRIT data files in the form of short

univariate strokes. Aisoshad can be used to make simple yet nice line

art illustrations of geometry that is based solely on isoparametric

curves.

Aisoshad employs a simple shader to determine the density of the

isoparametric strokes as well as the thickness etc.  Output of aisoshad

can be piped into the irit2ps postscript postprocessor.

Command line options

--------------------

aisoshad [-o OutName] [-m] [-i] [-F PolyOpti FineNess]

[-f PolyOpti SampTol] [-r RndrMdl] [-c CosPwr] [-s SdrPwr]

[-l Lx Ly Lz] [-R Random] [-d AdapDir] [-t SrfZTrans]

[-M MinSubdiv] [-D AdapDist] [-w AdapIsoWidth] [-S WidthScale]

[-W] [-u] [-Z ZbufSize] [-b] [-z] DFiles

-o OutName: Name of output file. Default is stdout.

-m: More talkative mode. Prints processing information.

-i: Solve symbolic products using interpolations.  Faster but the

generated output is not as compact as possible.

-I #IsoLines: Specifies number of isolines per surface, per direction.

-F PolyOpti FineNess: Optimality of polygonal approximation of

surfaces. See the variable POLY_APPROX_OPT for the meaning of

FineNess. Default is 0 and 20.0 (no optimal sampling with fineness

of 20.0 (real number)).

-f PolyOpti SampTol: Controls the method used to approximate curves

into polylines. If PolyOpti == 0, equally spaced intervals are

used.  For PolyOpti == 1, SampTol (real number) specifies the

maximal allowed deviation tolerance of the piecewise linear

approximation from the original curve.

Default is 0 64 (uniform sampling with 64 samples).

-r: Selects the rendering model of the shader as follows:

1. Cosine shader, diffuse only, light source regular.

2. Cosine shader, diffuse only, light source as two lights from

opposite directions.

3. Cosine shader, has specular term, light source regular.

4. Cosine shader, has specular term, light source as two lights.

5. Shader emphasizing the silhouette areas of the model.

6. Shader estimating distance decay from a point light source.

-c CosPower: Controls the cosine shader's power.

-s SdrPower: Controls the shader's relative influence.

-l Lx Ly Lz: Sets the light source position/direction.

-R Random: Controls the levels of randomness that the isoparametric

curves perturb.  Low levels of randomness would leave visible

artifacts while too high levels would disturb the shading.  Should be

greater than one or negative one to disable.

-d AdapDir: Sets the isoparametric directions of the strokes. Either

0, 1, or 2 for U direction, V direction or both U and V directions,

respectively.

-t SrfZTrans: The amount to translate the created line strokes in Z,

in order to prevent Z fighting with the rendered object itself.

-M MinSubdiv: Sets the minimal number of subdivision to enforce

during the isoparametric strokes' construction.  This flag should be

used rarely and typically MinSubdiv should be low and close to one.

-D AdapDist: Sets the distance between adjacent isocurves. The smaller

AdapDist is, the denser the coverage of the strokes will be.

-w AdapIsoWidth: Sets the default width attribute of the generated

strokes.

-S WidthScale:  Controls the relative variance of the width of the

strokes in variable width strokes.

-W: If set, enables variable width strokes.

-u: If set, maps the strokes to screen space. Otherwise, strokes

are mapped back to object space.

-Z ZbufSize: Sets the size of the (square) Z buffer to set.

-b: If set, generates a binary IRIT data file that holds the

strokes.  Otherwise, IRIT text file will be created.

-z: Print version number and current defaults.

Usage

-----

Aisoshad is a simple line art illustration tool that generates strokes

that follow the isoparametric curves. It processes freeform geometry such

as surfaces and dumps geometry with attributes that makes

nice line illustrations. Aisoshad is geared mainly toward its use with

irit2ps to create postscript illustrations. Here is a simple example:

aisoshad -Z -500 -F 0 50 -s 10 -c 1 -D 0.3 -r 5 wglass.itd view.imd |

irit2ps -W 0.05 -d 0.2 0.6 -u - > wglass1.ps

that creates line art illustrations of a wine glass wglass.itd with

hidden strokes removed via a Z-buffer of size 500 that will be displayed on

screen, polygonal fineness of 50 for the surface of the glass, shader power

of 10 and cosine power of 1, isoparametric curves maximal distance of 0.3,

and shader number 5 that emphasizes silhouettes.  The output of the shader

is piped to a irit2ps filter to postscript that sets the width of the strokes

to be a function of depth.

Here is another example:

aisoshad -Z 700 -R 4 -F 0 50 -l 1 1 1 -D 0.02 -r 2 wglass.itd view.imd

irit2ps -W 0.005 -d 0.2 0.6 -u - > wglass2.ps

that creates line art illustrations of a wine glass wglass.itd with

hidden strokes removal via a Z-buffer of size 700 that is allocated off-line

in core memory, polygonal fineness of 50 for the surface of the glass,

a light source at (1, 1, 1), isoparametric curves maximal distance of 0.02,

and a cosine shader number 2.  The output of the shader is piped to a irit2ps

filter to postscript that sets the width of the strokes to be a function of

depth.

Transparent objects, or objects with the "transp" attribute, would generate

strokes as regular surfaces but would not participate in the hidden strokes

removal.  An "AdapIsoDir" attribute that is found on some surface object

would override the global isoparametric direction's setup of strokes as is

set via the 'd' option.

See also the illustrt, izebra, lineshad, and

irit2ps tools.

IZebra - Simple zebra style, parallel curve based rendering

-----------------------------------------------------------

Introduction

------------

Izebra is a filter that processes IRIT data files into a 2D striped,

zebra style illustration that gives the user an illusionary depth cue.  The

output is also an IRIT data file in the form of freeform curves.  Izebra

can be used to make simple yet nice art illustrations of geometry that is

based on a specific style inspired by the artist Victor Vasarely.

Izebra employs a Z buffer to determine the density and warping of the

stripes.  Output of izebra can be piped into the irit2ps postscript

postprocessor.

Command line options

--------------------

IZebra [-o OutName] [-m] [-O ImgOper] [-F PolyOpti FineNess] [-u]

[-I NumIters] [-Z ZbufSize] [-B CbcBspSize] [-D DataSrf]

[-A StripeAngle] [-b] [-s Stripes] [-S ZScale] [-d ZInitDepth]

[-z] DFiles

-o OutName: Name of output file. Default is stdout.

-m: More talkative mode. Prints processing information.

-O ImgOper: By default, the Z buffer is employed directly.  However,

once the Z buffer is fully evaluated and before beginning the stripes

processing, one can apply a filter to the Z map of the Z buffer. The

filter can be a first order Roberts derivative if "-O 1", a second

order Laplacian if "-O 2", or an inverted depth if "-O 3".

-F PolyOpti FineNess: Optimality of polygonal approximation of

surfaces. See the variable POLY_APPROX_OPT for the meaning of

FineNess. Default is 0 and 20.0 (no optimal sampling with fineness

of 20.0 (real number)).

-u: Forces a unit matrix. That is, input data are not

transformed at all.

-I NumIters: Puts a bound on the number of iterations in the

numerical processing stage.

-Z ZbufSize: ZbufSize sets the size of the (square) Z buffer to set.

-B CbcBspSize: Sets the mesh size of the constructed uniform cubic

B-spline grid, if no data surface is specified by '-D'.

-D DataSrf: If specified, provides the name of the uniform cubic

B-spline to load and warp.  Overrides the '-B' option.

-A StripeAngle: Sets the angle of the stripes with respect to the

horizontal line, in degrees.

-b: If set, generates a binary IRIT data file that holds the

stripes.  Otherwise, an IRIT text file will be created.

-s Stripes: If set, prescribes the number of strips to extract as iso

parametric curves of the warped B-spline surface.  Otherwise, the

warped B-spline surface itself is dumped out.

-S ZScale: A relative factor to control the effect of the depth

on the warping amount.  This should be around one.

-d ZInitDepth: By default, the Z buffer is initialized to a depth of

zero which amounts to no warping of the B-spline surface.  Here is

a proper way to prescribe a different background depth (which will

cause warping in the surface).

-z: Print version number and current defaults.

Usage

-----

Izebra is a simple stripes art illustration tool that generates stripes

that follow a warped B-spline surface as its isoparametric curves. It

processes the given geometry, such as surfaces, into a Z map of a Z buffer

and warps a B-spline surface that is placed over it, with a warping amount

that is a function of the locally detected depth.  IZebra dumps out stripes

geometry that makes nice illusionary illustrations.  IZebra is geared

mainly toward its use with irit2ps to create postscript illustrations.

Here is a simple example:

izebra -m -Z 500 -B 150 -I 10 -F 0 100 -A 140 -S 0.35 pawn.itd |

irit2ps -f 0 300 -u -B -0.45 -0.75 0.65 0.75 -W 0.004 -I 0:250 - > pawn.ps

creates striped illustrations of a pawn chess piece, with the aid

of a Z-buffer of size 500 by 500, a uniform cubic B-spline surface with mesh

size of 150 by 150, polygonal fineness of 100 for the surface of the pawn,

rotation of stripes of 140 degrees and Z scale factor of 0.35.  Ten iterations

will be conducted during the numerical processing of the data.

The output of izebra is piped by the irit2ps filter to postscript that

extracts 250 isoparametric curves out of the dumped warped surface and sets

the width of the strokes to be 0.004.

Here is another example:

izebra -m -Z 500 -B 200 -I 10 -F 0 100 -A -90 -S 0.4 teapot.itd |

irit2ps -f 0 200 -u -B -0.55 -0.35 0.55 0.35 -W 0.007 -I 0:150 - > teapot.ps

creates striped illustrations of the Utah Teapot, with the aid

of a Z-buffer of size 500 by 500, a uniform cubic B-spline surface with mesh

size of 200 by 200, polygonal fineness of 100 for the surface of the teapot,

rotation of stripes of -90 degrees and Z scale factor of 0.4.  Ten iterations

will be conducted during the numerical processing of the data.

The output of izebra is piped by the irit2ps filter to postscript that

extracts 150 isoparametric curves out of the dumped warped surface and sets

the width of the strokes to be 0.007.

See also the illustrt, aisoshad, lineshad, and

irit2ps tools.

LineShad - Simple line illustration filter

------------------------------------------

Introduction

------------

Lineshad is a filter that processes IRIT data files of freeform shapes

and dumps out modified IRIT data files in the form of short

univariate strokes. Lineshad can be used to make simple yet nice line

art illustrations of geometry that is based on arbitrarily stroked

curves on the surfaces.

Lineshad employs a simple shader to determine the density of the

isoparametric strokes as well as the thickness etc.  Output of lineshad

can be piped into the irit2ps postscript postprocessor.

Command line options

--------------------

lineshad [-o OutName] [-m] [-F PolyOpti FineNess] [-R RelStepSize]

[-f PolyOpti SampTol] [-r RndrMdl] [-c CosPwr] [-s SdrPwr]

[-i Intensity] [-l Lx Ly Lz] [-v Vx Vy Vz] [-w Width]

[-d Density] [-t SrfZTrans] [-S WidthScale] [-T Texture]

[-Z ZbufSize] [-b] [-z] DFiles

-o OutName: Name of output file. Default is stdout.

-m: More talkative mode. Prints processing information.

-F PolyOpti FineNess: Optimality of polygonal approximation of

surfaces. See the variable POLY_APPROX_OPT for the meaning of

FineNess. Default is 0 and 20.0 (no optimal sampling with fineness

of 20.0 (real number)).

-R RelStepSize:  Relative control (default to 1.0) on the step size

taken during the numerical marching on the surfaces in the different

strokes' patterns.

-f PolyOpti SampTol: Controls the method used to approximate curves

into polylines. If PolyOpti == 0, equally spaced intervals are

used.  For PolyOpti == 1, SampTol (real number) specifies the

maximal allowed deviation tolerance of the piecewise linear

approximation from the original curve.

Default is 0 64 (uniform sampling with 64 samples).

-r: Selects the rendering model of the shader as follows:

1. Dumps only the uniform point distribution.

2. Cosine shader, diffuse only, light source regular.

3. Cosine shader, diffuse only, light source as two lights from

opposite directions.

4. Cosine shader, has specular term, light source regular.

5. Cosine shader, has specular term, light source as two lights.

6. Shader emphasizing the silhouette areas of the model.

7. Shader estimating distance decay from a point light source.

-c CosPower: Controls the cosine shader's power.

-s SdrPower: Controls the shader's relative influence.

-i Intensity: Controls the global density of the constructed line art.

The larger Intensity is, the denser the drawing becomes.

-l Lx Ly Lz: Sets the light source position/direction.

-v Vx Vy Vz: Sets the viewing direction; typically, the Z axis.

-w Width: Sets the width of the generated strokes.

-d Density: Relative control (default to 1.0) of the density of the

uniform point distribution from which the strokes are developed.

-t SrfZTrans: Amount of created line strokes in Z to translate,

in order to prevent Z from fighting with the rendered object itself.

-S WidthScale:  Controls the relative variance of the width of the

strokes in variable width strokes.

-T Textures:  Selects the pattern of the strokes. Texture can be one

of:

1. "isoparam[,{0,1,2}w]":  Isoparametric curves will be created in

a similar way to the aisoshad tool.  Following the "isoparam"

string, one can optionally specify the isoparametric direction

as 0, 1 or 2 for U, V, or both, and a second 'w' character for

optional variable width.  This option extracts exact

isoparametric curves from the given surface.

2. "wood[,Dx,Dy,Dz]":  A strokes' style following layers of wood

will be used.  Optionally, a direction normal to the layers

can be specified, with a default being the Z axis.

3. "vood[,Ry,Rz]":  A variation on the wood texture, this time with

a layered orientation set via two rotation angles around Y and

Z.

4. "isomarch[,{0,1,2}]":  Similar to "isparam" but numerically

march on the surface in the isoparametric direction.  Again,

0, 1, or 2 stands for U, V, or both isoparametric directions.

5. "silhouette[,{t,n,tn}]":  Extract strokes emphasizing the

silhoeutte areas from the given viewing direction.  Strokes

can be extracted in the direction of the surface normal near

the silhouette area if option ",n" is given, tangent along

the surface if ",t", or both if ",tn".

6. "iTexture":  Employ a raster image as a texture image on the

surface with the gradient of the image serving as the strokes

direction. The name of the image itself (must be in urt rle

format) is expected in a "iTexture" attribute on the specific

object.

7. "curvature[,{0,1,2}]":  Develop strokes along lines of

curvatures.  Strokes are developed along the minimal curvature

if ",0", the maximal curvature if ",1", and both if ",2".

8. "CurveStroke":  An XY curve object is expected as a

"CurveStroke" attribute on the same object and serves as a

specification of motion in the parametric space of the surface

for each given point.

-Z ZbufSize: ZbufSize sets the size of the (square) Z buffer to set.

-b: If set, generates a binary IRIT data file that holds the

strokes.  Otherwise, an IRIT text file will be created.

-z: Print version number and current defaults.

Usage

-----

lineshad is a simple line art illustration tool that generates strokes

that follow the isoparametric curves. It processes freeform geometry such

as surfaces, and dumps geometry with attributes that makes

nice line illustrations. lineshad is geared mainly toward its use with

irit2ps to create postscript illustrations. Here is a simple example:

lineshad -Z -500 -F 0 50 -T "isoparam" -d 0.5 -c 10 -r 2 wglass.itd view.imd |

irit2ps -W 0.002 -u - > wglass3.ps

creates line art illustrations of a wine glass wglass.itd with

hidden strokes removal via a Z-buffer of size 500 that will be displayed on

screen, polygonal fineness of 50 for the surface of the glass, shader that

employs isoparametric curves, relative density of distribution of 0.5, and

cosine power of 10 of the cosine shader number 2.

Here is another example:

lineshad -Z -500 -F 0 50 -T "wood,1,1,1" -d 6 -c 10 -r 2 wglass.itd view.imd |

irit2ps -W 0.002 -u - > wglass4.ps

creates line art illustrations of a wine glass wglass.itd with

hidden strokes removal via a Z-buffer of size 500 that is allocated off-line

in core memory, polygonal fineness of 50 for the surface of the glass,

a light source at (1, 1, 1) for the wood strokes' style, relative point

distribution of 6, and a cosine power of 10 for the cosine shader number 2.

Transparent objects, or objects with the "transp" attribute, will generate

strokes as regular surfaces but will not participate in the hidden strokes

removal.   A string "itexture" attribute is expected if "itexture" strokes'

style is used.  A curve object as the "CurveStroke" attribute is expected if

"CurveStroke" is employed.  One can override the strokes' style as it is set

via the '-T' command line option by setting an "lTexture" string attribute

with the prefered strokes' style of this object.  One can modify the relative

density of some specific object by placing a real number attribute named

"PtsDensity" on the object.

See also the illustrt, izebra, aisoshad, and

irit2ps tools.

ihidden - Hidden Curve Removing Program

---------------------------------------

Introduction

------------

ihidden is a program to remove hidden curves from a given surface model.

Only freeform objects are processed in ihidden.

The program performs 3 passes over the input:

1. Preprocesses and extracts the different curves in a scene, boundary curves,

silhouette curves, isoparametric curves and discontinuity curves.

2. Solves for all the intersections of the different curves in the parametric

space, and at that point splits the curves into curve segments.

3. Applies a visibility test to each segment of curve.

This program can handle non self interesecting surfaces only.  Further,

surfaces that intersect other surfaces and are not properly trimmed into

a model are likely to result in the wrong answer as well.

The output of ihidden is in the form of curves. It is a regular

IRIT data file that can be viewed using any of the display

devices, for example.

Command line options

--------------------

ihidden [-q] [-H] [-I #UIso[:#VIso[:#WIso]]] [-s Stage] [-b]

[-o OutName] [-C CCTTol] [-S RSITol] [-Z ZBufSz] [-t AnimTime]

[-z] DFiles

-q: Quiet - provides no information on the progress if TRUE.

-H: Dumps both visible lines and hidden curves as separated objects.

Hidden curves will be dumped using a narrower line width.

-I #UIso[:#VIso[:#WIso]]: Specifies the number of isolines per

surface/trivariate, per direction. If #VIso is not specified,

#UIso is used for #VIso as well and so on.

-s: Specifies the step at which to stop this process, where step 3, as

described above, will complete the entire hidden curve removal

process and is the default.

-b: If set, generates a binary IRIT data file that holds the

strokes.  Otherwise, an IRIT text file will be created.

-o OutName: Name of output file. Default is stdout.

-C CCTTol: Tolerance of the curve curve intersection (CCI) process.

-S RSITol: Tolerance of the surface fineness in the ray surface

intersection (RSI) and visibility testing process as well as in the

silhouette extraction.

-Z ZBufSz: Size of the Z buffer in the visibility testing process.

-c: Clips data to screen (default). If disabled ('-c-'), data

outside the view screen ([-1, 1] in x and y) are also processed.

-t AnimTime: If the data contains animation curves, evaluate and

process the scene at time AnimTime.

-z: Prints version number and current defaults.

Some of the options may be turned on in ihidden.cfg. They can then be

turned off in the command line as '-?-'.

Configuration

-------------

The program can be configured using a configuration file named ihidden.cfg.

This is a plain ASCII file you can edit directly and set the parameters

according to the comments there. 'ihidden -z' will display the current

configuration as read from the configuration file.

The configuration file is searched in the directory specified by the

IRIT_PATH environment variable. For example,

'setenv IRIT_PATH /u/gershon/irit/bin/'.

If the IRIT_PATH variable is not set, the current directory is searched.

Usage

-----

As this program is not interactive, usage is quite simple, and the only

control available is using the command line options.

If a certain surface should contain more or less isoparametric curves,

a relative change could be applied to some specific object via the

"num_of_isolines" attribute.  If a "transp" attribute is found on some

object, it will generate all the curves but will not affect the visibility

(i.e. be fully transparent).

See also Poly3d-h.

Irender - Simple Scan Line Renderer

-----------------------------------

Introduction

------------

irender is a program to render IRIT scenes into images. It is

a software based Z buffer that is able to create images in few formats.

Several of its features includes parametric and volumetric texture mapping,

shadow computations, transparency and antialiasing.

Freeform objects are preprocessed into polygons with controlled fineness.

Command line options

--------------------

irender [-v] [-s XSize YSize] [-Z Znear Zfar] [-a Ambient] [-b R G B]

[-B] [-F PolyOpti FineNess] [-f PolyOpti SampPerCrv]

[-M Flat/Gouraud/Phong/None] [-P WMin [WMax]] [-S]

[-T] [-t AnimTime] [-A FilterName] [-d] [-l] [-n]

[-i rle/ppm{3,6}] [-o OutName] [-z] files

-v: Verbose mode. Prints informative messages as it progresses.

-s XSize YSize: Sets the size of the output image, in pixels.

Default to 512x512.

-Z Znear Zfar: Sets the near and far cliping planes with default

of no clipping.

-a Ambient: Sets the ambient lighting fraction. Between zero (no

ambient lighting) and one. Default to 0.2.

-b R G B: Sets the background color. Each of thre R,G,B colors is

an integer value between zero and 255. Default to black.

-B: Apply back face culling. Somewhat faster, but only correct for

closed objects. Default is no back face culling.

-F PolyOpti FineNess: Optimality of polygonal approximation of

surfaces. See the variable POLY_APPROX_OPT for the meaning of

FineNess. Default is 0 and 20.0 (no optimal sampling with fineness

of 20.0 (real number)).

-f PolyOpti SampTol: Controls the method used to approximate curves

into polylines. If PolyOpti == 0, equally spaced intervals are

used.  For PolyOpti == 1, SampTol (real number) specifies the

maximal allowed deviation tolerance of the piecewise linear

approximation from the original curve.

Default is 0 64 (uniform sampling with 64 samples).

-M Flat/Gouraud/Phong/None: Selects the shader to be used. Default to

Phong if has normals of vertices, Flat if no normals are found.

The None options exactly paints the objects with the given color,

applying no shader.

-P WMin [WMax]: Width of rendered polyline, in pixels. If only

WMin is specified, all polylines are set to have WMin width. Otherwise,

if WMax is prescribed as well, polylines' width is set to be

proportional to their depth with WMax is the width of closest polyline

and WMin the farest polyline.  Polylines and curves will be

ignored without the setting of this option.

-S: Enable shadow computation. No shadows will be rendered without -S.

The is no shadow support for this release of irender.

-T: Enable transparency computation. No transparent object will be

processed without -T.

-t AnimTime: If the data contains animation curves, evaluate and

process the scene at time AnimTime.

-A FilterName: Selects an antialiasing filter. FilterName can be one

of 'none', 'box', 'triangle', 'quadratic', 'cubic', 'catrom',

'mitchell', 'gaussian', 'sinc, and 'bessel'. Default is 'none'.

-d: Output will be in the form of Z depth instead of a color image.

Output will be 32 bits depth instead of RGBA.

-n: Reverses the normals of vertices and planes, globally.

-i rle/ppm{3,6}: Selects output image type. Currently the

Utah Raster Toolkit's (URT) rle format is being supported as well as

the PPM format.  PPM can be either P6 or P3 style.

-o OutName: Name of output file. By default the output goes to

stdout.

-z: Prints version number and current defaults.

Some of the options may be turned on in irender.cfg. They can be then

turned off in the command line as '-?-'.

Configuration

-------------

The program can be configured using a configuration file named irender.cfg.

This is a plain ASCII file you can edit directly and set the parameters

according to the comments there. 'irender -z' will display the current

configuration as read from the configuration file.

The configuration file is searched in the directory specified by the

IRIT_PATH environment variable. For example,

'setenv IRIT_PATH /u/gershon/irit/bin/'.

If the IRIT_PATH variable is not set, the current directory is searched.

Usage

-----

As this program is not interactive, usage is quite simple, and the only

control available is using the command line options.

Advanced Usage

--------------

One can specify several attributes that affect the way the scene is

rendered. The attributes can be generated within IRIT.

See also the ATTRIB IRIT command.

Surface color is controlled on two levels. If the object has an RGB

attribute, it is used. Otherwise, a color as set via the IRIT COLOR

command.

If a certain surface should be finer/coarser than the rest of the

scene, one can set a "resolution" attribute which specifies the

relative FineNess resolution of this specific surface. Further,

"u_resolution" and "v_resolution" might be similarly used to set

relative resolution for the u or v direction only.

The "crv_resolution" attribute controls the relative fineness of curves

as polylines. The "num_of_isolines" attribute controls the relative number

of isoparametric curves.

Example:

attrib( Ball, "rgb", "255,0,0" );

color( Sphere, white );

The cosine exponent of the phong shader can be set for a specific

object via the SRF_COSINE attribute, with 128 as default value.

An object can affect its diffuse and specular components via the

DIFFUSE and SPECULAR real attributes, with 0.4 as default value.

Example:

attrib( Ball, "srf_cosine", 16 );

attrib( Ball, "diffuse", 0.7 );

attrib( Ball, "specular", 1.0 );

An object can be drawn transparent instead of opaque, if it has a

"transp" attribute. A transparent value of one denotes a completely

transparent object, while a value of zero means a completely opaque

object. Transparent objects will be rendered as such if and only if the '-T'

command line option is set.

Example:

attrib( final, "transp", 0.5 );

Several types of texture mapping are supported. Parametric texture may be

attached to a parametric surface where the prescribed image is mapped onto

the rectangular parametric domain of the surface.

The parametric texture may be applied with the following options:

'D' x y z        Vector that will be rotated to Z along with the

texture coordinates. Applies to 'T' 1, 2 or 3.

Default to the Z axis.

'O' x y z        a point to which that texture Origin will be translated.

Applies to 'T' 1, 2 or 3. Default to origin.

'S' Su Sv {Sw}   Scales the coordinates in u and v.  Scale

factors of 1.0 would cover the entire surface

once.  Default to scale factors of 1.0.  If Sw is

specified for a polygonal object, each polygonal is

locally scaled based on its maximal projection on

one of the main, XY XZ or YZ, planes.

'A' a            Angle of rotation in degrees of texture map with

respect to main axis.  Applies to 'T' 1, 2 or 3.

Default to no rotations.

'T' TextureType  with 0 denotes regular parametric texture,

1 denotes spherical coordinates,

2 denotes spherical bijective coordinates,

3 denotes cylinderical coordinates,

4 denotes planar coordinates.

Regular parametric texture employs the inherited surface parametrization

of the freeform surface and can only be used on parametric surfaces.

Spherical, cylinderical, and planar coordinate transformations are useable

for all types of geometry from polygons to freeform surfaces and is

fairly straightforward with the origin as set by 'O' being the center

of the mapping while the direction set by 'D' controls the north pole

of the sphere,  the axis of the cylinder, and the normal of the plane.

Finally, the angle set by 'A' rotates the texture around this 'D' prescribed

axis.

The spherical bijective mapping is more complex.  An object identical

to the textured object should be found as an "PTextureBijectObj" that

contains the identical topology of the original object.  The original

object must be genus zero non convex, while the attribute object must be

a genus zero convex object with the origin as set via 'O', inside this

convex object.   It is likely that both the original object and its attribute

object will be a polygonal object.  Both objects must contain triangles only.

A bijective mapping is then conducted from every point on the original

non convex object to the convex attribute object and from there through

spherical mapping to the texture map.

Example:

attrib( Srf1,        "ptexture", "checker.ppm, S 1 1, A 45" );

attrib( Srf2,        "ptexture", "checker.ppm, S 1 3, T 1, O 1 1 1, D 0 0 1" );

attrib( Srf3Triangs, "ptexture", "checker.ppm, S 1 2, T 1, O 1 1 1, D 0 0 1" );

attrib( Srf3Triangs, "PTextureBijectObj", Srf3ConvexTriangles );

Srf1 is a parametrically textured map using spherical mapping,

Srf2 is a parametrically textured map using cylinderical

mapping and Srf3Triangs is a parametrically textured map using

spherical bijective mapping and Srf3ConvexTriangles is the convex

topologically equivalent object.

The program will automatically detect a ppm file from an rle according to

the file's name.  Note that regular parametric texture may be applied to

parametric surfaces only, whereas the spherical, cylinderical and planar

parametric textures may be used on all types of geometry.

A second type of texture mapping can be applied to all geometric objects.

Herein, a procedural texture mapping is employed. The currently supported

textures are

camouf         Camouflage style

checker        Checker style

chocolate      Chocolate chips style

contour        Parallel plane contouring

curvature      Gaussian/Mean etc. curvature

marble         Marble style

ncontour       Constant normal angle to major axis

orange         Bump mapping orange style

wood           Wood style

punky          Colorful punky style

A second parameter that must be provided for procedural textures is the

scaling factor of the texture, which can be either one parameter of uniform

scaling or a vector of three coefficients for scaling in x, y, and z.

For contour style, the scale denotes the spacing of the contouring

planes in X, Y and Z.  For ncontour style, the scale also denotes

the spacing of the adjacent constant normal contours.  Related attributes

are "texture_color" and "texture_width" that support the color and the

width of the textured strokes.

Example:

attrib( Obj1, "texture", "marble, 2" );

attrib( Obj2, "texture", "wood, 1 0.5 2.5" );

which sets Obj1 to have a marble procedural texture with a uniform

scaling factor of 2 and a wood texture for Obj2 with scaling factors

of (1, 0.5, 2.5) in x, y, and z.

In addition, the appearance of each procedural texture can be controlled by

optional parameters which are different for each texture. Each texture

parameter is recognized by a letter; to enter a parameter, add to the

attribute string the paramter letter followed by the value or values. Each

parameter should be separated by a comma.

Example:

attrib( Obj1, "texture", "wood, 2, b 0.3, o 5 5 5" );

sets Obj1 to have a wood procedural texture with a scaling factor

of 2, a Brightness level of 0.3, and the Origin point at (5,5,5).

The optional parameters are:

checker:

'z' x y z      a vector to which the Z axis will be rotated.

'o' x y z      a point to which the Origin will be translated.

'b' x          the brightness of the checker color scaling,

should be between 0 and 1.

'CP' f         To force a 2D checker plane orthogonal to the

vector that is specified via the 'z' option.

'C1' r g b     A second optional color for the checkerboard.

'C2' r g b     A third optional color for the checkerboard,

used in the second layer of the checker volume.

'C3' r g b     A fourth optional color for the checkerboard,

used in the second layer of the checker volume.

chocolate:

'W' w          the 'width' of chocolate piece (zero to half).

'd' x          the 'depth' of the bumps on the bump-mapping.

contour:

'W' w          the 'width' of contour.

'C' r g b      the color of the contour in RGB betweeo zero

and one ("C 1 1 1" fully is white).

curvature:

The curvature texture has no optional parameter, but the first scale

parameter has a special meaning.  A scale of

0          Paints convex regions in red, concave in

green, and saddle-like in yellow.

>0         Paints the Gaussian curvature in convex regions

in red to magenta, in concave regions in yellow

to green, and in saddle-like in cyan to blue.

<0         Paints the Mean curvature in positive mean

curvature regions in yellow to green and in

negative Mean curvature in red to magenta.

If this first scale parameter is non zero, its absolute value is used to

modify the blending speeds between the different colors.

marble:

'z' x y z      a vector to which the Z axis will be rotated.

'o' x y z      a point to which the Origin will be translated.

't' f s        the scale of the turbulence noise, and the

factor to multiply that noise.

'f' x          the 'frequency' of the marble layers.

ncontour:

'W' w          the 'width' of contour.

'C' r g b      the color of the contour in RGB betweeo zero

and one ("C 1 1 1" fully is white).

orange:

'd' x          the 'depth' of the bumps on the bump-mapping.

wood:

'z' x y z      a vector to which Z axis will be rotated.

'o' x y z      a point to which the Origin will be translated.

'b' x          the brightness of the wood color scaling,

should be between 0 and 1.

'c' f s        the scale of the noise in the wood center axis

and the factor by which to multiply that noise.

'w' n f        the number of angles to sample noise when creating

distortion in the circle shape of the wood

cylinders, and the factor by which to multiply that noise.

'f' x          the 'frequency' of the wood cylinders.

'r' f s        the scale of the wood-fibers noise, and the

factor by which to multiply that noise.

punky:

'b' x          the brightness/saturation of the punky color.

More Examples:

attrib( Obj1, "texture", "marble, 2, t 3.0 12.0, f 7.0" );

attrib( Obj2, "texture", "contour, 1 0.5 2.5, W 0.004, C 1 1 0" );

sets Obj1 to have a marble procedural texture with a uniform

scaling factor of 2, and new turbulance and frequency factors.

This also sets a contouring texture for Obj2 with scaling

factors of (1, 0.5, 2.5) in x, y, and z, in yellow color and width 0.004.

In addition, a scalar surface spanning the same parameteric domain as an

original surface may be used as a texture mapping function.  Herein, the

scalar function texture is evaluated at each UV parameter value and is mapped

through a color scale to yield the output color.  This type of texture is

useful for stress maps or analysis maps on top of freeform surfaces.

Several related attributes are supported: "stexture_scale" which prescribes

the color scale image (only its first column is employed), and

"stexture_bound" that sets the domain that will be clipped to the min max

values. Funally, "stexture_func" can hold the functions "sqrt" or "abs"

to be applied to the evaluated surface value.

Example:

attrib( Srf, "stexture", scrvtr( Srf, P1, off ) );

attrib( Srf, "stexture_scale", "color_scale.ppm" );

attrib( Srf, "stexture_func", "sqrt" );

attrib( Srf, "stexture_bound", "0.0 100.0" );

where scrvtr computes a scalar field to Srf that represents the

sum of the squares of the principle curvatures.  The evaluated scalar

texture surface's value is piped through a sqrt function.

The first column of the image of color_scale.ppm is used to set

the coloring scale for curvature bounds values between 0.0 and 100.0.

Both "stexture_scale" and "stexture_bound" are optional.  The default

color scale maps the min/max values from blue to red through green. The

default scalar surface texture bound is computed as the extreme values of

the "stexture" surface.

While the program has a default for lighting which is two light sources

at opposite directions at (1, 1, 1) and (-1, -1, -1), one can overwrite

this default. A POINT_TYPE object with LIGHT_SOURCE attribute denotes

a light source.  If irender detects one or more light sources in the

input stream, the default light sources are not created. Two types of

light sources may be prescribed, a parallel at infinity or a point at a

finite distance light source, distinguished by a TYPE attribute of

either POINT_POS or POINT_INFTY. A point light source can be colored;

an RGB attribute will set its color. A point light source will cast

shadows, if and only if, it has a SHADOW attribute (one needs to apply the

'-S' command line option as well for rendering shadows). Finally, one can

construct two mirrored light sources at opposite directions if the TWOLIGHT

attribute is added to the light source object.

Example:

Light1 = point( 0, 0, 10 );

attrib( Light1, "light_source", on );

attrib( Light1, "shadow", on );

attrib( Light1, "rgb", "255,0,0" );

attrib( Light1, "type", "point_pos" );

Light2 = point( 1, 1, 1 );

attrib( Light2, "light_source", on );

attrib( Light2, "twolight", on );

attrib( Light2, "type", "point_infty" );

constructs two lights sources with Light1 with red color positioned

at (0, 0, 10) and casting shadows, while Light2 will create two

mirrored white parallel lights sources in the direction of (1, 1, 1) and

(-1, -1, -1), as its irender's default.

3DS2IRIT - AutoCad 3DS Data To IRIT file filter

-----------------------------------------------

Converts '.3ds' data files to '.itd' IRIT data files.

Command line options

--------------------

3ds2Irit [-m] [-c ClrScale] [-o OutName] [-b] [-z] 3DSFile

-m: More information flag.

-c ClrScale:  Scaling the color values (intensity control).

-o OutName: Name of output file. By default the output goes to

stdout.

-b: If set, generates a binary IRIT data file that holds the

strokes.  Otherwise, an IRIT text file will be created.

-z: Print version number and current defaults.

Usage

-----

3ds2irit converts Autocad's 3DS data files into IRIT data files.  The

current version provides only partial support, mainly due to lack of

documentation examples on the dxf format and the convoluted way

freeform surfaces are saved.

Example:

3ds2irit file.3ds > file.itd

DAT2BIN - Data To Binary Data file filter

-----------------------------------------

Command line options

--------------------

dat2bin [-t] [-z] {[-c QuantVal]} DFiles

-t: Dumps data to stdout as text instead of binary.

-z: Print version number and current defaults.

-c: Optional option that is available only if compressed binary files

are supported.  Dumps data to stdout as compressed binary file

with a quanitization level of QuantVal.

Usage

-----

The user may sometimes wish to convert .itd data files into a binary form,

for example, for fast loading of files with large geometry. Binary files can

be somewhat larger and are unreadable in editors but are much faster to load.

A binary file must have a '.ibd' file type.

Example:

dat2bin b58polys.itd > b58polys.ibd

dat2bin -t b58polys.ibd | more

The above converts a text file b58polys.itd into a binary file b58polys.ibd

and shows the content of the binary file by converting it back to text. At

this time data through pipes must be in text. That is, the following is

illegal:

dat2bin b58polys.itd | xglmdrvs -

It should be remembered that the binary format is not documented and

it might change in the future. Moreover, it is machine dependent and can

very well may be unreadable between different platforms.

DAT2IRIT - Data To IRIT file filter

-----------------------------------

Converts '.itd' and '.ibd' data files to '.irt' IRIT scripts.

Optionally, if compressed binary files are supported, also handle

'.icd' compressed data files.

Command line options

--------------------

dat2irit [-z] DFiles

-z: Print version number and current defaults.

Usage

-----

Users may sometimes wish to convert .itd data files into a form that

can be fed back to IRIT - a '.irt' file. This filter does exactly that.

Example:

dat2irit b58.itd > b58-new.irt

DXF2IRIT - DXF (Autocad) To IRIT filter

---------------------------------------

Converts Autocad's, DXF data files into IRIT data files.

Command line options

--------------------

dxf2irit [-m] [-f] [-o OutName] [-z] DXFFile

-m: Provides some more information on the data file(s) parsed.

-f: Coerces floating end conditions to constructed freeform surfaces.

Default is open end conditions.

-o OutName: Name of output file. By default the output goes to

stdout.

-z: Prints version number and current defaults.

Usage

-----

dxf2irit converts Autocad's DXF data files into IRIT data files.

The current version provides only partial support for the conversion of

freeform surfaces, mainly due to lack of documentation examples on the

dxf format and the convoluted way freeform surfaces are saved.

Example:

dxf2irit file.dxf > file.itd

IGS2IRIT - IGES Data To IRIT file filter

----------------------------------------

Converts '.igs' data files to '.itd' IRIT data files.

Command line options

--------------------

IGS2Irit [-m] [-M] [-c] [-a] [-s] [-o OutName] [-b] [-z] IGSFile

-m: More information flag.

-M: Even more information flag - dumps all parsed entities.

-c: Clips trimmed surfaces to the minimal domain as prescribed by the

trimming curves.

-a: Dumps all.  Without this flag setting, only top level objects,

that are referenced by no other object, will be dumped out.

-s: Dumps surfaces only.  When set only (trimmed) surfaces are dumped.

-o OutName: Name of output file. By default the output goes to

stdout.

-b: If set, generates a binary IRIT data file that holds the

strokes.  Otherwise, an IRIT text file will be created.

-z: Print version number and current defaults.

Usage

-----

igs2irit converts IGES data files into IRIT data files.

Example:

igs2irit file.igs > file.itd

IRIT2DXF - IRIT To DXF (Autocad) filter

---------------------------------------

Converts IRIT data files into Autocad's, DXF data files.

Command line options

--------------------

irit2dxf [-s Scale] [-t Tx Ty Tz] [-i] [-f] [-F PolyOpti FineNess]

[-4] [-o OutName] [-T] [-a AnimTime] [-z] DFiles

-s Scale: Global scaling factor of the converted geometry.

-t Tx Ty Tz: a Vector of size three of translation factors along

the X, Y, and Z axes.

-i: Shows internal edges as well.

-f: Dumps freeforms as converted polygonal geometry.

-F PolyOpti FineNess: Optimality of polygonal approximation of

surfaces. See the variable POLY_APPROX_OPT for the meaning of

FineNess. See also -4.

-4: Forces four polygons per almost flat region in the surface to

polygon conversion. Otherwise two polygons only.

-o OutName: Name of output file. By default the output goes to

stdout.

-T: Talkative mode. Prints processing information.

-a AnimTime: If the data contains animation curves, evaluate and

process the scene at time AnimTime.

-z: Prints version number and current defaults.

Usage

-----

irit2dxf converts IRIT data files into Autocad's DXF data files.  The

current version provides only partial support for the direct conversion of

freeform surfaces, mainly due to lack of documentation examples on the

dxf format and the convoluted way freeform surfaces are saved.  Nonetheless,

freeform surfaces can be converted into polygons using the '-f' flag.

Example:

irit2dxf -z -t 1 2 3 -F 0 20 -4 -o file.dxf file.itd

IRIT2Hgl - IRIT To HPGL filter

------------------------------

Converts IRIT geometry into the HL Graphics Language used by HP's plotters.

Command line options

--------------------

irit2hgl [-t XTrans YTrans] [-I #UIso[:#VIso[:#WIso]]]

[-f PolyOpti SampTol] [-F PolyOpti FineNess] [-M] [-G] [-T]

[-a AnimTime] [-i] [-o OutName] [-z] DFiles

-t XTrans YTrans: X and Y translation. of the image. Default is (0, 0).

-I #UIso[:#VIso[:#WIso]]: Specifies the number of isolines per

surface/trivariate, per direction. If #VIso is not specified,

#UIso is used for #VIso as well and so no.

-f PolyOpti SampTol: Controls the method used to approximate curves

into polylines. If PolyOpti == 0, equally spaced intervals are

used.  For PolyOpti == 1, SampTol (real number) specifies the

maximal allowed deviation tolerance of the piecewise linear

approximation from the original curve.

Default is 0 64 (uniform sampling with 64 samples).

-F PolygonOpti FineNess: Optimality of polygonal approximation of

surfaces. See the variable POLY_APPROX_OPT for the meaning of

FineNess. See also -4. This enforces the dump of freefrom geometry

as polygons.

-M: Dumps the control mesh/polygon as well.

-G: Dumps the freeform geometry.

-T: Talkative mode. Prints processing information.

-a AnimTime: If the data contains animation curves, evaluate and

process the scene at time AnimTime.

-i: Internal edges (created by IRIT) - default is not to

display them, and this option will force their display.

-o OutName: Name of output file. By default the name of the first data

file from DFiles list is used.  See below on the output files.

-z: Prints version number and current defaults.

Usage

-----

Irit2Hgl converts freeform surfaces and polygons into polylines in a format

that can be used by HPGL.

Example:

irit2Hgl -M -f 0 16 saddle.itd > saddle.hgl

However, one can overwrite the viewing matrix by appending a new matrix

in the end of the command line, created by the display devices:

x11drvs b58.itd

irit2Hgl -M -f 0 16 b58.itd irit.imd > saddle.hgl

where irit.imd is the viewing matrix created by x11drvs.

Irit2IGS - IRIT To IGES filter

------------------------------

Converts IRIT data files into IGES/IGS data files.

Command line options

--------------------

Irit2igs [-m] [-o OutName] [-t AnimTime] [-u] [-z] IritFile

-m: More information flag.

-o OutName: Name of output file. By default the output goes to

stdout.

-t AnimTime: If has animation data, time of dump.

-u: Forces a unit transformation matrix.

-z: Prints version number and current defaults.

Usage

-----

Irit2IGS converts IRIT data files into IGES data files.

Example:

Irit2IGS -u -o file.igs file.itd

IRIT2IV - {\em IRIT} To SGI's Inventor filter

---------------------------------------------

IV is the format used by the Inventor modeling/rendering package from SGI.

Command line options

--------------------

irit2iv [-l] [-4] [-P] [-F PolyOpti FineNess] [-f PolyOpti SampTol]

[-T] [-t AnimTime] [-z] DFiles

-l: Linear - forces linear (degree two) surfaces to be approximated

by a single polygon along their linear direction.

Although most of the time linear direction can be represented exactly

using a single polygon, even a bilinear surface can have a freeform

shape (saddle-like) that is not representable using a single polygon.

Note that although this option will better emulate the surface shape,

it will create unnecessary polygons in cases where one is enough.

-4: Four - Generates four polygons per flat patch. Default is 2.

-P: Polygonize freeform shapes. Default is to leave freeform curves

and surfaces as is.

-F PolyOpti FineNess: Optimality of polygonal approximation of

surfaces. See the variable POLY_APPROX_OPT for the meaning of

FineNess. See also -4.

-f PolyOpti SampTol: Controls the method used to approximate curves

into polylines. If PolyOpti == 0, equally spaced intervals are

used.  For PolyOpti == 1, SampTol (real number) specifies the

maximal allowed deviation tolerance of the piecewise linear

approximation from the original curve.

Default is 0 64 (uniform sampling with 64 samples).

-T: Talkative mode. Prints processing information.

-t AnimTime: If the data contains animation curves, evaluate and

process the scene at time AnimTime.

-z: Prints version number and current defaults.

Usage

-----

Irit2Iv converts freeform surfaces and polygons into polygons and

saved in iv Inventor's ASCII file format.

Example:

irit2iv solid1.itd > solid1.iv

Surfaces are converted to polygons with fineness control:

irit2iv -F 0 16 - view.imd < saddle.itd > saddle.iv

Note the use of '-' for stdin.

IRIT2NFF - {\em IRIT} To NFF filter

-----------------------------------

Command line options

--------------------

irit2nff [-l] [-4] [-c] [-F PolyOpti FineNess] [-o OutName] [-T]

[-t AnimTime] [-g] [-z] DFiles

-l: Linear - forces linear (degree two) surfaces to be approximated

by a single polygon along their linear direction.

Although most of the time linear direction can be represented exactly

using a single polygon, even a bilinear surface can have a freeform

shape (saddle-like) that is not representable using a single polygon.

Note that although this option will better emulate the surface shape,

it will create unnecessary polygons in cases where one is enough.

-4: Four - Generates four polygons per flat patch. Default is 2.

-c: Output files should be filtered by cpp. When set, the usually

huge geometry file is separated from the main nff file that contains

the surface properties and view parameters. By default all data,

including the geometry, are saved into a single file with type extension

'.nff'. Use of '-c' will pull out all the geometry into a file with

the same name but a '.geom' extension, which will be included using the

'#include' command. The '.nff' file should, in that case, be

preprocessed using cpp before being piped into the nff renderer.

-F PolyOpti FineNess: Optimality of polygonal approximation of

surfaces. See the variable POLY_APPROX_OPT for the meaning of

FineNess. See also -4.

-o OutName: Name of output file. By default the name of the first data

file from the DFiles list is used.  See below on the output files.

-T: Talkative mode. Prints processing information.

-t AnimTime: If the data contains animation curves, evaluate and

process the scene at time AnimTime.

-g: Generates the geometry file only. See below.

-z: Prints version number and current defaults.

Usage

-----

Irit2Nff converts freeform surfaces into polygons in a format that can

be used by an NFF renderer. Usually, one file is created with the '.nff' type

extension. Since the number of polygons can be extremely large, a '-c'

option is provided, which separates the geometry from the surface

properties and view specification, but requires preprocessing by cpp.

The geometry is isolated in a file with the extension '.geom' and included

(via '#include') in the main '.nff' file. The latter holds the surface

properties for all the geometry as well as the viewing specification.

This allows for the changing of the shading or viewing properties while

editing small ('.nff') files.

If '-g' is specified, only the '.geom' file is created, preserving the

current '.nff' file. The '-g' flag can be specified only with '-c'.

In practice, it may be useful to create a low resolution approximation

of the model, change viewing/shading parameters in the '.nff' file until

a good view and/or surface quality is found, and then run Irit2Nff once more

to create a high resolution approximation of the geometry using '-g'.

Example:

irit2nff -c -l -F 0 8 b58.itd

creates b58.nff and b58.geom with low resolution (FineNess of 5).

Once done with parameter setting, a fine approximation of the model can

be created with:

irit2nff -c -l -g -F 0 64 b58.itd

which will only recreate b58.geom (because of the -g option).

One can overwrite the viewing matrix by appending a new matrix in the

end of the command line, created by a display device:

xgldrvs b58.itd

irit2nff -l -F 0 32 b58.itd irit.imd

where irit.imd is the viewing matrix created by xgldrvs.

Advanced Usage

--------------

One can specify surface qualities for individual surfaces of a model.

Several such attributes are supported by Irit2Nff and can be set within

IRIT. See also the ATTRIB IRIT command.

If a certain surface should be finer/coarser than the rest of the

scene, one can set a "resolution" attribute which specifies the

relative FineNess resolution of this specific surface. Further,

"u_resolution" and "v_resolution" might be similarly used to set

relative resolution for the u or v direction only.

The "crv_resolution" attribute controls the relative fineness of curves

as polylines. The "num_of_isolines" attribute controls the relative number

of isoparametric curves.

Example:

attrib( srf1, "resolution", 2 );

will force srf1 to have twice the default resolution, as set via the '-f'

flag.

Almost flat patches are converted to polygons. The rectangle can be

converted into two polygons (by subdividing along one of its diagonals) or

into four by introducing a new point at the center of the patch. This

behavior is controlled by the '-4' flag, but can be overwritten for

individual surfaces by setting a "twoperflat" or a "fourperflat" attribute.

NFF specific properties are controlled via the following attributes:

"kd", "ks", "shine", "trans", "index". Refer to the NFF manual for detail.

Example:

attrib( srf1, "kd", 0.3 );

attrib( srf1, "shine", 30 );

Surface color is controlled on two levels. If the object has an RGB

attribute, it is used. Otherwise, a color, as set via the IRIT COLOR

command, is used if set.

Example:

attrib( tankBody, "rgb", "244,164,96" );

Irit2Off - IRIT To OFF filter

-----------------------------

Converts IRIT data files into OFF data files.

Command line options

--------------------

Irit2Off [-l] [-4] [-n] [-F PolyOpti FineNess] [-E VrtxEps] [-o OutName]

[-m] [-z] DFiles

-l: Linear - forces linear (degree two) surfaces to be approximated

by a single polygon along their linear direction.

Although most of the time linear direction can be represented exactly

using a single polygon, even a bilinear surface can have a freeform

shape (saddle-like) that is not representable using a single polygon.

Note that although this option will better emulate the surface shape,

it will create unnecessary polygons in cases where one is enough.

-4: Four - Generates four polygons per flat patch. Default is 2.

-n: Vertex Normals - Dumps the normals of the vertices with the

coordinates.

-F PolyOpti FineNess: Optimality of polygonal approximation of

surfaces. See the variable POLY_APPROX_OPT for the meaning of

FineNess. See also -4.

-E VrtxEps: Epsilon to consider two vertices same.

-o OutName: Name of output file. By default the output goes to

stdout.

-m: More information flag.

-z: Prints version number and current defaults.

Usage

-----

Irit2Off converts IRIT data files into Geom View OFF data files.

Example:

Irit2Off -m -o file.off file.itd

IRIT2PLG - IRIT To PLG (REND386) filter

---------------------------------------

PLG is the format used by the rend386 real time renderer for the IBM PC.

Command line options

--------------------

irit2plg [-l] [-4] [-F PolyOpti FineNess] [-T] [-z] DFiles

-l: Linear - forces linear (degree two) surfaces to be approximated

by a single polygon along their linear direction.

Although most of the time linear direction can be represented exactly

using a single polygon, even a bilinear surface can have a freeform

shape (saddle-like) that is not representable using a single polygon.

Note that although this option will better emulate the surface shape,

it will create unnecessary polygons in cases where one is enough.

-4: Four - Generates four polygons per flat patch. Default is 2.

-F PolyOpti FineNess: Optimality of polygonal approximation of

surfaces. See the variable POLY_APPROX_OPT for the meaning of

FineNess. See also -4.

-T: Talkative mode. Prints processing information.

-z: Prints version number and current defaults.

Usage

-----

Irit2Plg converts freeform surfaces and polygons into polygons in a

format that can be used by the REND386 renderer.

Example:

irit2plg solid1.itd > solid1.plg

Surfaces are converted to polygons with fineness control:

irit2plg -F 0 16 - view.imd < saddle.itd > saddle.plg

Note the use of '-' for stdin.

IRIT2POV - IRIT To POVRAY raytracer filter

------------------------------------------

Command line options

--------------------

irit2pov [-l] [-4] [-C] [-F PolyOpti FineNess] [-f PolyOpti SampTol]

[-o OutName] [-g] [-p Zmin Zmax] [-P] [-M] [-T] [-t AnimTime]

[-I #UIso[:#VIso[:#WIso]]] [-s ObjSeq#] [-i Includes] [-z] DFiles

-l: Linear - forces linear (degree two) surfaces to be approximated

by a single polygon along their linear direction.

Although most of the time linear direction can be represented exactly

using a single polygon, even a bilinear surface can have a freeform

shape (saddle-like) that is not representable using a single polygon.

Note that although this option will better emulate the surface shape,

it will create unnecessary polygons in cases where one is enough.

-4: Four - Generates four polygons per flat patch. Default is 2.

-C: Constructs bicubic Bezier patches whenever possible as POVRAY

supports this type of surface.  Polynomial Bezier surfaces of

orders up to and including bicubic (order 4, degree 3) are degree

raised to bicubic.  Piecewise polynomials B-spline surfaces are

split into Bezier patches.  Higher order surfaces and rational

surfaces are always converted into polygons.

-F PolygonOpti FineNess: Optimality of polygonal approximation of

surfaces. See the variable POLY_APPROX_OPT for the meaning of

FineNess. See also -4, -C, and -l.

-f PolyOpti SampTol: Controls the method used to approximate curves

into polylines. If PolyOpti == 0, equally spaced intervals are

used.  For PolyOpti == 1, SampTol (real number) specifies the

maximal allowed deviation tolerance of the piecewise linear

approximation from the original curve.

Default is 0 64 (uniform sampling with 64 samples).

-o OutName: Name of output file. By default the name of the first data

file from the DFiles list is used.  See below on the output

files.

-g: Generates the geometry file only. See below.

-p Zmin Zmax: Sets the ratios between the depth cue and the width of

the dumped polylines. See also -P. Closer lines will be drawn

wider.

-P: Forces dumping polygons as polylines with thickness controlled

by -p.

-M: If -P (see -P and -p), then convert the control mesh/polygon

to polylines which are represented as a sequence of truncated

cones.

-T: Talkative mode. Prints processing information.

-t AnimTime: If the data contains animation curves, evaluate and

process the scene at time AnimTime.

-I #UIso[:#VIso[:#WIso]]: Specifies the number of isolines per

surface/trivariate, per direction. If #VIso or #WIso is not

specified, #UIso is used for #VIso etc.

-s ObjSeq#: Sets object sequence number if there is no object name.

Default 1.

-i Includes: Expands the comma's separated list of POVRAY include

file names into POVRAY include commands at the beginning of the

created POVRAY output file.

-z: Prints version number and current defaults.

Usage

-----

Irit2pov converts freeform surfaces into polygons in a format that can

be used by the POVRAY ray tracing program.

Two files are created, one with a '.geom' extension and

one with a '.pov' extension. Since the number of polygons can be extremely

large, the geometry is isolated in the '.geom' file and is included

(via '#include') in the main '.pov' file. The latter holds the surface

properties for all the geometry as well as viewing and POVRAY specific

commands. This allows for the changing of the shading or the viewing

properties while editing small ('.pov') files.

If '-g' is specified, only the '.geom' file is created, preserving the

current, possibly manually modified, '.pov' file.

In practice, it may be useful to create a low resolution approximation

of the model, change the viewing/shading parameters in the '.pov' file until

a good view and/or surface quality is found, and then run Irit2pov once more

to create a high resolution approximation of the geometry using '-g'.

Example:

irit2pov -l -F 0 5 b58.itd

creates b58.pov and b58.geom with low resolution (FineNess of 5).

At such low resolution it may very well happen that triangles will have

normals "over the edge" since a single polygon may approximate a highly

curved surface.

One can ray trace this scene using a command similar to:

POVRAY -Q0 +Ib58

Once done with a parameter setting for POVRAY, a fine approximation of the

model can be created with:

irit2pov -l -g -F 0 64 b58.itd

which will only recreate b58.geom (because of the -g option).

Interesting effects can be created using the depth cue support and polyline

conversion of irit2pov. For example,

irit2pov -P -p -0.0 0.5 solid1.itd

will dump solid1 as a set of polylines (represented as truncated cones in

POVRAY) with varying thickness according to the z depth. Another example

is

irit2pov -P -p -0.1 1.0 saddle.itd

which dumps the isolines extracted from the saddle surface with varying

thickness.

Each time a data file is saved in IRIT, it can be saved with the

viewing matrix of the last INTERACT by saving the VIEW_MAT object as well.

I.e.:

save( "b58", b58 );

However, one can overwrite the viewing matrix by appending a new matrix

in the end of the command line, created by the display devices:

xglmdrvs b58.itd                        // Also creates irit.imd

irit2pov -l -F 0 16 b58.itd irit.imd

where irit.imd is the viewing matrix created by xglmdrvs. The output name,

by default, is the last input file name, so you might want to provide an

explicit name with the -o flag.

Advanced Usage

--------------

One can specify surface qualities for individual surfaces of a model.

Several such attributes are supported by Irit2pov and can be set within

IRIT. See also the ATTRIB IRIT command.

If a certain surface should be finer/coarser than the rest of the

scene, one can set a "resolution" attribute which specifies the

relative FineNess resolution of this specific surface. Further,

"u_resolution" and "v_resolution" might be similarly used to set

relative resolution for the u or v direction only.

The "crv_resolution" attribute controls the relative fineness of curves

as polylines. The "num_of_isolines" attribute controls the relative number

of isoparametric curves.

Example:

attrib( srf1, "resolution", 2 );

will force srf1 to have twice the default resolution as set via the '-f'

flag.

Almost flat patches are converted to polygons. The rectangle can be

converted into two polygons (by subdividing along one of its diagonals) or

into four by introducing a new point at the patch center. This behavior is

controlled by the '-4' flag, but can be overwritten for individual surfaces

by setting "twoperflat" or "fourperflat".

POVRAY also supports bicubic Bezier patches and the '-C' option of

irit2pov supports that.  In such a case, the resolution that is requested

from POVRAY to polygonize these patches approximately follows the resolution

as selected via the '-F' flag of irit2pov.  Nevertheless, one can

override the requested resolution via the "steps", "u_steps", and "v_steps"

attributes to irit2pov data files that are transferred directly to

POVRAY's bicubic Bezier patches.  The "steps" attributes sets both

"u_steps" and "v_steps".

While the program has a default for lighting which is a point light

source at (1, 2, 10), one can overwrite this default.  A POINT_TYPE

object with LIGHT_SOURCE attribute in the data stream denotes a

light source.  If irit2pov detects one or more light sources in the

input stream, the default light sources are not created.  A point

light source can be colored, when an RGB attribute will set its

color.

Example:

l1 - point( 5, 5, 5 );

attrib( l1, "rgb", "255, 0, 0" );

creates a red light source at (5, 5, 5).

POVRAY specific properties are controlled via the following attributes:

"ambient", "diffuse", "brilliance", "phong", "phong_size", "specular",

"roughness", "metallic", "reflection", "crand", "conserve_energy",

"irid", "ior", "caustics", "dispersion", "dispersion_samples",

"fade_distance", "fade_power", "fade_color".

One can prescribe a whole property block of POV attributes via the

"texture", "pigment", "finish", "halo", and "normal".  The values of this

attributes must be strings as they are copied verbatim.  Refer to POVRAY's

manual for their exact meaning.

Example:

attrib( legs, "ambient", 0.1 );

attrib( pot, "matallic", "" );

attrib( table, "ior", 1.4 );

attrib( bird, "finish", "ambient 0 diffuse 1 specular 1" );

Surface color is controlled on two levels. If the object has an RGB

attribute, it is used. Otherwise a color as set via the IRIT COLOR

command is used, if set.

Example:

attrib( tankBody, "rgb", "244,164,96" );

Transparency is controlled via the "transp" attribute, with values

between zero and one.

Example:

attrib( Glass, "transp", 0.9 );

IRIT2PS - IRIT To PS filter

---------------------------

Command line options

--------------------

irit2ps [-l] [-4] [-s Size] [-I #UIso[:#VIso[:#WIso]]] [-F PolyOpti FineNess]

[-f PolyOpti SampTol] [-M] [-G] [-P] [-W LineWidth]

[-w WidenLen WidenWidth] [-b R G B] [-B X1 Y1 X2 Y2] [-c] [-C]

[-T] [-t AnimTime] [-N FontName] [-i] [-o OutName] [-d [Zmin Zmax]]

[-D [Zmin Zmax]] [-p PtType PtSize] [-u] [-z] DFiles

-l: Linear - forces linear (degree two) surfaces to be approximated

by a single polygon along their linear direction.

Although most of the time linear direction can be represented exactly

using a single polygon, even a bilinear surface can have a freeform

shape (saddle-like) that is not representable using a single polygon.

Note that although this option will better emulate the surface shape,

it will create unnecessary polygons in cases where one is enough.

-4: Four - Generates four polygons per flat patch. Default is 2.

-s Size: Controls the size of the postscript output in inches.

Default is to fill the entire screen.

-I #UIso[:#VIso[:#WIso]]: Specifies the number of isolines per

surface/trivariate, per direction. If #VIso or #WIso is not

specified, #UIso is used for #VIso etc.

-F PolygonOpti FineNess: Optimality of polygonal approximation of

surfaces. See the variable POLY_APPROX_OPT for the meaning of

FineNess. See also -4.

-f PolyOpti SampTol: Controls the method used to approximate curves

into polylines. If PolyOpti == 0, equally spaced intervals are

used.  For PolyOpti == 1, SampTol (real number) specifies the

maximal allowed deviation tolerance of the piecewise linear

approximation from the original curve.

Default is 0 64 (uniform sampling with 64 samples).

-M: Dumps the control mesh/polygon as well.

-G: Dumps the curve/surface (as freeform geometry). Default. See -I,

-C, -f for control on polyline approximation.

-P: Dumps the curve/surface (as polygons). See -F, -l, -4 for control

on polygonal approximation.

-W #LineWidth: Sets the line drawing width in inches. Default is

as thin as possible. This option will overwrite only those objects

that do not have a "width" attribute. See also -d. If LineWidth

is negative, its absolute value is used to scale the current width

of the object if it has one, or the default width otherwise.

-w WidenLen WidenWidth: Widens the end points of polylines if they

should be made wider, and if so, to what width.

-b R G B: Sets a colored background. RGB are three integers prescribing

the Red, Green, and Blue coefficients. If there is no -c (i.e. a gray

level drawing), this color is converted to a gray level using RGB to

T.V. Y(IQ) channel conversion.

-B X1 Y1 X2 Y2: Clips the drawing area outsize the bounding box

from (X1, Y1) to (X2, Y2).

-c: Creates a color postscript file.

-C: Curve mode. Dumps freeform curves and surfaces as cubic

Bezier curves. Higher order curves and surfaces and/or rationals are

approximated by cubic Bezier curves. This option generates data

files that are roughly a third of piecewise linear postscript files

(by disabling this feature, -C-), but it takes a longer time to compute.

-T: Talkative mode. Prints processing information.

-t AnimTime: If the data contains animation curves, evaluate and

process the scene at time AnimTime.

-N FontName: Sets the font to use when dumping text out of string

objects.

-i: Internal edges (created by IRIT) - the default is not to

display them, and this option will force displaying them as well.

-o OutName: Name of output file. Default is stdout.

-d [Zmin Zmax]: Sets the ratios between the depth cue and the width of

the dumped data. See also -W, -p. Closer lines/points will be drawn

wider/larger. Zmin and Zmax are optional. The object's bounding

box is otherwise computed and used.

-D [Zmin Zmax]: Same as -d, but depth cue the color or gray scale

instead of width. You might need to consider the sorting option

of the illustrt tool (-s of illustrt) for proper drawings.

Only one of -d and -D can be used.

-p PtType PtSize: Specifies the way points are drawn.

PtType can be one of H, F, C for Hollow circle, Full Circle, or

Cross. PtSize specifies the size of the point to be drawn, in inches.

Vectors will also be drawn as points, but with an additional thin

line to the origin. See also -d.

-u: Forces a unit matrix transformation, i.e. no transformation.

-z: Prints version number and current defaults.

Usage

-----

Irit2Ps converts freeform surfaces and polygons into a postscript file.

Example:

irit2ps solid1.itd > solid1.ps

Surfaces are converted to polygons with fineness control:

irit2ps -f 0 32 -c -W 0.01 saddle.itd > saddle.ps

creates a postscript file for the saddle model, in color, and with

lines 0.01 inch thick.

Advanced Usage

--------------

One can specify several attributes that affect the way the postscript

file is generated. The attributes can be generated within IRIT.

See also the ATTRIB IRIT command.

If a certain object should be thinner or thicker than the rest of the scene,

one can set a "width" attribute which specifies the line width in inches of

this specific object.

Example:

attrib( srf1, "width", 0.02 );

will force srf1 to have this width, instead of the default as set via the

'-W' flag.

If a (closed) object, a polygon, for example, needs to be filled, a "fill"

attribute should be set.

Example:

attrib( poly, "fill", true );

will fill poly.

If an object, a polygon, for example, needs to be painted/filled in a gray

level instead of black, a "gray" attribute should be set, with a value equal

to the gray level desired.

Example:

attrib( poly, "gray", 0.5 );

will draw/fill poly with %50 gray.

Dotted or dashed line effects can be created using a "dash" attribute which

is a direct postScript dash string. A simple form of this string is "[a b]"

in which a is the drawing portion (black) in inches, followed by b inches

of white space. See the postScript manual for more about the format of this

string. Here is an example for a dotted-dash line.

attrib( poly, "dash", "[0.006 0.0015 0.001 0.0015] 0" );

Surface color is controlled (for color postscript only - see -c) on two

levels. If the object has an RGB attribute, it is used. Otherwise, a color as

set via the IRIT COLOR command is used.

Example:

attrib( Ball, "rgb", "255,0,0" );

An object can be drawn as "tubes" instead of full lines. The ratio

between the inner and the outer radii of the tube is provided as the

TUBULAR attribute:

attrib( final, "tubular", 0.7 );

The depth cueing option of irit2ps could be disabled for individual

objects by placing an integer attribute "DepthCue" with the FALSE value:

attrib( final, "DepthCue", FALSE );

The "resolution" attribute controls the relative fineness of polygonal

approximation of surfaces, and "u_resolution" and "v_resolution" similarly

control this relative fineness along one parametric direction only.

The "crv_resolution" attribute controls the relative fineness of curves

as polylines. The "num_of_isolines" attribute controls the relative number

of isoparametric curves.

A string object can be dumped as text of a selected PS font (See -N).  The

string position is set via a "StrPos" vector attribute (default to the

origin), and "StrScale" real attribute to control the string height in

world unit (default to 0.1).  Text will always be dumped horizontally.

Example:

Text = "Some text";

attrib( Text, "StrPos", vector( 1, 2, 3 ) );

attrib( Text, "StrScale", 0.1 );

Will print the Text "Some text" at location (1, 2, 3).  The text

height be be 0.1.

IRIT2RAY - IRIT To RAYSHADE filter

----------------------------------

Command line options

--------------------

irit2ray [-l] [-4] [-G GridSize] [-F PolyOpti FineNess]

[-f PolyOpti SampTol] [-o OutName] [-g] [-p Zmin Zmax] [-P]

[-M] [-T] [-t AnimTime] [-I #UIso[:#VIso[:#WIso]]] [-s ObjSeq#]

[-z] DFiles

-l: Linear - forces linear (degree two) surfaces to be approximated

by a single polygon along their linear direction.

Although most of the time linear direction can be represented exactly

using a single polygon, even a bilinear surface can have a freeform

shape (saddle-like) that is not representable using a single polygon.

Note that although this option will better emulate the surface shape,

it will create unnecessary polygons in cases where one is enough.

-4: Four - Generates four polygons per flat patch. Default is 2.

-G GridSize: Usually objects are grouped as lists of polygons.

This flag will coerce the usage of the RAYSHADE grid structure,

with GridSize being used as the grid size along the object

bounding box's largest dimension.

-F PolygonOpti FineNess: Optimality of polygonal approximation of

surfaces. See the variable POLY_APPROX_OPT for the meaning of

FineNess. See also -4.

-f PolyOpti SampTol: Controls the method used to approximate curves

into polylines. If PolyOpti == 0, equally spaced intervals are

used.  For PolyOpti == 1, SampTol (real number) specifies the

maximal allowed deviation tolerance of the piecewise linear

approximation from the original curve.

Default is 0 64 (uniform sampling with 64 samples).

-o OutName: Name of output file. By default, the name of the first data

file from the DFiles list is used.  See below on the output

files.

-g: Generates the geometry file only. See below.

-p Zmin Zmax: Sets the ratios between the depth cue and the width of

the dumped polylines. See also -P. Closer lines will be drawn

wider.

-P: Forces dumping polygons as polylines with thickness controlled

by -p.

-M: If -P (see -P and -p), will then convert the control mesh/polygon

to polylines which are represented as a sequence of truncated

cones.

-T: Talkative mode. Prints processing information.

-t AnimTime: If the data contains animation curves, evaluate and

process the scene at time AnimTime.

-I #UIso[:#VIso[:#WIso]]: Specifies the number of isolines per

surface/trivariate, per direction. If #VIso or #WIso is not

specified, #UIso is used for #VIso etc.

-s ObjSeq#: Sets object sequence number if no object name. Default 1.

-z: Prints version number and current defaults.

Usage

-----

Irit2Ray converts freeform surfaces into polygons in a format that can

be used by the RAYSHADE ray tracing program.

Two files are created, one with a '.geom' extension and

one with a '.ray' extension. Since the number of polygons can be extremely

large, the geometry is isolated in the '.geom' file and is included

(via '#include') in the main '.ray' file. The latter holds the surface

properties for all the geometry as well as viewing and RAYSHADE specific

commands. This allows for the changing of the shading or viewing

properties while editing small ('.ray') files.

If '-g' is specified, only the '.geom' file is created, preserving the

current '.ray' file.

In practice, it may be useful to create a low resolution approximation

of the model, change the viewing/shading parameters in the '.ray' file until

a good view and/or surface quality is found, and then run Irit2Ray once more

to create a high resolution approximation of the geometry using '-g'.

Example:

irit2ray -l -F 0 5 b58.itd

creates b58.ray and b58.geom with low resolution (FineNess of 5).

At such low resolution it may very well happen that triangles will have

normals "over the edge" since a single polygon may approximate a highly

curved surface. That will cause RAYSHADE to issue an

"Inconsistent triangle normals" warning. This problem will not arise if

high fineness is used.

One can ray trace this scene using a command similar to:

RAYSHADE -p -W 256 256 b58.ray > b58.rle

Once done with the parameter setting for RAYSHADE, a fine approximation of the

model can be created with:

irit2ray -l -g -F 0 64 b58.itd

which will only recreate b58.geom (because of the -g option).

Interesting effects can be created using the depth cue support and polyline

conversion of irit2ray. For example,

irit2ray -G 5 -P -p -0.0 0.5 solid1.itd

will dump solid1 as a set of polylines (represented as truncated cones in

RAYSHADE) with varying thickness according to the z depth. Another example

is

irit2ray -G 5 -P -p -0.1 1.0 saddle.itd

which dumps the isolines extracted from the saddle surface with varying

thickness.

Each time a data file is saved in IRIT, it can be saved with the

viewing matrix of the last INTERACT by saving the VIEW_MAT object as well.

I.e.:

save( "b58", b58 );

However, one can overwrite the viewing matrix by appending a new matrix

in the end of the command line, created by the display devices:

os2drvs b58.itd                        // Also creates irit.imd

irit2ray -l -F 0 16 b58.itd irit.imd

where irit.imd is the viewing matrix created by os2drvs. The output name,

by default, is the last input file name, so you might want to provide an

explicit name with the -o flag.

Advanced Usage

--------------

One can specify surface qualities for individual surfaces of a model.

Several such attributes are supported by Irit2Ray and can be set within

IRIT. See also the ATTRIB IRIT command.

If a certain surface should be finer/coarser than the rest of the

scene, one can set a "resolution" attribute which specifies the

relative FineNess resolution of this specific surface. Further,

"u_resolution" and "v_resolution" might be similarly used to set

relative resolution for the u or v direction only.

The "crv_resolution" attribute controls the relative fineness of curves

as polylines. The "num_of_isolines" attribute controls the relative number

of isoparametric curves.

Example:

attrib( srf1, "resolution", 2 );

will force srf1 to have twice the default resolution, as set via the '-f'

flag.

Almost flat patches are converted to polygons. The rectangle can be

converted into two polygons (by subdividing along one of its diagonals) or

into four by introducing a new point at the patch center. This behavior is

controlled by the '-4' flag, but can be overwritten for individual surfaces

bu setting "twoperflat" or "fourperflat".

RAYSHADE specific properties are controlled via the following attributes:

"specpow", "reflect", "transp", "body", "index", and "texture". The value of

these attributes must be strings as it is copied verbatim. Refer to

RAYSHADE's manual for their meaning.

Example:

attrib( legs, "transp", "0.3" );

attrib( legs, "texture", "wood,2" );

attrib( table, "texture", "marble" );

attrib( table, "reflect", "0.5" );

An optional scale can be prescribed to textures. In the above example

wooden legs' (that are also transparent...) texture is selected with

a texture scaling factor of 2.

Surface color is controlled on two levels. If the object has an RGB

attribute, it is used. Otherwise a color as set via the IRIT COLOR

command is used, if set.

Example:

attrib( tankBody, "rgb", "244,164,96" );

IRIT2Scn - IRIT To SCENE (RTrace) filter

----------------------------------------

SCENE is the format used by the RTrace ray tracer. This filter was donated

by Antonio Costa (acc@asterix.inescn.pt), the author of RTrace.

Command line options

--------------------

irit2scn [-l] [-4] [-F PolyOpti FineNess] [-o OutName] [-g] [-T]

[-t AnimTime] [-z] DFiles

-l: Linear - forces linear (degree two) surfaces to be approximated

as a single polygon along their linear direction.

Although most of the time linear direction can be represented exactly

using a single polygon, even a bilinear surface can have a freeform

shape (saddle-like) that is not representable using a single polygon.

Note that although this option will better emulate the surface shape,

it will create unnecessary polygons in cases where one is enough.

-4: Four - Generates four polygons per flat patch.

-F PolyOpti FineNess: Optimality of polygonal approximation of

surfaces. See the variable POLY_APPROX_OPT for the meaning of

FineNess. See also -4.

-o OutName: Name of output file. By default the name of the first data

file from DFiles list is used.  See below on the output files.

-g: Generates the geometry file only. See below.

-T: Talkative mode. Prints processing information.

-t AnimTime: If the data contains animation curves, evaluate and

process the scene at time AnimTime.

-z: Prints version number and current defaults.

Usage

-----

Irit2Scn converts freeform surfaces and polygons into polygons in a format

that can be used by RTrace. Two files are created, one with a '.geom'

extension and one with a '.scn' extension. Since the number of polygons can

be extremely large, the geometry is isolated in the '.geom' file and is

included (via '#include') in the main '.scn' file. The latter holds the

surface properties for all the geometry as well as viewing and RTrace specific

commands. This allows for the changing of the shading or viewing

properties while editing small ('.scn') files.

If '-g' is specified, only the '.geom' file is created, preserving the

current '.scn' file.

In practice, it may be useful to create a low resolution approximation

of the model, adjust the viewing/shading parameters in the '.scn' file

until a good view and/or surface quality is found, and then run Irit2Scn once

more to create a high resolution approximation of the geometry using '-g'.

Example:

irit2scn -l -F 0 8 b58.itd

creates b58.scn and b58.geom with low resolution (FineNess of 5).

One can ray trace this scene after converting the scn file to a sff file,

using scn2sff provided with the RTrace package.

Once done with the parameter setting of RTrace, a fine approximation of the

model can be created with:

irit2scn -l -g -F 0 64 b58.itd

which will only recreate b58.geom (because of the -g option).

One can overwrite the viewing matrix by appending a new matrix

at the end of the command line, created by the display devices:

wntdrvs b58.itd

irit2scn -l -F 0 8 b58.itd irit.imd

where irit.imd is the viewing matrix created by wntdrvs. The output name,

by default, is the last input file name, so you might want to provide an

explicit name with the -o flag.

Advanced Usage

--------------

One can specify surface qualities for individual surfaces of a model.

Several such attributes are supported by Irit2Scn and can be set within

IRIT. See also the ATTRIB IRIT command.

If a certain surface should be finer/coarser than the rest of the

scene, one can set a "resolution" attribute which specifies the

relative FineNess resolution of this specific surface. Further,

"u_resolution" and "v_resolution" might be similarly used to set

relative resolution for the u or v direction only.

The "crv_resolution" attribute controls the relative fineness of curves

as polylines. The "num_of_isolines" attribute controls the relative number

of isoparametric curves.

Example:

attrib( srf1, "resolution", 2 );

will force srf1 to have twice the default resolution, as set via the '-f'

flag.

Almost flat patches are converted to polygons. The patch can be converted

into two polygons (by subdividing along one of its diagonals) or into four

by introducing a new point at the patch center. This behavior is controlled

by the '-4' flag, but can be overwritten for individual surfaces by setting

"twoperflat" or "fourperflat".

RTrace specific properties are controlled via the following attributes:

"SCNrefraction", "SCNtexture", "SCNsurface. Refer to the RTrace manual for

their meaning.

Example:

attrib( srf1, "SCNrefraction", 0.3 );

Surface color is controlled on two levels. If the object has an RGB

attribute, it is used. Otherwise a color as set via IRIT COLOR command

is used, if set.

Example:

attrib( tankBody, "rgb", "244,164,96" );

IRIT2Stl - IRIT To STL filter

-----------------------------

Command line options

--------------------

irit2stl [-l] [-4] [-r] [-F PolyOpti FineNess] [-E VrtxEps] [-s] [-S]

[-o OutName] [-m] [-u] [-z] DFiles

-l: Linear - forces linear (degree two) surfaces to be approximated

by a single polygon along their linear direction.

Although most of the time linear direction can be represented exactly

using a single polygon, even a bilinear surface can have a freeform

shape (saddle-like) that is not representable using a single polygon.

Note that although this option will better emulate the surface shape,

it will create unnecessary polygons in cases where one is enough.

-4: Four - Generates four polygons per flat patch. Default is 2.

-r: Regularize and triangulate the input data if not regularized

and with triangles only to begin with.

-F PolygonOpti FineNess: Optimality of polygonal approximation of

surfaces. See the variable POLY_APPROX_OPT for the meaning of

FineNess. See also -4.

-E VrtxEps:  Tolerance of two adjacent verices to be considered the

same.  Vertices that are considered the same are collapsed to an

identical location.

-s: Dumps each object as a separated "solid" - "endsolid" brackets.

-S: Dumps each object as a separated "solid" - "endsolid" brackets in

a separated stl file, with file name appended with numeric index.

-o OutName: Name of output file. By default the output goes to

stdout.

-m: More information flag.

-u: Forces a unit matrix. That is, input data are not

transformed at all.

-z: Prints version number and current defaults.

Usage

-----

Irit2Stl converts freeform surfaces and polygons into the STL

(Stereolithography) file format.  The STL data should be a closed solid in

general but no such validity check is conducted by irit2stl.

Example:

irit2stl -u solid2.itd > solid2.stl

Irit2Wrl - IRIT To IGES filter

------------------------------

Converts IRIT data files into IGS data files.

Command line options

--------------------

irit2wrl [-l] [-4] [-u] [-F PolyOpti FineNess] [-f PolyOpti SampTol]

[-o OutName] [-T] [-z] DFiles

-l: Linear - forces linear (degree two) surfaces to be approximated

by a single polygon along their linear direction.

Although most of the time linear direction can be represented exactly

using a single polygon, even a bilinear surface can have a freeform

shape (saddle-like) that is not representable using a single polygon.

Note that although this option will better emulate the surface shape,

it will create unnecessary polygons in cases where one is enough.

-4: Four - Generates four polygons per flat patch. Default is 2.

-u: Forces a unit matrix. That is, input data are not

transformed at all.

-F PolyOpti FineNess: Optimality of polygonal approximation of

surfaces. See the variable POLY_APPROX_OPT for the meaning of

FineNess. See also -4.

-f PolyOpti SampTol: Controls the method used to approximate curves

into polylines. If PolyOpti == 0, equally spaced intervals are

used.  For PolyOpti == 1, SampTol (real number) specifies the

maximal allowed deviation tolerance of the piecewise linear

approximation from the original curve.

Default is 0 64 (uniform sampling with 64 samples).

-o OutName: Name of output file. By default the output goes to

stdout.

-T: More talkative/information flag.

-z: Prints version number and current defaults.

Usage

-----

Irit2Wrl converts IRIT data files into Geom View OFF data files.

Example:

Irit2Wrl -m -o file.off file.itd

IRIT2Xfg - IRIT To XFIG filter

------------------------------

Command line options

--------------------

irit2xfg [-s Size] [-t XTrans YTrans] [-I #UIso[:#VIso[:#WIso]]]

[-f PolyOpti SampTol] [-F PolyOpti FineNess] [-M] [-G] [-T]

[-a AnimTime] [-i] [-o OutName] [-z] DFiles

-s Size: Size in inches of the page. Default is 7 inches.

-t XTrans YTrans: X and Y translation. of the image. Default is (0, 0).

-I #UIso[:#VIso]: Specifies the number of isolines per surface, per

direction. If #VIso is not specified, #UIso is used for #VIso as

well.

-f PolyOpti SampTol: Controls the method used to approximate curves

into polylines. If PolyOpti == 0, equally spaced intervals are

used.  For PolyOpti == 1, SampTol (real number) specifies the

maximal allowed deviation tolerance of the piecewise linear

approximation from the original curve.

Default is 0 64 (uniform sampling with 64 samples).

-F PolygonOpti FineNess: Optimality of polygonal approximation of

surfaces. See the variable POLY_APPROX_OPT for the meaning of

FineNess. See also -4. This enforces the dump of freefrom geometry

as polygons.

-M: Dumps the control mesh/polygon as well.

-G: Dumps the freeform geometry.

-T: Talkative mode. Prints processing information.

-a AnimTime: If the data contains animation curves, evaluate and

process the scene at time AnimTime.

-i: Internal edges (created by IRIT) - default is not to

display them, and this option will force their display.

-o OutName: Name of output file. By default, the name of the first data

file from DFiles list is used.  See output files below.

-z: Prints version number and current defaults.

Usage

-----

Irit2Xfg converts freeform surfaces and polygons into polylines in a format

that can be used by XFIG.

Example:

irit2Xfg -T -f 0 16 saddle.itd > saddle.xfg

However, one can overwrite the viewing matrix by appending a new matrix

at the end of the command line, created by the display devices:

x11drvs b58.itd

irit2Xfg -T -f 0 16 b58.itd irit.imd > saddle.xfg

where irit.imd is the viewing matrix created by x11drvs.

OBJ2IRIT - Wavefront OBJ format To IRIT data files

--------------------------------------------------

converts Waverfront's OBJ data files into IRIT data files.

Command line options

--------------------

obj2irit [-m] [-r] [-o OutName] [-z] OBJFile

-m: Provides some more information on the data file(s) parsed.

-r: Reverses all polygons' orientation in generated data.

-o OutName: Name of output file. By default, the output goes to

stdout.

-z: Prints version number and current defaults.

Usage

-----

obj2irit converts Wavefront's OBJ data files into IRIT data files.

The current version provides only partial support for the direct conversion

of freeform surfaces, mainly due to luck of examples of freeform surfaces

in obj format.

Example:

obj2irit -o file.itd file.obj

Off2IRIT - Geom View Off format To IRIT data files

--------------------------------------------------

Converts Geom View's Off data files into IRIT data files.

Command line options

--------------------

Off2irit [-o OutName] [-z] OffFile

-o OutName: Name of output file. By default the output goes to

stdout.

-z: Prints version number and current defaults.

Usage

-----

Off2irit converts Geom View's Off data files into IRIT data files.

Example:

Off2irit - < file.off > file.itd

STL2IRIT - Stl (stereo lithograph) data To IRIT file filter

-----------------------------------------------------------

Converts '.stl' stereolithography data files to '.irt' IRIT scripts.

Both binary and text STL files are supported.

Command line options

--------------------

stl2irit [-b] [-w] [-n] [-o OutName] [-z] STLFile

-b: The stl file is a binary stl.

-w: Perform an endian swap on all read data.  Little vs. Big

Endian is supported for binary STL files only.

-n: Flip orientation of all polygons by flipping their normals.

-o OutName: Name of output file. By default, the output goes to

stdout.

-z: Print version number and current defaults.

Usage

-----

stl2irit converts stereo-lithography STL data files into IRIT data

files.

Example:

stl2irit -o file.itd file.stl

DATAFILE Format

---------------

This section describes the data file format used to exchange data between

IRIT and its accompanying tools.

[OBJECT {ATTRS} OBJNAME

[NUMBER n]

| [POINT x y z]

| [VECTOR x y z]

| [CTLPT POINT_TYPE {w} x y {z}]

| [STRING "a string"]

| [MATRIX m00 ... m03

m10 ... m13

m20 ... m23

m30 ... m33]

;A polyline should be drawn from first point to last. Nothing is drawn

;from last to first (in a closed polyline, last point is equal to first).

| [POLYLINE {ATTRS} #PTS                   ;#PTS = number of points.

[{ATTRS} x y z]

[{ATTRS} x y z]

.

.

.

[{ATTRS} x y z]

]

;Defines a closed planar region. Last point is NOT equal to first,

;and a line from last point to first should be drawn when the boundary

;of the polygon is drawn.

| [POLYGON {ATTRS} #PTS

[{ATTRS} x y z]

[{ATTRS} x y z]

.

.

.

[{ATTRS} x y z]

]

;Defines a "cloud" of points.

| [POINTLIST {ATTRS} #PTS

[{ATTRS} x y z]

[{ATTRS} x y z]

.

.

.

[{ATTRS} x y z]

]

;Defines an instance - a geometric reference (by name, SRF13 below)

;and a transformation matrix to apply to this geoemtry

| [INSTANCE SRF13

m00 ... m03

m10 ... m13

m20 ... m23

m30 ... m33]

;Defines a Bezier curve with #PTS control points. If the curve is

;rational, the rational component is introduced first.

| [CURVE BEZIER {ATTRS} #PTS POINT_TYPE

[{ATTRS} {w} x y z ...]

[{ATTRS} {w} x y z ...]

.

.

.

[{ATTRS} {w} x y z ...]

]

;Defines a Bezier surface with #UPTS * #VPTS control points. If the

;surface is rational, the rational component is introduced first.

;Points are printed row after row (#UPTS per row), #VPTS rows.

| [SURFACE BEZIER {ATTRS} #UPTS #VPTS POINT_TYPE

[{ATTRS} {w} x y z ...]

[{ATTRS} {w} x y z ...]

.

.

.

[{ATTRS} {w} x y z ...]

]

;Defines a Bezier triangular surface with (#PTS + 1) * #PTS / 2 control

;points, of order ORDER. If the surface is rational, the rational

;component is introduced first. Note #PTS holds number of points along

;an edge and is exactly equal to ORDER. Points are printed sequentially.

| [TRISRF BEZIER {ATTRS} #PTS POINT_TYPE

[{ATTRS} {w} x y z ...]

[{ATTRS} {w} x y z ...]

.

.

.

[{ATTRS} {w} x y z ...]

]

;Defines a Bezier trivariate with #UPTS * #VPTS * #WPTS control

;points. If the trivariate is rational, the rational component is

;introduced first. Points are printed row after row (#UPTS per row),

;#VPTS rows, #WPTS layers (depth).

| [TRIVAR BEZIER {ATTRS} #UPTS #VPTS #WPTS POINT_TYPE

[{ATTRS} {w} x y z ...]

[{ATTRS} {w} x y z ...]

.

.

.

[{ATTRS} {w} x y z ...]

]

;Defines a Bezier multivariate of #Dim dimensions (#Dim = 1 for a

;curve, #Dim = 2 for a surface, #Dim = 3 for a trivariate, etc.)

;with (Dim1#PTS * ... * Dim1#PTS) control points. If the multivariate

;is rational, the rational component is introduced first.

| [MULTIVAR BEZIER {ATTRS} #Dim Dim1#PTS ... DimN#PTS POINT_TYPE

[{ATTRS} {w} x y z ...]

[{ATTRS} {w} x y z ...]

.

.

.

[{ATTRS} {w} x y z ...]

]

;Defines a B-spline curve of order ORDER with #PTS control points. If the

;curve is rational, the rational component is introduced first.

;Note that the length of knot vector is equal to #PTS + ORDER.

;If the curve is periodic, KVP prefix the knot vector that has length of

;'Length + Order + Order - 1'.

| [CURVE BSPLINE {ATTRS} #PTS ORDER POINT_TYPE

[KV{P} {ATTRS} kv0 kv1 kv2 ...]                    ;Knot vector

[{ATTRS} {w} x y z ...]

[{ATTRS} {w} x y z ...]

.

.

.

[{ATTRS} {w} x y z ...]

]

;Defines a B-spline surface with #UPTS * #VPTS control points, of order

;UORDER by VORDER. If the surface is rational, the rational component

;is introduced first.

;Points are printed row after row (#UPTS per row), #VPTS rows.

;If the surface is periodic in some direction, KVP prefix the knot vector

;that has length of 'Length + Order + Order - 1'.

| [SURFACE BSPLINE {ATTRS} #UPTS #VPTS UORDER VORDER POINT_TYPE

[KV{P} {ATTRS} kv0 kv1 kv2 ...]                ;U Knot vector

[KV{P} {ATTRS} kv0 kv1 kv2 ...]                ;V Knot vector

[{ATTRS} {w} x y z ...]

[{ATTRS} {w} x y z ...]

.

.

.

[{ATTRS} {w} x y z ...]

]

;Defines a B-spline triangular surface with (#PTS + 1) * #PTS / 2 control

;points, of order ORDER. If the surface is rational, the rational

;component is introduced first.

;Points are printed sequentially.

| [TRISRF BSPLINE {ATTRS} #PTS ORDER POINT_TYPE

[KV {ATTRS} kv0 kv1 kv2 ...]                   ;Knot vector

[{ATTRS} {w} x y z ...]

[{ATTRS} {w} x y z ...]

.

.

.

[{ATTRS} {w} x y z ...]

]

;Defines a B-spline trivariate with #UPTS * #VPTS * #WPTS control

;points. If the trivariate is rational, the rational component is

;introduced first. Points are printed row after row (#UPTS per row),

;#VPTS rows, #WPTS layers (depth).

;If trivariate is periodic in some direction, KVP prefix the knot vector

;that has length of 'Length + Order + Order - 1'.

| [TRIVAR BSPLINE {ATTRS} #UPTS #VPTS #WPTS UORDER VORDER WORDER POINT_TYPE

[KV{P} {ATTRS} kv0 kv1 kv2 ...]                ;U Knot vector

[KV{P} {ATTRS} kv0 kv1 kv2 ...]                ;V Knot vector

[KV{P} {ATTRS} kv0 kv1 kv2 ...]                ;W Knot vector

[{ATTRS} {w} x y z ...]

[{ATTRS} {w} x y z ...]

.

.

.

[{ATTRS} {w} x y z ...]

]

;Defines a B-spline multivariate of #Dim dimensions (#Dim = 1 for a

;curve, #Dim = 2 for a surface, #Dim = 3 for a trivariate, etc.)

;with (Dim1#PTS * ... * Dim1#PTS) control points. If the multivariate

;is rational, the rational component is introduced first.

| [MULTIVAR BSPLINE {ATTRS} #Dim Dim1#PTS ... DimN#PTS POINT_TYPE

[KV{P} {ATTRS} kv0 kv1 kv2 ...]                ;Dim1 Knot vector

.

.

.

[KV{P} {ATTRS} kv0 kv1 kv2 ...]                ;DimN Knot vector

[{ATTRS} {w} x y z ...]

[{ATTRS} {w} x y z ...]

.

.

.

[{ATTRS} {w} x y z ...]

]

;Defines a trimmed surface. Encapsulates a surface (can be either a

;B-spline or a Bezier surface) and prescribes its trimming curves.

;There can be an arbitrary number of trimming curves (either Bezier

; or B-spline). Each trimming curve contains an arbitrary number of

;trimming curve segments, while each trimming curve segment contains

;a parameteric representation optionally followed by a Euclidean

;representation of the trimming curve segment.

| [TRIMSRF

[SURFACE ...

]

[TRIMCRV

[TRIMCRVSEG

[CURVE ...

]

]

.

.

.

[TRIMCRVSEG

[CURVE ...

]

]

]

.

.

.

[TRIMCRV

[TRIMCRVSEG

[CURVE ...

]

]

.

.

.

[TRIMCRVSEG

[CURVE ...

]

]

]

]

;Defines a model. A model contains a set of (trimmed) surfaces along

;with a set of trimming curves that are shared by (at most) two

;surfaces each.

;The trimming curves must form closed loops in each surface.

| [MODEL #TrimSrfs #TrimSegs

;A surface in the model holds a regular surface and a set of

;closed loops that defines the trimming loops of the surface.

[MDLTSRF #Loops                        ;Number of trimming loops

[SURFACE ...

]

;Each trimming loop is a list of trimming curve segments.

;If the index is negative, it denotes the traversal of the

;curve in reverse order.

[MDLLOOP trim seg's indices]        ;Negative index - reversed

.

.

.

[MDLLOOP trim seg's indices]        ;Negative index - reversed

]

.

.

.

[MDLTSRF #Loops                         ;Number of trimming loops

[SURFACE ...

]

[MDLLOOP trim seg's indices]        ;Negative index - reversed

.

.

.

[MDLLOOP trim seg's indices]        ;Negative index - reversed

]

;The trimming curve segments can hold a parameteric curve of the

;in first surface, a parametric curve in the second surface, and a

;a Euclidean representation, in this order. A 3 bits mask 'CurveMask'

;says what is available, as one bit per curve type.

;'#1stSrf' and '#2ndSrf' specify the two surfaces that share

;this boundary trimming curve, with 0 denoting no surface.

[MDLTSEG CurveMask #1stSrf #2ndSrf        ;CurveMask = 5

[CURVE ...

]

[CURVE ...

]

]

.

.

.

[MDLTSEG CurveMask #1stSrf #2ndSrf        ;CurveMask = 7

[CURVE ...

]

[CURVE ...

]

[CURVE ...

]

]

]

]

POINT_TYPE -> E1 | E2 | E3 | E4 | E5 | E6 | E7 | E8 | E9 |

P1 | P2 | P3 | P4 | P5 | P6 | P7 | P8 | P9

ATTRS -> [ATTRNAME ATTRVALUE]

| [ATTRNAME]

| [ATTRNAME ATTRVALUE] ATTRS

Some notes:

* This definition for the text file is designed to minimize the

reading time and space. All information can be read without backward

or forward referencing.

* An OBJECT must never hold different geometry types or other entities.

I.e. CURVEs, SURFACEs, and POLYGONs must all be in different OBJECTs.

* Attributes should be ignored if not needed. The attribute list may have

any length and is always terminated by a token that is NOT '['. This

simplifies and disambiguates the parsing.

* Comments may appear between '[OBJECT ...]' blocks, or

immediately after OBJECT OBJNAME, and only there.

A comment body can be anything not containing the '[' or the

']' tokens (signals start/end of block). Some of the comments in

the above definition are illegal and appear there only for the sake

of clarity.

* It is preferable that geometric attributes such as NORNALs be saved on

the geometric structure level (POLYGON, CURVE or vertices) while graphical

and other attribures such as COLORs will be saved on the OBJECT level.

* Objects may be contained in other objects to an arbitrary level.

Here is an example that exercises most of the data format:

This is a legal comment in a data file.

[OBJECT DEMO

[OBJECT REAL_NUM

And this is also a legal comment.

[NUMBER 4]

]

[OBJECT A_POINT

[POINT 1 2 3]

]

[OBJECT A_VECTOR

[VECTOR 1 2 3]

]

[OBJECT CTL_POINT

[CTLPT E3 1 2 3]

]

[OBJECT STR_OBJ

[STRING "string"]

]

[OBJECT UNIT_MAT

[MATRIX

1 0 0 0

0 1 0 0

0 0 1 0

0 0 0 1

]

]

[OBJECT [COLOR 4] POLY1OBJ

[POLYGON [PLANE 1 0 0 0.5] 4

[-0.5 0.5 0.5]

[-0.5 -0.5 0.5]

[-0.5 -0.5 -0.5]

[-0.5 0.5 -0.5]

]

[POLYGON [PLANE 0 -1 0 0.5] 4

[0.5 0.5 0.5]

[-0.5 0.5 0.5]

[-0.5 0.5 -0.5]

[0.5 0.5 -0.5]

]

]

[OBJECT [COLOR 63] ACURVE

[CURVE BSPLINE 16 4 E2

[KV 0 0 0 0 1 1 1 2 3 4 5 6 7 8 9 10 11 11 11 11]

[0.874 0]

[0.899333 0.0253333]

[0.924667 0.0506667]

[0.95 0.076]

[0.95 0.76]

[0.304 1.52]

[0.304 1.9]

[0.494 2.09]

[0.722 2.242]

[0.722 2.318]

[0.38 2.508]

[0.418 2.698]

[0.57 2.812]

[0.57 3.42]

[0.19 3.572]

[0 3.572]

]

]

[OBJECT [COLOR 2] SOMESRF

[SURFACE BEZIER 3 3 E3

[0 0 0]

[0.05 0.2 0.1]

[0.1 0.05 0.2]

[0.1 -0.2 0]

[0.15 0.05 0.1]

[0.2 -0.1 0.2]

[0.2 0 0]

[0.25 0.2 0.1]

[0.3 0.05 0.2]

]

]

]

BUGS and LIMITATIONS

--------------------

As with any program of more than one line, this is far from perfect.

Some limitations, as well as simplifications, are laid out below.

*  If the intersection curve of two objects falls exactly on polygon

boundaries, for all polygons, the system will scream that the two objects

do not intersect at all. Try to move one by EPSILON into the other.

I probably should fix this one - it is supposed to be relatively easy.

*  Avoid degenerate intersections that result in a point or a line.

They will probably cause wrong propagation of the inner and outer parts of

one object relative to another. Always extend your object beyond the

other object.

*  If two objects have no intersection in their boundary, IRIT assumes

they are disjoint: a union simply combines them, and the other Boolean

operators return a NULL object. One should find a FAST way (3D Jordan

theorem) to find the relation between the two (A in B, B in A, A

disjoint B) and according to that, make a decision.

*  Since the Boolean sum implementation constructs ruled surfaces with

uniform speed, it might return a somewhat incorrect answer, given

non-uniform input curves.

*  The parser is out of hand and difficult to maintain. There are several

memory leaks there that one should fix.

*  The X11 driver has no menu support (any easy way to have menus using

Xlib!?).

*  IBM R6000 fails to run the drivers in -s- mode.

*  Rayshade complains a lot about degenerate polygons on irit2ray output.

To alleviate the problem, change the 'equal' macro in common.h in libcommon

of rayshade from EPSILON (1e-5) to 1e-7 or even lower.

*  On the motif-based drivers (xmtdrvs etc.) clicking the mouse left and

right of the scale's button produces stepped transformations. This

step size is constant, and is not proportional to the distance between

the mouse's position and the position of the button. The reason for the

flaw is incorrect callback information returned from the scale in

repetitive mode.

*  Binary data files are not documented, nor will they be. They might

change in the future and are in fact machine dependend. Hence, one

platform might fail to read another's binary data file.